3GPP TSG-RAN WG4 Meeting #114bis									                    R4-2503613

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.4.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [114bis][203] NonCol_intraB_ENDC_NR_CA_Ph2

Document for:	Information

Introduction

This email thread discusses the RRM core requirements of WI on R19 Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s).

Online handling:

(Online) Issue 1-1: The structure of the RRM requirements for Type 4 UE non-collocated

(Online) Issue 1-2: Update on RRM requirements for FR1 non-collocated intra-band EN-DC/NR-CA for Type 4 UE in Rel-19

Topic #1:	RRM requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503506 | Nokia, Nokia Shanghai Bell | Draft CR |
| R4-2503507 | Nokia, Nokia Shanghai Bell | Proposal 1: The draftCR on RRM requirements, if endorsed, need to be updated based on RF/RAN2 conclusion on the signalling. |
| R4-2503860 | ZTE Corporation, Sanechips | Draft CR |
| R4-2503881 | Huawei, HiSilicon | Proposal: Companies can provide the draft CRs for the identified RRM requirements, and the Type 4 UE capability and the description of “R19 type 4 capable UE is in non-collocated/collocated condition” with regard to BS signalling can be updated, once the UE capability and BS signalling are concluded in RF session. |
| R4-2503882 | Huawei, HiSilicon | Draft CR |
| R4-2504199 | Ericsson | Observation 1: We have assumed same logic and structure as existing release-18 text, with the addition of signaling element [nonCollocatedTypeNR-CA-r19] in brackets. The final signaling for release-19 is not yet agreed and the CRs will have to be updated accordingly when we reach agreement. Proposal 1: Approve CRs in [2] and [3] according to work split. |
| R4-2504200 | Ericsson | Draft CR |
| R4-2504201 | Ericsson | Draft CR |
| R4-2504292 | Samsung | Draft CR |
| R4-2503584 | Apple | Draft CR |

Open issues summary

Sub-topic 1: RRM requirements impact

(Online) Issue 1-1: The structure of the RRM requirements for Type 4 UE non-collocated

- Proposals

  - Option 1 (Ericsson): Assumed same logic and structure as existing release-18 text, with the addition of signaling element [nonCollocatedTypeNR-CA-r19] in brackets. The final signaling for release-19 is not yet agreed and the CRs will have to be updated accordingly when we reach agreement.

- Recommended WF:

To draft the RRM requirements for Type 4 UE non-collocated, the same logic and structure as existing release-18 text are assumed.

(Online) Issue 1-2: Update on RRM requirements for FR1 non-collocated intra-band EN-DC/NR-CA for Type 4 UE in Rel-19

Background: In RRM session, in RAN4#112bis meeting, the impacted RRM core requirements were identified in [R4-2416850], and in RAN4#113 meeting, CR split among companies was approved in [R4-2420090]. In RAN4#114 meeting, RAN4 agreed to endorse the draft CRs in RAN4#114bis meeting.

In RF session, regarding UE capability and network signalling for Type 4 UE, the case when maxMIMO-layers=4 is still under discussion, and other cases have reached conclusion.

- Proposals

- Option 1: Endorse the draft CRs in this meeting and update the UE capability and BS signaling based on RF/RAN2 conclusion later

  - Option 1a (Ericsson): Approves CRs according to work split.

  - Option 1b (Nokia): The draftCR on RRM requirements, if endorsed, need to be updated based on RF/RAN2 conclusion on the signalling.

  - Option 1c (Huawei): Companies can provide the draft CRs for the identified RRM requirements, and the Type 4 UE capability and the description of “R19 type 4 capable UE is in non-collocated/collocated condition” with regard to BS signalling can be updated, once the UE capability and BS signalling are concluded in RF session.

- Recommended WF:

- Endorse the draft CRs in this meeting and further update the UE capability and BS signalling based on RF/RAN2 conclusion.