3GPP TSG-RAN WG4 Meeting # 114bis											R4-2503621

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.22.4

Source:	Moderator (Samsung)

Title:	Topic summary for [114bis][221] NR_MIMO_Ph5_Part1

Document for:	Information

Introduction

This topic summary covers the contributions submitted under the following AI for first part of Rel-19 RRM for NR MIMO Phase 5:

- 7.22	NR MIMO Phase 5

- 7.22.3 RRM core requirements

  - 7.22.3.1 Enhancement for UE-initiated/event-driven beam management

Recommended issues to online discussion:

Issue 1-1

Issue 1-2

Issue 1-5

Issue 1-6

Issue 1-10

Issue 1-3

Issue 1-7&Issue 1-8

Issue 1-4

Topic #1: RRM requirements for UE-initiated/event-driven beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503127 | Qualcomm Technologies Ireland | Proposal 1: For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam. Observation 1: It is unclear whether RAN1 is still discussing the time restriction for channel measurement M {1, 3}. No agreement has been made in RAN1 #120. Proposal 2: RAN4 should define delay requirements for the measurement period of UE-initiated/event-driven beam management taking both possible values of M into account. Observation 2: In RAN1 #119 it was agreed that at least the case is supported in Rel-19 that the evaluation periodicity for event-2 is identical for the current and the new beam and that the evaluation periodicity equals the periodicity of the RS of the current and the new beam. Observation 3: The case of different periodicities of the current beam RS and the new beam RS(s) is still under discussion in RAN1 but has not been agreed yet. Proposal 3: RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements. Observation 4: Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring. Observation 5: In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted. Observation 6: For event-triggered periodic reporting of L3 measurements delay uncertainty occuring when inserting the measurement report to the TTI of the uplink DCCH and the case of no UL resources are excluded from the reporting delay. Proposal 4: Delay uncertainty till the first PUCCH can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting. Proposal 5: Option 3 in Issue 1-4 should be added to the core requirements that the UE shall send the beam report in the next available PUCCH resource to ensure that the reporting delay is kept as small as possible. Observation 7: How to define the details of the window-based approach was still under discussion in RAN1 #120, but no conclusion could be reached in RAN1. It seems in RAN2 the topic how to define the window has also not been discussed. Proposal 6: All further discussion on the details of the measurement procedure of the window-based approach in RAN4 should be postponed till RAN1 and/or RAN2 has concluded. Proposal 7: RAN4 should discuss whether a LS should be sent to RAN1 and RAN2 summarizing the RAN4 agreements for the measurement periods. Observation 8: In case that no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested. Proposal 8: RAN4 should not define requirements for the window-based approach whether the event has occurred M times. Observation 9: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-based L1-RSRP. Proposal 9: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. Proposal 10: The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management. |
| R4-2503312 | Xiaomi | Observation 1: If UE doesn’t consider the impact of new L1 result after L1 reporting is triggered, the definition of Tfirst UL channel will be the time interval from L1-RSRP RS which trigger L1 reporting to first PUCCH. Observation 2: If UE consider the impact of new L1 result after L1 report is triggered, starting point of Tfirst UL channel will be L1-RSRP RS which is nearest to first PUCCH. The time interval from L1-RSRP RS which triggers L1 reporting to L1-RSRP RS which is nearest to first PUCCH shall also be considered. Observation 3: If UE consider the impact of new L1 results, there are new L1 results, e.g. RS4, RS4, between PUCCH and PUSCH. There are also several possibilities and the situation is too complex. Proposal 1: For L1 reporting delay, didn’t consider the impact of L1 result after L1 reporting is triggered. Proposal 2: Fine with both Option 1 and Option 2. For Option 1, the definition of Tfirst UL channel will be the time interval from L1-RSRP RS which trigger L1 reporting to first PUCCH. Observation 4: RAN1 is also discussing the evaluation time and will have impact on RAN4 delay requirement. Proposal 3: For event-2, apply the same measurement periodicities of current beam and new beam(s) to define the requirements. Proposal 4: Define a unified requirement with M=1 to cover all cases. Proposal 5: Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress. |
| R4-2503546 | Apple | Reporting Delay Requirements - Applicability for event triggered L1-RSRP reporting should be discussed. - Options for applicability from RAN4#114 seem restrictive and hard to guarantee in practice - Measurement reporting delay requirement applicability - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period - RAN4 only discuss/define requirements for same periodicity of new beam and current beam, until further agreement on support of different periodicity in RAN1. - Components of measurement reporting delay for UEIBM are measurement period, time to PUCCH. - Measurement period for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } In FR1/ FR2 with resources not adjacent or overlapping: max{ Mc*N*Pc*TcurrBeam , MN*N*PN*TnewBeam } In FR2 with resources adjacent or overlapping: max{ Mc*N*2*Pc*TcurrBeam , MN*N*2*PN*TnewBeam } - Time to PUCCH can be defined as time to first PUCCH resource or periodicity of PUCCH resource for UEIBM - Tfirst-PUCCH or TPUCCH. - Measurement reporting delay for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } + Tfirst-PUCCH or TPUCCH. - It is not feasible to define core requirement for reporting delay with time window based triggering - Measurement reporting delay requirement for UE initiated beam report is defined for M=1. - To extend the requirements for case when M>1 is configured, add note longer delay is expected if M>1 is configured. - The NW is expected to configure resources for measurement and PUCCH accordingly to ensure timely indication/ request when event is triggered at the UE. - The UE is expected to make measurements until event trigger condition is met at the UE. CSI-RS Based measurements - CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell. - From UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. - RAN4 defines UEIBR reporting delay for CSI-RS based measurement without specifying intra-cell or inter-cell. TCI State Switching Requirements - UE initiated beam reporting doesn’t impact any of the delay components for TCI state switching delay. - There is no need to specify new unified TCI state switching delay requirements. Capabilities - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added. - Discuss later in the WI on compatibility of capabilities with LTM. |
| R4-2503598 | LG Electronics | Proposal 1: Define measurement period as follows: - M=1 - TSSB = Max{TSSB_current, TSSB_new} Proposal 2: RAN4 to specify reporting delay requirement as follows: - Reporting delay = Measurement time + UL_uncertainty - UE should transmit First PUCCH within the reporting delay - UL_uncertainty is the first available UL channel after the measurement period - Proposal 3: Specify only no-time window based requirment Proposal 4: Whether to update L1 report which is measured in L1 report needs to wait further RAN1 progress Proposal 5: For unified TCI switching delay requirements, RAN4 needs to wait further RAN1 progress |
| R4-2503665 | Samsung | Proposal 1-1: The evaluation periodicity, for event 2, the periodicity of current beam and new beam are the same. the evaluation periodicity can reuse the legacy L1-RSRP measurement period. Take SSB as example, the Tssb is the periodicity of the SSB-Index in indicated TCI state or QCLed with the indicated TCI state. Proposal 1-2: For M in L1_measurement periods, reuse the legacy value: SSB based: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise CSI-RS based: For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise; For aperiodic CSI-RS resources M=1 Proposal 1-3: For event-2, P in L1_measurement periods, reuse the same definition in existing requirements. Proposal 1: For event-2, for the basic L1_evaluation_period (Mcounter =1), reuse the existing L1 measurement periods. Only Update T_RS definition and remove TReport. Proposal 2: The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL . - Tfirst UL channel as from time point which the RS of L1-RSRP trigger L1 reporting to first PUCCH to the time point of the latest PUCCH occasion Proposal 3: RAN4 to specify requirements to cover both cases of no time window based and with time window based with configurable M. Proposal 4: After UE send first PUCCH, between first PUCCH and PUSCH, there may new L1-RSRP evaluation. No need to update L1-RSRP results for report. Proposal 5: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements. Proposal 6: For known conditions for TCI states for TCI states switching based on UEIBM, it can use as legacy. Proposal 7: No difference by using UE triggered L1 report or NW based L1 report. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching if known case. No unknown case is applied if the TCI states switching is based on UEIBM. Proposal 8: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. Proposal 9: For event trigger L1 reporting, do not define such capability in parallel per category like L3. |
| R4-2503755 | CMCC | Proposal 1: it is proposed that the event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on. Proposal 2: it is proposed to specify requirements to cover the case of no time window and the case with time window with configurable M. Proposal 3: it is proposed to define CSI-RS based L1-RSRP measurement requirements for both intra-cell and inter-cell. Proposal 4: it is proposed to define capabilities for support of UE-initiated/event-driven beam reporting. Proposal 5: it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO. |
| R4-2503869 | Huawei, HiSilicon | Observation 1: Based on latest RAN1 progress, for cross-CC case, the new beam RS and current beam RS are in the same CC. Proposal 1: RAN4 to prioritize the requirements for UEIBM for same periodicity between new beam(s) and current beam. Observation 2: When timeRestrictionForChannelMeasurement is configured, event evaluation should be based on the most recent result from both new beam and current beam. Otherwise, the event could be wrongly triggered (i.e. the results for comparation experience completely different channel). Proposal 1: When timeRestrictionForChannelMeasurement is configured, M =1, and the event evaluation should be based on the most recent result from both new beam and current beam Proposal 2: When timeRestrictionForChannelMeasurement is not configured, whether filtering is assumed for both new beam and current beam shall be discussed. Observation 3: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading. Proposal 3: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering. Proposal 4: For time window-based scenario with same periodicity between current and new beam, one event instance shall be evaluate based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance. Observation 4: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell. Proposal 5: CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. Observation 5: Do not combine UE capabilities across different WID, and postpone the discussion in RAN4 with more RAN1/2 progress. |
| R4-2503934 | Ericsson | - When the RS periodicity for current beam and new beam is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new. - RAN4 to wait for RAN1 progress on consideration of counter or timer in the UE initiated beam reporting in RAN4 requirements. - Event evaluation period for a SSB or CSI-RS can be decided in RAN1. - RAN4 to agree TTime to First UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time. Where, Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1. - Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS. - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2. - Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching. - UE Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added. - RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility. |
| R4-2504073 | Nokia | Observation 1: M value that is being discussed in RAN1 is the number of Event instances that should occur within a time window for the UE to determine that an UEI Event-2 has occurred. Observation 2: M that RAN1 is discussing has no relation to timeRestrictionForChannelMeasurement (M) used in the computation of RAN4 L1 RSRP measurement period, which is not being discussed in RAN1. Proposal 1: If timeRestrictionForChannelMeasurement configuration is supported when UE is configured with UEIBM event-triggered reporting, RAN4 to reuse M=1 or M=3 for computation of L1 RSRP event triggered measurement period depending on whether timeRestrictionForChannelMeasurement is configured, similar to legacy. Proposal 2: Only define requirements considering same periodicity of current beam and new beam unless there are further RAN1 agreements on the support of different periodicities for current beam and new beam. Proposal 3: The UE shall be able to send the PUCCH latest at the first PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. TL1-RSRP_event triggered_measurement_Period). Observation 3: RAN1 is discussing the approach (fixed window or sliding window) to be followed for the window-based UE event triggered report. Proposal 4: RAN4 to defer discussion on defining requirements for window based approach till RAN1 concludes the details of this approach. Observation 4: RAN1 is discussing various alternatives to reduce the beam application latency, some of which may have an impact on the TCI state switch delay. Proposal 5: RAN4 to wait for RAN1 progress on discussion of beam application latency reduction after a UEI beam report is sent before defining the related TCI state switching delay requirements. Proposal 6: UE behaviour in the interim between sending the 1st PUCCH and waiting for UL resources to send the UEI report is up to RAN1. Observation 5: No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management. Proposal 7: RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not defined a clear use case. Observation 6: RAN1 is discussing and defining UE capability for UEIBM. If any RAN4 specific UE capability is required, then this can be discussed once discussions in RAN1 are completed. Proposal 8: Do not combine capabilities across different L1 measurements. |
| R4-2504248 | ZTE Corporation, Sanechips | Observation 1: The current beam RS and the new beam RS(s) must be in the same CC, i.e. only single-CC beam reporting is allowed by RAN1. For the single-CC beam reporting, only same periodicity case is approved by RAN1. Proposal 1: Based on RAN1 progress, RAN4 only need to define measurement period requirements for the same periodicity case between current beam and candidate new beam(s). The agreements in last meeting for same periodicity case is enough, no need further discussion. Proposal 2: For Event-1, the measurement period requirements is same as Event-2 as only same periodicity case approved until now by RAN1. Proposal 3: For Event-7, the measurement period requirements is same as Event-2 as only same periodicity case approved until now by RAN1. Proposal 4: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. For the optional Case 2, the sample number M and timer X should be considered. Proposal 5: Reuse the style of legacy L3 measurement reporting delay to specify the L1 measurement reporting delay, i.e. the event triggered L1 measurement reporting delay shall be less than TL1_measure_period + Tfirst UL channel. Proposal 6: Based on RAN1 progress, for the TCI state switching triggered by UEIBR(may or may not receive TCI state switching command), UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored. Proposal 7: For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. Proposal 8: For CSI-RS based UEIBM, RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. |
| R4-2504602 | MediaTek inc. | Proposal 1: Legacy number of samples (M) for L1-RSRP measurement can be reused. This number can be revisited if RAN1 make a different agreement otherwise. Proposal 2: RAN4 not to discuss the scenario of different periodicities for the current and new beam since RAN1 has no further agreement whether to support it or not. - If RAN1 agree to support the different periodicities of current and new beam, consider the maximum periodicity: - Max {TSSB_current, TSSB_new} for TL1-RSRP_event triggered_measurement_Period_SSB - Max {TCSI-RS_current, TCSI-RS_new} for TL1-RSRP_event triggered_measurement_Period_CSI-RS Proposal 3: RAN4 to consider delay requirement of event 2 for both “no time window” and “with time window with configurable M”: Proposal 4: For the case of no time window: - Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period + Tfirst UL channel where, - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. Proposal 5: RAN4 to discuss how to define the delay requirement when only M out of N instances meet event 2 condition within the configured time window. Proposal 6: For the case of time window with configurable M (event instances threshold): - Option 1: Measurement reporting delay can re-use the same principle as TTT for the configurable time window. - Option 2: Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where, - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - C: event instances threshold is configured by NW. - Ttime_window: time window is configured by NW. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. Proposal 7: Reuse legacy measurement period for event 2 on UE-initiated/event-driven beam management. - SSB based L1-RSRP measurement for both intra-cell and inter-cell. - CSI-RS based L1-RSRP measurement for intra-cell. - Descope CSI-RS based L1-RSRP measurement for Inter-cell case. Proposal 8: RAN4 to consider skipping the fine timing sync (additional SSB reception) for known TCI state if RAN1 has further agreement on the reduction of additional SSB reception for TCI state of UE-initiated/event-driven beam. Otherwise, reuse the existing unified TCI state delay requirement. Proposal 9: Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4). Proposal 10: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency. |

Open issues summary

Sub-topic 1-1:

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

[Agreement in RAN4#114]

Agreement:

The agreements under this issue are for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1.

TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS

  - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4

- Proposals

  - Proposal 1: (Qualcomm)

    - For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

    - RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements.

  - Proposal 2: (Xiaomi, ZTE)

    - For event-2, apply the same measurement periodicities of current beam and new beam(s) to define the requirements.

  - Proposal 3: (Apple)

    - Measurement period for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }
In FR1/ FR2 with resources not adjacent or overlapping: max{ Mc*N*Pc*TcurrBeam , MN*N*PN*TnewBeam }
In FR2 with resources adjacent or overlapping: max{ Mc*N*2*Pc*TcurrBeam , MN*N*2*PN*TnewBeam }

  - Proposal 4: (LGE)

    - M=1

    - TSSB = Max{TSSB_current, TSSB_new}

  - Proposal 5: (Samsung)

    - The evaluation periodicity, for event 2, the periodicity of current beam and new beam are the same. the evaluation periodicity can reuse the legacy L1-RSRP measurement period.Take SSB as example, the Tssb is the periodicity of the SSB-Index in indicated TCI state or QCLed with the indicated TCI state.

    - For M in L1_measurement periods, reuse the legacy value: SSB based: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. CSI-RS based: For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise; For aperiodic CSI-RS resources M=1

    - For event-2, P in L1_measurement periods, reuse the same definition in existing requirements.

  - Proposal 6: (Huawei)

    - prioritize the requirements for UEIBM for same periodicity between new beam(s) and current beam.

    - When timeRestrictionForChannelMeasurement is configured, M =1, and the event evaluation should be based on the most recent result from both new beam and current beam

    - When timeRestrictionForChannelMeasurement is not configured, whether filtering is assumed for both new beam and current beam shall be discussed.

  - Proposal 7: (Nokia)

    - If timeRestrictionForChannelMeasurement configuration is supported when UE is configured with UEIBM event-triggered reporting, RAN4 to reuse M=1 or M=3 for computation of L1 RSRP event triggered measurement period depending on whether timeRestrictionForChannelMeasurement is configured, similar to legacy.

    - Only define requirements considering same periodicity of current beam and new beam unless there are further RAN1 agreements on the support of different periodicities for current beam and new beam

- Recommended WF

  - For M L1_measurement periods:

    - Option 1: (Samsung, Nokia, MediaTek, Apple)

      - reuse the legacy definition and requirements

    - Option 1a (Apple): Consider Mc and Mn for current and new beam respectively.

    - Option 2: (LGE)

      - Only M = 1

    - Option 3: (Huawei)

      - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam.

      - If timeRestrictionForChannelMeasurement is not configured, to be discuss

  - For P (sharing factor) L1_measurement periods:

    - Option 1: (Apple)

      - Separate Pc and Pn for current beam and new beam

    - Option 2: (Samsung)

      - reuse the legacy definition and requirements

  - For T_RS (TSSB or TCSI-RS) in L1_measurement periods

    - T_RS (TSSB or TCSI-RS):

      - Option1: (Qualcomm, Xiaomi, Samsung, Huawei, Nokia, ZTE, MediaTek)

        - the value of current beam and new beams are the same.

      - Option 2: (LGE, MediaTek (if RAN1 further agree to support different periodicities))

        - Max{TSSB_current, TSSB_new}

      - Option 3: (Ericsson)

        - When the RS periodicity for current beam and new beam is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new.

Issue 1-2: Components of Event triggered measurement reporting delay?

Issue 1-2: Components of Event triggered measurement reporting delay?

- Proposals

  - Proposal 1: (Qualcomm)

    - Delay uncertainty till the first PUCCH can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting.

    - The UE shall send the beam report in the next available PUCCH resource to ensure that the reporting delay is kept as small as possible.

  - Proposal 2: (Xiaomi)

    - For L1 reporting delay, didn’t consider the impact of L1 result after L1 reporting is triggered.

    - Fine with both “The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel” and reuse L3 definition “The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering”. For Option 1, the definition of Tfirst UL channel will be the time interval from L1-RSRP RS which trigger L1 reporting to first PUCCH.

  - Proposal 3: (Apple, LGE, Samsung, Ericsson, ZTE, MediaTek)

    - Measurement period + time to PUCCH

    - Proposal 3a: (Apple)

      - Time to PUCCH can be defined as time to first PUCCH resource or periodicity of PUCCH resource for UEIBM - Tfirst-PUCCH or TPUCCH

      - max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } + Tfirst-PUCCH or TPUCCH

    - Proposal 3b: (LGE, MediaTek)

      - UL_uncertainty is the first available UL channel after the measurement period

    - Proposal 3c: (Samsung)

      - Tfirst UL channel as from time point which the RS of L1-RSRP trigger L1 reporting to first PUCCH to the time point of the latest PUCCH occasion

    - Proposal 3d: (Ericsson)

      - TTime to First UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time. Where, Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1

  - Proposal 4: (CMCC)

    - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on.

  - Proposal 5: (Nokia)

    - The UE shall be able to send the PUCCH latest at the first PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. TL1-RSRP_event triggered_measurement_Period).

- Recommended WF

  - To decided inside there two options:

    - Option 1: (Apple, Xiaomi, LGE, Samsung, Ericsson, ZTE, MediaTek)

      - The event triggered measurement reporting delay is TL1-RSRP_measure + Tfirst UL channel

    - Option 2: (Xiaomi, CMCC)

      - The event triggered measurement reporting delay shall be less than TL1-RSRP_measure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering.

    - Option 3: (Qualcomm)

      - Tfirst UL channel should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting

  - To discuss whether to add the definition in core requirements:

    - The UE shall be able to send the PUCCH latest at the first PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. TL1-RSRP_event triggered_measurement_Period).

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M?

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with [eventInstanceCount-r19]?

Moderator: eventInstanceCount-r19 is used to replaced previous configurable M to avoid the collision of M definition in legacy L1-RSRP tables

- Proposals

  - Proposal 1: (Qualcomm)

    - All further discussion on the details of the measurement procedure of the window-based approach in RAN4 should be postponed till RAN1 and/or RAN2 has concluded. RAN4 should discuss whether a LS should be sent to RAN1 and RAN2 summarizing the RAN4 agreements for the measurement periods

    - RAN4 should not define requirements for the window-based approach whether the event has occurred M times.

  - Proposal 2: (Xiaomi)

    - Define a unified requirement with M=1 to cover all cases.

  - Proposal 3: (Apple)

    - Measurement reporting delay requirement for UE initiated beam report is defined for M=1.

    - To extend the requirements for case when M>1 is configured, add note longer delay is expected if M>1 is configured.

  - Proposal 4: (LGE)

    - Only define requirements for M=1.

  - Proposal 5: (Samsung, CMCC, Huawei, ZTE, MediaTek)

    - Specify requirements to cover both cases of no time window based and with time window based with configurable M

    - Option 5a: one event instance shall be evaluate based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance. (Huawei)

  - Proposal 6: (Nokia)

    - Defer discussion till RAN1 conclusion

  - Proposal 7: (MediaTek)

    - For the case of time window with configurable M (event instances threshold):

      - Alt 1: Measurement reporting delay can re-use the same principle as TTT for the configurable time window.

      - Alt 2: Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

        - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

        - C: event instances threshold is configured by NW.

        - Ttime_window: time window is configured by NW.

        - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

  - Proposal 8: (Ericsson)

    - RAN4 to wait for RAN1 progress on consideration of counter or timer in the UE initiated beam reporting in RAN4 requirements.

- Recommended WF

  - To decided inside there 3 options:

    - Option 1: (Qualcomm, Xiaomi, Apple, LGE)

      - Do not define requirements for eventInstanceCount-r19 > 1

      - Option 1a: (Apple): To extend the requirements for case when M>1 is configured, add note longer delay is expected if M>1 is configured.

    - Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek)

      - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

    - Option 3: (Nokia, Ericsson)

      - Wait for further RAN1 conclusion

  - For how to specify the requirements in option 2 above, to check, or companies in Option2 can provide further details in the meeting:

      - Alt 2: Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

        - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

        - C: event instances threshold is configured by NW.

        - Ttime_window: time window is configured by NW.

        - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Issue 1-4: Whether/how to specify the UE behaviour if new L1-RSRP during the UL resource waiting time?

Issue 1-4: Whether/how to specify the UE behaviour if new L1-RSRP during the UL resource waiting time?

- Proposals

  - Proposal 1: (Apple)

    - The UE is expected to make measurements until event trigger condition is met at the UE.

  - Proposal 2: (LGE)

    - Wait for further RAN1 progress

  - Proposal 3: (Samsung)

    - No need to update L1-RSRP results for report

  - Proposal 4: (Nokia)

    - Up to RAN1

- Recommended WF

  - TBA

Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM

Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM

- Proposals

  - Proposal 1: (Qualcomm, Nokia, MediaTek (descoped))

    - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

  - Proposal 2: (Apple)

    - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

    - RAN4 defines UEIBR reporting delay for CSI-RS based measurement without specifying intra-cell or inter-cell.

  - Proposal 3: (Samsung)

    - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

  - Proposal 4: (CMCC, Huawei, Ericsson)

    - Define CSI-RS based L1-RSRP measurement requirements for both intra-cell and inter-cell.

  - Proposal 5: (Ericsson)

    - Use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

  - Proposal 6: (ZTE)

    - needs to identify whether additional clarifications on the following points are necessary:

      - -  Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

      - -  The prerequisite on L3 measurement on NSC.

    - use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Recommended WF

  - To decide in these three options:

    - Option 1: Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management

    - Option 2: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

    - Option 3:

      - To define different condition and delay for inter-cell CSI-RS measurement including:

        - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

        - The prerequisite on L3 measurement on NSC.

        - Similar sharing factor as that in SSB for the sharing of intra-cell and inter-cell

Moderator’s observation: although some companies mentioned CSI-RS L1 measurement to support for intra-cell and inter-cell (Apple, Huawei, CMCC, Ericsson, ZTE, Samsung), I don’t think companies are in the same page. ZTE and Ericsson mentioned to add new sharing factor [and known condition definition] clearly. From Apple, Huawei, [CMCC], my understanding is the same requirements are transparent and same for both intra-cell and inter-cell, no need to add new sharing factor [and known condition definition]. Please feel free to modify the recommended WF.

Issue 1-6: Whether to specify new unified TCI switching delay requirements?

Issue 1-6: Whether to specify new unified TCI switching delay requirements?

- Proposals

  - Proposal 1: (Qualcomm, Apple, Samsung (for known case), Ericsson)

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

  - Proposal 2: (Xiaomi, Nokia, LGE)

    - Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

  - Proposal 3: (ZTE)

    - Based on RAN1 progress, for the TCI state switching triggered by UEIBR(may or may not receive TCI state switching command), UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

  - Proposal 4: (MediaTek)

    - RAN4 to consider skipping the fine timing sync (additional SSB reception) for known TCI state if RAN1 has further agreement on the reduction of additional SSB reception for TCI state of UE-initiated/event-driven beam. Otherwise, reuse the existing unified TCI state delay requirement.

- Recommended WF

  - Check whether to agree:

    - For MAC-CE based DL/UL TCI state switch delay

      - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson)

        - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

      - Option 2: (ZTE)

        - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc

      - Option 3: (MediaTek)

        - FFS on whether to remove Tfirst-SSB+TSSB-proc

      - Whether cover cases: target TCI state is known and unknown

    - For DCI based DL/UL TCI state switch delay

      - Option 1:

        - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

      - Option 2:

        - Whether to further reduce the delay, wait for further RAN1 progress

Moderator’s suggestion: For MAC-CE based, if target TCI state is unknown, the delay is added by TL1-RSRP, the TL1-RSRP is defined in legacy NW configured L1-RSRP reporting. Please the proponents of the case if target TCI state is unknow, what is the definition of TL1-RSRP in the delay.

Issue 1-7: Other conditions for L1 measurement and TCI state switching?

Issue 1-7: Other conditions for L1 measurement and TCI state switching?

- Proposals

  - Option 1: (Samsung)

    - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Recommended WF

  - Discuss Issue 1-7 and 1-8 together.

Issue 1-8: Assumption or applicability of event triggered L1 measurement:

Issue 1-8: Assumption or applicability of event triggered L1 measurement:

- Proposals

  - Proposal 1: (Apple)

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

- Recommended WF

  - Discuss Issue 1-7 and 1-8 together.

Issue 1-9: Requirements for event-1 and event-7?

- Proposals

  - Proposal 1: (ZTE)

    - For Event-1, the measurement period requirements is same as Event-2 as only same periodicity case approved until now by RAN1.

    - For Event-7, the measurement period requirements is same as Event-2 as only same periodicity case approved until now by RAN1.

- Recommended WF

  - Option 1: When requirements for Event-2 are agreed, agree to reuse all requirements for Event-1 and Event-7.

  - Option 2: others are not preclude

Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria?

Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria?

- Proposals

  - Proposal 1: (Apple, Ericsson, Nokia)

    - Defined by RAN1. To be discussed later if any RAN4 specific UE capabilities need to be added.

  - Proposal 2: (Samsung)

    - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

  - Proposal 3: (CMCC, MediaTek)

    - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3

Issue 1-11: Compatibility of capabilities with LTM

Issue 1-11: Compatibility of capabilities with LTM

- Proposals

  - Proposal 1: (Apple, Ericsson)

    - Discuss it later.

  - Proposal 2: (CMCC, Huawei, Nokia)

    - Not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO

  - Proposal 3: (MediaTek)

    - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

- Recommended WF

  - TBA

Sub-topic 1-2: Draft CRs

| R4-2504073 | Nokia | DraftCR for UEIBM reporting requirements |
| --- | --- | --- |

Topic #2: Spec structure of Rel-19 MIMO Phase 5

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503665 | Samsung | Proposal 10: Agree on the spec structure of core requirements for Rel-19 MIMO phase 5 as below table: Objective; Clauses; Description. UEIBM; 9.x1 event trigger Reporting based on L1-RSRP measurements 9.x1.1 Introduction 9.x1.2 Requirements applicability 9.x1.3 Measurement Reporting Requirements 9.x1.4 L1-RSRP measurement requirements 9.x1.4.1 SSB based L1-RSRP Reporting 9.x1.4.2 CSI-RS based L1-RSRP Reporting 9.x1.5 Measurement restriction for CSI-RS and SSB for L1-RSRP measurement 9.x1.5.1 Measurement restriction for SSB based L1-RSRP 9.x1.5.2 Measurement restriction for CSI-RS based L1-RSRP 9.x1.6 Scheduling availability of UE during L1-RSRP measurement; New clauses to specify the event trigger reporting based on L1-RSRP measurement. 9.x2 event trigger Reporting based on L1-RSRP measurements for a cell with different PCI from serving cell 9.x2.1 Introduction 9.x2.2 Requirements applicability 9.x2.3 Measurement Reporting Requirements 9.x2.4 L1-RSRP measurement requirements 9.x2.4.1 SSB based L1-RSRP Reporting 9.x2.5 Measurement restriction for SSB for L1-RSRP measurement 9.x2.5.1 Measurement restriction for SSB based L1-RSRP 9.x2.6 Scheduling availability of UE during L1-RSRP measurement; New clauses to specify the event trigger reporting based on L1-RSRP measurement for a cell with different PCI from serving cell. 8.15 Active downlink TCI state switching delay for unified TCI; Depends on the open issue: whether new TCI state switching or if existing requirements can be applied, FFS on any clarification. 8.16 Active uplink TCI state switching delay for unified TCI; Depends on the open issue: whether new TCI state switching or if existing requirements can be applied, FFS on any clarification. CJTC reporting; [9.x Inter-TRP time misalignment measurement and reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements 9.x.4 Inter-TRP time misalignment measurement requirements 9.x.4.1 CSI-RS based Inter-TRP time misalignment Reporting 9.x.5 Measurement restriction for CSI-RS Inter-TRP time misalignment 9.x.6 Scheduling availability of UE during Inter-TRP time misalignment]; New clauses Whether to add core requirements for CJTC reporting for ‘CJTC-Dd’, depends on the conclusion of open issues.. [9.x Inter-TRP frequency offset measurement and reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements 9.x.4 Inter-TRP frequency offset measurement requirements 9.x.4.1 CSI-RS based Inter-TRP frequency offset Reporting 9.x.5 Measurement restriction for CSI-RS Inter-TRP frequency offset 9.x.6 Scheduling availability of UE during Inter-TRP frequency offset measurement]; New clauses Whether to add core requirements for CJTC reporting for ‘CJTC-F’, depends on the conclusion of open issues.. [9.x Inter-TRP phase offset measurement and reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements 9.x.4 Inter-TRP phase offset measurement requirements 9.x.4.1 CSI-RS based Inter-TRP phase offset Reporting 9.x.5 Measurement restriction for CSI-RS Inter-TRP phase offset 9.x.6 Scheduling availability of UE during Inter-TRP phase offset measurement]; New clauses Whether to add core requirements for CJTC reporting for ‘CJTC-P’, depends on the conclusion of open issues.. Asymmetric DL sTRP/UL mTRP deployment; 7 Timing 7.1 UE transmit timing 7.1.1 Introduction 7.1.2 Requirements 7.1.2.1 Gradual timing adjustment; Add the timing requirements for two TA extension.. 7.5 Maximum Transmission Timing Difference 7.5.1 Introduction 7.5.7 Minimum requirements for multi-TRP 7.6 Maximum Receive Timing Difference 7.6.1 Introduction 7.6.8 Minimum requirements for Multi-TRPs; For two TA extension, update the condition and capability, some parts depend on further RAN1 progress. 8.16 Active uplink TCI state switching delay for unified TCI 8.16.1 Introduction 8.21 Active downlink TCI state switching delay for unified TCI for single-DCI mTRP 8.21.1 Introduction; Whether to add new TCI state switching delay for unified TCI and single-DCI mTRP, depends on the decision of open issue. Even no new TCI state switching delay requirements, FFS on whether to add the applicability rule in existing requirements.. |
| R4-2504602 | MediaTek inc. | Observation 4: RRM Spec impact for CR drafting for NR_MIMO_Ph5 (Part2): Scenario; Impacted clauses. - 2TA enhancement for asymmetric DL sTRP/ UL mTRP; 7.1 UE transmit timing 7.1.1 Introduction 7.1.2 Requirements 7.1.2.1 Gradual timing adjustment. - Active uplink TCI state switching delay for unified TCI for single-DCI mTRP; 8.16. - CJT calibration using delay/frequency offset measurements and reporting; New sections: 9.X CJT calibration using delay/freq offset measurement for reporting. 9.X.1 Introduction 9.X.2 Requirements applicability 9.X.3 Measurement Reporting Requirements 9.X.4 Delay offset measurement requirements 9.X.5 Frequency offset measurement requirements 9.X.6 Measurement restriction for CSI-RS 9.X.7 Scheduling availability of UE. |

Issue 2-1: Spec structure of Rel-19 MIMO Phase 5

Issue 2-1: Spec structure of Rel-19 MIMO Phase 5

- Proposals

  - Proposal 1: (Samsung)

    - Agree on the spec structure of core requirements for Rel-19 MIMO phase 5 as below table in R4-2503665

  - Proposal 2: (MediaTek)

    - RRM Spec impact for CR drafting for NR_MIMO_Ph5 (Part2) in R4-2504602

- Recommended WF

  - Use Table in R4-2503665 and R4-2504602 as baseline to discuss the spec impact for Rel-19 MIMO Phase 5. The spec impact and work split for CR work is expected to be discussed and agreed during the meeting.