3GPP TSG-RAN WG4 Meeting # 114bis										                 R4-2503630

Wuhan, China, 07 – 11 April 2025

Agenda item:			7.28.3

Source:	Moderator (Nokia)

Title:	Topic summary for [114bis][220] NR_XR_Ph3

Document for:	Information

# Introduction

This summary includes the proposals and issues for discussion related to Rel-19 enhancements for extended reality for NR Phase 3, under thread [114bis][220] NR_XR_Ph3.

This summary is based on the contributions submitted to the agenda items:

- 7.28  XR for NR Phase 3

  - 7.28.1  General aspects

  - 7.28.2  RRM core requirements

  - 7.28.3  Moderator summary and conclusions

The issues for discussion are organized in topics which are as follows:

- Topic 1 Scenarios for measurement skipping

- Topic 2 Measurements with gaps

- Topic 3 XR skipping requirements impact

- Topic 4 Time offset for measurement skipping processing

- Topic 5 Need/feasibility of UE assistance information

- Topic 6 Additional signalling aspects

- Topic 7 CR work and clauses with spec impact

The following issues are proposed to be prioritized for discussion

- Issue 1-1: Deployment scenarios

- Issue 1-2: UE power class

- Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Issue 1-5: L1/L2 triggered mobility measurements and procedures

- Issue 3-1: Principles for measurement delay calculation

- Issue 3-2: Formulas for measurement delay extension without DRX

- Issue 3-4: Conditions for measurement skipping

- Issue 3-5: Measurement gap configurations relevant to skipping

- Issue 2-1: General approach for defining requirements for measurements with gaps

- Issue 2-2: Types of measurement gaps

- Issue 5-1: General on UAI

- Issue 6-1: Semi-static schemes

- Issue 4-1: Time offset requirement for measurement gap skipping

- Issue 4-2: Time offset requirement for cancelling skip indication

- Issue 6-2: Skipping indication on frequency domain

Issues for email discussion

- Issue 7-1: Update work split

- Issue 7-2: Comments on CRs

The latest work item description can be found at:

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm

Previous way forwards for information

- R4-2414027, Workplan for Rel-19 XR phase 3, Nokia

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia

- R4-2416864, WF on RRM requirements for XR_Ph3 , Nokia

- R4-2420103, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements, To: RAN1, cc: RAN2, Source: Qualcomm

- R4-2502700, WF on RRM requirements for NR_XR_Ph3, Nokia

# Agreed workplan (from R4-2414027 for information)

| RAN4 meeting | Tasks |
| --- | --- |
| RAN4 #112 - August 2024 | - Approval of workplan - Initial discussion on scenarios for measurement skipping based on RAN1 conclusions. |
| RAN4 #112bis - October2024 | - Conclusions on scenarios for measurement skipping based on RAN1 conclusions. - Discussion on the need for UE assistance information regarding measurements occasions needed. |
| RAN4 #113 - November 2024 | - Discussion on RRM requirements with measurement skipping. - Discussion on the necessary UE assistance information. |
| RAN4 #114 - February 2025 | - Conclusion on RRM requirements with measurement skipping. - Discussion on the need for UE assistance information regarding measurement skipping. - Agreement of CR work split. |
| RAN4 #114bis - April 2025 | - Conclusion on the necessary UE assistance information. - First Draft CRs for RRM core requirements. - Draft CRs for 38.133 expected. - First big Draft CR. |
| RAN4 #115 - May 2025 | - Discussion on remaining open issues for RRM core |
| RAN4 #116 - August 2025 | - Discussion on remaining open issues for RRM core - Revision of CRs for RRM core requirements. - Planning of RRM performance. - Initial discussion on RRM test cases. |

# RRM requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2504504 | Nokia | Collection of XR_Ph3 agreements for information Moderator: document for information only. No proposals |
| R4-2503236 | InterDigital, Inc. | NR_XR_Ph3 remaining issues Observation 1: If the DCI skipping order applies to the gaps affecting the cells configured in a particular band, then at least inter-band NR-DC with gaps per UE and per FR can be supported. Proposal 1: If Inter-band NR-DC gap skipping is to be supported, then the rule of per band applicability for DCI gap skipping order is required and sufficient. Observation 2: As Type-1 MG is a mandatory feature for Rel-19, the other types of MG shall be treated as a second priority. Proposal 2: At least Type-1 MG shall be covered in Rel-19 timeframe. We don’t have a strong opinion on the WID coverage for second priority MG types, if times allows. Observation 3: Option 1 offers a more intuitive and direct way of computing the delay as it adds up the skipped measurements. Proposal 3: Support Option 1: - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, Ceil( 8 * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. Observation 4: As the measurement delay will be precisely defined, and the supported gaps are network configured, the need for extra conditions is not clear. Proposal 4: Support Option 2: Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration. Observation 5: The UAI agreed assumptions do not justify UAI introduction. Proposal 5: Do not introduce UAI, and thus: - Support Option 1 on the measurement occasions: - Option 1: Do not define UAI related to measurement occasions. Support Option 2 on the channel conditions: - Option 2: do not provide information on channel conditions Proposal 6: Support Option 1 - Option 1: Do not consider semi-static solutions Proposal 7: Support Proposal 3 from the above list: Proposal 3: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. |
| R4-2503574 | Apple | Discussion of RRM impact of XR for NR Phase 3 Observation 1: if only dynamic solution is supported, it could be challenging to enable it in DC scenario. Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario. Proposal 2: priorities of each gap type: Cap types; First prio; Second prio; Don’t include. Type 1; √. Type 2; √. NCSG; √. MUSIM; √. Positioning; √. Rel 18 concurrent gap; √. PreConfigured; √. Proposal 3: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. Proposal 4: max number of occasions cancelled during a measurement period shall be defined. Proposal 5: support option 4 for simplicity. Open for option 2 and 3: - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation (preferred). |
| R4-2503760 | CMCC | Discussion on XR (eXtended Reality) for NR Phase 3 Proposal 1: for types of measurement gaps, it is proposed that Type 1 MG, Type 2 MG, NCSG, Pre-MG, concurrent gaps are first priority. Proposal 2: taking FR1 inter-frequency measurements with gap as an example, for the case of no DRX, the measurement delay extension due to measurement skipping is proposed as following: - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter Proposal 3: for time offset, there is no need to have condition for the optional capability of 3ms. Proposal 4: it is proposed that the content of UE assistance information could be the number and/or location of measurement gap occasions not used for measurement. Proposal 5: for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes. |
| R4-2503870 | Huawei, HiSilicon | Discussion on RRM requirements for Rel-19 XR Observation 1: Compared with DCI-based solution, semi-static approach has following benefits: - It is less complicated to UE implementation that UE can have a stable long-term measurement pattern. - Semi-static schemes can support the case that MG occasion skipping without impacting RRM performance. If NW can indicate the positions of MG occasions to be skipped, UE can arrange the measurement pattern to guarantee that there is no impact on measurement delay. - A single RRC signal has less overhead than DCI which is per-occasion indication. Proposal 1: RAN4 to conclude whether to introduce semi-static indication by RAN4#114bis. Proposal 2: RAN4 to consider following approach to move forward: - RAN4 to introduce semi-static indication to skip measurement gap. - RAN4 discuss whether UE is allowed to only support semi-static indication. Observation 2: When the MG is used to perform measurement for NR and LTE, gNB has no idea whether the following MG occasion is for NR or LTE, which is up to UE implementation. Observation 3: Longer processing time may be needed to cancel a MG scheduling for LTE measurement, which involving cross-RAT processing. Proposal 3: 3 ms optional capability only apply when UE is only configured with NR measurement. Proposal 4: Use following formula to define the delay extension. The all candidate carriers using the MG shall be extended. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. Proposal 5: Introduce UAI on number of MG occasions that can be skipped within measure period without measurement delay extension. Note: It applies to both DCI-based solution and semi-static solution if introduced. Observation 4: The interval between available measurement occasions shall be defined as conditions for delay extension. Upon exceeding the maximum interval, UE may need to restart the measurement instead of simply extending the delay. Observation 5: The interval between available measurement occasions is hard to be predefined with constant value which depends on UE implementation on maintaining the timing/AGC, and the MGRP configurations and number of carriers need to be measured Proposal 6: Introduce UAI maximum interval between available measurement occasions for delay extension. UE is allowed to restart measurement upon exceeding the interval instead of only extending the delay. Proposal 7: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. |
| R4-2503922 | Ericsson | On RRM requirements for XR Proposal 1: MGs associated with PRS (positioning) measurements shall not be cancelled, e.g., because positioning can be used for emergency and also because these measurements are handled by the core network node (LMF). Proposal 2: MUSIM MGs shall not be cancelled. Proposal 3: Network controlled pre-configured MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs. Proposal 4: Concurrent MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs. Proposal 5: FFS the impact of the following issues for UL XR operation and their relation to the cancelled MGs: - Issue 1: Ensuring that the UE has the correct UL timing prior to cancelling MGs, - Issue 2: Guard period for DL-to-UL switching, - Issue 3: Impact of RF switching from DL to UL and back. Proposal 6: For transmitting UL XR data during the cancelled MG occasion, the guard period for DL-to-UL switching shall be within the duration of the cancelled MG. Proposal 7: For transmitting UL XR data during the cancelled MG occasion, the time of RF switching from DL to UL and back, shall be within the duration of the cancelled MG. Proposal 8: When receiving a DCI scheduling UL XR data during a cancelled MG occasion, it is assumed that the UE will have the correct UL timing prior to the transmission of the UL XR data during the cancelled MG occasion. Proposal 9: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133, without the need to signal it in UAI. Proposal 10: No time-domain patterns are provided in UAI, e.g., because of scheduling impact and the potential ambiguity in patterns indicated by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle, and the UE is not required to perform measurements more frequently than once per DRX cycle, which should also be followed for XR. Proposal 11: DRX configuration needs to be considered in discussions on MG cancellation, e.g., because: - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided. - UE is not expected to monitor control channel beyond DRX on (and changing this assumption for XR may not be justified), - UE can be configured with more than one DRX configurations, - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations. Proposal 12: RAN4 does not need to further discuss other options for MG cancellation control such as semi-static schemes. Proposal 13: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: Condition; FR1: TPSS/SSS_sync_inter. No DRX; Max(600 ms, Ceil(8+XPSS/SSS,gaps) Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320 ms; Max(600 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320 ms; Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter. Condition NOTE1; FR2-1: TPSS/SSS_sync_inter. No DRX; Max(600 ms, Ceil(Mpss/sss_sync_inter + XPSS/SSS,gaps) Max(MGRP , SMTC period)) CSSFinter. DRX cycle ≤ 320 ms; Max(600 ms, Ceil(1.5 (Mpss/sss_sync_inter + XPSS/SSS,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320 ms; Ceil(Mpss/sss_sync_inter+ XPSS/SSS,gaps) DRX cycle CSSFinter. Proposal 14: The maximum numbers of extensions of inter-frequency cell detection period are for - FR1: - XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, - XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, - XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms. - FR2: - XPSS/SSS,gaps,max=1.5Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, - XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, - XPSS/SSS,gaps,max=0.5 Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms. Proposal 15: UE behaviour when the maximum number of extensions for cell detection is exceeded: - Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement. Proposal 16: Inter-frequency SSB time index detection requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively: Condition NOTE1; FR1: TSSB_measurement_period_inter NOTE2. No DRX; Max(120 ms, Ceil(3+Xind,gaps) Max(MGRP , SMTC period)) CSSFinter. DRX cycle ≤ 320 ms; Max(120 ms, Ceil((3+Xind,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320 ms; Ceil(3+Xind,gaps) DRX cycle CSSFinter. Condition; FR2-1: TSSB_measurement_period_inter. No DRX; Max(200 ms, Ceil(MSSB_index_inter + Xind,gaps) Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320 ms; Max(200 ms, Ceil(1.5 (MSSB_index_inter + Xind,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320 ms; Ceil(MSSB_index_inter + Xind,gaps) DRX cycle CSSFinter. Proposal 17: The maximum numbers of extensions of inter-frequency SSB time index detection period are: - FR1: - Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, - Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and - Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms. - FR2: - Xind,gaps,max=1.5 MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, - Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and - Xind,gaps,max=0.5 MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms. Proposal 18: UE behaviour when the maximum number of extensions for inter-frequency SSB time index detection is exceeded: - Upon exceeding Xind,gaps,max the UE shall restart the time index detection. Proposal 19: Inter-frequency measurement period requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively: Condition NOTE1; FR1: TSSB_measurement_period_inter NOTE2. No DRX; Max(200 ms, Ceil(8 + Xmeas) Max(MGRP , SMTC period)) CSSFinter. DRX cycle ≤ 320 ms; Max(200 ms, Ceil((8 + Xmeas) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320 ms; Ceil(8 + Xmeas) DRX cycle CSSFinter. Condition; FR2-1: TSSB_measurement_period_inter. No DRX; Max(400 ms, Ceil(Mmeas_period_inter + Xmeas) Max(MGRP , SMTC period)) CSSFinter. DRX cycle ≤ 320 ms; Max(400 ms, Ceil(1.5 (Mmeas_period_inter + Xmeas) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320 ms; Ceil(Mmeas_period_inter + Xmeas) DRX cycle CSSFinter. Proposal 20: The maximum numbers of extensions of inter-frequency cell detection period are for - FR1: - XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, - XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, - XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms. - FR2: - Xmeas=1.5 Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, - Xmeas= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, - Xmeas=0.5 Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms. Proposal 21: UE behaviour when the maximum number of extensions for inter-frequency measurements is exceeded: - upon exceeding Xmeas,max the UE shall restart the measurement. Proposal 22: RAN4 to discuss MG configurations relevant for MG cancellation, e.g., - minimum MGL, e.g., MGL≥3 ms, - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays. Proposal 23: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.: - Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known. Proposal 24: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3. |
| R4-2504027 | vivo | Further discussion on RRM requirements for XR Proposal 1: Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective. Proposal 2: Similar to type 1 MG, Type 2 MG can be considered if only per-UE gap or per-FR gap are configured. Proposal 2: Option 1 is used to define requirements for gap skipping. Proposal 3: Similar to type 1 MG, Type 2 MG can be considered if only per-UE gap or per-FR gap are configured. Proposal 4: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. Proposal 5: Requirements in clause 9.15 NR inter-frequency L1 measurement are updated for gap skipping. Proposal 6: The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information. Proposal 7: NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement so that UE can provide corresponding gap patterns that meet the percentage. Proposal 8: The maximum interval between two measurement occasions available for RRM measurements can be defined as UAI. Proposal 9: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured. Proposal 10: The information about gaps not used for measurement are reported as UE capability rather than UE assistance information. Proposal 11: Send an LS to RAN2 and RAN1 on RAN4 agreements about UE assistance information. Proposal 12: RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported. Proposal 13: RAN4 agrees that for DCI-based triggering scheme, the gaps to be skipped during a time period is indicated by RRC in advance. Proposal 14: RAN4 LS to RAN1 and RAN2 the agreements on introduction of RRC-based triggering scheme and early indication for DCI-based triggering scheme, and additionally on only gap skipping is considered in the WI. Proposal 15: RAN4 doesn’t need to discuss the gap skipping in frequency domain as it is already clear based on RAN1 agreements. |
| R4-2504252 | ZTE Corporation, Sanechips | Discussion on RRM aspects of R19 XR Proposal 1: The gap/restriction skipping and the support of NR-DC are two separate objectives in the WID and assigned to different groups. Since same challenge as EN/NE-DC, apply aligned principle to NR-DC and EN/NE-DC, i.e. not support DCI-based measurement gap skipping for NR-DC. Proposal 2: RAN4 to define the measurement delay extension for inter-frequency measurement with gap as: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions overlaping with SMTC in the measurement period. - Use this formula as example and follow similar approach for the other cases. Proposal 3: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. Proposal 4: Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered. Proposal 5: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration. Proposal 6: No need to define additional condition for the optional capability of 3 ms. Proposal 7: Based on the guidance of UAI approved in last meeting, the following types of UAI are helpful to NW decision: - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation. |
| R4-2504291 | Samsung | General discussion on XR requirement Proposal 1: Deprioritize NR-DC for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements Proposal 2: Take the formula: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter as starting point to define the measurement delay extension due to measurement skipping for inter-frequency measurements with gap - FFS: The maximum number of skipped MG occasions in the measurement period to limit Nskip - FFS: CSSFinter Proposal 3: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified Proposal 4: Do not consider semi-static solutions |
| R4-2504481 | MediaTek inc. | Discussion on RRM requirements for XR Proposal 1: RAN4 should de-prioritize NR-DC. Proposal 2: RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic. Proposal 3: RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types, which are Type-2, NCSG, Con-current GAP, and Pre-MG. Proposal 4: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic. Proposal 5: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period)) × CSSFinter’, where Nskip is the number of skipped MG occasions in the measurement period. Proposal 6: RAN4 shall discuss and define the max number of occasions cancelled during a measurement period. Proposal 7: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period. Proposal 8: When a UE is configured with Per-FR gap, the MG skipping applies only to the FR associated with that Per-FR gap, otherwise, when Per-UR gap is configured, the MG skipping applies to both FRs. |
| R4-2504505 | Nokia | Analysis of skip ratio on XR capacity and mobility performance Moderator: Simulation paper for information, without proposals |
| R4-2504508 | Nokia | Discussion on XR ph3 RRM requirements Proposal 1: NR-DC is not supported for measurement gap skipping for XR enhancements. Observation 1: Existing L1/L3 sharing factor P is not defined for the case where gaps are overlapping with all SSB occasions. Proposal 2: RAN4 not to define requirements for measurement gap skipping for the case of measurement gap fully overlapped with L1 measurements. Proposal 3: RAN4 to consider the following gap types for measurement gap skipping: Gap types; First priority; Second priority; Don’t include. Type 1; Yes. Type 2; Don’t include. NCSG; Yes, Rel-17 NCSG. MUSIM; Don’t include. Positioning; Don’t include. Rel 18 concurrent gap; Don’t include. PreConfigured; Don’t include. Observation 2: The measurement delay of option 2 is 44,4 % larger than the delay for Option 1 when CSSF=5 and Nskip=5. Proposal 4: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms ´ CSSFinter, Ceil(8 * Kgap) ´ Max(MGRP , SMTC period) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period)), where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. Proposal 5: RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority. Observation 1: It is possible to increase skipping ratio to up to 50% for 2 layers and 80% for 5 layers without impacting the measurement delay of 1 frequency layer. Proposal 6: High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is - Nskip ≤8 ´ (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤ 5 ´ (CSSF - Nhigh) for intra-frequency measurements - where Nhigh is the number of high priority layers. Observation 2: Our simulations results show benefit of using about 60% skipping ratio. Observation 3: In at least some scenarios skipping ratio can be increased to about 75% without significant mobility degradation. Observation 4: Limiting the maximum number of occasions that can be skipped in the specification will have an impact on the network deployment flexibility. Proposal 7: No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration. Proposal 8: No condition is needed for the optional capability of 3 ms. Proposal 9: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where - X=5ms as baseline and X=3ms as optional UE capability. Observation 5: The number of samples needed to complete a measurement depends on the UE experience SNR. Observation 6: The experienced SNR is different for each frequency layer. Observation 7: If single UAI report is done, the network cannot distinguish the desired skipping ratio for each of the configured measurement gaps. Proposal 10: Define UAI reporting where the UE indicates a ratio of occasions that can be skipped without measurement delay extension per measurement object. Observation 8: RAN1 already confirmed working assumption regarding Alt 1-1. Observation 9: The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL. Observation 10: RAN4 already agreed on 5 ms time offset for measurement skipping as a baseline. Observation 11: Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1) Observation 12: Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic. Observation 13: Semi-static pattern cannot adapt quickly to channel conditions (e.g. decrease skipping ratio as response to reaching cell edge). Observation 14: RAN1 already made down selection between DCI and RRC schemes Proposal 11: Follow RAN1 agreement and consider DCI scheme only. Proposal 12: Capture in RAN4 specifications following RAN1 agreement: - It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications |
| R4-2504606 | Qualcomm Incorporated | RRM requirements for XR enhancements Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Proposal 2: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI atleast X ms before the MG occasion, where - X=5ms as baseline and X=3ms as optional UE capability. - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Proposal 3: For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples. Proposal 4: If the UE indicates the MG to be per-FR, the MG skipping indication is only valid for the gap instance in the same FR with scheduled XR transmission, otherwise, the indication applies to all the carriers. |
| R4-2504622 | Meta | Discussion on RRM Requirements for XR Proposal 1: Deprioritize NR-DC scenarios. Proposal 2: Support Type-1 measurement gaps and deprioritize other types. Proposal 3: Support Option 2, e.g., no conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration. Proposal 4: Regarding UAI, support Option 5 in the WF. |

## Open issues summary

### Topic 1 Scenarios for measurement skipping

#### Issue 1-1: Deployment scenarios

- Background information: In RAN4 #112 bis the following agreement regarding deployment scenarios was reached R4-2416864:

< Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation

- Background information: In RAN4 #113 the following agreement regarding deployment scenarios was reached R4-2420103:

< Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID <Way forward>: - Further discussion needed for - FR2-2 - NR-DC

- Background information: In RAN4 #114 the following agreement regarding deployment scenarios was reached R4-2502700:

< Agreement>: - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements <Way forward>: - Further discussion needed for - NR-DC

- Proposals form companies

  - Proposals not supporting NR-DC

    - Proposal 1 (Meta) Deprioritize NR-DC scenarios.

    - Proposal 2 (Nokia): NR-DC is not supported for measurement gap skipping for XR enhancements.

    - Proposal 3 (MediaTek): RAN4 should de-prioritize NR-DC.

    - Proposal 4 (Samsung): Deprioritize NR-DC for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements.

    - Proposal 5 (ZTE): The gap/restriction skipping and the support of NR-DC are two separate objectives in the WID and assigned to different groups. Since same challenge as EN/NE-DC, apply aligned principle to NR-DC and EN/NE-DC, i.e. not support DCI-based measurement gap skipping for NR-DC.

  - Proposals supporting NR-DC

    - Proposal 6 (Interdigital): If Inter-band NR-DC gap skipping is to be supported, then the rule of per band applicability for DCI gap skipping order is required and sufficient.

  - Proposals where other working groups are asked for decision

    - Proposal 7 (vivo): Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective.

- Merged proposals from Moderator

  - Option 1: NR-DC is not supported

  - Option 2: NR-DC is de-prioritized

  - Option 3: Inter-band NR-DC is supported and per-band applicability for DCI skipping order is required and sufficient.

  - Option 4: Send LS to RAN1 and RAN2

- Recommended WF

  - Majority of the companies support deprioritizing or precluding NR-DC. Please check if Option 1 or Option 2 can be accepted.

#### Issue 1-2: UE power class

- Proposals

  - Option 1 (Ericsson): The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

- Recommended WF

  - It is the first time that power class is discussed. Please give comments. For the proponent of Option 1, please clarify if this proposal includes intra-frequency / inter-RAT measurements as well.

#### Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Background information: In RAN4 #114 the following agreement regarding deployment scenarios was reached R4-2502700:

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions ## Issue 1-4: L1 measurements with scheduling restrictions and procedures ## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

- Proposals from companies

  - Proposal 1 (Nokia): RAN4 not to define requirements for measurement gap skipping for the case of measurement gap fully overlapped with L1 measurements.

- Recommended WF

  - Please discuss Proposal 1.

#### Issue 1-5: L1/L2 triggered mobility measurements and procedures

- Background information: In RAN4 #114 the following agreement regarding deployment scenarios was reached R4-2502700:

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions ## Issue 1-4: L1 measurements with scheduling restrictions and procedures ## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

- Proposals from companies

  - Proposal 1 (vivo): Requirements in clause 9.15 NR inter-frequency L1 measurement are updated for gap skipping.

- Recommended WF

  - Discuss the proposal.

### Topic 2 Measurements with gaps

#### Issue 2-1: General approach for defining requirements for measurements with gaps

- Proposals

  - Proposal 1 (MediaTek): RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types, which are Type-2, NCSG, Con-current GAP, and Pre-MG.

- Recommended WF

  - Please discuss the proposal. We can use the open points for the discussion on the next issue 2-2.

#### Issue 2-2: Types of measurement gaps

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Company proposals on the priorities for RAN4 to work are summarized using based on the following table

| Gap types | First priority | Second priority | Shall not be cancelled |
| --- | --- | --- | --- |
| Type 1 | Moderator: Type 1 is omitted since it is already agreed |  |  |
| Type 2 | CMCC, Apple | vivo (one MG only, per FR or per-UE) | Meta, Nokia |
| NCSG | CMCC, Apple | Nokia, Ericsson | Meta, vivo |
| MUSIM |  | Apple | Meta, Nokia, Ericsson, vivo |
| Positioning |  |  | Meta, Nokia, Ericsson, Apple, vivo |
| Rel 18 concurrent gap | CMCC, Apple |  | Meta, Nokia, vivo |
| PreConfigured | CMCC, Apple | Ericsson, vivo? | Meta, Nokia |

- Recommended WF

  - Recommended to further progress on other issues before coming back to gap types.

### Topic 3 XR skipping requirements impact

#### Issue 3-1: Principles for measurement delay calculation

- Proposals

  - Proposal 1 (Apple): as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

  - Proposal 2 (Qualcomm): For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

- Recommended WF

  - Discuss the proposals.

#### Issue 3-2: Formulas for measurement delay extension without DRX

- Proposals

  - Option 1 (Nokia, vivo, Interdigital): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Max(200 ms × CSSFinter, Ceil( 8  * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

  - Option 2b (MediaTek, ZTE, Ericsson, Huawei, CMCC): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

  - Option 2a (Samsung): Take the formula: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter as starting point to define the measurement delay extension due to measurement skipping for inter-frequency measurements with gap

    - FFS: The maximum number of skipped MG occasions in the measurement period to limit Nskip

    - FFS: CSSFinter

- Recommended WF

  - Please discuss based on Option 1, 2, and 2b

#### Issue 3-3: Formulas for measurement delay extension with DRX

- Proposals

  - Option 1 (Moderator/Nokia): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - DRX cycle ≤ 320 ms

      - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

    - DRX cycle > 320 ms

      - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

  - Option 2 (Ericsson): Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

    - DRX cycle ≤ 320 ms

      - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

    - DRX cycle > 320 ms

      - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

- Recommended WF

  - Discussion first for measurement period in FR1.

  - Extension to PSS/SSS detection and SSB index detection can be done in 2nd step.

  - Discuss based on outcome of case without DRX.

  - Extension to FR2 is discussed in a next step.

#### Issue 3-4: Conditions for measurement skipping

- Proposals from companies

  - Proposals for no conditions

    - Proposal 1 (Meta): no conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

    - Proposal 2 (Nokia): No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration

    - Proposal 3 (ZTE): No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

    - Proposal 4 (Interdigital): No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

  - Proposals on max number of occasions

    - Proposal 5 (MediaTek): RAN4 shall discuss and define the max number of occasions cancelled during a measurement period.

    - Proposal 6 (Samsung, vivo): For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified

    - Proposal 7 (Apple): max number of occasions cancelled during a measurement period shall be defined.

    - Proposal 8 (Ericsson): The maximum number of extensions are

|  | PSSS/SSS detection | SSB index detection | Measurement period |
| --- | --- | --- | --- |
| No DRX or max(DRX cycle, SMTC period, MGRP)≤40 ms | FR1: 12 FR2: 1.5×Mpss/sss_sync | FR1: 5 FR2: 1.5×MSSB_index | FR1: 12 FR2: 1.5×Mmeas_period |
| DRX cycle ≤ 320 ms | FR1: 8 FR2: Mpss/sss_sync | FR1: 3 FR2: MSSB_index | FR1: 8 FR2: Mmeas_period |
| DRX cycle > 320 ms | FR1: 5 FR2: 0.5×Mpss/sss_sync | FR1: 2 FR2: 0.5× MSSB_index | FR1: 5 FR2: 0.5× Mmeas_period |

    - Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

    - Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

    - Upon exceeding Xmeas,max the UE shall restart the measurement.

  - Proposals on maximum separation between two occasions shall be

    - Proposal 9 (Ericsson): Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

      - Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

- Proposals merged by the Moderator

  - Option 1: No conditions are defined

  - Option 2: Define the maximum number of gaps skipped during the measurement period.

  - Option 3: Define a maximum separation between two non-skipped gaps for measurement

- Recommended WF

  - Discussion on the conditions is needed. If the group agrees that conditions are necessary, we can further discuss the limits taking Proposal 8 as an example.

#### Issue 3-5: Measurement gap configurations relevant to skipping

- Options

  - Option 1 (Ericsson): RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

    - minimum MGL, e.g., MGL≥3 ms,

    - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

- Recommended WF

  - Please discuss if there is need for limiting which MG configurations can be skipped.

#### Issue 3-6: DRX

- Proposal

  - Proposal 1 (Ericsson): DRX configuration needs to be considered in discussions on MG cancellation, e.g., because:

    - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided.

    - UE is not expected to monitor control channel beyond DRX on (and changing this assumption for XR may not be justified),

    - UE can be configured with more than one DRX configurations,

    - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations.

  - Recommended WF

    - Discuss the proposal.

#### Issue 3-7: Impact on different frequency layers

- Proposals

  - Proposal 1 (Nokia): RAN4 to define requirements such that the network can configure which frequency layers are measured with higher priority

  - Proposal 2 (Nokia): High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is

    - Nskip ≤ 8 × (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤  5 × (CSSF - Nhigh) for intra-frequency measurements

    - where Nhigh is the number of high priority layers.

- Recommended WF

  - Discuss the proposals.

#### Issue 3-8: UL XR impact

- Proposals

  - Proposal 1 (Ericsson) : FFS the impact of the following issues for UL XR operation and their relation to the cancelled MGs:

    - Issue 1: Ensuring that the UE has the correct UL timing prior to cancelling MGs,

    - Issue 2: Guard period for DL-to-UL switching,

    - Issue 3: Impact of RF switching from DL to UL and back.

  - Proposal 2 (Ericsson): For transmitting UL XR data during the cancelled MG occasion, the guard period for DL-to-UL switching shall be within the duration of the cancelled MG.

  - Proposal 3 (Ericsson): For transmitting UL XR data during the cancelled MG occasion, the time of RF switching from DL to UL and back, shall be within the duration of the cancelled MG.

  - Proposal 4 (Ericsson): When receiving a DCI scheduling UL XR data during a cancelled MG occasion, it is assumed that the UE will have the correct UL timing prior to the transmission of the UL XR data during the cancelled MG occasion.

- Recommended WF:

  - Discuss the proposals.

### Topic 4 Time offset for measurement skipping processing

#### Issue 4-1: Time offset requirement for measurement gap skipping

- Background information: Agreement from RAN4 #113 (R4-2420103)

## Issue 4-1: Time offset requirement for measurement gap skipping <Agreement>: - Baseline feature 5 ms - Optional capability 3 ms, FFS if any condition is needed

- Proposals

  - Option 1 (Huawei): 3 ms optional capability only apply when UE is only configured with NR measurement.

  - Option 2 (Nokia, ZTE, CMCC): No need to define additional condition for the optional capability of 3 ms

- Recommended WF

  - Discuss the options.

#### Issue 4-2: Time offset requirement for cancelling skip indication

- Background information: RAN1 agreement

A component of an FG can have Option 1 as a first value and Option 2 as a second value: - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Note: UE indicates only one value of this component.

- Proposals

  - Proposal 1 (Qualcomm, Nokia): The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where

    - X=5ms as baseline and X=3ms as optional UE capability.

    - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

- Recommended WF

  - Please consider if Proposal 1 can be agreed.

### Topic 5 Need/feasibility of UE assistance information

Sub-topic description

RAN1 has sent an LS (R1-2405736) to RAN4, requesting RAN4 to decide whether to introduce any UE assistance information related to measurement occasions. This sub-topic includes issues relating to the need and feasibility of such UE assistance information.

Open issues and candidate options before meeting:

#### Issue 5-1: General on UAI

- Agreement from RAN4 #114

Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability. <Way forward>: - Proposals related to measurement occasions - Option 1: Do not define UAI related to measurement occasions. - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation. - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability. - Proposals related to channel conditions - Option 1: Provide information on channel conditions - Option 2: do not provide information on channel conditions

- Proposals from the companies

  - Proposals not supporting defining UAI

    - Proposal 1 (Interdigital): Do not define UAI related to measurement occasions.

    - Proposal 2 (Interdigital): Do not provide information on channel conditions

    - Proposal 3 (Ericsson): The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133, without the need to signal it in UAI.

    - Proposal 4 (Ericsson): No time-domain patterns are provided in UAI, e.g., because of scheduling impact and the potential ambiguity in patterns indicated by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle, and the UE is not required to perform measurements more frequently than once per DRX cycle, which should also be followed for XR.

  - Proposals on measurement occasions

    - Proposal 5 (Meta): Regarding UAI, support Option 5 in the WF.

    - Proposal 6 (Nokia): Define UAI reporting where the UE indicates a ratio of occasions that can be skipped without measurement delay extension per measurement object.

    - Proposal 7 (ZTE): Based on the guidance of UAI approved in last meeting, the following types of UAI are helpful to NW decision:

      - Option 2: The UE indicates a pattern of unused gaps

      - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

      - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation.

    - Proposal 8 (vivo): The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information.

    - Proposal 9 (vivo): The maximum interval between two measurement occasions available for RRM measurements can be defined as UAI.

    - Proposal 10 (vivo): RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

    - Proposal 11 (Huawei): Introduce UAI on number of MG occasions that can be skipped within measure period without measurement delay extension.

      - Note: It applies to both DCI-based solution and semi-static solution if introduced.

    - Proposal 12 (Huawei): Introduce UAI maximum interval between available measurement occasions for delay extension. UE is allowed to restart measurement upon exceeding the interval instead of only extending the delay.

    - Proposal 13 (CMCC): it is proposed that the content of UE assistance information could be the number and/or location of measurement gap occasions not used for measurement.

    - Proposal 14 (Apple): support option 4 for simplicity. Open for option 2 and 3.

  - Proposals regarding UE capabilities

    - Proposal 15 (vivo): The information about gaps not used for measurement are reported as UE capability rather than UE assistance information.

  - Other proposals

    - Proposal 16 (vivo): Send an LS to RAN2 and RAN1 on RAN4 agreements about UE assistance information.

- Proposals merged by the Moderator

  - Options related to measurement occasions

    - Option 1: Do not define UAI related to measurement occasions.

    - Option 2: The UE indicates a pattern of unused gaps

    - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

      - Option 3a: The UE indicates a ratio/number of occasions that can be skipped per measurement object

    - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability.

    - Option 6: The UE indicates the maximum interval between two available measurement occasions

    - Option 7: The information about gaps not used for measurement is reported as UE capability rather than UE assistance information.

  - Proposals related to channel conditions

    - Option 1: Provide information on channel conditions

    - Option 2: do not provide information on channel conditions

- Recommended WF

  - Discussion based on the options merged by the Moderator.

    - If we agree on UAI reporting, please consider also Proposal 10 on the relationship to DRX configuration and Proposal 16 on sending an LS to RAN2 and RAN1.

### Topic 6 Additional signalling aspects

#### Issue 6-1: Semi-static schemes

- Proposals from companies

  - General proposals

    - Proposal (Huawei) RAN4 to conclude whether to introduce semi-static indication by RAN4#114bis.

  - Proposals not supporting semi-static schemes

    - Proposal 2 (Nokia): Follow RAN1 working assumption and consider DCI only.

    - Proposal 4 (Samsung): Do not consider semi-static solutions.

    - Proposal 8 (Ericsson): RAN4 does not need to further discuss other options for MG cancellation control such as semi-static schemes.

    - Proposal 13 (Interdigital): Do not consider semi-static solutions

  - Proposals supporting semi-static schemes

    - Proposal 1 (Qualcomm): In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well.

    - Proposal 3 (MediaTek): RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

    - Proposal 5 (vivo): RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported.

    - Proposal 6 (vivo): RAN4 agrees that for DCI-based triggering scheme, the gaps to be skipped during a time period is indicated by RRC in advance.

    - Proposal 7 (vivo): RAN4 LS to RAN1 and RAN2 the agreements on introduction of RRC-based triggering scheme and early indication for DCI-based triggering scheme, and additionally on only gap skipping is considered in the WI.

    - Proposal 10 (Huawei): RAN4 to consider following approach to move forward:

      - RAN4 to introduce semi-static indication to skip measurement gap.

      - RAN4 discuss whether UE is allowed to only support semi-static indication.

    - Proposal 11 (CMCC): for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes.

    - Proposal 12 (Apple): In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

- Proposals merged by the Moderator

  - Further discussion based on recommended options

    - Option 1: Do not consider semi-static solutions

    - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution.

      - Option 2a: RRC pattern-based

      - Option 2b: MAC-CE bitmap for skipping indication

      - Option 2c: RRC configuration patterns with MAC-CE based activation

    - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

      - Option 3a: UE is allowed to only support semi-static indication.

- Recommended WF

  - Discuss the proposals.

#### Issue 6-2: Skipping indication on frequency domain

- Proposals

  - Option 1: Follow RAN1 agreements

    - Option 1a (Nokia): Capture in RAN4 specifications following RAN1 agreement:

      - It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction.

      - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

    - Option 1b (vivo): RAN4 doesn’t need to discuss the gap skipping in frequency domain as it is already clear based on RAN1 agreements.

  - Option 2: Further RAN4 discussion needed

    - Option 2a (Qualcomm) If the UE indicates the MG to be per-FR, the MG skipping indication is only valid for the gap instance in the same FR with scheduled XR transmission, otherwise, the indication applies to all the carriers.

    - Option 2b (MediaTek): When a UE is configured with Per-FR gap, the MG skipping applies only to the FR associated with that Per-FR gap, otherwise, when Per-UR gap is configured, the MG skipping applies to both FRs.

    - Option 2c (ZTE): For the gap/restriction skipping impact in frequency domain,

      - For the gap skipping, depend on whether the UE capable of per-FR gap

        - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

        - Otherwise, no need to distinguish different FR.

      - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

    - Option 2d (Huawei, Interdigital): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

- Recommended WF

  - Issue has been agreed in RAN1. Please discuss if Options 1/1a/1b are ok or if further discussion is needed.

#### Issue 6-3: Network indication

- Proposals

  - Proposal 1 (vivo):

    - NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement so that UE can provide corresponding gap patterns that meet the percentage.

- Recommended WF

  - Please discuss the proposal.

#### Issue 7-4: Partial measurement skipping

- Proposals

  - Proposal 1 (ZTE): Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered.

- Recommended WF

  - Please discuss the proposal.

# Draft CRs

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503921 | Ericsson | draftCR38133 Inter-frequency measurement requirements with XR |
| R4-2503575 | Apple | Inter-RAT measurements E-UTRAN TDD measurements |
| R4-2504029 | vivo | Draft CR on NR inter-frequency L1 measurement for XR |
| R4-2504257 | ZTE Corporation, Sanechips | Draft CR on measurement gap skipping in intra-frequency measurement with gaps for R19 XR |
| R4-2504509 | Nokia | Draft CR38.133 Introduction of measurement gap cancellation |

### Topic 7 CR work and clauses with spec impact

#### Issue 7-1: Update work split

- Background information: In RAN4 #114 the following worksplit and list of affected clauses was agreed R4-2502700:

| Clause | Topic | Volunteer |
| --- | --- | --- |
| 8.xr (new) Or 9.1 | Time offset for DCI based measurement skipping | Nokia |
| 9.2.6 | Intra-freq measurements with gaps | ZTE |
| 9.3.4 | Inter-freq measurements with gaps | Ericsson |
| 9.4.2 | Inter-RAT measurements E-UTRAN FDD measurements | Huawei |
| 9.4.3 | Inter-RAT measurements E-UTRAN TDD measurements | Apple |
| NOTE: The full list of clauses with potential impact due to XR skipping is found in the Ad-hoc minutes R4-2502580 |  |  |

.

- Recommended WF

  - Check if L1 impact is foreseen based on agreement for Issue 1-4.

  - Check if impact on TS 38.133, clause 9.15 can be agreed based on agreement for Issue 1-5.

#### Issue 7-2: Comments on CRs

- List of CRs for commenting

  - R4-2503921, Ericsson, draftCR38133 Inter-frequency measurement requirements with XR

  - R4-2503575, Apple, Inter-RAT measurements E-UTRAN TDD measurements

  - R4-2504029, vivo, Draft CR on NR inter-frequency L1 measurement for XR

  - R4-2504257, ZTE Corporation, Sanechips, Draft CR on measurement gap skipping in intra-frequency measurement with gaps for R19 XR

  - R4-2504509, Nokia, Draft CR38.133 Introduction of measurement gap cancellation

- Recommended WF

  - Collect comments

  - To be updated based on agreements during RAN4 #114bis