3GPP TSG-RAN WG4 Meeting #114bis                                                           R4-2504631

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.23.4

Source:	Moderator (Samsung)

Title:	Topic summary for [114bis][306] NR_duplex_evo_General

Document for:	Information

# Introduction

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved [RP-234035], with WID further revised in the follow-up RAN plenary [RP-240789]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”.

Over RAN4#114, there was extensive discussion on SBFD general aspects and potential new requirements for SBFD operation for FR1 and FR2-1, which results in the progress captured in the approved WF [R4-2502310].

This document is provided for the moderator summary of thread [306] on Rel-19 work item on evolution of NR duplex operation (SBFD) for general aspect and potentially new requirements for SBFD, in which the following highlighted agenda items are supposed to be covered specifically:

- Evolution of NR duplex operation: Sub-band full duplex (SBFD) [NR_duplex_evo] - General aspects (including RAN4 aspects for SBFD system parameters) [NR_duplex_evo-Core] - BS RF requirements [NR_duplex_evo-Core] - Potentially new requirements for SBFD operation for FR1 and FR2-1 [NR_duplex_evo-Core] - Modification of existing Tx requirements for FR1 and FR2-1 [NR_duplex_evo-Core] - Modification of existing Rx requirements for FR1 and FR2-1 [NR_duplex_evo-Core] - RRM core requirements [NR_duplex_evo-Core] - RRM requirements for UE-to-UE CLI handling [NR_duplex_evo-Core] - RRM impacts for SBFD operation [NR_duplex_evo-Core] - Moderator summary and conclusions

# Topic #1: General issues

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503285 | Tejas Network Limited | Proposal 1: For 50 MHz and 100 MHz bandwidth cases, UL subband sizes must conform to those specified in Table 5.3.2-1 of TS 38.104. In DUD configurations, the two downlink subbands (DL1 and DL2) must either be equal in size or differ by no more than 1 RB. Proposal 2: For FR1 WA BS, the declared DL subband sizes supporting SBFD shall be tested, while no restrictions should apply to other FR1 BS classes or FR2 BS. |
| R4-2503405 | CMCC | Proposal 1: There is no need to set restrictions on subband configurations for MR and LA BS. Proposal 2: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared for WA BS. Proposal 3: The maximum value of the guard band size should be specified in the spec. |
| R4-2503466 | CATT | Proposal 1: No limitation for DL/UL sub-band declaration for MR class. Proposal 2: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. Proposal 3: DUD and DU pattern can have different UL SB size declaration. Proposal 4: The restriction of DL/UL subband configurations is not applied to FR1 MR BS. Proposal 5: For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case. • This restriction is not applied to FR1 MR BS. Proposal 6: There should be a limit on the maximum number of RBs for the guardband. Proposal 7: For SBFD, at least one DL subband size should be no less than the DL initial BWP size. |
| R4-2503823 | vivo | Proposal 1: BS channel bandwidth 100MHz for DUD case can be supported for FR2-1 gNB SBFD. Proposal 2: Only apply the restriction of SBFD configuration to FR1 WA BS, not for FR1 MR/LA and FR2-1 SBFD capable BS product. Proposal 3: It is not necessary to introduce the following restriction on DL subband size: o For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size). o FR2 minimum CBW to support SBFD is 100MHz. |
| R4-2504007 | Ericsson | Observation 1 Given that there are two patterns (DUD and DU/UD) for SBFD, for a specific channel BW, a maximum of two specific UL subband sizes implies that four analog filters per subarray are needed. Proposal 1 For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. Proposal 2 For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. Proposal 3 Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. Proposal 4 For FR1 WA SBFD capable BSs, a maximum of two specific UL subband sizes is allowed to be declared for a specific channel BW supported for SBFD. Proposal 5 For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), there is no need to restrict the maximum number of specific UL subband sizes for a specific channel BW supported for SBFD. Proposal 6 For FR1 MR SBFD capable BSs, when the TRP output power is greater than 33 dBm, a maximum of two specific UL subband sizes are allowed to be declared for a specific channel BW supported for SBFD; when the TRP output power is no greater than 33 dBm, analog filters are not expected to be used, and the maximum number of specific UL subband sizes for a specific channel BW supported for SBFD is not restricted. Proposal 7 Use draft CR R4-2504011 as baseline for introduction of SBFD feature in TS 38.104. |
| R4-2504011 | Ericsson | Draft CR to 38.104 for introduction of SBFD |
| R4-2504091 | Huawei, HiSilicon | Observation: For UL resource muting, there is no transmit quality or MPR requirement impact can be expected for PUSCH as comb-2 pattern is already supported since LTE. Proposal 1: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. • Different sizes for specific UL subband can be supported for DUD and DU/UD, respectively. Proposal 2: The restrictions on SBFD UL/DL subband configuration do not need to be considered for MR BS. |
| R4-2504193 | Nokia | Proposal 1: RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work. Proposal 2: If SSBs are transmitted in SBFD slots, RAN4 to consider 200MHz channel bandwidth for FR2-1 DUD case. A note can be added in the specification to indicate that FR2-1 SBFD DUD operation with 100 MHz channel bandwidth requires the SSB to be transmitted in DL-only or non-SBFD slots. Proposal 3: The restrictions agreed for FR1 WA BS are also applicable for FR1 MR BS. Proposal 4: For a specific channel BW supported for SBFD, maximum of 2 specific UL subbands sizes are allowed to be declared. Proposal 5: One of the 2 specific UL subband sizes can be applicable to the DUD configuration, and the other to the DU/UD configuration. Proposal 6: The restriction on maximum number of specific UL subband sizes to be declared is also applicable to the FR1 MR BS. Proposal 7: The restriction on maximum number of specific DL subband sizes to be declared is also applicable to the FR1 MR BS. Proposal 8: If requirement set applicability table for SBFD-capable BS will be implemented in the specification, the reflected requirement sets for SBFD-capable BS can be referenced to the new clause 12 that will include the requirements for SBFD-capable BS. Proposal 9: To introduce requirements for SBFD-capable BS in TS 38.104 in a separate clause 12. |
| R4-2504218 | Samsung | Proposal 1: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. • The specific UL subband sizes supported shall be the same for DUD and DU/UD if BS supports. • This restriction shall be applied to FR1 WA BS only. Proposal 2: For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case. • This restriction shall be applied to FR1 WA BS only. Proposal 3: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs. Proposal 4: The RAN4 evaluation on the signal quality impact from the new waveform can be concluded, i.e., no new MRP is required for PUSCH with resource muting for CLI handling, if no critical issue further identified. Proposal 5: The applicability of RAN4 requirement for SBFD capable BS shall be introduced, with the following table used as baseline: Table x.x: Requirement set applicability for SBFD-capable BS Requirement; Requirement Set for non-SBFD symbols; Requirement Set for SBFD symbols. BS type 1-H; BS type 1-O; BS type 2-O; BS type 1-H; BS type 1-O; BS type 2-O. BS output power; 6.2; TBD. Output power dynamics; 6.3; TBD. Transmit ON/OFF power; 6.4; TBD. Transmitted signal quality; 6.5; TBD. Occupied bandwidth; 6.6.2; TBD. ACLR; 6.6.3; TBD. Operating band unwanted emissions; 6.6.4; TBD. Transmitter spurious emissions; 6.6.5; TBD. Transmitter intermodulation; 6.7; NA; NA; TBD; NA; NA. In-channel adjacent subband leakage; NA; TBD. Reference sensitivity level; 7.2; TBD. Dynamic range; 7.3; TBD. In-band selectivity and blocking; 7.4; TBD. Out-of-band blocking; 7.5; TBD. Receiver spurious emissions; 7.6; TBD. Receiver intermodulation; 7.7; TBD. In-channel selectivity; 7.8; TBD. In-channel adjacent subband blocking; TBD. Performance requirements; 8; TBD. Radiated transmit power; 9.2; 9.2; 9.2; TBD; TBD; TBD. OTA base station output power; 9.3; 9.3; TBD; TBD. OTA output power dynamics; 9.4; 9.4; TBD; TBD. OTA transmit ON/OFF power; 9.5; 9.5; TBD; TBD. OTA transmitted signal quality; 9.6; 9.6; TBD; TBD. OTA occupied bandwidth; 9.7.2; 9.7.2; TBD; TBD. OTA ACLR; NA; 9.7.3; 9.7.3; NA; TBD; TBD. OTA out-of-band emission; 9.7.4; 9.7.4; TBD; TBD. OTA transmitter spurious emission; 9.7.5; 9.7.5; TBD; TBD. OTA transmitter intermodulation; 9.8; NA; TBD; NA. OTA in-channel adjacent subband leakage; NA; NA; TBD; TBD. OTA sensitivity; 10.2; 10.2; NA; TBD; TBD; NA. OTA reference sensitivity level; 10.3; 10.3; TBD; TBD. OTA dynamic range; 10.4; NA; TBD; NA. OTA in-band selectivity and blocking; 10.5; 10.5; TBD; TBD. OTA out-of-band blocking; NA; 10.6; 10.6; NA; TBD; TBD. OTA receiver spurious emission; 10.7; 10.7; TBD; TBD. OTA receiver intermodulation; 10.8; 10.8; TBD; TBD. OTA in-channel selectivity; 10.9; 10.9; TBD; TBD. Radiated performance requirements; 11; 11; TBD; TBD. OTA in-channel adjacent subband blocking; NA; NA; TBD; TBD. Proposal 6: The further sub-clauses of Chapter 12 “requirements for SBFD-capable BS” are provided as below: - Requirements for SBFD-capable BS: 12. 1 General 12.1.1 Applicability of requirements for SBFD-capable BS 12. 2 Operating bands and channel arrangement 12.2.1 General 12.2.2 Operating bands 12.2.3 BS channel bandwidth 12. 3 Conducted transmitter characteristics for SBFD-capable BS 12.3.1 General 12.3.2 Base station output power 12.3.3 Output power dynamics 12.3.4 Transmit ON/OFF power 12.3.5 Transmitted signal quality 12.3.6 Unwanted emissions 12.3.7 Transmitter intermodulation 12.3.8 In-channel adjacent subband leakage 12. 4 Conducted receiver characteristics for SBFD-capable BS 12.4.1 General 12.4.2 Reference sensitivity level 12.4.3 Dynamic range 12.4.4 In-band selectivity and blocking 12.4.5 Out-of-band blocking 12.4.6 Receiver spurious emissions 12.4.7 Receiver intermodulation 12.4.8 In-channel selectivity 12.4.9 In-channel adjacent subband blocking 12. 5 Conducted performance requirements for SBFD-capable BS 12. 6 Radiated transmitter characteristics for SBFD-capable BS 12.6.1 General 12.6.2 Radiated transmit power 12.6.3 OTA base station output power 12.6.4 OTA output power dynamics 12.6.5 OTA transmit ON/OFF power 12.6.6 OTA transmitted signal quality 12.6.7 OTA unwanted emissions 12.6.8 OTA transmitter intermodulation 12.6.9 OTA in-channel adjacent subband leakage 12. 7 Radiated receiver characteristics for SBFD-capable BS 12.7.1 General 12.7.2 OTA sensitivity 12.7.3 OTA reference sensitivity level 12.7.4 OTA dynamic range 12.7.5 OTA in-band selectivity and blocking 12.7.6 OTA out-of-band blocking 12.7.7 OTA receiver spurious emissions 12.7.8 OTA receiver intermodulation 12.7.9 OTA in-channel selectivity 12.7.10 OTA in-channel adjacent subband blocking 12. 8 Radiated performance requirements for SBFD-capable BS |
| R4-2504339 | Qualcomm Germany | Observation 1: Given that UL subband size needs to be at least 50 MHz and following RAN4 agreement that that DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB, DUD configuration for FR2-1 with 100MHz channel bandwidth would not be feasible. Proposal 1: In FR2-1 bands, the BS channel bandwidth for DUD configuration shall be no less than 200MHz. Observation 2: Restrictions on in-field operation of certain configurations is not within the scope of RAN4 specs, but could be captured via regional regulations (e.g., Harmonized Standards in Europe). Observation 3: It is challenging for RAN4 to agree on a reference RF solution for interference handling in SBFD-capable BS to be used to test SBFD configurations for medium range base stations. Observation 4: In TR 38.858, no challenges were reported from implementation point of view on the feasibility of SBFD-capable local area BS. Proposal 2: If it is feasible to capture RAN4 specs, RAN4 to consider the below only for wide area base stations • The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared and tested. Proposal 3: To limit the discussion of DL/UL subband configurations support for SBFD to FR1 WA BS at this stage, as other BS classes and FR2-1 WA BS have more flexibility in terms of interference cancellation. |
| R4-2504368 | ZTE Corporation, Sanechips | Proposal 1: regarding the declaration and flexibility for SBFD operation, we propose to define the conformance testing as following way: 3) FR1 WA BS capable of SBFD operation, BS need to declare its supported SBFD configuration and test all of these SBFD configuration; 4) For other BS class capable of SBFD operation, BS need to declare its supported SBFD configuration and only test the worst case which follows the legacy principle (e.g. typical transmission bandwidth (5MHz or 20MHz) or minimum supported DL sub-band configuration) Proposal 2: regarding the DL/UL subband configurations support for SBFD, to set the maximum UL sub-band size as 3 for FR1 WA SFBD and don’t see this necessity to apply this feature to other SBFD BS class. Proposal 3: propose not to mandate the additional restriction for DL sub-band band. Proposal 4: propose to conduct the MPR evaluation for PUSCH with resource muting for CLI solution. Proposal 5: for Case B, propose to configure the guard symbols following the the legacy TDD DL-UL configuration. Proposal 6: for Case C, whether the guard period to be contained within DL symbols or SBFD symbols could be left up to the implementation, however this should be made clear in the conformance testing specification for testing. Proposal 7: propose to consider the SBFD symbols together with the legacy UL symbols for dutycycle evaluation for HPUE cases including HPUE in single band, EN-DC or NR-DC. |

## Open issues summary

### Sub-topic 1-1: Declaration and flexibility for SBFD operation

##### Issue 1-1-1: Declaration and flexibility for SBFD operation

- [Moderator] In RAN4#113 and 114, the following agreements are achived:

Agreement in RAN4#113: - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications Agreement in RAN4#114: - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - This restriction shall not be applied to FR1 LA BS - FFS FR1 MR BS

- Observations/proposal on the declaration and flexibility for SBFD operation:

  - Observation 1 (Qualcomm): Restrictions on in-field operation of certain configurations is not within the scope of RAN4 specs, but could be captured via regional regulations (e.g., Harmonized Standards in Europe).

- Proposals on the BS classes in the above declaration rule:

  - Option 1 (CATT, vivo, Qualcomm): No limitation for DL/UL sub-band declaration for MR class.

  - Option 2 (Nokia): This restriction is also applicable to FR1 MR BS.

- Proposals on how conformance testing is performed:

  - Option A (ZTE):

    - FR1 WA BS capable of SBFD operation, BS need to declare its supported SBFD configuration and test all of these SBFD configuration;

    - For other BS class capable of SBFD operation, BS need to declare its supported SBFD configuration and only test the worst case which follows the legacy principle (e.g. typical transmission bandwidth (5MHz or 20MHz) or minimum supported DL sub-band configuration)

  - Option B (Ericsson):

    - For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

    - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

      - Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

- Moderator Recommendation:

  - Determine the applicability for MR BS firstly;

  - For the principle of how conformance testing is performed, it could be determined in the performance part discussion and can be deprioritized.

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

- [Moderator] Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. In RAN4#113, the following agreement is provided:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

In which option 2b is given in RAN4#112:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

##### Issue 1-2-1: Restriction on the number of UL subband sizes supported

- [Moderator] For the restriction-2 (i.e., restriction on the number of UL subband sizes supported), the following questions to be further discussed:

- For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

  - Maximum 3 specific UL subband sizes allowed: Samsung, CMCC, CATT, ZTE, Huawei

  - Maximum 2 specific UL subband sizes allowed: Nokia, Ericsson

- FFS different sizes for specific UL subband for DUD and DU/UD.

  - The same size for DUD and DU/UD: Samsung

  - DUD and DU/UD can have different UL subband size: CATT, Huawei, [Ericsson]

  - One for DUD and one for DU/UD: Nokia

  - Note: Need to check Ericsson’s view on this. It is not clearly indicated in Ericsson’s paper, but it mentioned that 2 UL subband sizes means 4 analog filters to be supported.

- FFS the restriction is also applied to FR1 MR BS

  - No (i.e., only apply to FR1 WA BS): Samsung, CMCC, CATT, Qualcomm, ZTE, Huawei

  - Yes (also appliable to FR1 MR BS): Nokia

  - Yes if TRP output power is greater than 33dBm: Ericsson

- Moderator Recommendation:

  - For MR BS, conclude the discussion by considering the Ericsson proposal as a compromised solution.

  - For the number of specific UL subband sizes and different size for DUD and DU/UD, a compromise solution is needed.

##### Issue 1-2-2: Restriction on the number of DL subband sizes supported

- [Moderator] For the restriction-3 (i.e., Restriction on the number of DL subband sizes supported), the following questions to be further discussed:

- FFS the restriction is also applied to FR1 MR BS

  - No (i.e., only apply to FR1 WA BS): Samsung, Tajas Network, CMCC, CATT, Qualcomm, ZTE, Huawei

  - Yes (also appliable to FR1 MR BS): Nokia

- Moderator Recommendation:

  - Conclude “the restriction is NOT applied to FR1 MR BS” based on majority view.

##### Issue 1-2-3: Restriction on guardband size for SBFD

- [Moderator] Based on RAN4#112bis agreement, the following options for restriction on guardband size are identified, while no further agreement achieved in RAN4#113:

Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

- Companies’ preference:

  - Option 3: CMCC, CATT

  - Option 3a: Samsung

- Moderator Recommendation:

  - Conclude this issue by down select Option 3 and 3a. If Option 3 is followed, the proponent may further suggest how RAN4 shall determine the maximum allowed number of RBs for guardband.

##### Issue 1-2-4: Restriction on DL subband size

- [Moderator] Based on last meeting’s discussion, the following WF is approved for further discussion on the following restrictions:

Way Forward: - FFS the following restriction on DL subband size: - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

- Companies’ preference:

  - Option 1 (CATT): at least one DL subband size should be no less than

    - the DL initial BWP size.

    - SSB BW + sync raster step size.

  - Option 2 (ZTE): Don’t mandate the additional restriction for DL sub-band band.

- Moderator Recommendation:

  - Further discussion is needed:

    - If SSB can also be located in the non-SBFD symbols, seems Option 2 make more sense.

##### Issue 1-2-5: FR2-1 CBW support for SBFD DUD case

- [Moderator] For whether FR2-2 CBW 100MHz can be support for DUD case, it is closely related to Issue 1-2-4. So, this issue is moved to under this sub-topic for this meeting (although the resultant conclusion is related to CBW rather than subband size).

- [Moderator] In RAN4#114, the following are agreed on FR2-1 CBW and SCS support for SBFD:

Agreement: - In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz. - RAN4 to further study on 100MHz for DUD case to identify if any issue.

- Proposals on 100MHz for DUD case:

  - Option 1 (vivo): BS channel bandwidth 100MHz for DUD case can be supported for FR2-1 gNB SBFD.

  - Option 2 (Qualcomm, Nokia): In FR2-1 bands, the BS channel bandwidth for DUD configuration shall be no less than 200MHz.

    - Option 2a (Nokia): A note can be added in the specification to indicate that FR2-1 SBFD DUD operation with 100 MHz channel bandwidth requires the SSB to be transmitted in DL-only or non-SBFD slots.

- Moderator Recommendation:

  - Determine Issue 1-2-4 firstly and then this issue can be determined accordingly.

### Sub-topic 1-3: Requirement applicability for SBFD

##### Issue 1-3-1: Requirement set applicability table for SBFD-capable BS

- [Moderator] On the requirement set applicability table for SBFD-capable BS, the following way forward is approved in RAN4#114:

Way Forward: - RAN4 further study the applicability of RAN4 requirement for SBFD capable BS, by taking the table provided in R4-2501967 as example to be further study on. NOTE: The table does not imply that the BS can be declared 1-H for non-SBFD and 1-O for SBFD, or vice versa.

- Proposal for the requirement set applicability table for SBFD-capable BS:

  - Proposal 1 (Samsung): The requirement set applicability table for SBFD-capable BS is updated as (updated rows have been highlighted):

- Table x.x: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | TBD |  |  |
| Output power dynamics | 6.3 |  |  | TBD |  |  |
| Transmit ON/OFF power | 6.4 |  |  | TBD |  |  |
| Transmitted signal quality | 6.5 |  |  | TBD |  |  |
| Occupied bandwidth | 6.6.2 |  |  | TBD |  |  |
| ACLR | 6.6.3 |  |  | TBD |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | TBD |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | TBD |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | TBD | NA | NA |
| In-channel adjacent subband leakage | NA |  |  | TBD |  |  |
| Reference sensitivity level | 7.2 |  |  | TBD |  |  |
| Dynamic range | 7.3 |  |  | TBD |  |  |
| In-band selectivity and blocking | 7.4 |  |  | TBD |  |  |
| Out-of-band blocking | 7.5 |  |  | TBD |  |  |
| Receiver spurious emissions | 7.6 |  |  | TBD |  |  |
| Receiver intermodulation | 7.7 |  |  | TBD |  |  |
| In-channel selectivity | 7.8 |  |  | TBD |  |  |
| In-channel adjacent subband blocking |  |  |  | TBD |  |  |
| Performance requirements | 8 |  |  | TBD |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | TBD | TBD | TBD |
| OTA base station output power |  | 9.3 | 9.3 |  | TBD | TBD |
| OTA output power dynamics |  | 9.4 | 9.4 |  | TBD | TBD |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | TBD | TBD |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | TBD | TBD |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | TBD | TBD |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | TBD | TBD |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |  | TBD | TBD |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | TBD | TBD |
| OTA transmitter intermodulation |  | 9.8 | NA |  | TBD | NA |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | TBD | TBD |
| OTA sensitivity | 10.2 | 10.2 | NA | TBD | TBD | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | TBD | TBD |
| OTA dynamic range |  | 10.4 | NA |  | TBD | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | TBD | TBD |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | TBD | TBD |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | TBD | TBD |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | TBD | TBD |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | TBD | TBD |
| Radiated performance requirements |  | 11 | 11 |  | TBD | TBD |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | TBD | TBD |

- Moderator Recommendation:

  - RAN4 discuss and further refine/endorse the table in this meeting.

### Sub-topic 1-4: BS RF specification structure for SBFD requirements

##### Issue 1-4-1: BS RF specification structure for SBFD requirements

- [Moderator] Based on RAN4#114 discussion, the follow way forward is achieved, in which a separate chapter for requirements of SBFD-capable BS in TS 38.104 used as baseline:

<From RAN4#114> Agreement: - RAN4 shall capture RF requirements in SBFD-capable BS in TS 38.104. To use a separate chapter for requirements for SBFD-capable BS in TS 38.104 is the baseline, while details can be FFS by taking below structure as the example. Foreword 1 Scope 2 References 3 Definitions, symbols and abbreviations 4 General 5 Operating bands and channel arrangement 6 Conducted transmitter characteristics 7 Conducted receiver characteristics 8 Conducted performance requirements 9 Radiated transmitter characteristics 10 Radiated receiver characteristics 11 Radiated performance requirements 12 Requirements for SBFD-capable BS

- Proposed sub-clauses of Chapter 12 “requirements for SBFD-capable BS”:

  - Samsung (R4-2504218):

- Requirements for SBFD-capable BS:

12. 1     General

12.1.1		Applicability of requirements for SBFD-capable BS

12. 2     Operating bands and channel arrangement

12.2.1		General

12.2.2		Operating bands

12.2.3		BS channel bandwidth

12. 3     Conducted transmitter characteristics for SBFD-capable BS

12.3.1		General

12.3.2		Base station output power

12.3.3		Output power dynamics

12.3.4		Transmit ON/OFF power

12.3.5		Transmitted signal quality

12.3.6		Unwanted emissions

12.3.7		Transmitter intermodulation

12.3.8       In-channel adjacent subband leakage

12. 4     Conducted receiver characteristics for SBFD-capable BS

12.4.1		General

12.4.2		Reference sensitivity level

12.4.3		Dynamic range

12.4.4		In-band selectivity and blocking

12.4.5		Out-of-band blocking

12.4.6		Receiver spurious emissions

12.4.7		Receiver intermodulation

12.4.8		In-channel selectivity

12.4.9       In-channel adjacent subband blocking

12. 5     Conducted performance requirements for SBFD-capable BS

12. 6     Radiated transmitter characteristics for SBFD-capable BS

12.6.1		General

12.6.2		Radiated transmit power

12.6.3		OTA base station output power

12.6.4		OTA output power dynamics

12.6.5		OTA transmit ON/OFF power

12.6.6		OTA transmitted signal quality

12.6.7		OTA unwanted emissions

12.6.8		OTA transmitter intermodulation

12.6.9       OTA in-channel adjacent subband leakage

12. 7     Radiated receiver characteristics for SBFD-capable BS

12.7.1		General

12.7.2		OTA sensitivity

12.7.3		OTA reference sensitivity level

12.7.4		OTA dynamic range

12.7.5		OTA in-band selectivity and blocking

12.7.6		OTA out-of-band blocking

12.7.7		OTA receiver spurious emissions

12.7.8		OTA receiver intermodulation

12.7.9		OTA in-channel selectivity

12.7.10     OTA in-channel adjacent subband blocking

12. 8     Radiated performance requirements for SBFD-capable BS

  - Ericsson (draft CR R4-2504011):

12	Requirements for SBFD-capable BS	2

12.1	General	2

12.2	Conducted transmitter characteristics for SBFD-capable BS	3

12.2.1	General	3

12.2.2	Base station output power for SBFD	3

12.2.3	Output power dynamics for SBFD	3

12.2.4	Transmit ON/OFF power for SBFD	4

12.2.5	Transmitted signal quality for SBFD	4

12.2.6	Unwanted emissions for SBFD	4

12.2.7	Transmitter intermodulation for SBFD	4

12.2.8	Transient period between SBFD and non-SBFD	5

12.2.9	In-channel adjacent subband leakage	5

12.3	Conducted receiver characteristics for SBFD-capable BS	5

12.3.1	General	5

12.3.2	Reference sensitivity level	5

12.3.3	Dynamic range	5

12.3.4	In-band selectivity and blocking	5

12.3.5	Out-of-band blocking	5

12.3.6	Receiver spurious emissions	5

12.3.7	Receiver intermodulation	5

12.3.8	In-channel selectivity	5

12.3.9	In-channel adjacent subband blocking	5

12.4	Radiated transmitter characteristics for SBFD-capable BS	5

12.4.1	General	5

12.4.2	Radiated transmit power	5

12.4.3	OTA base station output power	5

12.4.4	OTA output power dynamics	5

12.4.5	OTA transmit ON/OFF power	5

12.4.5.1	General	5

12.4.5.2	OTA transmitter OFF power	5

12.4.5.3	OTA transient period	5

12.4.6	OTA transmitted signal quality	5

12.4.7	OTA unwanted emissions	6

12.4.8	OTA transmitter intermodulation	6

12.4.9	OTA Transient period between SBFD and non-SBFD	6

12.4.10	OTA in-channel adjacent subband leakage	6

12.5	Radiated receiver characteristics for SBFD-capable BS	6

12.5.1	General	6

12.5.2	OTA sensitivity for SBFD	6

12.5.3	OTA reference sensitivity level	6

12.5.4	OTA dynamic range	6

12.5.5	OTA in-band selectivity and blocking	6

12.5.6	OTA out-of-band blocking	6

12.5.7	OTA receiver spurious emissions	6

12.5.8	OTA receiver intermodulation	6

12.5.9	OTA in-channel selectivity	6

12.5.10	OTA in-channel adjacent subband blocking	6

- Moderator Recommendation:

  - Offline discussion firstly to merge two proposals on the table by collecting more comments from other companies.

### Sub-topic 1-5: Draft CR and work splitting

##### Issue 1-5-1: Draft CR and work splitting

- [Moderator] Ericsson provided a draft CR to 38.104 for introduction of SBFD [R4-2504011]. Based on that, views can be collected for CR drafting and work splitting from companies will be discussed in this meeting.

- Moderator Recommendation:

  - Views can be collected for dCR R4-2504011.

  - Based on the structure decided in this Issue 1-4-1, the work splitting on CR drafting can be determined in this meeting.

### Sub-topic 1-6: CLI handling impact on RF requirement

##### Issue 1-6-1: CLI handling impact on RF requirement

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved. Based on previous meeting’s discussion, there is no agreement to either revisit or confirming no revisit the “FFS” bullet.

<From RAN4#112> - Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Further proposals are provided:

  - Proposal 1 (Nokia): RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Moderator Recommendation:

  - It has been proposed and discussed for several meetings, but not easily to be concluded.

### Sub-topic 1-7: Update for HPUE duty cycle evaluation

##### Issue 1-7-1: Update for HPUE duty cycle evaluation

- Proposal:

  - Proposal 1 (ZTE): propose to consider the SBFD symbols together with the legacy UL symbols for dutycycle evaluation for HPUE cases including HPUE in single band, EN-DC or NR-DC.

    - Some initial update as follows:

- If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class: - - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or - - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - - if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower; - - shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4; - - else if the UE does not support a power class with higher maximum output power than PC2; or - - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or - - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or - - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - - if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the power class 2 or lower; - - shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4; - - else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

- Moderator Recommendation:

  - Discussion on Proposal 1 firstly. It should be noted TS 38.101-1 has not been included in the impacted specification in WID.

# Topic #2: PUSCH with resource muting for CLI handling

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2504043 | Ericsson | Observation 1 MPR remains unaffected by the introduction of UL resource muting. |
| R4-2504091 | Huawei, HiSilicon | Observation: For UL resource muting, there is no transmit quality or MPR requirement impact can be expected for PUSCH as comb-2 pattern is already supported since LTE. |
| R4-2504455 | Apple | Observation 1: In all cases that have been simulated, UL muting has minimal impact on EVM or MPR. |
| R4-2504483 | Qualcomm Incorporated | Observation 1: Resource muting for the studied QPSK waveforms at the left edge of the channel have less than 0.5 dB impact on MPR. Proposal 1: Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with allocations at the edges of the channel have minimal impact on the transmit waveform quality. Observation 2: Resource muting for the studied QPSK waveforms with full allocation has negligible impact on MPR. Proposal 2: Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with full allocations have no impact on the transmit waveform quality. Proposal 3: Resource muting should be studied for the remainder of QPSK allocations along with other modulations to gauge its impact on transmit waveform quality. |
| R4-2504218 | Samsung | Proposal 4: The RAN4 evaluation on the signal quality impact from the new waveform can be concluded, i.e., no new MRP is required for PUSCH with resource muting for CLI handling, if no critical issue further identified. |
| R4-2504368 | ZTE Corporation, Sanechips | Proposal 4: propose to conduct the MPR evaluation for PUSCH with resource muting for CLI solution. |

## Open issues summary

- [Moderator] Triggered by incoming LS from RAN1 [R4-2500006], RAN4 is supposed to check impact on transmit signal quality/MPR.

### Sub-topic 2-1: RF requirement impact for PUSCH with resource muting

##### Issue 2-1-1: RF requirement impact for PUSCH with resource muting

- Following proposal/observations provided:

  - PAPR increasement from new waveform:

    - MPR remains unaffected.

  - Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with allocations at the edges of the channel: SEM limited

    - Qualcomm: less than 0.5 dB impact on MPR

    - Apple: no negative impact observed

    - Huawei: no MPR performance degradation

  - Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with full allocations: ACLR limited

    - Qualcomm: negligible impact on MPR

    - Apple: No impact

- To be evaluated:

  - Potential scope mentioned by Apple: 256QAM

  - Potential scope mentioned by Qualcomm: other modulation than QPSK.

- Moderator Recommendation:

  - Based on existing results, the RAN4 evaluation on the signal quality impact from the new waveform can be concluded:

    - LS to RAN1 can be drafted based on existing results.

    - Evaluation on remaining scope can be continued, and the issues can be raised if found in future.

# Topic #3: SBFD adjacent channel coexistence

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503235 | Charter Communications, Inc | Proposal 1: A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%), and by reducing PCMAX of the SBFD UE power by XdB. |
| R4-2504300 | Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom | Proposal 1: At least for FR1 macro BS it is confirmed the deployment scenario of SBFD-SBFD co-existence in adjacent channels shall be within the scope of Rel-19. |

### Sub-topic 3-1: SBFD-SBFD co-existence in adjacent channel

##### Issue 3-1-1: SBFD-SBFD co-existence scenario within Rel-19 scope or not

- [Moderator] In RAN4#114, companies have different view on whether SBFD-SBFD co-existence scenario is within Rel-19 scope or not.

- Proposal:

  - Proposal 1 (Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom): At least for FR1 macro BS it is confirmed the deployment scenario of SBFD-SBFD co-existence in adjacent channels shall be within the scope of Rel-19.

- Moderator Recommendation:

  - Conclude this issue by reviewing other companies’ views firstly.

### Sub-topic 3-2: SBFD-TDD co-existence in adjacent channel

##### Issue 3-2-1: Fair coexistence between SBFD and TDD networks

- [Moderator] In RAN4#114, it is proposed to consider some mechanism on limiting maximum UE transmit power in SBFD symbols/slots to limit UE-to-UE CLI in the adjacent legacy TDD networks, thus making the transmit power can be capped by different maximum values for SBFD symbols/slots and non-SBFD symbols/slots.

Transmit power=minPCMAX, Ptx,  &non-SBFD symbols/slotsminPCMAX,SBFD, Ptx,  &SBFD symbols/slots

Based on the analysis by Charter, the DUD configuration (40%, 20%, 40%) and reducing Pcmax of SBFD UE by XdB are analyzed.

- Proposal:

  - Proposal 1 (Charter): A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%), and by reducing PCMAX of the SBFD UE power by XdB.

- Moderator Recommendation:

  - Proponent of proposal 1 can further clarify the RAN4 impact expected from this proposal.

# Topic #4: New BS RF requirements for SBFD

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503286 | Tejas Network Limited | Observation 1: The placement of the transient period should be treated as a gNB implementation aspect rather than a strict RAN4 requirement. This decision should consider key factors such as network synchronization, interference management, and scheduling flexibility. Input from RAN1 may be useful to ensure alignment with overall system design. Proposal 1: Recognize the placement of the transmitter transient period as an implementation-specific decision, with no direct impact on RAN4 standards. However, given its influence on network synchronization and guard period design, it may be beneficial to seek input from RAN1 to ensure alignment with overall system considerations. |
| R4-2503404 | CMCC | Proposal 1: Transient period between SBFD and non-SBFD could be set in guard symbol(s), and wait for the conclusion for the location of the guard period from RAN1. Proposal 2: Identify the assumption of adjacent network performance before defining sub-band Tx leakage. Proposal 3: Legacy ACLR requirements could be used as starting point. |
| R4-2503469 | CATT | Observation 1: It can take time to switch from the NR DL full-band to SBFD DL sub-band, from NR UL full-band to SBFD UL sub-band, from SBFD DL sub-band to NR DL full-band and from SBFD UL sub-band to NR UL full-band. Proposal 1: DL sub-band switched to DL full-band and UL sub-band switched to UL full-band and vice versa all need to consider the transient period. Proposal 2: Location of transient period between SBFD and non-SBFD shall be located within the SBFD slot. Proposal 3: The Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104 can be reused for absolute PSD limit Proposal 4: The naming of this requirement can be ASLR absolute limit. Proposal 5: The in-channel adjacent sub-band leakage ratio limit can be 45dB. Proposal 6: The in-channel adjacent sub-band blocking requirement should consider the interference from inter-sectors BSs and inter-site BSs Proposal 7: The in-channel adjacent sub-band blocking requirement should be defined based on feasibility consideration. Proposal 8: In-channel adjacent sub-band blocking requirement is to be specified under an assumed isolation value between two sectors. Proposal 9: The isolation between two sectors can be assumed to be 70dB. |
| R4-2504008 | Ericsson | Observation 1 The interfering signals of In-channel selectivity (ICS) for SBFD come from UE and are placed on the UL subband, while the interfering signal of in-channel adjacent subband blocking comes from other sectors and is placed on the DL subband. Observation 2 In-channel selectivity (ICS) cannot cover in-channel adjacent subband blocking. Observation 3 In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and has not been studied so far. Proposal 1 Reuse the concept of the existing ACLR (absolute PSD value) when specifying SBFD in-channel adjacent sub-band leakage requirement. Proposal 2 Specify -13 dBm/MHz as the requirement level for the SBFD in-channel adjacent sub-band leakage. Proposal 3 Define in-channel adjacent subband blocking requirement without considering CLI handling schemes. Proposal 4 For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors. Proposal 5 The location of transient period between SBFD and non-SBFD symbols are as follows: Case-A: as part of regular NTA_offset in the UL part(s) of SBFD symbol(s). Case-B: in guard symbol(s) reserved in the DL part(s) of SBFD symbol(s). Case-C: in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission. Case-D: as part of regular NTA_offset in non-SBFD UL symbol(s). |
| R4-2504092 | Huawei, HiSilicon | Observation 1: Assuming 45dB ACLR and 53dBm BS output power over 100MHz, roughly -12dBm/Hz level residual noise level in the adjacent channel can be expected. However, it will get higher for U6G since the ACLR requirement is relaxed to 38dB. Observation 2: Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques. Observation 3: How to accommodate the CLI handling capability with the in-channel adjacent subband interference should be further discussed. Proposal 1: Suggest as follows in order to specify in-channel adjacent subband ACLR requirements covering both DU and DUD SBFD configurations. • The ACLR definition can be reused for the DU case, while the CACLR definition can be reused for the DUD case. • The existing absolute limit values cannot be reused w/o any relaxation, otherwise RAN4 can consider to have both relative and absolute limit and adopt whichever less stringent as the applied requirements. |
| R4-2504194 | Nokia | Proposal 1: On the transmitter transient period for SBFD, adopt the text proposal from the way forward. Proposal 2: Use the existing ACLR absolute limits as the new in-channel adjacent subband leakage ratio requirements. Proposal 3: Change the name of the in-channel adjacent subband leakage ratio to in-channel adjacent subband leakage. Proposal 4: Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes. Proposal 5: Reasonable expected gNB-to-gNB isolation should be the worst-case scenario to ensure functionality in the field. |
| R4-2504219 | Samsung | Transient period requirement for SBFD Proposal 1: For the location of transient periods Case-B, it is in guard symbol(s) reserved before UL non-SBFD symbols, and the guard symbol(s) in this case should be reserved in SBFD symbol(s). Proposal 2: For the location of transient periods Case-C, both (1) guard symbols reserved in non-SBFD DL symbol(s) and (2) blanked SBFD symbols in the beginning of SBFD UL transmission shall be allowed, depending on NW configuration. Proposal 3: In RAN4 requirement, no need to specify the detailed location of the transmitter transient period (i.e., within the SBFD symbol or not), because in practice the length and location of TA_offset and guard symbols are provided by gNB configuration. Proposal 4: After clarifying the remaining issues for transient period, RAN4 shall send LS to RAN1 to give the information about RAN4 agreements, and the drafted LS is given in the Appendix. In-channel adjacent subband requirements Proposal 5: Define the requirement as “in-channel adjacent subband leakage requirement” by removing “ratio”. Proposal 6: Take the below absolute basic limit values for in-channel adjacent subband leakage requirement: BS category / BS class; ACLR absolute basic limit. Category A Wide Area BS; -8 dBm/MHz. Category B Wide Area BS; -10 dBm/MHz. Medium Range BS; -20 dBm/MHz. Local Area BS; -27 dBm/MHz. Proposal 7: For in-channel adjacent subband blocking (ICASB), the requirement shall be defined similar to BS in-band blocking requirement. Proposal 8: For FR1 WA BS, -35 dBm is adopted as the requirement level for the SBFD in-channel adjacent subband blocking. |
| R4-2504341 | Qualcomm Germany | Proposal 1: RAN4 to not define the location of the transient period between SBFD and non-SBFD symbol in the core specification. Proposal 2: In case RAN4 agreed to define the location of the transient period, RAN4 to agree to define the guard symbols in the blanked SBFD symbol in the beginning of the SBFD UL transmission for case C and at the end of the SBFD transmission for case B. Observation 1: The in-channel subband sub-band Tx leakage and blocking depend on the self-interference, inter-site and inter-sector interference considerations; thus, they are implementation dependent. Proposal 3: RAN4 to agree on reusing ACLR absolute basic limit given in Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2-1, for the new in-channel adjacent subband leakage. Proposal 4: RAN4 consider the impact of inter-CLI handling schemes when analysing feasible in-channel adjacent subband blocking requirement. |
| R4-2504369 | ZTE Corporation, Sanechips | Proposal 1: propose to reuse the absolute CACLR limit as staring point for SBFD in-band carrier leakage requirement and further discuss how to address the following two cases: 1) if the UL sub-band size is too narrow for the DL sub-band ACLR1 measurement compared with the legacy CACLR requirement defined in TS38.104 2) absolute ACLR limit is too stringent compare with relative ACLR requirement. Proposal 2a: if any further evaluation for in-channel blocking requirement are needed, then some beam coordination among different BSs should be considered otherwise we failed to the CLI coordination information exchange defined in RAN3. Proposal 2b: if in-channel blocking requirement is higher than legacy in-band blocking requirement, propose to check the antenna gain of each interference link between aggressor and victim BS and also check the in-channel blocking alleviation if beam coordination among BSs are considered. Proposal 3: for the feasibility of in-channel blocking requirement, analog filter could be assumed for FR1 WA SBFD and no analog filter could be assumed for FR1 MR, LA SBFD and FR2 SBFD BS. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1: Requirements for Transient period between SBFD and non-SBFD

- [Moderator] In RAN4#114, the following WF is achieved for “Location of transient period between SBFD and non-SBFD”, while companies provide vies on the following FFS points in this meeting:

Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below:

##### Issue 4-1-1: Location of transient period for Case-B (from SBFD symbol to UL symbol)

- [Moderator] As agreed in last meeting, the location of transient periods shall be in guard symbol(s) reserved before UL non-SBFD symbols, but still “FFS guard symbol(s) in this case is SBFD symbol or not”.

- Proposals:

  - Option 1 (Ericsson, Samsung, CATT, Qualcomm, ZTE): guard symbol(s) should be reserved in SBFD symbol(s)

    - Option 1a (Ericsson): in guard symbol(s) reserved in the DL part(s) of SBFD symbol(s)

  - Option 2 (CMCC): wait for RAN1 conclusion.

- Moderator Recommendation:

  - Agree on Option 1 and check Option 1a can be further agreed.

##### Issue 4-1-2: Location of transient period for Case-C (from DL symbol to SBFD symbol)

- [Moderator] As WF achieved in last meeting, FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission.

- Proposals:

  - Option 1: guard symbols reserved in non-SBFD DL symbol(s)

  - Option 2 (CATT, Qualcomm): blanked SBFD symbols in the beginning of SBFD UL transmission shall be allowed

  - Option 3 (Samsung, ZTE, Ericsson): both Option 1 and Option 2 shall be allowed, depending on NW configuration.

    - Option 3a (ZTE): Should be clear in the conformance testing specification for testing

  - Option 4 (CMCC): wait for RAN1 conclusion.

- Moderator Recommendation:

  - After clarifying the location of transient period is not needed to be specified in core requirement, adopt Option 3 as compromised solution.

##### Issue 4-1-3: RAN4 requirement impact

- [Moderator] After clarifying the location of transient period between SBFD and non-SBFD symbols, how this impact RAN4 requirement needs to be discussed.

- Proposals:

  - Option 1 (Qualcomm, Samsung, Tejas Networks): implementation dependent and no need to be specified in core requirement:

    - Proposal 1 (Samsung): No need to specify the detailed location of the transmitter transient period (i.e., within the SBFD symbol or not), because in practice the length and location of TA_offset and guard symbols are provided by gNB configuration.

    - Proposal 2 (Tejas Networks): Recognize the placement of the transmitter transient period as an implementation-specific decision, with no direct impact on RAN4 standards. However, given its influence on network synchronization and guard period design, it may be beneficial to seek input from RAN1 to ensure alignment with overall system considerations.

- Moderator Recommendation:

  - Option 1 is concluded.

##### Issue 4-1-4: Text proposal for transient period requirement applicability on SBFD subbands

- [Moderator] WF below is agreed in last meeting:

Way Forward: - Text proposal below is taken as the baseline for further discussion, with the understanding that the content can be further updated based on the future agreement on the location of transient period between SBFD and non-SBFD: =================== Start of Text Proposal =================== 6.4.2B Transmitter transient period for SBFD 6.4.2.1B General Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS. The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1. The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases: Table 6.4.2.1B-1: Transmitter transient period for SBFD Frequency regions for transition; BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; BS transmitter OFF-to-ON. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. =================== End of Text Proposal ===================

- Proposals:

  - Option 1 (Nokia): adopt TP in the way forward

- Moderator Recommendation:

  - TP in the way forward (R4-2502310) can be endorsed.

### Sub-topic 4-2: LS to RAN1 on RAN4 agreement of transient period

##### Issue 4-2-1: LS to RAN1 on RAN4 agreement of transient period

- Drafted LS to RAN1 (Samsung):

## 1 Overall description On RAN4 requirements for SBFD transient period at gNB, RAN4 has made the following agreements Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations. Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases: Frequency regions for transition; BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; BS transmitter OFF-to-ON.. Particularly for the location of transient period, RAN4 has identified four cases of transitions to/from SBFD symbols as illustrated in Figure 1: Figure 1: Illustration of four cases of transitions to/from SBFD symbols And the following agreement is achieved on the location of transient period for the above four cases: Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s).. NOTE: further update the above highlighted content after further agreement is achieved in RAN4#114bis. RAN4 respectfully asks RAN1 take the above RAN4 agreement into account, and provide further information on the location of transient period if any. ## 2 Actions To RAN1 ACTION: RAN4 respectfully asks RAN1 take the above RAN4 agreement into account, and provide further information on the location of transient period if any.

- Moderator Recommendation:

  - After the issue of location of transient period is concluded, RAN4 can refine the content of LS.

### Sub-topic 4-3: In-channel adjacent subband leakage

- [Moderator] In RAN4#113, RAN4 agree to introduce new requirement for in-channel adjacent subband leakage ratio, and in RAN4#114, the following agreement is achieved:

Agreement: - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit.

##### Issue 4-3-1: Requirement naming

- Proposals on requirement naming:

  - Option 1 (Samsung, Nokia): in-channel adjacent subband leakage requirement (by removing “ratio”).

  - Option 2 (CATT): ASLR absolute limit

- Moderator Recommendation:

  - Proposal 1 is adopted to align with the absolute PSD limit used instead of the ratio.

##### Issue 4-3-2: Assumption for in-channel adjacent subband leakage requirement

- Proposals on requirement naming:

  - Proposal 1 (CMCC): Identify the assumption of adjacent network performance before defining sub-band Tx leakage.

- Moderator Recommendation:

  - Need to discuss the assumption further.

##### Issue 4-3-3: Absolute limit for in-channel adjacent subband leakage

- Absolute limit for in-channel adjacent subband leakage

  - Option 1 (Ericsson, Nokia, CMCC, Qualcomm, CATT): legacy ACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104)

  - Option 2 (Samsung): 5dB relaxation on ACLR absolute basic limit

  - Option 3 (ZTE): Reuse the absolute CACLR limit as staring point for SBFD in-band carrier leakage requirement and further discuss how to address the following two cases:

    - if the UL sub-band size is too narrow for the DL sub-band ACLR1 measurement compared with the legacy CACLR requirement defined in TS38.104

    - absolute ACLR limit is too stringent compare with relative ACLR requirement.

  - Option 4 (Huawei): Suggest as follows in order to specify in-channel adjacent subband ACLR requirements covering both DU and DUD SBFD configurations.

    - The ACLR definition can be reused for the DU case, while the CACLR definition can be reused for the DUD case.

    - The existing absolute limit values cannot be reused w/o any relaxation, otherwise RAN4 can consider to have both relative and absolute limit and adopt whichever less stringent as the applied requirements.

- Moderator Recommendation:

  - Options are discussed further.

##### Issue 4-3-4: Necessity of in-channel adjacent subband leakage ratio

- The necessity of in-channel adjacent subband leakage ratio:

  - Option 1 (CATT): can be 45dB

- Moderator Recommendation:

  - No need to further discuss the necessity of in-channel adjacent subband leakage ratio, based on agreement from RAN4#114.

### Sub-topic 4-4: In-channel adjacent subband blocking

- [Moderator] In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband blocking requirement, while no need to introduce new requirement for in-channel adjacent subband selectivity, and no further agreement achieved in RAN4#114:

Agreement from RAN4#113: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration.

##### Issue 4-4-1: In-channel adjacent subband blocking requirement

- Proposals on the level of in-channel adjacent subband blocking requirement:

  - For FR1 WA BS:

    - Option 1 (Samsung): -35 dBm.

    - Option 2 (Ericsson): -27 dBm.

- Moderator Recommendation:

  - Discuss on the above proposals/options firstly.

##### Issue 4-4-2: Assumption of gNB-to-gNB CLI handling

- Proposals/Observations:

  - Proposal 1 (Nokia): Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes.

  - Proposal 2 (Nokia): Reasonable expected gNB-to-gNB isolation should be the worst-case scenario to ensure functionality in the field.

  - Proposal 3 (Qualcomm): RAN4 consider the impact of inter-CLI handling schemes when analysing feasible in-channel adjacent subband blocking requirement.

  - Proposal 4a (ZTE): if any further evaluation for in-channel blocking requirement are needed, then some beam coordination among different BSs should be considered otherwise we failed to the CLI coordination information exchange defined in RAN3.

  - Proposal 4b (ZTE): if in-channel blocking requirement is higher than legacy in-band blocking requirement, propose to check the antenna gain of each interference link between aggressor and victim BS and also check the in-channel blocking alleviation if beam coordination among BSs are considered. [More simulation results to be provided later on]

  - Proposal 5 (ZTE): for the feasibility of in-channel blocking requirement, analog filter could be assumed for FR1 WA SBFD and no analog filter could be assumed for FR1 MR, LA SBFD and FR2 SBFD BS.

  - Observation 1 (Huawei): Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

  - Observation 2 (Huawei): How to accommodate the CLI handling capability with the in-channel adjacent subband interference should be further discussed.

  - Observation 3 (Ericsson): In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and has not been studied so far.

  - Proposal 6 (Ericsson): Define in-channel adjacent subband blocking requirement without considering CLI handling schemes.

  - Proposal 7 (CATT): In-channel adjacent sub-band blocking requirement is to be specified under an assumed isolation value between two sectors, e.g., 70dB.

- Moderator Recommendation:

  - Discuss on the above proposals/options firstly.