3GPP TSG-RAN WG4 Meeting #114bis				                                                                                                                    R4-2504687

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.24.5

Source:	Moderator (CMCC)

Title:	Topic summary for [114bis][129] A-IoT_device

Document for:	Information

Introduction

This summary focuses on the R19 ambient IOT work item under agenda 7.24.1, 7.24.2.2, 7.24.4, including general, device and OTA requirements.  The summary document in RAN4#114 meeting is R4-2500686. The way forward agreed in RAN4#114 is R4-2502858.

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| R4-2503216 | A-IoT general aspects | Huawei, HiSilicon | 7.24.1 |
| R4-2503314 | Discussion on AIoT system parameters | Xiaomi | 7.24.1 |
| R4-2503392 | Discussion on A-IoT general requirements | CMCC | 7.24.1 |
| R4-2503472 | Discussion on system parameters for A-IoT | CATT | 7.24.1 |
| R4-2503673 | Discussion on system parameters for ambient IoT | Spreadtrum,UNISOC | 7.24.1 |
| R4-2503813 | Discussion on the systeme parameter of AIoT | vivo | 7.24.1 |
| R4-2503932 | (new) TS 38.191 Ambient IoT device radio transmission and reception | CMCC | 7.24.1 |
| R4-2504096 | 3GPP_TS 38.194 skeleton | Huawei, HiSilicon | 7.24.1 |
| R4-2504207 | on system parameter | OPPO | 7.24.1 |
| R4-2504352 | Discussions on General aspect for A-IoT | ZTE Corporation, Sanechips | 7.24.1 |
| R4-2504399 | A-IoT general overview | Ericsson | 7.24.1 |
| R4-2503316 | Discussion on AIoT device 1 RF requirement | Xiaomi | 7.24.2.2 |
| R4-2503391 | Discussion on A-IoT device RF requirements | CMCC | 7.24.2.2 |
| R4-2503438 | Discussion on Ambient IoT device 1 RF requirements | CATT | 7.24.2.2 |
| R4-2503559 | Ambient IoT device backscattering and SFO error | Qualcomm Incorporated | 7.24.2.2 |
| R4-2503671 | Discussion on RF requirements for ambient IoT device 1 | Spreadtrum,UNISOC | 7.24.2.2 |
| R4-2503815 | Discussion on the RF requirement for device 1 | vivo | 7.24.2.2 |
| R4-2504063 | RF requirements of ambient IoT device 1 | Sony | 7.24.2.2 |
| R4-2504206 | on RF requirements for device 1 | OPPO | 7.24.2.2 |
| R4-2504354 | Discussion on RF requirement of Ambient IoT device | ZTE Corporation, Sanechips | 7.24.2.2 |
| R4-2504400 | A-IoT device requirement overview | Ericsson | 7.24.2.2 |
| R4-2504516 | Discussion on RF requirement for A-IoT device 1 | LG Electronics UK | 7.24.2.2 |
| R4-2504557 | On the RF requirements for Ambient IoT Device | Huawei, HiSilicon | 7.24.2.2 |
| R4-2503138 | Discussion on OTA testing for A-IoT device 1 | Ericsson-LG Co., LTD | 7.24.4 |
| R4-2503393 | Discussion on A-IoT OTA requirements | CMCC | 7.24.4 |
| R4-2503494 | on OTA tests for ambient IoT devices | Huawei, HiSilicon | 7.24.4 |
| R4-2503817 | Discussion on the OTA test method for device | vivo | 7.24.4 |
| R4-2504056 | Views on Ambient IoT Testability | Qualcomm Incorporated | 7.24.4 |
| R4-2504064 | Consideration on the OTA test of ambient IoT device 1 | Sony | 7.24.4 |
| R4-2504151 | Discussion on OTA test method for A-IoT device | CAICT | 7.24.4 |
| R4-2504204 | on OTA test aspect | OPPO | 7.24.4 |

New TS Skeleton

R4-2503932  (new) TS 38.191 Ambient IoT device radio transmission and reception	CMCC

R4-2504096             3GPP_TS 38.194 skeleton	Huawei, HiSilicon

Proposal 1 (vivo):

- Only the unique test setup for AIoT is captured in the new TS, and the duplicate content as TR38.870 is directly referenced in the corresponding Clause of the TR..

- The outcome of detailed test method in this WI is capture in TR 38.870, and two new clause is added:

  - Clause 10: testing methodologies for A-IoT

  - B.6 MU assessment for A-IoT

Recommended WF:

Take the TS skeleton for device in R4-2503932 as a starting point and further discuss the details.

Take the TS skeleton for BS and CW in R4-2504096 as a starting point and further discuss the details.

System parameters

- Topic 2-1: R2D bandwidth

Issue 2-1-1: R2D transmission bandwidth

Agreement in RAN4#114: - Reuse the following definition of R2D transmission bandwidth in TR38.769 - Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D. - RAN4 will re-visit the two agreed bullets above, if RAN1 has the different conclusion. - Define R2D transmission bandwidth configuration: - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS - FFS on other RB numbers - Further discuss the R2D channel bandwidth considering additional guard band. - If RAN1 has the agreement different than above, RAN4 will revisit the agreements

Proposal 1 (Huawei): RAN4 specify maximum Btx,R2D # of PRBs. Set the Maximum Btx,R2D as 4 PRBs.

Proposal 2 (Xiaomi): RAN4 only define the RF requirements for minimum Btx,R2D # of PRBs associated to the M values, no maximum Btx,R2D # of PRBs are defined in RAN4 spec.

Proposal 3 (CMCC): For R2D transmission bandwidth, limit the max transmission bandwidth to 4PRB

Proposal 4 (CATT): RAN4 to specify A-IoT requirements based on continuous RB allocation in Rel-19.

Proposal 5 (Spreadtrum):

- It is needed to specify the maximum Btx,R2D# of PRBs.

- Define R2D transmission bandwidth configuration: 1RB, 2RB, 3RB, 4RB, 8RB and 16RB with 15 kHz SCS.

Proposal 6 (OPPO): To introduce 12 PRBs as the largest CBW and the number of PRBS will be 1,2,3…12.

Proposal 7 (Sony): RAN4 waits for RAN1 to determine the minimum RB number for each M value and if any additional RB numbers for D2R transmission BW will be considered.

Recommended WF:

In last RAN4 meeting, it was agreed to define R2D transmission bandwidth configuration as 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS.

Discuss whether to define larger number of PRB as maximum transmission bandwidth configuration, for example 8PRB, 12PRB, 16PRB with 15KHz SCS.

Issue 2-1-2: R2D channel bandwidth

Agreement in RAN4#114: - Define R2D channel bandwidth as: - Channel bandwidth, BCH,R2D from a reader perspective: The frequency resources used for transmitting R2D, and guard band. - Define R2D transmission bandwidth configuration: - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS - FFS on other RB numbers - Further discuss the R2D channel bandwidth considering additional guard band. - If RAN1 has the agreement different than above, RAN4 will revisit the agreements

Proposal 1 (Huawei)：With 90% spectrum utilization, define R2D CBW as 200kHz，400kHz，600kHz and 800kHz；

- For 200kHz R2D CBW, it includes 180kHz transmission BW and 10kHz guard band on each side.

- For 400kHz R2D CBW, it includes 360kHz transmission BW and 20kHz guard band on each side.

- For 600kHz R2D CBW, it includes 540kHz transmission BW and 30kHz guard band on each side.

- For 800kHz R2D CBW, it includes 720kHz transmission BW and 40kHz guard band on each side.

Proposal 2 (CMCC): for R2D, consider 200kHz, 400kHz, 600kHz, 800kHz CBW with 90% spectrum efficiency assumption for 1PRB, 2PRB, 3PRB, 4PRB.

Proposal 3 (CATT): The guardband adjacent to the low channel edge should be symmetric to the guardband adjacent to the high channel edge.

The channel bandwidth, transmission bandwidth configuration, and guardband for different number of RBs are:

| Transmission bandwidth configuration NRB | 1RB | 2RB | 3RB | 4RB |
| --- | --- | --- | --- | --- |
| Channel bandwidth (kHz) | 200 | 400 | 600 | 800 |
| Transmission bandwidth configuration Ntone 15kHz | 12 | 24 | 36 | 48 |
| Guardband (kHz) | 10 | 20 | 30 | 40 |

Proposal 4 (Spreadtrum): Channel bandwidth for R2D needs to consider transmission bandwidth and guard band for SFO. There is no need to consider guard band for co-existence study for R2D.

Proposal 5 (vivo): Consider same spectrum utilization of NB-IoT is applied to R2D, i.e., the channel bandwidth for 1/2/3/4RB could be 200/400/600/800 kHz.

Proposal 6 (OPPO):

- Use 200kHz as channel bandwidth for 1 PRB D2R channel bandwidth.

- To introduce 12 PRBs as the largest CBW and the number of PRBS will be 1,2,3…12.

Proposal 7 (ZTE): propose to define the minimum carrier bandwidth for R2D transmission as following with 10kHz, 20kHz, 30kHz, 40kHz GB on each side of R2D transmission reserved for 1/2/3/4 PRB transmission.

Proposal 8 (Ericsson): Use 10% as ratio between guard band and channel bandwidth to design channel bandwidth as starting point

Table 1: Channel bandwidth for A-IoT

| R2D transmission bandwidth | Guard band | Channel bandwidth |
| --- | --- | --- |
| 1 RB (180kHz) | 20 kHz | 200kHz |
| 2 RB (360kHz) | 40 kHz | 400kHz |
| 3 RB (540kHz) | 60kHz | 600kHz |
| 4 RB (720kHz) | 80kHz | 800kHz |

Recommended WF:

Define R2D channel bandwidth with assumption of 90% spectrum utilization.

- Topic 2-2: D2R bandwidth

Issue 2-2-1: D2R bandwidth

Agreement: - D2R channel bandwidth for A-IoT device - Define D2R channel bandwidth for A-IoT device considering the following aspects: - [1SB or] 2SB - D2R maximum data rate (depending on RAN1 conclusion) - SFO - FDMA, i.e., considering the different device 1 has different frequency shift - Reference Filter performance - Other aspects are not precluded

Proposal 1 (Huawei):

- Consider FDMA and SFO (assumed as 10^5ppm) effect, D2R CBW can be calculated as follows:

- D2R Channel BW = (Frequency shift * 2*1.1 + (2SB transmission bandwidth *1.1)/2) /0.9

- The 1.1 factor accounts for 10% SFO effect.

- The 0.9 divisor presents the 90% filter spectrum utility (10% guardband).

- The potential channel BW become too many when accounting for combinations of transmission bandwidth, chip length, and FDMA frequency shift.

- Minimize the kinds of D2R channel bandwidth.

Proposal 2 (Xiaomi):

- D2R bandwidth is defined according to DSB signal not SSB signal.

- RAN4 begins the CBW discussion with one specific chip rate assumption, for example 7.5kcps, for the general methodology. The final CBW decision can be updated according to RAN1 decision.

- Nominal channel bandwidth is defined without consideration of SFO. The RF requirements can be defined considering SFO impact.

- FDMA is not considered for D2R CBW from device side.

- RAN4 should decide if D2R CBW from reader side needs be defined to consider FDMA.

Proposal 3 (CMCC):

- there is no need for D2R CBW requirements unless strong motivation is identified for CBW definition.

- For D2R transmission bandwidth, define D2R transmission bandwidth of 1SB, and define SEM related requirements from the edge of some typical transmission bandwidth which is an equation. This definition applies for both small frequency shift and FDMA cases. For example:

  - D2R transmission bandwidth of 1SB=baseband chip rate

Proposal 4 (CATT):

- We can decide the channel bandwidth of D2R after RAN1 has conclusions on D2R maximum data rate and SFO.

- It is recommended that the DL and UL RB numbers of the device are symmetrical.

Proposal 5 (Spreadtrum):

- Define D2R channel bandwidth for A-IoT device considering the following aspects:

- 2SB

- D2R maximum data rate (depending on RAN1’s conclusion)

- SFO

- 3rd harmonics interference

- SFO for guard band for D2R is equal to 2*(1/10)*transmission bandwidth plus 2*(1/10)* the largest small frequency shift.

Proposal 6 (vivo): Confirm that the D2R channel bandwidth is defined based on 2SB transmission

Proposal 7 (OPPO): Define 15kHz as D2R channel bandwidth. Wait for RAN1 to define the frequency shift and it can be defined as channel raster for D2R.

Proposal 8 (ZTE):

- propose RAN4 to discuss how to use these above factors to be decide the D2R bandwidth e.g. any evaluation criteria.

- propose not to reuse the PRB configuration or sub-carrier spacing related concept for D2R transmission.

Proposal 9 (Ericsson):

- Use the occupied bandwidth requirement to quantify the channel bandwidth considering the data rate and other imperfection.

- Align the simulation assumption for D2R channel bandwidth simulation.

Proposal 10 (Sony): The bandwidth of D2R needs to cover both lower SB and upper SB and also consider a guardband to acoomandate the frequency error from the tag.

Recommended WF:

Define D2R channel bandwidth for A-IoT device considering the following aspects:

- Based on 2SB transmission

- Assume 90% filter spectrum utility (10% guardband)

- Further discuss whether to consider SFO impact

- Further discuss whether to consider FDMA (i.e. frequency shift)

- D2R data rate: discuss whether to define one specific chip rate assumption, for example 7.5kcps, for the general methodology. The final CBW decision can be updated according to RAN1 decision.

- Discuss whether and how to minimize the kinds of D2R channel bandwidth

- Topic 2-3: Channel raster

Issue 2-3-1: R2D channel raster

Agreement in RAN4#114: - Do not define channel spacing for R2D transmission - Define channel raster for R2D transmission. - Details need further discussion.

Proposal 1 (Huawei):

- R2D Channel raster from A-IoT BS perspective is 10 kHz.

- R2D channel raster offset can be -7.5kHz or +2.5kHz.

Proposal 2 (CMCC): Define channel raster as 15KHz for reader.

Proposal 3 (CATT): The channel raster of R2D should be 10kHz.

Proposal 4 (Spreadtrum): R2D channel raster can reuse legacy NR channel raster.

Proposal 5 (vivo): The enhanced channel raster is used for R2D, i.e., 10kHz.

Proposal 6 (OPPO): To reuse the NR channel raster for R2D.

Proposal 7 (ZTE): for R2D transmission in Standalone operation mode, to define the channel raster as 100KHz; for R2D transmission in in-band operation mode, to define the channel raster as 100KHz*N+delta_offset where delta_offset could be as [-47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5]KHz.

Proposal 8 (Ericsson):

- For standalone operation, the legacy NR channel raster should be followed

- For in-band operation, additional A-IoT channel raster should be designed, considering the orthogonality with macro NR BS.

Recommended WF:

For standalone spectrum operation, define 10KHz as R2D channel raster (i.e. same as enhanced channel raster)

For in-band spectrum operation, discuss whether additional channel raster is needed.

Discuss whether an offset for raster is needed:

- R2D channel raster offset can be -7.5kHz or +2.5kHz.

- Topic 2-4: Others

Issue 2-4-1: Operating band

Agreement in RAN4#114: - NR band n8 can be used as an example band, and other FDD bands below 1GHz are not precluded.

Proposal 1 (ZTE): propose RAN4 to discuss the supported frequency range at device side to further determine the supported NR bands for A-IoT system e.g. band n5.

Recommended WF:

In last RAN4 meeting, it was already agreed other FDD bands below 1GHz are not precluded. No discussion is needed.

Issue 2-4-2: Deployment scenario

Agreement in RAN4#114: - To specify RAN4 RF requirements based on the assumption that NR UE is only outdoor in Rel-19. - FFS on whether and how to capture the side condition of outdoor UE assumption in the specifications

Proposal 1 (Xiaomi): No need to capture the NR UE outdoor assumption in RAN4 TS specifications.

Proposal 2 (CATT): RAN4 can focus on specifying requirements for the scenario where all NR UEs are out door without capturing side conditions of outdoor UE assumption.

Recommended WF:

Do not capture the side condition of outdoor UE assumption in the specifications

Issue 2-4-3: Foffset for Ambient IoT

Proposal (Huawei): Apply the Foffset for Ambient IoT as Table 6:

Table 6: Foffset for Ambient IoT

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Ambient IoT for R2D CBW 200/400/600/800kHz | 100 kHz |
| Note：Foffset is start from A-IoT Channel Bandwidth edge |  |

As described in TS36.104, for NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset. A-IoT with one PRB transmission bandwidth is with similar narrow bandwidth, it can be applied to A-IoT. In section 2.1, the ACLR for OOK-4 OFDM signal (both pre- and post-filter, excluding PA degradation) is significantly worse than NB-IoT. This necessitates frequency offset for A-IoT operation. Figure 4 Diagram of A-IoT ACLR Foffset

Recommended WF:

TBA

Device RF requirements

- Topic 3-1: Transmit output power

Issue 3-1-1: Transmit output power

Agreement in RAN4#114: - Measurement configuration including input CW power and potential other conditions need to be defined.

Proposal 1 (Xiaomi):

- Device 1 output power is defined as EIPR not TRP.

- Peak EIRP can be considered as the requirement for device 1. If more than one peak EIRP directions are defined can be discussed further.

- The maximum input CW power to calculate device 1 radiated power can be assumed as 3 dBm.

- Radiated output power uses BPSK modulation.

Proposal 2 (CMCC): from coverage point of view, suggest to define max backscattering power under the max input power from CW and all devices should achieve such max power.

Proposal 3 (CATT):

- The terminology of Device 1 Tx output power requirements could be defined as Radiated power.

- Device 1 radiated power requirement should be defined as the maximum backscattered power of Device 1 at the maximum CW Tx power and at a designated distance.

Proposal 4 (Qualcomm):

- Use -7 dB as antenna efficiency for Ambient IoT device 1 in normative work

- Ambient IoT device 1 matched load reflection coefficient is -8 dB

- Ambient IoT device 1 unmatched load reflection coefficient is -1 dB

- Ambient IoT device 1 backscattering loss used to define normative requirements is 18.5 dB.

- Define a range of CW input power for Ambient IoT transmit output power requirement

Proposal 5 (Spreadtrum):

- The maximum target distance for D2R between device and reader needs to be considered (i.e., 10m).

- The maximum backscatter modulated power needs to be defined. The distance between CW and device needs to be defined. -34dBm for the minimum backscatter modulated power could be a starting point.

- There is no need to define backscattering loss.

Proposal 6 (vivo):

- Only one incoming CW power level configuration at device antenna is enough to verify the backscatter loss, and 0 dBm can be considered for simplicity.

- Use min peak EIRP to define the radiated power of device and the -6 dBm under 0 dBm CW power can be considered.

- Whether the requirement for antenna patten is necessary or not (e.g., orientation tolerance, spherical coverage) needs more discussion.

Proposal 7 (Sony):

- To facilitate the work in OTA test setup, it is proposed that RAN4 shall first focus on determining the corresponding performance metric (e.g., TRP/TIS or EIRP/EIS) and test condition (free space or installed on a platform).

- To determine the backscattering loss of the tag, the RCS of the tag and the modulation scheme needs to be taken into account.

Proposal 8 (OPPO):

- The reflection efficiency and the exact reflected power can be tested in one OTA test setup.

- A unified requirement can be defined.

- To define the output power of device as 22dBm E.R.P over 15kHz channel bandwidth.

Proposal 9 (ZTE): propose to discuss the maximum input power for CW signal and backscattering gain for device.

Proposal 10 (Ericsson):

- Specify the modulation efficiency as possible backscatter requirement

- Specify the spherical coverage requirement for min modulation efficiency associated with Xth percentile.

- Further discuss whether the requirement should be differentiated when device is targeted to be mounted to metal surfaceVery nic

- Further discussion whether the device should be declared to be mounted on a metallic surface.

Proposal 11 (LGE): Transmitted output power of A-IoT Device 1 can be defined by one of the following three formulas:

- Option1 Backscattering power

- Backscattering power= CW Received power-Ploss_iot_device +Antenna Gain

- Option2 Backscattering loss

- Ploss_iot_device =2*Ploss backscattering modulator

- Option3 Backscattering efficiency

- Backscattering efficiency=Refelction powerInput power×100(%)

Proposal 12 (Huawei): Consider to evaluate the backscatter loss at given CW incident power. The limit on the backscatter loss and the corresponding incident power level are TBD.

Recommended WF:

Performance metric of transmit output power:

- minimum peak EIRP

- FFS on spherical coverage associated with Xth percentile

Transmit output power is defined as the maximum backscattered power of Device 1 at certain input CW Tx power

Backscattering power= Input CW Tx power-Backscattering loss

- For the Input CW Tx power to define requirements:

  - FFS on the whether to use one value or a range of CW Tx power

  - FFS on the value(s) of CW Tx power

- Consider following aspects to determine backscattering loss to define requirements:

  - modulation loss

  - antenna efficiency

  - backscattering circuitry loss

  - impairment margin

- Topic 3-2: Modulation quality

Issue 3-2-1: SFO requirement

Proposal 1 (Qualcomm): RAN4 to discuss feasible method candidates how to specify device SFO requirement

Proposal 2 (Spreadtrum): There is no need to define the modulation quality requirements and small frequency shift error. SFO needs to be defined (i.e., 105ppm).

Proposal 3 (Sony): The frequency error requirement will need to be specified to support FDMA scheme.

Proposal 4 (Ericsson): Specify the occupied bandwidth test to test the SFO imperfection.

Recommended WF:

Define SFO requirement as [10%].

- If RAN1 has agreement on max SFO different than 10%, RAN4 will revisit the agreements.

Further discuss how to test the SFO requirement.

Issue 3-2-2: Modulation quality requirements

Agreement in RAN4#114: - Further discuss whether and how to define the modulation signal quality requirements

Proposal 1 (Xiaomi):

- EVM requirement is defined for Device 1.

- Device 1 Tx signal SNR is defined [10] dB higher than receiver DEMOD SNR threshold.

Proposal 2 (CMCC):

- for BPSK modulation, use legacy UE EVM requirement as starting point.

- for OOK signal, further check whether can reuse legacy RFID plusewidth and modulation depth requirements.

Proposal 3 (CATT): For modulation quality, a less stringent EVM requirements than BPSK EVM requirement could be defined for Device 1 (only applies for ON chip).

Proposal 4 (Spreadtrum): There is no need to define the modulation quality requirements and small frequency shift error. SFO needs to be defined (i.e., 105ppm).

Proposal 5 (Sony): For BPSK, the EVM requirement can be defined, while for OOK, Some requirements to ensure the modulation quality of the backscattered signal need to be studied and specified.

Proposal 6 (ZTE): For A-IoT device EVM requirement, we propose to define the EVM requirement similar as RFID with OOK signal considered. For BPSK based D2R design, the legacy EVM requirement need also to be revisited.

Proposal 7 (Ericsson): BLER can be tested for the backscattered modulation quality as a directional requirement.

Proposal 8 (Huawei): When evaluating potential modulation quality metrics, the reference demodulation process should be confirmed. The effect of the quality metric on the demodulation performance should be quantified.

Recommended WF:

Consider the following proposal as starting point:

- For BPSK modulation, define EVM requirements, FFS on whether to reuse legacy UE EVM requirement.

- For OOK signal, consider reusing RF ID requirements, i.e. RFID plusewidth and modulation depth

- Topic 3-3: Emission requirements

Issue 3-3-1: Emission requirements

Agreement in RAN4#114: - Consider one composite emission requirements covering the emission frequency range of SEM and spurious emission as starting point. - Further discuss whether to define TRP or EIRP requirement for unwanted emission.

Proposal 1 (Xiaomi): Emission requirement is only tested for no small frequency shift mode.

Proposal 2 (CMCC):

- for SEM, use device leakage simulation assumption as starting point and further consider whether to allow certain relaxation considering RF impacts.

- use legacy NR UE spurious emission requirements as starting point.

Proposal 3 (CATT): Emission requirements should be defined for Device 1. Meanwhile, the requirement should cover the emission range of SEM and spurious emissions.

Proposal 4 (Spreadtrum): Define TRP requirements for unwanted emissions

Proposal 5 (vivo):

- The emission mask of AIoT is verified under maximum D2R data rate and the hormonic need to be considered when defining the requirement.

- The unwanted emission is defined on the EIRP peak direction which is same as the radiated power.

Proposal 6 (Sony): Re-use the legacy spurious emission requirement for AIoT device 1 as a starting point to determine the RF spectrum emission requirement.

Proposal 7 (ZTE): for device emission mask, propose to consider ETSI EN 302 208 V3.4.1 (2023-12) 4.3.5.3 at low freq (2W power) as starting point.

Proposal 8 (Ericsson): The TRP metric should be used for SEM and Spurious requirement.

Proposal 9 (Huawei): Define EIRP emission requirement to be evaluated at the max gain direction.

Recommended WF:

- Discuss whether to define EIPR or TRP as performance metric for emission requirements

- Spurious emission: reuse legacy NR UE spurious emission requirement and further discuss the OOB boundary.

- SEM requirements: consider use device leakage simulation assumption as starting point, e.g. 25dBc or 16dB

- Emission requirement is only tested for no small frequency shift mode.

- Topic 3-4: Reference sensitivity

Issue 3-4-1: Reference sensitivity

Agreement: - For R2D reference sensitivity, consider following options: - Option 1: define reference sensitivity using legacy approach, e.g. PREFSENSdBm=-174dBm+10×log10BW+NF+IM+SNR - Option 2: define reference sensitivity based on link budget and/or physical limits of the receiver circuitry. - Other options are not precluded.

Proposal 1 (Xiaomi): Success rate can be considered as the criteria for device 1 REFSENS requirement.

Proposal 2 (CMCC):

- legacy SNR+N methodology is not applicable since actual devices’ sensitivity is much high than that calculated from SNR+N.

- REFSENSE requirement can be defined considering device capability or based on device declaration.

Proposal 3 (Spreadtrum):

- Support option 2, based on link budget to define reference sensitivity.

- The max target distance between Reader and device for R2D should be considered (i.e., 10m). -28dBm for the reference sensitivity could be a starting point.

Proposal 4 (vivo):

It is suggested that the Rx requirement of device is defined based on 10% miss detection rate.

Consider following two options for REFSENS definition:

- Option 1: RAN4 only define the test methods and performance metric, the REFSENS can be declared by manufacturer

- Option 2: Define REFSENS for device 1 based on the 15m coverage design target in R19, e.g., -13.8 dBm.

Proposal 5 (Sony):

- RAN4 can consider adopting X% miss detection rate as the performance metric to define the REFSENS requirements.

- RAN4 needs to determine the reference architecture of the R2D receiver and then decide the proper approach to derive the REFSENS.

Proposal 6 (OPPO):

- The calculated REFSENS has large gap comparing to the value used for co-exist simulation.

- Use declaration method for the REFSENS for device 1.

Proposal 7 (Ericsson):

- Specify the REFSENS and declaration based on EH sensitivity if there is no impact on test feasibility design.

- Discuss whether to set the REFSENS with option 2.

Proposal 8 (Huawei):

- In order to determine the receiver sensitivity requirement, allow companies to report the achievable sensitivity levels based on measurements.

- The AIoT inventory procedure can be used to evaluate the receiver sensitivity with success rate target of >=[90%]. The detailed parameters of the PRDCH and PDRCH (i.e. RMC) are TBD.

- Consider to define multiple levels of sensitivity requirements, for example:

- Table 1: Device Sensitivity Levels

| Sensitivity Level | REFSENS (dBm) |
| --- | --- |
| L1 | max gain direction：≤ [-30~-35] other directions：TBD |
| L2 | max gain direction：≤ [-40~-45] other directions：TBD |

- RAN4 to discuss how to define the Rx sensitivity requirements for different orientation angles.

- Additional margin should be included in the sensitivity requirements to account for the performance variation in the operating frequency range.

Recommended WF:

- Define minimum requirements of REFSENS based on link budget, FFS on the target coverage

- Declaration of REFSENS can also be considered

- FFS on whether and how to consider EH sensitivity

- Topic 3-5: Others

Issue 3-5-1: Maximum input power

Agreement in RAN4#114: - Further discuss the maximum input power for A-IOT device for R2D signal.

Proposal 1 (Xiaomi): Maximum input power requirement is not defined for device 1.

Proposal 2 (CMCC):

- max input power is defined from R2D signal.

- use 1m distance assumption from reader to device. Define the max input power requirements and update the definition of reader BS class based on above assumption.

- max input power is tested with the target that device could response inventory command.

Proposal 3 (CATT): The MCL should be discussed first to check if maximum input power requirement is needed. If the MCL needs to be determined, the value of MCL in TR 38.769 could be considered.

Proposal 4 (Spreadtrum): In order to avoid envelop detector being saturated, maximum input power is needed to define. The specific value needs to be further discussed.

Proposal 5 (Sony): The maximum input level of R2D needs to be defined to ensure the RF-ED receiver operates in the proper region, where -20 dBm may be considered a starting point.

Proposal 6 (OPPO): For device 1 RX requirement, only define REFSENS requirement.

Proposal 7 (Ericsson): Discuss the necessity of specifying the maximum input power by collecting the square law appliance range.

Proposal 8 (Huawei): It’s recommended to set the maximum input level to [-3] dBm for AIoT devices.

Recommended WF:

Since more companies support to define maximum input level requirements, it is recommended that:

Define maximum input level, consider following options:

- Option 1: using [1m] distance assumption from reader to device

- Option 2: using square law appliance range

Issue 3-5-2: Occupied bandwidth

Agreement in RAN4#114: - Further discuss whether to define the occupied bandwidth for device 1.

Proposal 1 (Xiaomi):

If OBW needs to be defined according to regulation, it can be defined as following,

- The occupied bandwidth for all transmission bandwidth configurations shall be less than the channel bandwidth.

Proposal 2 (Spreadtrum): Occupied bandwidth needs to be defined. The specific value is dependent on the channel bandwidth of D2R.

Proposal 3 (vivo): The occupied bandwidth is not defined for AIoT device and the concern on the impact to other radio system can be addressed by emission mask.

Proposal 4 (ZTE): The legacy OBW requirement could be used as starting point unless some OBW issues identified by the simulation.

Proposal 5 (Ericsson): Specify the occupied bandwidth test to test the SFO imperfection.

Recommended WF:

Further discuss whether to define occupied bandwidth

OTA test method

In last meeting, the scope of test methodology is listed as below:

Issue 4-1-1: Scope of OTA test methodology Agreement: - For OTA test, the starting point of test methodology for Ambient IoT includes - Chamber setup - Position of Device - Test procedures - Minimum range/ QZ (Quiet Zone) size/QZ calibration - Preliminary MU assessment - Note: The performance metrics to be discussed in RF (RRM/Demodulation if any) requirements. And the test methodology takes sensitivity and backscatter power as example for discussion to develop test methodology.

- Topic 4-1: General

Issue 4-1-1: testing frequencies

Proposal 1 (HW): only test low, middle and high frequencies within a band and average the performance metrics from the three frequencies

Recommended WF:

TBA

- Topic 4-2: Test setup

Issue 4-2-1: testing antenna and CW antenna relationship

Proposal 1 (Sony): RAN4 adopt one of the options as baseline to develop the OTA test method for AIoT device in Rel-19:

  - Option 1: Both testing antenna and CW antenna has a fixed angel offset and scan together around the DUT

  - Option 2: The position of CW antenna is fixed but testing antenna is scanning around the DUT

Proposal 2 (HW):

- The reader is placed under spherical coordinate in (, , R), (0, 0, R), (30, 0, R), (60, 0, R), and (90, 0, R), for a given CW position. The CW position is (45, 180, R), subject to conclusions from RF core requirements discussion. Both CW and reader are located above the DUT

Proposal 3 (OPPO): Take the anechoic chamber TRP test method as starting point and further consider the CW node

Recommended WF：

The position of CW antenna is fixed but testing antenna is scanning around the DUT.

FFS whether only hemisphere or the whole sphere

Issue 4-2-2: how to avoid the isolation between CW node and measurement antenna

Proposal 1 (Ericsson): IsoCW2M

- 96dB assuming omnidirectional antennas at CWT and reader.

- 66dB assuming 15dBi high directive antennas at CWT and reader

Proposal 2 (HW): IsoCW2M 50dB

Proposal 3 (vivo):

- IsoCW2M 50dB

- TE vendor to feedback on whether the TE can extract D2R power when the CW source is transmitting.

- if the TE cannot cancel the CW easily, the following procedure can be considered to extract the measurement results of D2R:

- Measure the power P1 (D2R+ CW) when device exist,

- Remove the device and measure the power P2 (CW only)

- Calculate the D2R measurement results = P1 – P2

Proposal 4 (Qualcomm): To ensure that the received D2R signal in measurement antenna is X dB higher than CW signal leakage to measurement. FFS on X value

Recommended WF：

- Analyze the feasibility of TE extracting D2R power when CW source is transmitting.

- Meanwhile analyze the candidate isolation between CW node and measurement antenna with following target:

  - To ensure that the received D2R signal in measurement antenna is X dB higher than CW signal leakage to measurement. FFS on X value

Issue 4-2-3: 3D scan and measurement grid

Proposal 1 (Ericsson): coarse measurement gird of 30 degree for TRP and 45 degree for TRS measurement.

Proposal 2 (CMCC): still use the 3D-scan for TRP performance metric but with more sparse measurement grids

Proposal 3 (HW):

- Taking 3D scan with measurement grid as baseline to get sensitivity level and backscatter results for each grid point. Whether all grid points are needed and which results are used to formulate the RF requirements are up to RF requirements discussion.

- 30 degree step in 

Proposal 4 (Sony): take 3D scanning test approach as baseline.  45° step size on measurement grid to reduce the test time and cost.

Proposal 5 (CAICT): adopt coarser grid configurations for A-IoT testing, such as a 30° grid for power measurements and 30°/45° grids for sensitivity measurements.

Recommended WF：

For TRP and TRS requirements, still use 3D-scan but with coarser measurement grid.

- For TRP, candidate values include 30 degree, 45 degree, others are not excluded

- For TRS, candidate values include 30 degree, 45 degree, others are not excluded

Issue 4-2-4: tag mounting material

Proposal 1 (vivo): The device need to mount on the non-RF-reflective and non-RF-conductive material

Recommended WF：

Proposal 1

- Topic 4-3: Position of device

Issue 4-3-1: Position of device

Proposal 1 (CMCC): the longest dimension of tag is suggested to be in x-y plane considering tag mainly use dipole antenna.

Proposal 2 (HW): The position of A-IoT device (DUT) is on the X-Y plane inside the quiet zone with its long side aligned with X-axis, short side with Y-axis and its reflective side facing Z-axis. And the CW and reader are located above the DUT

Proposal 3 (Qualcomm): The free space DUT orientation 1 specified in Table J.2-1 of TS 38101-2 can be assumed as the default position of device.

Proposal 4 (vivo): The default device orientation and corresponding EIRP peak direction are declared by manufacturer if the core requirement is only defined at EIRP peak direction.

Recommended WF：

TBA

- Topic 4-4: Minimum range/ QZ (Quiet Zone) size/QZ calibration

Last meeting agreements are listed as below

Take following as a starting point and further discuss the details. - The traditional test system of AC for handheld UE radiated testing can be considered as the starting point for A-IoT device testing. - The following test setup parameters shall be defined for A-IoT device OTA testing: - DCW2D: distance between CW node and DUT - DMA2D: distance between measurement antenna and DUT - IsoCW2M: Isolation between CW node and measurement antenna

Issue 4-4-1: ripple test for Quiet Zone

Proposal 1 (HW): reuse the same quiet zone ripple test procedure as in 38.870

Proposal 2 (Qualcomm): reuse the same quiet zone ripple test procedure as in 38.870

Proposal 3 (CAICT):  reuse the Ripple test procedure as in 38.870

Recommended WF：

Reuse the QZ Ripple test procedure as in 38.870

Issue 4-4-2: Quiet Zone and minimum range length

Proposal 1 (Ericsson): 30cm size and 90cm minimum range length

Proposal 2 (HW): 30cm size  and 90cm minimum range length as in 38.870

Proposal 3 (Sony): Define the QZ size of AIoT device type 1 as 30 cm (same as FR1 TRP/TIS test) as starting point, and RAN4 can revisit this QZ zone size if other test condition than free space would be required. Re-sue the same minimum range as legacy TRP/TIS test for the test antenna and also the CW/energy source antenna in AIoT device type 1 in Rel-19

Proposal 4 (CAICT): Reuse 30cm QZ size, minimum range length

Recommended WF：

Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length.

Issue 4-4-3: distance DCW2D and DMA2D

Proposal 1 (Ericsson): DCW2D and DMA2D of 1.5m with 90cm minimum range length based on 30cm quiet zone assumption.

Proposal 2 (HW): DCW2D and DMA2D equals to minimum range length or larger

Proposal 3 (vivo): Both the DCW2D and DMA2D are set to 1m

Proposal 4 (Sony): the same minimum range as legacy TRP/TRS testing.

Proposal 5 (CAICT): Use minimum range length for 30cm QZ as the requirement for DCW2D and DMA2D

Proposal  6 (Qualcomm): To follow the minimum range length for TRP and TRS OTA testing with 30cm quiet zone size defined in Table A.3.4-1 of TS 38.161

Recommended WF：

Distance DCW2D equals to DMA2D.

Both are in the range [0.9, 1.5]m, further downselect to one.

- Topic 4-5: Test procedure

Issue 4-5-1: testing performance metric of reference sensitivity

Proposal 1 (Qualcomm): measuring device’s respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna. FFS on how to set the side condition.

Proposal 2 (HW): Determine whether DUT can send the correct response with correct timing relationship

Recommended WF:

Measuring device’s respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna.

FFS on how to set the side condition.

Issue 4-5-2: Test procedure for receiver RF requirements

Proposal 1 (HW): agree on following test procedures for sensitivity and backscatter test.

For sensitivity:

Step 1：Conduct the calibration procedure as defined in clause 7.3 of 38.870

Step 2:   Place the DUT inside the QZ following the positioning guideline defined for A-IoT device.

Step 3：Set the target test frequency and transmit power for signal generator and CW signal.

- The transmit power of signal generator shall be set such that the received power at DUT’s antenna is lower than the minimum reference sensitivity requirement of device.

- The CW signal level is set as -10 dBm at DUT’s antenna

Step 4: Signal generators shall transmit the defined signalling (e.g. paging) and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response.

Step 5:  Determine whether DUT can send the correct response with correct timing relationship. If correct response is received, the sensitivity of this test point shall be calculated by use of the transmit power of signal generators and calibrated pathloss. Otherwise, repeat from step 3 and increase the transmit power of signal generator by 0.5 dB.

Step 6: Repeat step 3~5 for different measurement grid point.

- Note 1: The selected command used for test could be refined based on progress in other working groups.

Proposal 2 (Qualcomm):

For sensitivity:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Energy source transmit the energy signal until energy storage is full and then turn off the energy source based on the known ON-OFF pattern.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 5: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min

- Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: Calculate the number of times the DUT can correctly sent the response. FFS on how to specify the performance metric.

Recommended WF:

The main difference between proposal 1 and 2 is whether to list the step of how to set energy signal. Wait for the conclusion of Topic 4-6.

Issue 4-5-3: Test procedure for Tx RF requirements

Proposal 1 (HW): agree on following test procedures for sensitivity and backscatter test.

For backscatter:

Step 1：Conduct the calibration procedure as defined in clause 7.3 of 38.870.

Step 2:   Place the DUT inside the QZ following the positioning guideline defined for A-IoT device.

Step 3：Set the target test frequency and transmit power for signal generator and CW signal.

- The transmit power of signal generator shall be set such that the received power at DUT’s antenna is equal to minimum sensitivity of device for the measurement grid + 5dB

- The CW signal level is set as TBD

Step 4: Signal generators shall transmit the defined signalling (e.g. paging) and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response.

Step 5:  Determine whether DUT can send the correct response with correct timing relationship. If correct response is received, the backscatter power at DUT’s antenna shall be calculated by calibrating input power at DUT’s antenna. Alternatively, backscatter loss can be calculated by subtracting the transmitted power.

Step 6: Repeat step 3~5 for different measurement grid point.

- Note 1: The selected command used for test could be refined based on progress in other working groups.

- Note 2: The power level of CW signal to be updated based on RF requirement discussion.

Proposal 2 (Qualcomm):

For backscatter:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Energy source transmit the energy signal until energy storage is full and then turn off the energy source based on the known ON-OFF pattern.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device.

- Step 5: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP)

Recommended WF:

The main difference between proposal 1 and 2 is whether to list the step of how to set energy signal. Wait for the conclusion of Topic 4-6.

- Topic 4-6: Energy source and energy harvesting

Issue 4-6-1: energy source and energy harvesting

Proposal 1 (HW): adopt the following statement in the A-IoT specification that “the DUT is assumed to have sufficient power for each test point. It is up to device manufacturers to provide detailed conditions/methods to guarantee that the DUT would not fail a test due to lack of sufficient power supply. Such detailed conditions/methods would not be specified in the specifications”.

Proposal 2 (Qualcomm):

- It is necessary to place an energy source in the chamber. Either CW node or reader node or a separate node/antenna can be used for energy signal provider. The power level of energy source shall be specified.

- It is necessary to know the ON-OFF pattern for the device and energy source can provide the energy signal when device is in OFF symbol and energy source will have to be muted when device is in ON symbol. ON-OFF pattern can be based on the declaration.

Proposal 3 (Sony): RAN4 needs to confirm if any information related to energy harvest needs to be specified or declared during the OTA test.

Proposal 4 (CAICT):  It should be stated in the spec that device vendor should provide the testing lab with detailed method and procedure for supplying energy to the device to ensure the smooth execution of testing and accuracy of the results.

Proposal 5 (Ericsson): A-IoT device 1 should be tested while backscattering a CW signal from an external CWT and using RF energy harvesting to recharge its Energy Storage Unit (ESU).

Recommended WF:

TBA

Issue 4-6-2: CW power level

Proposal 1 (Sony): the power density of CW signal and energy source needs to be defined for the OTA test of AIoT device 1 to trigger the device during the test.

Recommended WF:

TBA

- Topic 4-7: MU

Issue 4-7-1:MU assessment

Proposal 1 (Huawei): use Table 1 and 2 in R4-2503494 as baseline for MU estimation

Proposal 2 (CMCC): use table in R4-2503393as baseline for discussion

Recommended WF:

Use following as baseline for further discussion.

| UID | Description of uncertainty contribution | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of transmitter chain | Reused |
| 2 | Insertion loss of transmitter chain | Reused |
| 3 | Influence of the measurement antenna cable | Reused |
| 4 | CW for backscatter measurement or R2D for sensitivity measurement: uncertainty of the absolute output level | FFS |
| 5 | Sensitivity measurement: output level step resolution | Reused |
| 6 | Measurement distance | Reused |
| 7 | Quality of quiet zone | Wait for quiet zone size conclusion |
| 8 | DUT sensitivity drift | FFS |
| 9 | Uncertainty related to the use of phantoms | No needs |
| 10 | Coarse sampling grid | Wait for sampling grid conclusion |
| 11 | Random uncertainty | Reused |
| 12 | Frequency Response | Reused |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 13 | Uncertainty of network analyzer | Reused |
| 14 | Mismatch of transmitter chain | Reused |
| 15 | Insertion loss of transmitter chain | Reused |
| 16 | Mismatch in the connection of calibration antenna | Reused |
| 17 | Influence of the calibration antenna feed cable | Reused |
| 18 | Influence of the measurement antenna cable | Reused |
| 19 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | Reused |
| 20 | Measurement distance | Reused |
| 21 | Quality of quiet zone | Wait for quiet zone size conclusion |
| Systematic Errors |  |  |
| 22 | Systematic Error related to TRS grids | Reused |