3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503111

Wuhan, China, 07th – 11th April. 2025

3GPP TSG-RAN1 Meeting #120	R1-2501633

Athens, Greece, 17th – 21st February 2025

Title:	LS on time location of on-demand SSB for SCell

Release:	Rel-19

Work Item:	Netw_Energy_NR_enh

Source:	RAN WG1

To:	RAN WG4

Cc:	RAN WG2

Contact Person:

Name:	Seonwook Kim

E-mail Address:	sseonwook.kim@lgepartner.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		-

1. Overall Description:

RAN1 discussed the relation in terms of time location between the always-on SSB and on-demand SSB and agreed the following in RAN1#120.

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

2. Actions:

To RAN WG4

ACTION: 	RAN1 respectfully ask RAN4 to take the above into consideration.

3. Date of Next TSG-RAN1 Meetings:

TSG RAN1 Meeting #120bis	7 - 11 April 2025					Wuhan, China

TSG RAN1 Meeting #121		19 - 23 May 2025				St. Julian, Malta