3GPP TSG-RAN4 Meeting #115	R4-2505507

# St Julian, Malta, 19th – 23rd May, 2025

Source: 	Qualcomm Incorporated

Title: 	 FR2-1 L3 measurement delay by optimizing RX beam sweeping factor

Agenda item:	 7.17.2.1

Document for:	Discussion

# Introduction

In this paper, we present our views of FR2-1 L3 delay reduction by optimizing RX beam sweeping factor.

# Discussion

## Prior Agreements

A subset of agreements and open issues from recent meetings [1][4][5] are listed below.

### Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in RRM session (Monday): - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell. - Leave the decision to RAN2. Further discuss the details for the LS to RAN2, and the LS can only be approved if the content is agreeable. - Details of the LS can refer to R4-2504962. Agreement in the ad-hoc: - Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4. Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting. Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2. ### Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement in ad-hoc session: - UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4. Further discuss this issue. ### Issue 1-1-3: SSB processing delay/time for processing multiple beams received in a SMTC Agreement in RRM session (Monday): - No need to add additional processing time in the delay requirement for scenario 5. ## Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF ### Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Further discuss this issue. ### Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF Further discuss this issue. ### Issue 1-2-3: Requirement at transition between the normal L3 measurement and FBS L3 measurement Agreement: - Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement. Applicability requirement: Updated Agreement in online session: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. UE Power class: Agreement in online session: Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion. Other clarification on WID: Agreement in online session: Remove {FFS: “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”}. Issue 1-2-1: FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature Agreement in online session: - The conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode. And it does not assume that the condition of in active mode is same as that for Rel-18 multi-Rx simultaneous reception. Issue 1-2-2: FFS: UE’s mobility status, e.g., whether HST is precluded or not Agreement in online session: - RAN4 to consider UE in non-HST case as first priority. - Note: whether or how HST case could use the outcome of the WI discussion can be FFS after concluding on non-HST case. These extra FFS parts will NOT delay the WI completion. Issue 1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF Which scenarios are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 6: PSCell addition - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Scenario 8: SCell activation - Scenario 9: SCG activation - Scenario 10: CGI identification - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded. Agreement: - Scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion. Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF: Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment. Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF Agreement: - RAN4 is to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19. - The detailed definition can be postponed to the end of the core part discussion. Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions): Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19. Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability: - FFS: details of the capability

## Impact of Channel Condition on Triggering Fast Beam Sweeping

RAN4 has agreed to use serving cell’s signal quality (RSRP and/or RSRQ) based triggering condition to activate/deactivate multi-RX for L3 measurement. It is useful for network to know UE’s fast beam sweeping related multi-RX status. Network could utilize this info to change SMTC config for the UE. For example, a UE that can support fast beam sweeping can be configured with longer SMTC periodicity to increase throughput.

Observation 1: It is useful for network to know UE’s fast beam sweeping related multi-RX status. Network could utilize this info to change SMTC config for the UE.

Existing RLM/BFD relaxation mechanisms have introduced signaling and procedures to report RLM/BFD relaxation status. These signaling and procedures are shown in appendix A. These reports for conveying RLM/BFD relaxation status can be reused or served as a baseline for conveying beam sweeping status.

Observation 2: 3GPP has introduced reporting configurations to convey RLM/BFD relaxation status.

RAN4 can adopt proposal 1 to allow UE to inform its beam sweeping status to network.

Proposal 1: Network configures report for the status of fast beam sweeping factor application.

    - The existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

## UE Capability Regarding RX Beam Sweeping Factor

RAN4 discussed different options of UE capability regarding RX beam sweeping factor during the last meeting. The following recommended way forwards show the options that were proposed by companies during the last meeting.

- Recommended WF - Introducing the UE capability of reduced N to indicate the BSF value(s) or fraction value(s). FFS the value(s). - Discuss option 3 in regard to the UE capability design: - Different BC - Number of Carriers/MOs to measured - Intra-frequency/inter-frequency - Support “per UE” or “per band” capability.

A FR2-1 UE can enable different mechanisms to reduce beam sweeping factor and reduce L3 measurement delay. It can activate multiple modules and one RX beam per module to simultaneously deploy multiple RX beams. It can also generate multiple RX beams simultaneously in the same module. The combination of multiple modules and multiple beams in the same module is a possibility, too.

UE’s capability to implement these mechanisms will vary across bands. That means, just because UE can support multiple FR2 RX beam in one FR2 band (e.g., 28 GHz), it does not mean that the UE will be able to support this feature in a different FR2 band (e.g., 39 GHz). Hence, the UE capability to reduce beam sweeping factor in this work item should be per-band capability.

Observation 3: A FR2-1 UE can enable following different mechanisms to reduce beam sweeping factor and reduce L3 measurement delay.

- Simultaneous activation of multiple modules.

- Simultaneous activation of multiple RX beams within the same module.

- Combination of above two.

Observation 4: UE’s ability to implement different mechanisms of observation 4 may vary among different FR2 bands.

Proposal 2: RAN4 introduces per-band UE capability for FR2 RX beam sweeping factor optimization and L3 measurement delay reduction.

# Conclusions

Observation 1: It is useful for network to know UE’s fast beam sweeping related multi-RX status. Network could utilize this info in different purposes, e.g., to change SMTC config for the UE.

Observation 2: 3GPP has introduced reporting configurations to convey RLM/BFD relaxation status.

Observation 3: A FR2-1 UE can enable following different mechanisms to reduce beam sweeping factor and reduce L3 measurement delay.

- Simultaneous activation of multiple modules.

- Simultaneous activation of multiple RX beams within the same module.

- Combination of above two.

Observation 4: UE’s ability to implement different mechanisms of observation 4 may vary among different FR2 bands.

Proposal 1: Network configures report for the status of fast beam sweeping factor application.

    - The existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

Proposal 2: RAN4 introduces per-band UE capability for FR2 RX beam sweeping factor optimization and L3 measurement delay reduction.

# Appendix A: Reporting of BFD/RLM Relaxation State via UAI

UE’s reporting regarding BFD and RLM relaxation status can be summarized through the following:

- bfd-RelaxationReportingConfig > OtherConfig

  - UE uses the config to report the relaxation state of BFD measurements.

- rlm-RelaxationReportingConfig > OtherConfig

  - UE uses the config to report the relaxation state of RLM measurements.

The requirements for providing RLM/BFD relaxation status are defined as follows in RAN2 and RAN4 specs [2] [3].

NR UE employs the following timers to report RLM/BFD relaxation status [2].

# References

[1] R4-2414026, Way Forward for NR_RRM_Ph5, Apple, RAN4 #112

[2] 38.331 V 17.7.0

[3] 38.133 V 17.11.0

[4] R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN4 #112bis.

[5] R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, RAN4 #114bis.