3GPP TSG-RAN WG4 Meeting #115	R4-2505604

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	6.14.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN B252 UE RF requirements

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration for IoT-NTN B252 UE RF requirements is provided for discussion.

- Discussion

2.1		Spectrum WI scope for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario; - Support UE Categories NB1, NB2, M1; - Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. The specification work of this WI shall leverage the studies and requirements for NR/LTE NTN n256/B256, n255/B255, n254/B254, where applicable.

2.2		Previous Meeting Agreements

In RAN4#114bis, the approved WF [2] provided the guidance for following works. The partial content is listed below for reference.

Issue 2-1-1: UE A-MPR for UE-to-UE Coexistence - Agreement. - Reuse the existing framework and do not specify any A-MPR requirement for Cat M1 and Cat NB1/NB2 to meet the -30 dBm/MHz and -40 dBm/MHz requirements. - Specify at least one NS value(s)

In RAN4#113, the approved WF [3] provided the guidance for following works. The partial content is listed below for reference.

Issue 2-1-2: UE Maximum Output Power for Cat M1 and NB1/NB2 Agreement: - The MOP of this band will be aligned with the agreements in other Rel-19 NTN related WIs.

2.3		UE RF requirements

2.3.1	B252 A-MPR for UE-to-UE Coexistence

Currently, B252 PC3 TX A-MPR values are not needed when UE-to-UE coexistence specification is relaxed requirements of -30dBm/MHz or -40dBm/MHz. However, because NTN HPUE for supporting B252 will be available soon, the B252 HPUE TX A-MPR could be necessary. Therefore, to be forward compatible with B252 HPUE, it is necessary to reuse band n252 UE-to-UE coexistence network signalling (e.g., NS_06N, NS_07N) for B252.

Proposal 1: To be forward compatible with B252 HPUE, it is at least necessary to use band n252 UE-to-UE coexistence network signalling indicators (e.g., NS_06N, NS_07N) for B252.

Proposal 2: Further discuss whether band n252 NS_08N indicator would be also needed for B252.

2.3.1	B252 TX MOP and power tolerance

It was agreed that the B252 TX MOP and associated tolerance will be aligned with the agreements in other Rel-19 NTN related WIs.

Observation 1: It was agreed that the B252 TX MOP will be aligned with the agreements in other Rel-19 NTN related WIs. However, the R19 IoT-NTN HPUE TX MOP and power tolerance for TX power enhancement are still under discussion.

Observation 2: If B252 TX MOP requirements are not completed on time because of being to align the agreements in other Rel-19 NTN related Wis, the R19 IoT-NTN B252 spectrum WID schedule may be delayed.

Proposal 3: Based on consideration of previous WF agreement and B252 WID schedule, firstly, to reuse IoT-NTN B256 TX MOP requirements for B252 MOP, and then update B252 TX MOP in R19 maintenance to align with the agreements in other Rel-19 NTN related WIs.

3 Conclusion

In this paper, our views on IoT-NTN band B252 UE RF requirements are provided.

Proposal 1: To be forward compatible with B252 HPUE, it is at least necessary to use band n252 UE-to-UE coexistence network signalling indicators (e.g., NS_06N, NS_07N) for B252.

Proposal 2: Further discuss whether band n252 NS_08N indicator would be also needed for B252.

Observation 1: It was agreed that the B252 TX MOP will be aligned with the agreements in other Rel-19 NTN related WIs. However, the R19 IoT-NTN HPUE TX MOP and power tolerance for TX power enhancement are still under discussion.

Observation 2: If B252 TX MOP requirements are not completed on time because of being to align the agreements in other Rel-19 NTN related Wis, the R19 IoT-NTN B252 spectrum WID schedule may be delayed.

Proposal 3: Based on consideration of previous WF agreement and B252 WID schedule, firstly, to reuse IoT-NTN B256 TX MOP requirements for B252 MOP, and then update B252 TX MOP in R19 maintenance to align with the agreements in other Rel-19 NTN related WIs.

4 Reference

- RP-241662, “New WID on Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- R4-2505235, “WF on NR-NTN and IoT-NTN bands”, Apple

- R4-2420465, “WF on requirements for NR IoT NTN bands”, Inmarsat