3GPP TSG-RAN WG4 Meeting #115	R4-2505661

Malta, 19 May – 23 May, 2025

Agenda item:	7.26.4

Source: 	vivo

Title: 	Discussion on RRM impacts for R19 A-IoT device

Document for:	Discussion

- Introduction

For the Ambient IoT, the study item [1] was completed and the corresponding WI was approved in [2] and further updated in [3]. The RRM discussion is scheduled to start from RAN4#114bis meeting. The WF of the previous meeting is in [4]. In this contribution, we provide our considerations on potential RRM impacts for Ambient-IoT.

- Discussion

The RRM scope in [3] is copied below for reference:

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

The target device and deployment scenario are included in the general scope of the WID and copied below where the device type to be specified and deployment scenario to be considered is described:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only.

The topic was initially discussed at RAN4 114bis meeting. The following agreement were achieved based on [4].

Issue 1-1: A-IoT paging

Agreement:

RAN4 not to define RRM requirements for A-IoT paging

Issue 1-2: A-IoT data transmission

Agreement:

Excepting the timing issue discussed in Issue 1-4, no other potential RRM requirement impact is identified for “A-IoT data transmission”

Issue 1-3: A-IoT random access procedure

Agreement:

RAN4 to include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. FFS the details

Issue 1-4: timing

Agreement:

Further discuss whether there is RRM impact on transmit timing requirements of device pending on RAN1 conclusion.

Regarding the timing relationship between D2R and R2D, RAN1 has the following agreements at previous RAN1 meeting.

From RAN1 agreement, it can be learned that for AIot device, the starting time of D2R is determined by the Toffset1 and the corresponding R2D. From RRM perspective, the initial transmission timing accuracy for the device could be specified.

Proposal 1: The initial transmission timing accuracy for the device could be specified.

- Conclusions

In this contribution we provide our analysis for A-IoT WI and have the following observations and proposals:

Proposal 1: The initial transmission timing accuracy for the device could be specified.

References

[1] RP‑234058 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[4] R4-2504905, WF on RRM requirements for Ambient_IoT_Solutions, CMCC, RAN4 114bis