# 3GPP TSG-RAN WG4 Meeting #115   	R4-2505797

# Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on other RRM core requirements in NTN for NR Phase 3

Agenda Item:	7.31.3.2

Document for:	Discussion

# 1Introduction

In the last RAN4#114 meeting, discussion on RRM requirements for Rel-19 NR NTN phase3 addressed and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further discussion on other RRM core requirements in NTN for NR Phase 3 and provide our proposals.

- Discussion

- SMTC configuration related issues

## Issue 1-0: SMTC configuration and actual SSB transmission period FFS: Wait for RAN2 progress and discuss the following - Decide whether the following needs to be captured only in the RAN4 specification as part of the requirement applicability rule, or if the group will send an LS to RAN2 to ensure the following constraint is included in their specifications. - If different neighbour cells in the same frequency layer have different SSB periodicities, and if SMTC is not configured to match the largest SSB periodicity among the cells, the periodicity and active patterns of the neighbour cells shall be informed to the UE to assist with UE measurement. - Further discuss the following if the decision needs to be made in RAN4 RRM: - For UEs supporting DL-CE feature in NR Rel-19, it shall be mandatory the support to 4 SMTCs per frequency layer ## Issue 1-6: SMTC configuration Conclusion: No discussion in this meeting.

- The impact of SMTC configuration was analyzed by companies in last meeting, but discussion is deferred pending RAN2’s more progress. In RAN2#129bis meeting, some progresses are made accordingly. And, a LS on SMTC enhancements [2] from RAN2 was received, where RAN2 is asking RAN4 to evaluate the feasibility to support the SMTC enhancements in Rel-19 time frame. We copy the relevant text below:

- Agreements in RAN2#129bis:

- 3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

- 4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

- 5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

LS out approved in RAN2#129bis: - Overall description For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe. - Actions To RAN4: ACTION: RAN2 kindly asks RAN4 to provide feedback on the feasibility to support those agreements in Rel-19 timeframe.

- In the LS, two main bullets were captured. The first one is about the configuration of two different SMTC periodicities (with different offsets) in one frequency layer. The second one is about the configuration of more than 4 SMTCs per frequency for idle/connected UEs. We understand the motivation of SMTC enhancement is to facilitate UE measurement due to beam hopping. The different SMTCs can help UE detect the different SSB transmission in different cells.

- In the following, we analyse the SMTC enhancement impact on RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode from RAN4 RRM perspective respectively.

- RRC_CONNECTED mode

- In RRC_CONNECTED mode, many requirements has a dependency on SMTC. However, the configuration of SMTCs with two different periodicities is also not brand new. NR already support dual SMTC configurations since Rel-15, i.e. smtc1 and smtc2. The smtc2 only apply to intra-frequency measurement, and also has shorter periodicity than smtc1. And during Rel-15 time frame, RAN4 took complete discussion on how to reflect the dual SMTC configuration in the requirements. The main issues need to be clarify is:

- When UE knows the PCI of the target cell, clarify the SMTC used in the related requirements is the one used by the cell being identified;

- When UE does not know the PCI of the target cell, i.e. during PSS/SSS detection, clarify which SMTC is supposed to be used.

- Here below, we capture the related requirements:

- For requirements of handover, the parameter Trs, a component for the interruption time calculation, has dependency on SMTC. The current spec clarify that “If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell”.

- For requirements of RRC Re-establishment, the parameter TSMTC is used to drive the Time to identify target NR cell. The current spec clarify that “If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell”.

- For requirements of RRC Connection Release with Redirection, the parameter of Trs used to derive the time to identify target NR cell has has dependency on SMTC. The current spec clarify with a NOTE1 in Table 6.2C.3.2.1-1 that “If the UE has been provided with higher layer signalling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell”.

- For requirements of RLM, LR (BFD and CBD) and L1-RSRP measurement, the scaling factor P introduced to account for the measurement opportunity sharing between measurement gap, SSB/CSI-RS and SMTC. It also has dependency on SMTC. For FR1, the related clarification is “If the higher layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.”

- For requirements of L3 measurement, SMTC is used to drive the measurement delay for at least measurement without measurement gap case. The SMTC is clarified with a NOTE that “If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified”. Furthermore, for the requirement of cell identification, when UE does not know the PCI of the target cell, the clarification is “if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1”.

- Observation 1: For the case of dual SMTC configuration in RRC_CONNECTED mode, RAN4 specified the applicability condition to clarify the SMTC periodicity used in the related requirement.

- The requirements for NR NTN inherits all the above mentioned requirements. Even though the relationship between the two different SMTCs configured for NR NTN is not clear yet. But from our understanding, if we take the similar logic as for dual SMTC in R15, the spec impact could be similar.

- Proposal 1: To support the SMTC enhancements on two different SMTC periodicities configuration in RRC_CONNECTED mode, RAN4 to update the applicability conditions for SMTC taken the same principle for dual SMTC as baseline.

- RRC_IDLE/INACTIVE mode

- In most of RRM requirements for RRC_IDLE/INACTIVE mode, DRX cycle is used to define the measurement delay. The SMTC periodicity is only used under some cases. Based on the statement in the LS, we can see that the configuration of two different SMTC periodicities (with different offsets) in one frequency layer is only for neighbor cell measurements. So, we go through the requirements for RRC_IDLE mode, and capture the related text below:

#### 4.2C.2.3 Measurements of intra-frequency NR cells Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1 and FR2-NTN. 0.32; 1; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. NOTE 2: The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.. Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh DRX cycle length [s]; Tdetect,NR_Intra_enh [s] (number of DRX cycles); Tmeasure,NR_Intra_enh [s] (number of DRX cycles); Tevaluate,NR_Intra_enh [s] (number of DRX cycles). 0.32; 2.56 x M2 (8 x M2)Note 1; 0.32 x M3 (1 x M3) Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; 5.12 (8); 0.64 (1); 1.92 (3). 1.28; 8.96 (7); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). NOTE 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5.

#### 4.2C.2.4 Measurements of inter-frequency NR cells The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in table 4.2C.2.4-1 under the following conditions: - TSMTC_intra = TSMTC_inter = 160 ms; where - TSMTC_intra is the periodicity of the SMTC configured for the intra-frequency carrier if no identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier; TSMTC_intra is the periodicity of the smtc2-LP configured for the intra-frequency carrier if at least one identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed for TSMTC_intra. If the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. - TSMTC_inter is the actual SMTC periodicity used by the inter-frequency cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the inter-frequency carrier is assumed for TSMTC_inter. If the actual SSB transmission periodicity is greater than the SMTC configured for the inter-frequency carrier, longer Tdetect, NR_inter is expected. - SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and - - SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS 38.304 [1].

#### 4.2C.2.5 Maximum interruption in paging reception At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. At inter-frequency cell re-selection with TN carrier, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. - ========omitted the unrelated part========= #### Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

- As highlighted, in the text, the current spec already clarify UE behavior when UE is configured with different SMTC periodicities. We understand these clarifications are for smtc2-LP, which is introduced in Rel-16 TEI, at first. The smtc2-LP is configurable to both intra-frequency and inter-frequency measurement. Compare with the legacy smtc, the smtc2-LP can only be set with strictly larger periodicity values while same offset. UE will select either smtc or smtc2-LP based on whether a target neighboring cell in the PCI list is detected. Current applicability conditions clarify the specific SMTC which is supposed to be used in the requirements to make sure UE can satisfy the requirements.

- Observation 2: For the case of smtc2-LP configuration in RRC_IDLE mode, RAN4 specified the applicability condition to clarify the SMTC periodicity used in the related requirement.

- When the smtc2-LP is configured, UE would get total two sets of SMTC configurations with different periodicities but same offset. We can see the similarity between the function of smtc2-LP and the SMTC enhancements to be introduced in NTN, which support two sets of SMTCs with different periodicities and different offsets. From our perspective, to support the SMTC enhancements on two different SMTC periodicities configuration, RAN4 can follow the similar logic as for smtc2-LP in Rel-16. The spec impact is clear.

- Proposal 2: To support the SMTC enhancements on two different SMTC periodicities configuration in RRC_IDLE/INACTIVE mode, RAN4 to update the applicability conditions for SMTC taken the same principle for smtc2-LP as baseline.

- The LS also mentioned that RAN2 agreed to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. Based on current UE capability, UE can at most support 4 SMTCs per frequency. Going back to R17 discussion, RAN4 already considered the case that the number of configured SMTCs per Frequency layer is beyond UE capability. And the agreement was achieved for RRC_IDLE/INACTIVE mode that:

- For RRC Idle/Inactive mode: - UE can be configured with more SMTCs than its capability. - UE performs RRM measurement for the broadcasted SMTCs up to its capability in terms of the number of supporting SMTCs at a given period of time. - If the number of SMTCs broadcasted by NW is beyond the UE capability, UE may select SMTCs sequentially until all of the SMTCs can be measured . - The selection of SMTCs to be used is up to UE implementation, e.g. RSRP, equal priority, etc. - (note) longer measurement delay is expected, if UE is configured with more SMTCs than its capability

- Proposal 3: The current TS 38.133 has already cover the scenario when UE is configured more SMTCs than UE capability for RRC_IDLE/INACTIVE mode.

- All in all, to support the SMTC enhancements, although many RRM requirements can be influenced, if we follow the same principle applied for smtc2-LP and smtc2, we think the work load in RRM is not very high. Also, to support cells in the same frequency layer with different SSB periodicity in DL CE scenario, we think the different SMTC periodicities is a reasonable solution. If go with single SMTC configuration, we think the SMTC will be configured to match the lowest SSB periodicity or the longest SSB periodicity. For the case SMTC periodicity shorter than the SSB periodicity, there might be no SSB available for some SMTC occasion, which would lead to UE cannot meet the requirement because UE cannot obtain enough measurement samples. For the case SMTC periodicity longer than the SSB periodicity, it would decrease the measurement efficiency and has no good for UE power consumption.

- Proposal 4: RAN4 to confirm the feasibility to support the agreements on SMTC enhancements in Rel-19 timeframe.

- In last meeting, company raised concern that NTN UE configured with same SMTCs periodicity but different SSB periodicity could have impact on UE behavior. Based on the latest RAN2 agreements, SMTCs with different periodicities could be supported and also NW can provide assistance information on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. From our side, it could solve the concern. For the number of SMTCs UE mandatory to support, we prefer to keep the legacy number, i.e. 2. RAN2 already agreed that the number of SMTC/gaps a UE needs to consider at any time will not be increased further, and is working on how to handle the coverage issue with limited UE capability. So, we prefer not to enforce UE to support 4 SMTCs per frequency layer.

- Proposal 5: UEs supporting DL-CE feature in NR Rel-19 is not mandatory the support to 4 SMTCs per frequency layer.

- Other issues

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i) Agreement (online): - In RRC re-establishment requirements, for case where the SMTC for the inter-frequency carrier is not provided, the value of TSMTC,i is assumed to be 160ms: - Discuss the applicability of the requirement for with the updated value of TSMTC,i to 160ms after the related RAN1/2 agreement. FFS: Wait for RAN2 progress and discuss the following - The requirements for intra-frequency and inter-frequency RRC re-establishment are applicable provided that the configured SMTC period is not shorter than the periodicity of the actual SSB transmission of the target cell. ## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Agreement (online): - In RRC re-establishment requirements, for the case SSB Ês/Iot < -8dB and unknown NR cell, - Make the following updates - Remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2 for the applicable scenarios, and the value of TSMTC and TSMTC,i is FFS - Tidentify_intra_NR: MAX (800 ms, [22] x TSMTC) for intra-frequency carrier - Tidentify_inter_NR,i: MAX (800 ms, [22] x TSMTC,i) for inter-frequency carrier ## Issue 1-5: Paging Further discussion: - For the paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms: - No update is needed, and K is 5.

- Paging interruption in RRC Idle/Inactive:

- In last meeting, some companies propose to reduce the value of K used in the paging interruption requirements. The main concern is on the case K=5 that the total interruption time could be very long consider the SSB periodicity of 160ms. However, we understand that the current requirement, which is defined based on SMTC periodicity, has already support the case SSB periodicity with 160ms. Furthermore, the K=5 only apply for the case that the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO. In this scenario, UE need the time to search the target cell as well as the time to do fine T/F tracking in the new serving cell. We understand the fine T/F tracking time is 2 SMTC periodicity, and the search time is 3 SMTC periodicity which take from Tsearch in HO for AGC settling and PSS/SSS detection. From our perspective, these are all reasonable components for UE implementation. So, we prefer not to reduce the value of factor K in 4.2C.2.5.

- Proposal 6: For the paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms, no update is needed.

- Conclusion

- Observation 1: For the case of dual SMTC configuration in RRC_CONNECTED mode, RAN4 specified the applicability condition to clarify the SMTC periodicity used in the related requirement.

- Proposal 1: To support the SMTC enhancements on two different SMTC periodicities configuration in RRC_CONNECTED mode, RAN4 to update the applicability conditions for SMTC taken the same principle for dual SMTC as baseline.

- Observation 2: For the case of smtc2-LP configuration in RRC_IDLE mode, RAN4 specified the applicability condition to clarify the SMTC periodicity used in the related requirement.

- Proposal 2: To support the SMTC enhancements on two different SMTC periodicities configuration in RRC_IDLE/INACTIVE mode, RAN4 to update the applicability conditions for SMTC taken the same principle for smtc2-LP as baseline.

- Proposal 3: The current TS 38.133 has already cover the scenario when UE is configured more SMTCs than UE capability for RRC_IDLE/INACTIVE mode.

- Proposal 4: RAN4 to confirm the feasibility to support the agreements on SMTC enhancements in Rel-19 timeframe.

- Proposal 5: UEs supporting DL-CE feature in NR Rel-19 is not mandatory the support to 4 SMTCs per frequency layer.

- Proposal 6: For the paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms, no update is needed.

- Reference

- R4-2504913, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm

- R2-2503068, LS on SMTC enhancements, RAN2