3GPP TSG-RAN WG4 Meeting #115							         		      R4-2505834

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.10.3.3.1	Tx requirements	[NR_IoT_NTN_req_test_enh-Core]

Title: 	NR and GSM PAs measurements for NB-IoT-NTN HPUE

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

In R4#114b, the way forward [1] captured the tentative assumptions for MPR evaluation for PC2 and PC1 NB-IoT-NTN. In [2], we already demonstrated that a PC2 NR PA may be able to reach PC1 levels based on UTRA ACLR. In this contribution, we evaluated the NB-IoT SEM on top of GSM and UTRA ACLR for both NR and GSM PAs and make proposals for MPR.

# Discussion

## 2.1 Way forward input

The way forward [1] captured the tentative agreements on HPUE PA calibration points for MPR evaluation for PC2 and PC1 targets. The two options are summarized in italics below.

MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC2 and PC1: WF (Ad-hoc agreement): Tentative PA Calibration points

- Option 1:

  - DFT-s-OFDM QPSK 20 MHz 100RB0

  - [NR ACLR: 31dBc]

  - 4dB post PA loss

  - 1dB MPR

  - Carrier leakage: 28 dBc

  - IQ image: 28 dBc

  - CIM3: 60 dBc

- Option 2:

  - DFT-s-OFDM 1RE (Single Tone), [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK]

  - [2] or [3] dB post PA loss

  - 0dB MPR

  - Carrier leakage 28dBc

  - IQ image:28 dBc

  - CIM3: 60 dB

- For PC2, both options can be considered, both Testing and simulation may be considered.

- For PC1, Option 2 as baseline. Option 1 may be considered by interested parties, , both Testing and simulation may be considered.

In our view there is still a fundamental difference between option 1 and option 2 approaches for calibration:

- Option 1 sets a known linearity calibration level on an existing PA. This is a PC2 NR PA calibrated for the usual NR waveform and then tested with NB-IoT waveforms. The power capability to meet the NB-IoT SEM and UTRA ACLR is then registered. This enables assessment of how much more power a NR PA that is already supported in a smartphone can deliver for waveforms intended for NB-IoT-NTN. Note that, given option 1 does not state a 1dB MPR calibration power, one could use a PC2 NRPA for both PC2 and PC1 while there is 5dB difference in PCmax. We thus assessed a PC3 NRPA for PC2 NB-IoT-NTN

- Option 2 seems to tackle the problem the other way around by choosing one NB-IoT waveform that would meet the target power class. We assume this waveform just meets NB-IoT SEM and UTRA ACLR and then checks the power capability for other waveforms. In our view, this option 2 corresponds to a virtual PA that can be “designed” by simulation but does necessarily correspond to an existing PA.

Since we do not see consistency in option 2, and option 1 does not have a 1dB MPR power target we decided to measure an NR PA and a GSM PA as these are the natural target for hardware reuse in a smartphone. Each PA is measured in two operation modes:

- A Band 40/41 NR PA calibrated with DFT-s-OFDM QPSK 20 MHz 100RB0 at 30dB ACLR and 26dBm (4dB post PA losses) consistent with a PC3 NR operation. Labelled NR PC3 PA.

- A Band 40/41 NR PA calibrated with DFT-s-OFDM QPSK 20 MHz 100RB0 at 31dB ACLR and 29dBm (4dB post PA losses) consistent with a PC2 NR operation. Labelled NR PC2 PA.

- A GSM mid-band PA at its nominal voltage of 3.5V (since there is no agreed calibration point for saturated GSM PAs). Labelled GSM Vnom PA.

- A GSM mid-band PA at its maximum voltage of 4.5V. Labelled GSM Vmax PA.

Each PA was measured for five waveforms and checked for NB-IoT SEM, GSM ACLR and UTRA ACLR:

- Pi/2 BPSK with one edge RE labelled 1NB1RE0

- Pi/2 BPSK with one middle RE labelled 1NB1RE5

- QPSK with one edge RE labelled 2NB1RE0

- QPSK with one middle RE labelled 2NB1RE5

- QPSK with one RB labelled 2NB1RB.

To provide some reference in terms of NB-IoT MPR, see the TS-36.102 PC3 and PC5 table below:

Table 6.2B.2-1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 3 and 5

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-11 |  |  |  |
| MPR | 0 dB |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |

Observations:

- There is no reference for Pi/2 BPSK or Pi/4 QPSK, only QPSK in 36.102 and 16QAM additionally in 36.101.

- A good reference is QPSK 1RB MPR of 2dB as it does not depend on TRx impairments and is thus a good representation of PA linearity.

- In 36.101, Cat NB1 and NB2 PC3 was derived from a PC3 LTE PA and did not assume lower post-PA losses.

- It is likely that the middle 3 tones (3-5 and 6-8) and the middle 1 tone can achieve beyond 23dBm for PC3.

## 2.2 Measured power capability per PA and waveform types

The following results are based on measurements and are corrected for the anticipated post-PA losses for HD-FDD. These are further corrected by 0.5dB for the NR PA as the output power is measured after a switch while the GSM PA is measured directly at the PA output.

Since the NR post-PA loss assumption is 4dB and includes the usual post-PA filter, it is safe to assume 3dB post PA losses for HD-FDD where the acoustic filter is replaced by a simple low pass filter for harmonics. Further optimization may be feasible with specific bypass switches.

Proposal for HD-FDD post PA losses: MPR for NB-IoT-NTN is based on a 3dB post-PA losses assumption.

Based on this assumption ,Table 1 provides the antenna power level of the various PA modes and the five waveforms that are limited by SEM as both the GSM and UTRA ACLR show large margins as shown in [2]. Please note that in terms of absolute power the measured values accuracy is within ±0.3dB.

Table 1: Power capability for NB-IoT waveforms versus PA type

| PA type/mode |  |  | NR PC3 | NR PC2 | GSM Vnom | GSM Vmax |
| --- | --- | --- | --- | --- | --- | --- |
| Waveform | position | Modulation | Pout [dBm] at NB-IoT SEM limit |  |  |  |
| 1NB1RE0 | Edge | Pi/2 BPSK | 25.9 | 29.5 | 28.7 | 30.5 |
| 1NB1RE5 | Middle | Pi/2 BPSK | 27 | 30.8 | 30.5 | 32 |
| 2NB1RE0 | Edge | QPSK | 25.7 | 29.3 | 28.2 | 29.8 |
| 2NB1RE5 | Middle | QPSK | 26.9 | 30.7 | 30 | 31.5 |
| 2NB1RB | Full | QPSK | 22.7 | 25.8 | 25.4 | 26.3 |

Observations:

- The difference between the PC3 and PC2 NR PA for 1RB QPSK waveform is about 3dB as expected

- The GSM PA at nominal voltage has slightly lower capability than the PC2 NR PA: they both fall 0.5dB short from reaching 31dBm for the best waveform.

- The PC3 NR PA delivers 22.7dBm which would correspond to 0.3dB MPR instead of the 2dB allowed for PC3 NB-IoT. For the best power, its power reaches above PC2.

- Pi/2 BPSK power is only 0.2dB higher than QPSK with less constellation states and thus may not be very useful.

- Based on a QPSK middle allocation as the reference for MPR:

  - A NR PC3 PA can support PC2 for NB-IoT-NTN with 0.5dB boosting

  - A NR PC2 PA can support PC1 for NB-IoT-NTN with an MPR of 0.5dB

  - A NR GSM PA can support PC1 for NB-IoT-NTN with an MPR of 1dB at nominal voltage and 0dB at a higher voltage that may be compatible with non-smartphone UEs.

Proposal for PC2 NB-IoT NTN:

- PC2 NB-IoT-NTN MPR is based on a PC3 NR PA (PC3 NR calibration) with 3dB post PA losses following the MPR/Boost table below to be completed for 3RE and 6RE, and other 1RE positions.

- FFS if 0.5dB optional boosting is specified

- Keeping Pi/2 BPSK is FFS as the power may not be significantly higher than QPSK and MPR is the same.

| Modulation | Pi/2 PSK |  |  | QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0 | 5 | others | 0 | 5 | others |
| MPR [/ boost option] | ≤ [0.5] dB | ≤ [0] dB / ≥ [0.5dB] | TBD | ≤ [0.5] dB | ≤ [0] dB / ≥ [0.5dB] | TBD |
| Tone positions for 3 Tones allocation | TBD | TBD | TBD | TBD | TBD | TBD |
| MPR | TBD | TBD | TBD | TBD | TBD | TBD |
| Tone positions for 6 Tones allocation | TBD | TBD | TBD | TBD | TBD | TBD |
| MPR | TBD | TBD | TBD | TBD | TBD | TBD |
| Tone positions for 12 Tones allocation | 0-11 |  |  | 0-11 |  |  |
| MPR | TBD |  |  | ≤ [3.5] dB |  |  |

Proposal for PC1 NB-IoT NTN for smartphone UEs:

- PC1 NB-IoT-NTN MPR is based on a PC2 NR PA (PC2 NR calibration) or GSM PA with 3dB post PA losses following MPR table below to be completed for 3RE and 6RE, and other 1RE positions.

- Non smartphone UEs without thermal, DC current or battery voltage range constraints can achieve 0.5dB higher MPR

- Keeping Pi/2 BPSK is FFS as the power may not be significantly higher than QPSK and MPR is the same.

| Modulation | Pi/2 PSK |  |  | QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0 | 5 | others | 0 | 5 | others |
| MPR | ≤ [1.5] dB | ≤ [0.5] dB | TBD | ≤ [2] dB | ≤ [0.5] dB | TBD |
| Tone positions for 3 Tones allocation | TBD | TBD | TBD | TBD | TBD | TBD |
| MPR | TBD | TBD | TBD | TBD | TBD | TBD |
| Tone positions for 6 Tones allocation | TBD | TBD | TBD | TBD | TBD | TBD |
| MPR | TBD | TBD | TBD | TBD | TBD | TBD |
| Tone positions for 12 Tones allocation | 0-11 |  |  | 0-11 |  |  |
| MPR | TBD |  |  | ≤ [5.5] dB |  |  |

# Conclusion

In this contribution, we have discussed the MPR and A-MPR for PC1.5 contiguous ULCA and made the following proposals.

Proposal for HD-FDD post PA losses: MPR for NB-IoT-NTN is based on a 3dB post-PA losses assumption.

Proposal for PC2 NB-IoT NTN:

- PC2 NB-IoT-NTN MPR is based on a PC3 NR PA (PC3 NR calibration) with 3dB post PA losses following the MPR/Boost table below to be completed for 3RE and 6RE, and other 1RE positions.

- FFS if 0.5dB optional boosting is specified

- Keeping Pi/2 BPSK is FFS as the power may not be significantly higher than QPSK and MPR is the same.

| Modulation | Pi/2 PSK |  |  | QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0 | 5 | others | 0 | 5 | others |
| MPR [/ boost option] | ≤ [0.5] dB | ≤ [0] dB / ≥ [0.5dB] | TBD | ≤ [0.5] dB | ≤ [0] dB / ≥ [0.5dB] | TBD |
| Tone positions for 3 Tones allocation | TBD | TBD | TBD | TBD | TBD | TBD |
| MPR | TBD | TBD | TBD | TBD | TBD | TBD |
| Tone positions for 6 Tones allocation | TBD | TBD | TBD | TBD | TBD | TBD |
| MPR | TBD | TBD | TBD | TBD | TBD | TBD |
| Tone positions for 12 Tones allocation | 0-11 |  |  | 0-11 |  |  |
| MPR | TBD |  |  | ≤ [3.5] dB |  |  |

Proposal for PC1 NB-IoT NTN for smartphone UEs:

- PC1 NB-IoT-NTN MPR is based on a PC2 NR PA (PC2 NR calibration) or GSM PA with 3dB post PA losses following MPR table below to be completed for 3RE and 6RE, and other 1RE positions.

- Non smartphone UEs without thermal, DC current or battery voltage range constraints can achieve 0.5dB higher MPR

- Keeping Pi/2 BPSK is FFS as the power may not be significantly higher than QPSK and MPR is the same.

| Modulation | Pi/2 PSK |  |  | QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0 | 5 | others | 0 | 5 | others |
| MPR | ≤ [1.5] dB | ≤ [0.5] dB | TBD | ≤ [2] dB | ≤ [0.5] dB | TBD |
| Tone positions for 3 Tones allocation | TBD | TBD | TBD | TBD | TBD | TBD |
| MPR | TBD | TBD | TBD | TBD | TBD | TBD |
| Tone positions for 6 Tones allocation | TBD | TBD | TBD | TBD | TBD | TBD |
| MPR | TBD | TBD | TBD | TBD | TBD | TBD |
| Tone positions for 12 Tones allocation | 0-11 |  |  | 0-11 |  |  |
| MPR | TBD |  |  | ≤ [5.5] dB |  |  |

# References

[1] R4-2505114 WF on HPUE RF requirements for IoT-NTN, Vivo, R4#114b

[2] R4-2504604 Initial power boosting result for NB-IoT-NTN for a PC2 NR PA, Skyworks Solutions Inc., R4#114b