3GPP TSG-RAN WG4 Meeting #115	 R4-2505851

Malta, 19th – 23rd May, 2025

Agenda Item:	7.11.7

Source:	Eutelsat Group

Title: 	RRM performance requirements for UEs with directional antenna

Document for:	Discussion and Decision

- Background

Rel-18 introduced FR2-NTN with VSAT UE (directional antenna) however, the RRM requirements for satellite switch with resynchronisation were not captured in the RRM specification.

The WID "Introduction of Ku bands for NR NTN" comprises provision for RRM work (fixed VSAT served by NGSO and GSO, mobile VSAT served by GSO) and was updated at RAN#106 [1] to include RRM support for mobile VSAT connectivity to NGSO and further updated at RAN#107 [2].

Issue 1-6: NR SAN FR2-NTN – NR SAN FR2-NTN Handover (section 6.1C.1.3)

Agreement:

- Reuse the same value of T_sat_beam as in Ka band for Ku band

- For VSAT UEs operating in FR1-NTN numerology, not specify the RRM requirements for TN-NTN mobility, inter-RAT mobility [or satellite switching with re-sync].

During RAN4#114-bis the applicability of “Satellite switch with re-synchronization” feature to VSAT type UE was questioned and suggested to be removed from the scope of Ku band RRM [3]. The agreements are as follows:

This contribution provides some further detail on why satellite switch with re-synchronization is important to satellite network operators and outlines the RRM specifications required to support this feature for UEs with directional antenna (e.g., Ku band VSAT UE).

- Discussion

3GPP TS 38.300 [4] section 16.14.3.2 provides the stage 2 description of satellite switch with re-synchronization (further referred to ‘Satellite Switch’ for convenience in this document). This procedure, specified in Rel-18, is highly desirable in any NTN network as it avoids the need for Layer 3 signalling with the associated overhead (“signalling storm”) when all devices in a Quasi-Earth Fixed Cell (QEFC) are required to simultaneously switch to a new satellite. This feature is as critical to a satellite network operator as handover.

Observation 1: For NGSO case the satellite switch procedure is required to prevent a layer 3 signalling storm when all users in a ‘cell’ need to change serving satellite simultaneously.

16.14.3.2.3          Satellite switch with re-synchronization

Upon both hard and soft satellite switch over in the quasi-Earth fixed scenario with the same SSB frequency and the same gNB, the satellite switch with re-synchronization procedure is supported. The satellite switch with re-sync avoids a L3 mobility for UEs in the cell by maintaining the same PCI on the geographical area covered by quasi-Earth fixed beam.

For soft satellite switch over, the UE can start synchronizing with the target satellite before the source satellite ends to serve the cell. It is not required for the UE to be connected to source satellite when the UE switches to target satellite.

For hard satellite switch over, the UE can only start synchronizing with the target satellite after the source satellite ends to serve the cell.

When both CHO and Satellite switch with re-synchronization are configured, it is up to UE implementation which procedure to initiate, if both of them are triggered simultaneously.

For the re-synchronization to the target satellite, random access can be triggered by a PDCCH order via the target satellite.

Satellite switch with re-synchronisation stage 2 description from TS 38.300 [4]

The satellite switch procedure is described below and illustrated for information in Figure 1 (adapted from [6]) which will now be briefly described. When enabled in the network, information required for satellite switch is provided in SIB 19 (satSwitchWithReSync) including ntn-Config (NTN configuration parameters for the new satellite), t-ServiceStart (time information on when the target [new] satellite is going to start serving the area) and ssb-TimeOffset (time offset of the SSB from target satellite – not illustrated). The parameter “t-Service” indicates when a cell provided via the serving [outgoing] satellite is going to stop serving the area – (in effect, “t-ServiceStop”) – this parameter is not specific to Satellite Switch but is a legacy NTN parameter which is used by the feature.

Figure 1: Hard/Soft satellite switch with re-synchronization processes

PCI: Physical Cell Identity / SR: Scheduling Request / TAT: Time Alignment Timer

The key points to note in Figure 1 and from the text extract from [4]:

- Satellite switch is inherently RACH-less; RACH provides a fall-back mechanism if a timeout occurs.

- Satellite switch can be applied together with conditional handover (CHO) schemes.

- Soft satellite switch requires coverage from the two satellites overlapping in time, and a UE capable of either synchronizing to the target satellite simultaneously to maintaining the connection to the source satellite, or able to fast switching between the two satellites: the UE starts synchronizing with the ‘new’ (target) satellite before the ‘outgoing’ (source) satellite stops serving the UE (T-Start < T-Service).

- Hard satellite switch procedure makes no assumption about the UE capability, it assumes one satellite in-view to the UE or no satellite coverage overlap in time; the UE starts synchronizing with the ‘new’ (target) satellite [only] after the ‘outgoing’ (source) satellite has stopped serving the UE (T-Start >=T-Service).

Observation 2: Hard satellite switch with re-synchronization may be supported by any NTN UE, including UE with single beam directional antennas, without restriction.

Observation 3: Soft satellite switch with re-synchronization may be supported by NTN UEs with a single beam directional antenna that can switch rapidly between two different satellites (source and target) and/or a by VSAT that supports more than one receive beam.

The present RRM specification [5] already incorporates 6.1C.3 NR SAN Satellite switching with re-synchronization. The text in section 6.1C.3.2 that follows only specifies NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization.

Observation 4: Satellite switching with re-synchronization is already specified for FR-1 NTN.

The scenario where multiple VSAT UE in the same cell need to switch between two satellites is depicted in Figure 2. In this scenario, UE with directional antenna (“spatial filter”), an allowance should be made for receive antenna switching time. For an electronically steered antenna (ESA);  the text in Section 6.1C.1.3 of [5] uses term “TSat_beam” to refer to this time. In practice, ESA beam switching time is typically a few 100 microseconds, and can be assumed to be less than 1 ms. Note: the VSAT UE would not be required to transmit to more than one satellite during the coverage overlap period.

The agreement at RAN4#109 [7], to set “TSat_beam” = 3*Trs (where Trs is as defined in [5] clause 6.1C.1.3.2 Interruption time) for ESA can be applied to the satellite switch procedure; the rationale for setting the time “TSat_beam” for handover also applies in this case. In the case of ESA, the 3*Trs value provides time for the UE to make adjustments to the peak of beam direction due to any difference in calculated angle vs. actual angle due to UE, satellite location uncertainty and/or steering angle uncertainty.

Observation 5: The value of TSat_beam agreed in the context of handover, presently used in clause 6.1C.1.3 of [5], is suitable for use with the satellite switch with resynchronization procedure.

Figure 2: Satellite switch scenario for UE with directional antenna

A possible approach to the RRM specification and example network implementation to support both ‘soft’ and ‘hard’ switch is outlined below.

- For VSAT UE that support Rel-18 capabilities: hardSatelliteSwitchResyncNTN-r18 or softSatelliteSwitchResyncNTN-r18 the existing FR1 procedures can be adapted to allow for UE antenna (spatial filter) repointing time.

Observation 6: The SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization provides a sound basis for introducing the same procedure for FR2-NTN case

Proposal 1: Introduce a new section, based upon 6.1C3.2 NR SAN FR1 – NR SAN FR1 into [5] 6.1C.3.3 NR SAN FR2-NTN – NR SAN FR2-NTN Satellite switching with re-synchronization to define the core requirement for both hard and soft satellite switch.

A draft text proposal for a new section is provided in Annex A for further discussion (e.g., if mechanical antenna should be included in ‘soft switch’).

- Conclusion

This contribution has shown that the satellite switch with re-synchronization (‘satellite switch’) procedure is applicable to Ku band and VSAT UE in general. A VSAT type employing ESA capable of fast receive beam switching is capable of supporting soft switch.

Observation 1: For NGSO case the satellite switch procedure is required to prevent a layer 3 signalling storm when all users in a ‘cell’ need to change serving satellite simultaneously.

Observation 2: Hard satellite switch with re-synchronization may be supported by any NTN UE, including UE with single beam directional antennas, without restriction.

Observation 3: Soft satellite switch with re-synchronization may be supported by NTN UEs with a single beam directional antenna that can switch rapidly between two different satellites (source and target) and/or a by VSAT that supports more than one receive beam.

Observation 4: Satellite switching with re-synchronization is already specified for FR-1 NTN.

Observation 5: The value of TSat_beam agreed in the context of handover, presently used in clause 6.1C.1.3 of [5], is suitable for use with the satellite switch with resynchronization procedure.

Observation 6: The SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization provides a sound basis for introducing the same procedure for FR2-NTN case

Proposal 1: Introduce a new section, based upon 6.1C3.2 NR SAN FR1 – NR SAN FR1 into [5] 6.1C.3.3 NR SAN FR2-NTN – NR SAN FR2-NTN Satellite switching with re-synchronization to define the core requirement for both hard and soft satellite switch.

To sum-up, satellite switch with re-synchronization is a critical feature, as important as handover, as it provides a mechanism for moving multiple users without layer 3 signalling in a quasi-fixed earth system context.

# Annex A: Draft text proposal

The highlighted parts of the text represent the changes compared with the equivalent clauses for FR1.

<Start of Text Proposal>

#### 6.1C.3.3	NR SAN FR2-NTN – NR SAN FR2-NTN Satellite switching with re-synchronization

The requirements in this clause are applicable to both hard and soft switch over quasi-earth fixed scenario from NR SAN FR2-NTN cell to NR SAN FR2-NTN cell. The UE is assumed to be a VSAT with directional antenna which acts as a spatial filter. The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target NR SAN cell during Dswitch_unchangedPCI, otherwise interruption time may be longer than the requirements in clause 6.1C.3.2.2 for hard satellite switch and satellite switch delay may be longer than the requirements in clause 6.1C.3.2.3 for soft satellite switch.

Requirements for soft satellite switching are applicable for UEs that support softSatelliteSwitchResyncNTN-r18 [14] when network configures soft satellite switching with resynchronization [2].

Requirements for hard satellite switching are applicable for UEs that support hardSatelliteSwitchResyncNTN-r18 or softSatelliteSwitchResyncNTN-r18 [14] when the network configures hard satellite switching with resynchronization [2]; or for UEs that support hardSatelliteSwitchResyncNTN-r18 but do not support softSatelliteSwitchResyncNTN-r18 when the network configures soft satellite switching with resynchronization.

##### 6.1C.3.3.1	Satellite switching delay

When the UE receives a broadcast message implying satellite switching within re-synchronization, the UE shall be ready to start the transmission of the new uplink PRACH channel or transmission of the new uplink PUSCH channel within Dswitch_unchangedPCI msec.

Where:

-	Dswitch_unchangedPCI equals to the interruption time stated in clause 6.1C.3.2.2 for hard satellite switch and satellite switch delay stated in clause 6.1C.3.2.3 for soft satellite switch.

##### 6.1C.3.3.2	Interruption time for hard satellite switch with re-sync

The interruption time is the time between t-service and the time when the UE is ready to receive any DL channel/signal and transmit any UL channel/signal from/to the target satellite .

When intra-frequency hard switch to NR SAN cell is commanded,

the interruption time shall be less than Tinterrupt

Tinterrupt = TSat_beam  + Tsearch + Tprocessing + T∆ + Tmargin ms

Otherwise, no interruption time requirement is applied.

Where:

-	Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = [Tfirst_SSB] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	TSat_beam is additional time for UE to steer the downlink spatial domain reception filter to the target cell.

-	For UE indicating ‘electronic’ via UE capability ntn-VSAT-AntennaType-r18, Tsat_beam is 3*Trs

-	For UE indicating ‘mechanical’ via UE capability ntn-VSAT-AntennaType-r18, Tsat_beam is Oangle / 22.5 s, where Oangle is the angle offset observed from UE in degree between the satellite for the serving cell and the satellite for the target cell.

-	T∆ is same as the one defined in section 6.1C.1.2.2.1.

-	Tprocessing is time for UE processing. Tprocessing can be up to 10 ms.

-	Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms.

-	Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, and if ssb-TimeOffset is configured, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the SSB-TimeOffset reference point as defined in TS 38.331 [2] to UE.

UE is allowed to skip measurements for other cells and satellites than the target satellite and source satellite from T-service until the satellite switch completion.

##### 6.1C.3.3.3	Satellite switch delay for soft satellite switch with re-sync

The Satellite switch delay is from t-serviceStart to the time instance when UE is ready to receive any DL channel/signal and transmit any UL channel/signal from/to the target satellite.

When intra-frequency soft switch to NR SAN cell is commanded,

the satellite switch time shall be less than Tsoft_switch

Tsoft_switch = max(t-service-t-serviceStart, TSat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

Where:

-	Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when the handover command is received by the UE. If the target cell Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	T∆ is same as the one defined in section 6.1C.2.2.2.1.

-	Tprocessing is time for UE processing. Tprocessing can be up to 10 ms.

-	Tmargin is same as the one defined in section 6.1C.2.2.2.1.

-	Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

-	TSat_beam is additional time for UE to steer the downlink spatial domain reception filter to the target cell.

-	For UE indicating ‘electronic’ via UE capability ntn-VSAT-AntennaType-r18, Tsat_beam is 3*Trs

[-	For UE indicating ‘mechanical’ via UE capability ntn-VSAT-AntennaType-r18, Tsat_beam is Oangle / 22.5 s, where Oangle is the angle offset observed from UE in degree between the satellite for the serving cell and the satellite for the target cell.]

During the time period from t-serviceStart to t-service, scheduling restriction as defined in clause 9.2C.5.3 is allowed, with the exception that the locations of SSB symbols of target satellite where scheduling restriction applies are determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the SSB-TimeOffset reference point as defined in TS 38.331 [2] to UE.

UE is allowed to skip measurements for other cells and satellites than the target satellite and source satellite from t-serviceStart to the satellite switch completion.

The requirement in this clause applies and UE is required to keep the connection (DL and UL) with the source NGSO satellite, under the following conditions:

-	SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side.

<End of Text Proposal>

Note: Parameter should be t-serviceStart not t-seviceStart (missing ‘r’ in service’); the ‘typo’ occurs in three places in clause 6.1C.3.2.3.

# 4	References (Sections 1-3)

[1] RP-243314 “Revised WID: Introduction of Ku bands for NR NTN”, RAN#106 Madrid (December 2024)

[2] RP-250799 “Revised WID: Introduction of Ku Bands for NR NTN”, RAN #107, Incheon (March 2025)

[3] R4-2504897, WF on RRM requirements for NR_NTN_Ku_bands, (April 2025).

[4] TS 38.300, "NR; Overall description; Stage 2" - v18.5.0 (March 2025)

[5] TS 38.133 "NR; Requirements for support of radio resource management" - v19.0.0 (April 2025)

[6] R2-2312279, “Major issues for satellite switch with PCI unchanged”, Qualcomm Corporation, RAN2 #124 (November 2023).

[7] R4-2318182 “Topic summary for [109][226] NR_NTN_enh”, Moderator (Qualcomm Incorporated), RAN4#109, Chicago (November, 2023)