3GPP TSG-RAN WG4 Meeting #115				R4-2505890

Malta, MT, 19th – 23rd May, 2025

Agenda item:	8

Source: 	Apple

Title: 	Views on Rel-19 RAN4 UE feature list for NR

Document for:	Discussion

- Introduction

In this document, we share our initial views on Rel-18 NR UE features.

- NR_ENDC_RF_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | Support of CBW extension based MPR reduction | Indicates support of CBW extension - Extension is fixed to 0.5 CBW/NRB - Extension can be symmetric or asymmetric |  | Yes |  | The feature cannot be supported. | Per Band | No | FR1 only |  |  | Optional with capability signaling |
| 46. NR_ENDC_RF_Ph4 | 46-2 | Support of CBW extension based MPR reduction | Indicates support of CBW extension - Size of extension is flexible and based on limited number of fixed values - Extension can be symmetric or asymmetric |  | Yes |  | The feature cannot be supported. | Per Band | No | FR1 only |  | TBD values of CBW extension size | Optional with capability signaling |
| 46. NR_ENDC_RF_Ph4 | 46-3 | Support of 6 MIMO layers in the downlink | Indicates support of 6 MIMO layers in the downlink for FWA devices only. |  | Yes |  | The feature cannot be supported. | Per UE | No | FR1 only |  | It is for FWA devices only. | Optional with capability signaling |

- NonCol_intraB_ENDC_NR_CA_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NonCol_intraB_ENDC_NR_CA_Ph2 | 47-1 | Support of intra-band non-collocated NR CA operation | Indicates the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table x.x.x.x/Table x.x.x.x in 38.133 and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table x.x.x.x in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1. | 33-1 (RAN4 R18 feature) | yes |  | The feature cannot be supported. | Per BC | N/A | FR1 only for Band n77/n78 |  | The UE supports up to maximum 8 layers per CC in collocated condition and maximum 4 layers per CC in non-collocated condition. | Optional with capability signaling |
|  | 47-2 | Support of intra-band non-collocated EN-DC operation | Indicates the UE supports TDD-TDD inter-band non-collocated EN-DC operation with MTTD/MRTD requirements according to Table x.x.x.x/Table x.x.x.x in 38.133 and UE RF requirements for inter-band non-collocated EN-DC including 7.10B in 38.101-3. And the UE also supports TDD-TDD inter-band EN-DC operation with MRTD according to Table x.x.x.x in 38.133 and UE RF requirements for inter-band EN-DC except for 7.10B in 38.101-3. | 33-2 (RAN4 R18 feature) | yes |  | The feature cannot be supported. | Per BC | N/A | FR1 only for Band n77/n78 |  | The UE supports up to maximum 6/8 layers per CC in collocated condition and maximum 4 layers per CC in non-collocated condition. | Optional with capability signaling |

- NR_ATG_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_ATG_enh | 48-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_RRM_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-1 | Fast Rx beam sweeping for FR2-1 L3 measurement delay reduction | - Supports beam sweeping factor reduction for L3 measurement for FR2-1. |  | Yes | N/A |  | Per band | TDD only | FR2-1 only |  | Candidate values: {2,4,6} for FR2-1 Note: It is only supported for power class 3. | Optional with capability signalling |
| 49. NR_RRM_Ph5 | 49-2 | One serving carrier measurement per-band for intra-frequency measurements without measurement gap | - Support of measuring serving cell and neighbor cells on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured for measurement in a same band |  | Yes |  | UE does not support measuring serving cell and neighbor cells on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured for measurement in a same band. | Per-UE | No | Yes |  | UE is required to meet the serving cell and neighbor cells measurement requirements in TS38.133 [section 9.2.3.2, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, and 9.1.5.1.4] if the feature is supported. | Optional with capability signaling |
| 49. NR_RRM_Ph5 | 49-3 | Parallel measurement on three serving carriers for intra-frequency measurements without measurement gap | Support of measuring serving cell and neighbor cells on three serving carriers for intra-frequency measurements without measurement gap. - There are separated indications of supporting this feature for different cases: {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, and {FR1+FR2 NR-DC}. |  | Yes |  | UE does not support measuring serving cell and neighbor cells on three serving carriers for intra-frequency measurements without measurement gap. | Per-UE | No | Yes |  | UE is required to meet the serving cell and neighbor cells measurement requirements in TS38.133 [section 9.1.5.1.1, 9.1.5.1.2, and 9.1.5.1.3] if the feature is supported. | Optional with capability signaling |
| 49. NR_RRM_Ph5 | 49-4 | Fast SCell activation based on early measurement report | Supports fast SCell activation specified in TS38.133 clause 8.3.2A | 39-8 or 39-9 in R18 RAN4 feature list | No | N/A | UE cannot meet fast SCell activation requirements as specified in TS38.133 clause 8.3.2A | Per-UE | No | No |  |  | Optional without capability signaling |

- NR_demod_Ph5-Perf

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_demod_Ph5-Perf | 50-1 |  |  |  |  |  |  |  |  |  |  |  |  |

No new UE feature is expected from NR_demod_Ph5 WI

- NR_MIMO_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_MIMO_Ph5 | 51-1 |  |  |  |  |  |  |  |  |  |  |  |  |

TBD if RAN4 capability is needed for RRM.

- Netw_Energy_NR_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. Netw_Energy_NR_enh | 52-1 | OD-SSB measurement | Lower bound for measurement in FMW. | TBA | Yes | N/A | The lower bound is not needed in minimum measurement interval |  | No | No |  | A lower bound is needed and based on OD-SSB periodicity of [10] ms | Optional with capability signaling |
|  | 52-2 | OD-SSB activation | Fast additional processing timing for OD-SSB RRM measurement | TBA | Yes | N/A | UE needs 5ms as baseline. |  | No | No |  | Candidate values: [2 or 3]ms. | Optional with capability signaling |

- NR_LPWUS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53.NR_LPWUS | 53-1 | Wake-up receiver | - Indicating support to receive wake-up signals on a certain band - Indicating capability: Type 1 or Type 2 |  | Yes |  | The feature cannot be supported. | Per Band | No | Yes |  |  | Optional with capability signaling |

- NR_Mob_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_Mob_Ph4 | 54-1 | Number of CSI-RS resources for L1-RSRP measurement within a slot | - The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for L1-RSRP measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the number of CSI-RS resources for L1 measurement within a slot. | Per BC | No | Yes | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} Note: It is also counted in FG 2-24 | Optional with capability signaling |
|  | 54-2 | Number of total CSI-RS resources to be measured | - The max number of total CSI-RS resources of serving cells and neighboring cells across all frequency layers for L1 measurement. Both LTM candidate cell(s) and serving cells for CSI-RS based L1-RSRP measurement are counted, regardless of whether CSI-RS based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the total number of CSI-RS resources of serving cells and neighboring cells across all frequency layers for L1 measurement. | Per BC | No | No | N/A | Candidate values: {2,4,8,12,16,32,64} Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
|  | 54-3 | Skipping SSB based L1 measurement in FR2-1 | - Support of skipping SSB based L1 measurement for FR2-1 for CSI-RS based L1 measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | N/A | The network cannot have optimized measurement configuration | Per UE | no | FR2-1 only | N/A | - For a UE not supporting the capability, SSB based L1 measurement is necessary for FR2-1. - The CSI-RS resources for L1 measurement should be at least Type-D QCL’ed with the SSB for L1 measurement. - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped: - The CSI-RS resources for L1 measurement should be Type-D QCL’ed with the SSB for L3 measurement. - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor. | Optional with capability signalling |

- NR_XR_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55.NR_XR_Ph3 | 55-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_NTN_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56.NR_NTN_Ph3 | 56-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_FR1_lessthan_5MHz_BW_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57. NR_FR1_lessthan_5MHz_BW_Ph2 | 57-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_PC2_RedCap_UE

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_PC2_RedCap_UE | 58-1 |  |  |  |  |  |  |  |  |  |  |  |  |

No capability is needed.

- NR_LBCA_Sw

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_LBCA_Sw | 59-1 | Switching period for switching between a LB FDD band and an SDL band | UE to indicate support of LB-LB carrier aggregation via switching between a LB FDD band and an SDL band. Switching_Period_For_FDD-SDL indicates the length the switching time between the FDD band and the SDL band: 35us represents 35 us of switching time, 140us represents 140us of switching time, as specified in TS 38.101-1. |  |  |  | Operation of Carrier aggregation between the FDD and the SDL via switching is not supported | per band pair per band combination |  | Applicable to FR1 only |  |  | Optional with capability signalling |

- NR_FR1_7MHz_BW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_FR1_7MHz_BW | 60-1 | UE support of 7MHz CBW | Indicates UE support of 7MHz CBW |  | Yes |  | Network does not know if UE supports 7MHz CBW | Per UE |  | Applicable to FR1 only |  |  | Optional with capability signalling |

- NR_IoT_NTN_req_test_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. NR_IoT_NTN_req_test_enh | 61-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_AIML_air

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_AIML_air | 62-1 |  |  |  |  |  |  |  |  |  |  |  |  |

As RAN1 feature list discussion is ongoing, it remains to be seen if any RAN4 capability is needed.

- NR_duplex_evo

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_duplex_evo | 63-1 |  |  |  |  |  |  |  |  |  |  |  |  |

In RRM session, it is assumed that for DUD case with method #1 and resource type#2, if UE can support simultaneous PDSCH reception on the two DL subbands for DUD, it can support simultaneous L1-CLI-RSSI measurement on the two DL subbands based on 1 sample. So the need of a RAN4 capability depends on RAN1 capability discussion.