# 3GPP TSG-RAN WG4 Meeting #115	R4-2505936

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.18.3.2

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#114bis, and the open issues have been summarized in the topic summary for [114bis][208] NR_RRM_Ph5_Part1 [2].

In this contribution, we continue the discussion for FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

# Discussion

Issue 1-2-1: revision on solution 1 (measure one serving CC per band)

“Network indication of a specific SCC per-band for measurement” is Option 1 (Apple, HW, ZTE, CATT): dynamic indication based on MAC CE Option 2 (QC, OPPO): semi-static indication based on RRC signaling Option 3 (ZTE, Nokia, Ericsson, MTK): leave to RAN2 to decide.

The agreement for solution 1 was:

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap.

In the previous agreements, which kind of network indications can be assumed are still unclear, e.g., dynamic indication or semi-static indication. In our view, the indication to enable this CSSF enhancement can be a semi-static indication, since it will not change that often after configuring to UE. However, for network indication to measurement one SCC, we think the benefit for this R19 CSSF enhancement is obvious if dynamic indication can be used, e.g., MAC CE. It means network can configure multiple MOs for different SCCs with reporting to UE during SCell additions, and dynamic indication can allow network to guide UE measurement on different SCCs from time to time without RRC reconfiguration. The main motivation for switching the to-be-measured SCC dynamically is to monitor the interference level change (e.g., RSRQ) among the different SCCs for future SCell activation/deactivation. However, the legacy R15 spec requires UE to keep monitoring all the SCells without any priority, and that will cause unnecessary measurement delay(due to the searcher limitation) and UE power consumption. Thus, here, to balance the UE measurement load/delay and the efficient SCell activation/deactivation due to interference level change on different SCCs, we prefer to use dynamic network indication of a specific SCC per-band for measurement.

Proposal 1: network indication to enable/disable this CSSF enhancement of solution 1 can be semi-static while network indication of a specific SCC per-band for measurement can be dynamic.

Issue 1-2-2: Expand solution 1 to FR1 case

[Way forward]: - FFS: - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band. - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

Based on the agreement for solution 1 in RAN4#114 meeting, we prepared one draft CR of CSSF enhancement with solution 1 in this meeting [4].

In addition, RAN4 agreed in #113 meeting that, after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable. In our view, the solution 1 is controlled by network, e.g., enable such CSSF enhancement and indicate which SCC to be measure in certain band, and therefore it would be also beneficial to expand it to FR1 case as well. If the channel conditions are quite different from network perspective and individual measurement shall be performed on all CCs in the same band, network can also disable such feature to UE and UE will follow the Rel-15 spec to perform serving and neighbor cell measurements. Based on the discussion in last meeting, we also agree that the intra-band contiguous case would be the most typical scenario to apply solution 1 for FR1 due to the channel similarity. Thus, we propose to expand the solution 1 to FR1 intra-band contiguous CA.

Proposal 2: expand the solution 1 to FR1 intra-band contiguous CA.

Issue 1-4-1: UE capability for CSSF enhancement

In last RAN4 meeting, it was agreed that,

Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Regarding the UE capability of solution 1, we assume UE needs a separated CSSF enhancement capability from the 3 searcher solution, and if both FR1 and FR2 are considered for solution 1, we believe it would be more flexible to allow UE indicating a per-FR capability since FR1 and FR2 probably has quite different architecture and design on the chipset. Thus, the solution 1 capability can be designed as an optional per-FR capability.

Proposal 3: to have a new optional per-UE capability which can be differentiated between different FRs for CSSF enhancement solution 1 in Rel-19.

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC)

[Way forward]: - FFS: - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF. - Note: 3-searchers will be used by UE if UE report this capability. - FFS: Other restriction (triggered by Huawei): - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple) - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple) - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT) - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

For how to use 3 searcher, we think the current option 3 is the easiest way for speeding up the RRM measurement. Even though it cannot enhance a specific SCC measurement, the baseline gain is reduce the measurement delay for each SCC by half. For instance, in FR1 NR-DC case that UE has PCC+PSCC+3SCCs, the CSSF for SCC measurement is 2*3, and option 3 can improve the CSSF to 3. In [3], we also had option 1 and 2 on the table,

Discussion: - RAN4 to discuss the detailed CSSF design: - Option 1: - The 3-searcher solution is PCC occupied one searcher and all SCC share two searchers. - Option 2: - For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - Use a dedicated searcher (the 3rd searcher) for speeding up the prioritized carriers(s), which is provided by NW. - Option 3: - For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing.

If option 2 is used, it will make the allocation mechanism very complicated. When it allocates a dedicated searcher for a specific SCC, then the resource of other SCCs needs to be reallocated, e.g., in the above example, the target SCC CSSF will be 1, PSCC will be 2, and other 2SCCs will be 2*2; and when considering other scenarios, it will be different from scenario to scenario. From our perspective, we prefer a general solution for all scenario to limit the UE complexity.

Proposal 4: Option 3 in WF R4-2502592 is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

- Note: 3-searchers will be used by UE if UE report this capability.

Regarding the other restriction, some companies thought like in R15, the sections of CSSF outside MG and CSSF inside MG may be mixed together to interpret that UE can have more than 3 searchers in total, e.g., FR1 measurement with 3 searcher outside MG is colliding on time domain with FR2 gap based measurement. Even though we don’t agree R15 means we can put these two sections together to derive 3 searchers, we still think such restriction clarification can help us to avoid the case that requires UE to support more than 3 searchers. Thus, we support option 3.

Proposal 5: UE is able to support per-FR measurement gaps and new 3-searcher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR.

# Detailed design for capability of CSSF enhancement

In previous meeting, the agreements for solution 1 and solution 3 under CSSF enhancement were:

Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19. Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Based on the template shared by moderator of the feature list thread, the following detailed capability design is proposed as in the annex of this paper.

Proposal 6: the detailed design for capability of CSSF enhancement can be referred to annex in this paper.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: network indication to enable/disable this CSSF enhancement of solution 1 can be semi-static while network indication of a specific SCC per-band for measurement can be dynamic.

Proposal 2: expand the solution 1 to FR1 intra-band contiguous CA.

Proposal 3: to have a new optional per-UE capability which can be differentiated between different FRs for CSSF enhancement solution 1 in Rel-19.

Proposal 4: Option 3 in WF R4-2502592 is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

- Note: 3-searchers will be used by UE if UE report this capability.

Proposal 5: UE is able to support per-FR measurement gaps and new 3-searcher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR.

Proposal 6: the detailed design for capability of CSSF enhancement can be referred to annex in this paper.

# References

[1] R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#114bis

[2] R4-2503618, Topic summary for [114bis][208] NR_RRM_Ph5_Part1, Moderator (Apple), RAN4#114bis

[3] R4-2504916, Ad-hoc minutes for NR_RRM_Ph5, Apple, RAN4#114bis

[4] R4-25xxxxx, Draft CR for Rel-19 CSSF enhancement solution 1, Apple, CATT, Huawei, HiSilicon, Qualcomm, Ericsson, ZTE, MediaTek, Samsung, RAN4#115

# Annex – UE capability of CSSF enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | One serving carrier measurement per-band for intra-frequency measurements without measurement gap | Support of measuring serving cell and neighbor cells on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured for measurement in a same band |  | Y |  | UE does not support measuring serving cell and neighbor cells on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured for measurement in a same band. | Per-UE | No | Yes |  | UE is required to meet the serving cell and neighbor cells measurement requirements in TS38.133 [section 9.2.3.2, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, and 9.1.5.1.4] if the feature is supported. | Optional with capability signaling |
| 49. NR_RRM_Ph5 | 49-y | Parallel measurement on three serving carriers for intra-frequency measurements without measurement gap | Support of measuring serving cell and neighbor cells on three serving carriers for intra-frequency measurements without measurement gap. There are separated indications of supporting this feature for different cases: {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, and {FR1+FR2 NR-DC}. |  | Y |  | UE does not support measuring serving cell and neighbor cells on three serving carriers for intra-frequency measurements without measurement gap. | Per-UE | No | Yes |  | UE is required to meet the serving cell and neighbor cells measurement requirements in TS38.133 [section 9.1.5.1.1, 9.1.5.1.2, and 9.1.5.1.3] if the feature is supported. | Optional with capability signaling |