3GPP TSG-RAN WG4 Meeting #115           	R4-2505979

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.31.7

Source:	Samsung

Title:	View on demodulation requirement for NR NTN

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WI was endorsed to enhance the NTN for NR with the following objectives， which can be categorized as following

- Objective 1: Downlink coverage enhancements

- Objective 2: Uplink Capacity/Cell Throughput Enhancement

- Objective 3: Signalling of intended service area of a broadcast service

- Objective 4: Support of regenerative payload

- Objective 5: Support RedCap UEs with NR NTN operating in FR1-NTN bands

As stated in the WID,  RAN4 needs to specify the necessary performance requireemnts for enhancements listed above. According to meeting agenda, this is the 1st meeting to discuss the performance part of NR NTN phase 3. In this contribution, the overall view on the test scope of demodulation requirement for NR NTN phase 3 is provided.

# 2	Test Scope

2.1	Objective 1: Downlink coverage enhancements

The detailed objective 1 as following

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Notes for this objective - SSB channel enhancement for improved link level performance is not considered - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO set-1 @600km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

DL coverage enhancements are needed to accommodate satellite payload constraints which may be unable to have all its beams active with the EIRP density per beam at a given time due to limited power and limited feeder link bandwidth. The objective is mainly including link level enhancements for FR1-NTN related PDCCH and PDSCH. Based on RAN1 evaluation, both PDSCH and PDCCH inter-slot repetition are supported. There is a performance impact from UE side.

Proposal 1: For DL coverage enhancement, introduce PDSCH and PDCCH demodulation requirement in both FR1-NTN and FR2-NTN.

2.2	Objective 2: Uplink Capacity/Cell Throughput Enhancement

The detailed objective 2 as following

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Down-select and specify one of the following Orthogonal Cover Codes (OCC) techniques to enhance DFT-s-OFDM PUSCH: - Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 or 4 - Inter-symbol(s) time domain OCC with OCC length 2 or 4 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4 - Notes for this objective - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

The coverage of NTN satellites is very wide, and considering device density, it is expected that a large number of UEs will be within a satellite’s coverage. To improve the Uplink capacity, OCC techniques has been introduced. Based on RAN1 agreement, inter-slot OCC with OCC length 2 was supported. Therefore, additional block spread processing is required after DFT processing. Therefore, there is performance impact for BS side. It is straightforward to introduce PUSCH demodulation performance requirements to support OCC processing.

Proposal 2: For Uplink Capacity enhancement, introduce PUSCH demodulation performance to support OCC processing based DFT-s-OFDM waveform

2.3	Objective 3: Signalling of intended service area of a broadcast service

The detailed objective 3 as following

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN [RAN3]

This objective is related with signaling indicate the intended service area. Based on WID, there is no RAN4 impact. Therefore, there is no performance impact foreseen for both BS side and UE side.

Proposal 3: There is no performance impact for objective 4: “Signalling of intended service area of a broadcast service”

2.4	Objective 4: Support of regenerative payload

The detailed objective 4 as following

- Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3, RAN2] - Note: if any additional necessary stage-3 specifications impact for e.g., NGAP is identified, RAN3 will handle it.

Support of non-terrestrial network architecture with 5G system functions on board the NTN vehicle (i.e. regenerative payloads) provides new architecture option(s) besides the transparent payload as specified in Rel-17 and Rel-18, which makes the deployment of non-terrestrial network more flexible. Support of regenerative payload brings some benefits on radio resource handling in Uu, and radio resource coordination between the gNBs via the ISL. For the support of real time connectivity between 2 UEs and between network and UE via the space segment with/without ISL, a regenerative payload (5G system functions on board satellite) is required.

This is no performance impact foreseen from both BS side and UE side

Proposal 4: There is no performance impact for objective 4: “Supporting of regenerative payload”

2.5	Objective 5: Support RedCap UEs with NR NTN operating in FR1-NTN bands

The detailed objective 5 as following

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1 - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs - Specify the processing rules to mitigate issues caused by TA mismatch at least for the following collision cases [RAN1]: - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Define the RF and RRM requirements for UE [RAN4] - Notes for this objective - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

The support of RedCap devices (e.g. handheld and IoT) operating in FR1 band NR-NTN networks can offer enhanced service capabilities (wideband/broadband) compared to IoT-NTN while ensuring low-complexity devices. Global coverage would clearly benefit RedCap devices. RF, RRM and demodulation requirements were defined for RedCap devices only for terrestrial networks in releases 17 and 18. In general, it is straightforward to introduce PDSCH demodulation performance requirements for RedCap devices to support NTN scenarios. For FR1 NR NTN, considering a UE to support NTN feature should meet the requirement defined for TN and NTN, specified in 38.101-4 and 38.101-5, respectively. Considering there is no enhancement compared with TN scenario, RAN4 only define limited PDSCH performance requirement for NTN scenario.  For RedCap devices, considering the test coverage, whether to introduce new demodulation requirements for RedCap devices need further discussion.

Proposal 5: FFS to introduce new demodulation requirements for RedCap devices to support NTN scenario.

# 3	Test Setup

In this section, the initial test setup for SAN demodulation requirement related with Uplink Capacity enhancement

PUSCH repetition type

For UL capacity enhancement, it is mainly based on repetition transmission scheme.  In NR, Both PUSCH repetition type A and type B are supported. RAN4 has already introduced the repetition type A in Rel-16 URLLC WI and also requirement in NTN scenario. Given repetition type A is a typical configuration in the field network, we prefer to only consider repetition type A for specifying PUSCH requirement with OCC.

Proposal 6: Only consider repetition type A for specifying PUSCH requirement with OCC

OCC length

Both OCC length 2 and OCC length 4 are supported for multiple UE multiplexing. To simply the test and align the we prefer to only consider OCC length as 2 for specifying PUSCH requirement with OCC.

Proposal 7:  Only consider OCC length as 2 for specifying PUSCH requirement with OCC

RV version

According to RAN1 agreement as following

118bis agreement - For RV cycling for OCC with PUSCH - For inter-slot OCC for OCC length 2 and for inter-slot OCC length 4 in option 1 - Same RV value is used in one OCC group 119 agreements - For RV cycling for OCC with DG-PUSCH, the following are considered - Option 1: RV cycling is used across OCC groups - Note 1: RV cycling is applied when the number of repetitions is greater than OCC length

According to RAN1 agreement, if the OCC length is same as number repetitions, only one OCC group is available with same RV value. If the number of repetitions is greater than OCC length, RV cycling is applied. The number of OCC groups depends on the number of repetitions.

Proposal 8: Consider one OCC group as starting point, the initial RV value can be configured as 0. FFS on considering two OCC groups with number of repetitions as 2 to verify the RV cycling function across OCC groups, the initial RV value for two OCC groups as 0,2.

Number of repetitions

Regarding the number of repetitions, up to 32 number of repetitions can be supported. In current NTN, 2 is configured to specify the requirement for PUSCH repetition for both 15KHz and 30KHz SCS. And 4 and 8 slots are configured for requirements for PUSCH with joint channel estimation, in 15KHz and 30KHz, separately.  As staring point, we prefer to configure number of repetitions as 2. While we are also open whether other values will be considered, in order to verify the RV cycling across OCC groups as following

Proposal 9:  Configure number of repetitions as 2 for staring point, FFS to configure number of repetitions as 4 to verify the RV cycling function across OCC groups

BW

Regarding the BW, similar as the configuration for PUSCH requirement with DFT-s-OFDM waveform, we prefer to only consider the minimum number of channel bandwidth for each SCS.

Proposal 10: Only consider the minimum number of channel bandwidth for each SCS for specifying PUSCH requirement with OCC

- 15KHz, 5MHz

- 30KHz, 10MHz

Antenna configuration

In Rel-17 FR1-NTN, both 1Rx and 2Rx are considered for specifying PUSCH requirement with test applicability rule introduced. Similarly, we prefer to consider both 1Rx and 2Rx for PUSCH requirement with DFT-s -OFDM waveform.

Proposal 11: Consider both 1Rx and 2Rx for specifying PUSCH requirement with OCC with test applicability rue for different Rx number

MCS for two UEs

For MCS, although there is no limitation of MCS scheduling for two UEs, we prefer to configure the same MCS for two UEs

Proposal 12: Consider the same MCS configured for two UEs for specifying PUSCH requirement with OCC

Channel Model for two UEs

For channel model, we prefer to configure the same channel model for two UEs.

Proposal 13: Consider the same channel model configured for two UEs for specify PUSCH requirement with OCC

Frequency offset Assumption

One of key factor to enable inter-slots OCC is that UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. Based on RAN1 assumption, we prefer to configure the frequency offset is 0Hz for two UEs. While we are also open to further discuss the frequency offset assumption.

Proposal 14: Consider there is no frequency offset configured for two UEs as baseline assumption, FFS on frequency offset assumption.

Others

For other simulation, the existing test parameters for PUSCH requirement with DFT-s-OFDM waveform can be considered as reference.

Proposal 15: Considering the existing test parameters for PUSCH with DFT-s-OFDM waveform requirement as reference.

# 4	Conclusion

In this contribution, the view on the demodulation requirement for NR NTN enhancement is provided

Proposal 1: For DL coverage enhancement, introduce PDSCH and PDCCH demodulation requirement in both FR1-NTN and FR2-NTN.

Proposal 2: For Uplink Capacity enhancement, introduce PUSCH demodulation performance to support OCC processing based DFT-s-OFDM waveform

Proposal 3: There is no performance impact for objective 4: “Signalling of intended service area of a broadcast service”

Proposal 4: There is no performance impact for objective 4: “Supporting of regenerative payload”

Proposal 5: FFS to introduce new demodulation requirements for RedCap devices to support NTN scenario.

Proposal 6: Only consider repetition type A for specifying PUSCH requirement with OCC

Proposal 7:  Only consider OCC length as 2 for specifying PUSCH requirement with OCC

Proposal 8: Consider one OCC group as starting point, the initial RV value can be configured as 0. FFS on considering two OCC groups with number of repetitions as 2 to verify the RV cycling function across OCC groups, the initial RV value for two OCC groups as 0,2.

Proposal 9:  Configure number of repetitions as 2 for staring point, FFS to configure number of repetitions as 4 to verify the RV cycling function across OCC groups

Proposal 10: Only consider the minimum number of channel bandwidth for each SCS for specifying PUSCH requirement with OCC

- 15KHz, 5MHz

- 30KHz, 10MHz

Proposal 11: Consider both 1Rx and 2Rx for specifying PUSCH requirement with OCC with test applicability rue for different Rx number

Proposal 12: Consider the same MCS configured for two UEs for specifying PUSCH requirement with OCC

Proposal 13: Consider the same channel model configured for two UEs for specify PUSCH requirement with OCC

Proposal 14: Consider there is no frequency offset configured for two UEs as baseline assumption, FFS on frequency offset assumption.

Proposal 15: Considering the existing test parameters for PUSCH with DFT-s-OFDM waveform requirement as reference.

# References

RP-240775, Non-Terrestrial Networks (NTN) for NR Phase 3,