3GPP TSG-RAN WG4 Meeting #115                                                                                                      R4-2506028

Malta, MT, 19th –23rd May, 2025

Source: 	CATT

Title: 	Discussion on framework simplification for co-location/co-existence requirements

Agenda item:	10.1

Document for:	Approval

- Introduction

In RAN4#114-bis meeting, A way forward for [114bis][331] BSRF_Simplify_CoLo_Coex was agreed and remaining issues in WF need further discussion.

In this contribution, we provide our views on framework simplification for co-location/co-existence requirements.

- Discussion

As per the WF [1], the remaining open issues concerning notes are shown as below:

We decide to choose Option 1 (i.e, Maintain tables for co-existence and co-location in each specification but simplify them). The following aspects need be considered when drafting the CRs: - All notes deleted from the legacy tables are properly reflected in new table format.

In order to analyse the note in note columns in co-existence requirement table, we can define partial overlapping, full overlapping and frequency gap as following for discussion.

Partial overlapping means that the DL operating band or UL operating band is part of co-existence frequency range, and the DL operating band or UL operating band partially covers co-existence frequency range.

Full overlapping means that the DL operating band or UL operating band full covers co-existence frequency range.

Frequency gap means that frequency offset of the upper frequency or lower frequency of the DL operating band or UL operating band to co-existence frequency range, and no overlapping.

Notes for frequency range of co-existence downlink operating band of co-existence systems or bands

As attached doc file for frequency range of co-existence downlink operating band, for NR operating band there are four cases where this requirement does not apply as below:

1) full overlapping, 2) partial overlapping, the rest is XXX-YYY MHz, 3) partial overlapping, only one frequency is overlapping, 4) frequency gap is less than 5MHz, where 5MHz is maximum, other frequency gap is 1MHz.

The typical examples for notes analysis are copied in following table.

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PCS1900 | 1930 – 1990 MHz | -47 dBm | 100 kHz | This requirement does not apply to BS operating in band n2, n25 or band n70. | n2 | 1850 – 1910 | 1930 – 1990 | FDD | No | Full overlapping |
|  |  |  |  |  | n25 | 1850 – 1915 | 1930 – 1995 | FDD | No | Full overlapping |
|  |  |  |  |  | n70 | 1695 – 1710 | 1995 – 2020 | FDD | No | Frequency gap is 5Mz from 1990 to 1995, the DL co-existence frequency range is located 5MHz below DL operating band. |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 or NR Band n18 | 860 – 890 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n18. | n18 | 815 – 830 | 860 – 875 | FDD | No | Partial overlapping, the rest is 875-890MHz. |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 925 – 960 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n8 or n100. | n8 | 880 – 915 | 925 – 960 | FDD | No | Full overlapping |
|  |  |  |  |  | n100 | 874.4 – 880 | 919.4 – 925 | FDD | No | Partial overlapping, only one frequency 925MHz is overlapping, the rest is 925MHz to 960MHz. |

We can summarize the above typical example cases as below:

For frequency range of downlink operating band of co-existence systems or bands, for NR operating band, if the frequency gap between NR downlink operating band and downlink operating band of co-existence systems or bands is less than or equal to [5MHz], or NR downlink operating band full covers downlink operating band of co-existence systems or bands, or NR downlink operating band partially covers downlink operating band of co-existence systems or bands (including only one frequency), this requirement does not apply to NR BS operating in this NR operating band.

Based on above analysis, we propose,

Proposal 1: State the following note in note column for frequency range of downlink operating band of co-existence systems or bands as below:

For NR operating band, if the frequency gap between NR downlink operating band and downlink operating band of co-existence systems or bands is less than or equal to [5MHz], or NR downlink operating band full covers downlink operating band of co-existence systems or bands, or NR downlink operating band partially covers downlink operating band of co-existence systems or bands (including only one frequency), this requirement does not apply to NR BS operating in this NR operating band.

Notes for frequency range of co-existence uplink operating band of co-existence systems or bands

As attached doc file for frequency range of co-existence uplink operating band, we analyse the notes for frequency range of co-existence uplink operating band of co-existence systems or bands from following three parts: full overlapping, partial overlapping and the rest is XXX-YYY MHz, and frequency gap.

The detailed notes analysis for note is in attached doc file.

1) Full overlapping:

There are two types of full overlapping, the first type is that NR uplink operating band full covers uplink operating band of co-existence systems or bands, the second type is that NR downlink operating band full covers uplink operating band of co-existence systems or bands.

For first type, the typical example is copied as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCS1800 | 1710 – 1785 MHz | -61 dBm | 100 kHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.2.2. | n3 | 1710 – 1785 | 1805 – 1880 | FDD | Full overlapping | No |

As the above example for first type, the NR uplink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to BS operating in NR operating band, since it is already covered by the requirement “Protection of the BS receiver of own or different BS”.

So, we have the following observation for frequency range of uplink operating band of co-existence systems or bands:

Observation 1: For frequency range of uplink operating band of co-existence systems or bands:

For NR operating band, if NR uplink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band, since it is already covered by the requirement “Protection of the BS receiver of own or different BS”.

For second type, the typical example is copied as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1447.9 – 1462.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n74, n75, n92, n94 or n109. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Full overlapping | Same as UL operating band |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | Full overlapping | No |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |

As above, for NR operating band n75, n92, n94, n109, the NR downlink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to BS operating in NR operating band.

So, we have the following observation for frequency range of uplink operating band of co-existence systems or bands:

Observation 2: For frequency range of uplink operating band of co-existence systems or bands:

For NR operating band, if NR downlink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band.

2) Partial overlapping, the rest is XXX-YYY MHz

For partial overlapping, there are two type of partial overlapping, first type is that co-existence requirement for the rest XXX-YYY MHz need to be applicable, and the second type is that co-existence requirement for rest XXX-YYY MHz doesn’t need to be applicable.

The first type is the co-existence requirement for NR operating band n28, n67, n68, n1, n2, and n5, the co-existence requirement applies for the rest part of partial overlapping of frequency range of uplink operating band of co-existence systems or bands with NR uplink operating band or NR downlink operating band.

The analysis for NR operating band n28 is as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 68 or NR Band n68 | 698-728 MHz | -49 dBm | 1 MHz | For BS operating in Band n28, this requirement applies between, while the rest is covered in clause 6.6.5.2.2. | n28 | 703 – 748 | 758 – 803 | FDD | Partial overlapping, the rest is 698-703MHz, this requirement applies between 698 MHz and 703 MHz. | No |

As above, for band n28, the co-existence requirement should be applicable for the rest 698 MHz - 703 MHz, and this note is included in note 2 in Table 6.6.5.2.3-1 of WF as below:

NOTE 2: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 or NR Band n68 UL operating band, where requirement applies for between 698 MHz and 703 MHz

The analysis for NR operating band n67 is as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -49 dBm | 1 MHz | For BS operating in Band n67, it applies for 703 MHz to 736 MHz. | n67 | N/A | 738 – 758 | SDL | No | Partial overlapping, the rest is 703-738MHz, but it(requirement) applies for 703 MHz to 736 MHz, what is the reason for the rest 2MHz(738-736)? |
| NR Band n83 | 703 – 748 MHz | -49 dBm | 1 MHz | For BS operating in Band n67, it applies for 703 MHz to 736 MHz. | n67 | N/A | 738 – 758 | SDL | Partial overlapping, the rest is 703-738MHz, but it applies for 703 MHz to 736 MHz. what is reason for the rest 2MHz(738-736)? | No |

As above, for band n67, the co-existence requirement should be applicable for the rest 703 MHz -736MHz, and this note is included in note 4 in Table 6.6.5.2.3-1 of WF as below:

NOTE 4: For BS operating in Band n67, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz.

But, there is a question about co-existence requirement for band n67, why co-existence requirement applies for 703 MHz to 736 MHz instead of 703-738MHz?

The analysis for NR operating band n68 is as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -49 dBm | 1 MHz | . For BS operating in Band n68, it applies for 728 MHz to 733 MHz. | n68 | 698 – 728 | 753 – 783 | FDD | Partial overlapping, the rest is 728-748MHz, but it(requirment) applies for 728 MHz to 733 MHz. what is reason for the rest 16MHz(748-733)? | No |
| NR Band n109 | 703 –733MHz | -49 dBm | 1 MHz | For BS operating in Band n68, it applies for 728 MHz to 733 MHz. | n68 | 698 – 728 | 753 – 783 | FDD | Partial overlapping, the rest is 728-733MHz, it applies for 728 MHz to 733 MHz. | No |

As above, for band n68, the co-existence requirement should be applicable for the rest 728 MHz to 733 MHz, and this note is not included in any notes. So, it is reasonable to add Note 5 in Table 6.6.5.2.3-1 of WF for band n68.

NOTE 5:	For BS operating in Band n68, specific solutions may be required to fulfil the spurious emissions

limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n109 UL operating band, where requirement applies for 728 MHz to 733 MHz.

So, we propose,

Proposal 2: Add add Note 5 in Table 6.6.5.2.3-1 for band n68 as below:

NOTE 5:	For BS operating in Band n68, specific solutions may be required to fulfil the spurious emissions

limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n109 UL operating band, where requirement applies for 728 MHz to 733 MHz.

The analysis for NR operating band n5, n2 and n1 is as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 – 1915 MHz | -49 dBm | 1 MHz | For BS operating in Band n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.6.5.2.2. | n2 | 1850 – 1910 | 1930 – 1990 | FDD | Partial overlapping, the rest is 1910-1915MHz. | No |
| UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -49 dBm | 1 MHz | For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.5.2.2. | n5 | 824 – 849 | 869 – 894 | FDD | Partial overlapping, the rest is 814-824MHz. | No |
| E-UTRA Band 65 or NR Band n65 | 1920 – 2010 MHz | -49 dBm | 1 MHz | For BS operating in Band n1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in clause 6.6.5.2.2. | n1 | 1920 – 1980 | 2110 – 2170 | FDD | Partial overlapping, the rest is 1980-2010MHz. it applies for 1980 MHz to 2010 MHz. | No |

As above, for band n2, the co-existence requirement should be applicable for the rest 1910 MHz to 1915 MHz. For band n5, the co-existence requirement should be applicable for the rest 814 MHz to 824 MHz. And for band n1, the co-existence requirement should be applicable for the rest 1980 MHz to 2010 MHz. The above is not included any notes, like band n68, so it is reasonable to add Note 6 for band n2, Note 7 for band n5, and Note 8 for band n1.

So, we propose,

Proposal 3: Add Note 6 in Table 6.6.5.2.3-1 for band n2 as below:

NOTE 6:	For BS operating in Band n2, specific solutions may be required to fulfil the spurious emissions

limits for NR BS co-existence with Band XXV/25/n25 UL operating band, where requirement applies for 1910 MHz to 1915 MHz.

Proposal 4: Add Note 7 in Table 6.6.5.2.3-1 for band n5 as below:

NOTE 7:	For BS operating in Band n5, specific solutions may be required to fulfil the spurious emissions

limits for NR BS co-existence with Band XXVI/26/n26 UL operating band, where requirement applies for 814 MHz to 824 MHz.

Proposal 5: Add Note 8 in Table 6.6.5.2.3-1 for band n1 as below:

NOTE 8:	For BS operating in Band n1, specific solutions may be required to fulfil the spurious emissions

limits for NR BS co-existence with Band 65/n65 UL operating band, where requirement applies for 1980 MHz to 2010 MHz.

The second type is the co-existence requirement for NR operating bands except n28, n67, n68, n1, n2, and n5, the co-existence requirement does not apply for NR operating bands if the DL operating band or UL operating band partially covers uplink operating band of co-existence systems or bands. There are many cases for second type in co-existence requirement table, and typical example is copied as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876 – 915 MHz | -61 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.2.2. | n8 | 880 – 915 | 925 – 960 | FDD | Partial overlapping, the rest is 876-880MHz | No |

As above, for band n8, the NR uplink operating band (880 – 915MHz) and uplink operating band of co-existence system or bands (876 – 915 MHz) are partial overlapping, the rest frequency range  is 876-880MHz, but for rest 876-880MHz, co-existence requirement is not applicable, which is different from that for NR operating band n28, n67, n68, n1, n2, and n5. We think that a general note for second type is needed to be defined.

So, we have the following observation for frequency range of uplink operating band of co-existence systems or bands:

Observation 3: For frequency range of uplink operating band of co-existence systems or bands:

For NR operating band except n28, n67, n68, n1, n2, and n5, if NR downlink operating band or NR uplink operating band partially covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band.

At same time, there is a case for co-existence band n82 as following table, and it is hard to interpret the note in note column.

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band n82 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz. For BS operating in Band n68, it applies for 728 MHz to 733 MHz. | n28 | 703 – 748 | 758 – 803 | FDD | Why? | Why? |
|  |  |  |  |  | n67 | N/A | 738 – 758 | SDL | No | Why? |
|  |  |  |  |  | n68 | 698 – 728 | 753 – 783 | FDD | Why? | Why? |

As above, RAN4 need to interpret the note in note column to properly reflect it in new format table.

Proposal 6: RAN4 need to discuss the note in note column in the following table to properly reflect it in new format table.

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band n82 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz. For BS operating in Band n68, it applies for 728 MHz to 733 MHz. | n28 | 703 – 748 | 758 – 803 | FDD | Why? | Why? |
|  |  |  |  |  | n67 | N/A | 738 – 758 | SDL | No | Why? |
|  |  |  |  |  | n68 | 698 – 728 | 753 – 783 | FDD | Why? | Why? |

3) Frequency gap.

The analysis for NR operating band n29 for frequency gap is copied as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 699 – 716 MHz | -49 dBm | 1 MHz | For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). | n29 | N/A | 717 – 728 | SDL | No | Frequency gap is 1MHz, the UL co-existence frequency is located at 1 MHz below the Band n29 downlink operating band |
| E-UTRA Band 17 | 704 – 716 MHz | -49 dBm | 1 MHz | For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). | n29 | N/A | 717 – 728 | SDL | No | Frequency gap is 1MHz, the UL co-existence frequency is located at 1 MHz below the Band n29 downlink operating band. |
| E-UTRA Band 85 or NR Band n85 | 698 – 716 MHz | -49 dBm | 1 MHz | For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). | n29 | N/A | 717 – 728 | SDL | No | Frequency gap is 1Mhz from 716MHz to 717MHz. the UL co-existence frequency range is located at 1 MHz below the Band n29 downlink operating band. |

As above, the frequency gap for band n29 is 1MHz, the co-existence requirement need to apply 1 MHz below the Band n29 downlink operating band. The above note is included in note 3 in Table 6.6.5.2.3-1 of WF as below:

NOTE 3: For a BS operating in Band n29, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band.

The analysis for NR operating band n28 for frequency gap is copied as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 27 | 807 – 824 MHz | -49 dBm | 1 MHz | This requirement also applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band (Note 4). | n28 | 703 – 748 | 758 – 803 | FDD | No | Frequency gap is 4MHz, the UL co-existence frequency is located at 4 MHz above the Band n28 downlink operating band. |

As above, the frequency gap for band n28 is 4MHz, the co-existence requirement need to apply to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band. The above note is included in note 2 in Table 6.6.5.2.3-1 of WF as below:

NOTE 2: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 or NR Band n68 UL operating band, where requirement applies for between 698 MHz and 703 MHz

The analysis for frequency gap for band n87, n5 and n26 is copied as below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 88 or NR Band n88 | 412 – 417 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in Band n87. | n87 | 410 - 415 | 420 – 425 | FDD | Partial overlapping, the rest is 415-417MHz. | Frequency gap is 3MHz, the UL co-existence frequency range is located at 3MHz below band n87 downlink operating band, but no special note? |
| E-UTRA Band 106 or NR Band n106 | 896 – 901 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in Band n5 or n26. | n5 | 824 – 849 | 869 – 894 | FDD | No | Frequency gap is 2MHz, the UL co-existence frequency range is located at 2MHz above DL operating band. |
|  |  |  |  |  | n26 | 814 – 849 | 859 – 894 | FDD | No | Frequency gap is 2MHz, the UL co-existence frequency range is located at 2MHz above DL operating band. |

As above, for co-existence uplink operating band, the maximum frequency gap is 4MHz, for NR operating bands except n29 and n28, if the frequency gap is less than or equal to 4MHz, the co-existence requirement does not apply.

We can summarize that for frequency range of uplink operating band of co-existence systems or bands, for NR operating band except n29 and n28, if the frequency gap between NR downlink operating band and uplink operating band of co-existence systems or bands is less than or equal to [4MHz], this requirement does not apply to NR BS operating in this NR operating band.

Observation 4: For frequency range of uplink operating band of co-existence systems or bands:

For NR operating band except n29 and n28, if the frequency gap between NR downlink operating band and uplink operating band of co-existence systems or bands is less than or equal to [4MHz], this requirement does not apply to NR BS operating in this NR operating band.

Based on above observation 1 to 4, we propose,

Proposal 7: State the following note in note column for frequency range of uplink operating band of co-existence systems or bands as below:

For NR operating band, if NR uplink operating band or NR downlink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band.

For NR operating band except n28, n67, n68, n1, n2, and n5, if NR downlink operating band or NR uplink operating band partially covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band.

For NR operating band except n29 and n28, if the frequency gap between NR downlink operating band and uplink operating band of co-existence systems or bands is less than or equal to [4MHz], this requirement does not apply to NR BS operating in this NR operating band.

Based on above proposals for co-existence requirement, we can use following simplified tables for co-existence requirement.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Operating band to co-exist with | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Frequency range of co-existence downlink operating band | GSM850, CDMA850 or GSM900 | -57 | 100 kHz | For NR operating band, if the frequency gap between NR downlink operating band and downlink operating band of co-existence systems or bands is less than or equal to [5MHz], or NR downlink operating band full covers downlink operating band of co-existence systems or bands, or NR downlink operating band partially covers downlink operating band of co-existence systems or bands (including only one frequency), this requirement does not apply to NR BS operating in this NR operating band. Note 1 |
|  | DCS1800 or PCS1900 | -47 | 100 kHz |  |
|  | Other | -52 | 1 MHz |  |
| Frequency range of co-existence uplink operating band | GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | 100 kHz | For NR operating band, if NR uplink operating band or NR downlink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band. For NR operating band except n28, n67, n68, n1, n2, and n5, if NR downlink operating band or NR uplink operating band partially covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band. For NR operating band except n29 and n28, if the frequency gap between NR downlink operating band and uplink operating band of co-existence systems or bands is less than or equal to [4MHz], this requirement does not apply to NR BS operating in this NR operating band. Note 1, Note 2, Note 3, Note 4, Note 5, Note 6, Note 7. |
|  | Other | -49 | 1 MHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in this table does not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for the excluded frequency range may be covered by local or regional requirements. NOTE 2: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 or NR Band n68 UL operating band, where requirement applies for between 698 MHz and 703 MHz NOTE 3: For a BS operating in Band n29, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 4: For BS operating in Band n67, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. NOTE 5: For BS operating in Band n68, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n109 UL operating band, where requirement applies for 728 MHz to 733 MHz. NOTE 6: For BS operating in Band n2, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXV/25/n25 UL operating band, where requirement applies for 1910 MHz to 1915 MHz. NOTE 7: For BS operating in Band n5, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXVI/26/n26 UL operating band, where requirement applies for 814 MHz to 824 MHz. NOTE 8: For BS operating in Band n1, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 65/n65 UL operating band, where requirement applies for 1980 MHz to 2010 MHz. |  |  |  |  |

Proposal 8: Use following simplified tables for co-existence requirement

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Operating band to co-exist with | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Frequency range of co-existence downlink operating band | GSM850, CDMA850 or GSM900 | -57 | 100 kHz | For NR operating band, if the frequency gap between NR downlink operating band and downlink operating band of co-existence systems or bands is less than or equal to [5MHz], or NR downlink operating band full covers downlink operating band of co-existence systems or bands, or NR downlink operating band partially covers downlink operating band of co-existence systems or bands (including only one frequency), this requirement does not apply to NR BS operating in this NR operating band. Note 1 |
|  | DCS1800 or PCS1900 | -47 | 100 kHz |  |
|  | Other | -52 | 1 MHz |  |
| Frequency range of co-existence uplink operating band | GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | 100 kHz | For NR operating band, if NR uplink operating band or NR downlink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band. For NR operating band except n28, n67, n68, n1, n2, and n5, if NR downlink operating band or NR uplink operating band partially covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band. For NR operating band except n29 and n28, if the frequency gap between NR downlink operating band and uplink operating band of co-existence systems or bands is less than or equal to [4MHz], this requirement does not apply to NR BS operating in this NR operating band. Note 1, Note 2, Note 3, Note 4, Note 5, Note 6, Note 7. |
|  | Other | -49 | 1 MHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in this table does not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for the excluded frequency range may be covered by local or regional requirements. NOTE 2: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 or NR Band n68 UL operating band, where requirement applies for between 698 MHz and 703 MHz NOTE 3: For a BS operating in Band n29, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 4: For BS operating in Band n67, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. NOTE 5: For BS operating in Band n68, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n109 UL operating band, where requirement applies for 728 MHz to 733 MHz. NOTE 6: For BS operating in Band n2, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXV/25/n25 UL operating band, where requirement applies for 1910 MHz to 1915 MHz. NOTE 7: For BS operating in Band n5, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXVI/26/n26 UL operating band, where requirement applies for 814 MHz to 824 MHz. NOTE 8: For BS operating in Band n1, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 65/n65 UL operating band, where requirement applies for 1980 MHz to 2010 MHz. |  |  |  |  |

As above analysis, the new format table can simplify the co-existence/co-location requirement, but for other readers, e.g., product design engineers, it is not very straightforward and convenient. For other readers, they first need to distinguish between frequency gap, full overlapping, partial overlapping, and then check special notes at end of table, this process is also a bit complicated. RAN4 can further consider the convenience for readers.

Proposal 9: RAN4 can further consider the convenience for readers.

- Conclusion

This contribution provides analysis on framework simplification for co-location/co-existence requirements, and the following proposal is concluded as follows:

Proposal 1: State the following note in note column for frequency range of downlink operating band of co-existence systems or bands as below:

For NR operating band, if the frequency gap between NR downlink operating band and downlink operating band of co-existence systems or bands is less than or equal to [5MHz], or NR downlink operating band full covers downlink operating band of co-existence systems or bands, or NR downlink operating band partially covers downlink operating band of co-existence systems or bands (including only one frequency), this requirement does not apply to NR BS operating in this NR operating band.

Observation 1: For frequency range of uplink operating band of co-existence systems or bands:

For NR operating band, if NR uplink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band, since it is already covered by the requirement “Protection of the BS receiver of own or different BS”.

Observation 2: For frequency range of uplink operating band of co-existence systems or bands:

For NR operating band, if NR downlink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band.

Proposal 2: Add add Note 5 in Table 6.6.5.2.3-1 for band n68 as below:

NOTE 5:	For BS operating in Band n68, specific solutions may be required to fulfil the spurious emissions

limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n109 UL operating band, where requirement applies for 728 MHz to 733 MHz.

Proposal 3: Add Note 6 in Table 6.6.5.2.3-1 for band n2 as below:

NOTE 6:	For BS operating in Band n2, specific solutions may be required to fulfil the spurious emissions

limits for NR BS co-existence with Band XXV/25/n25 UL operating band, where requirement applies for 1910 MHz to 1915 MHz.

Proposal 4: Add Note 7 in Table 6.6.5.2.3-1 for band n5 as below:

NOTE 7:	For BS operating in Band n5, specific solutions may be required to fulfil the spurious emissions

limits for NR BS co-existence with Band XXVI/26/n26 UL operating band, where requirement applies for 814 MHz to 824 MHz.

Proposal 5: Add Note 8 in Table 6.6.5.2.3-1 for band n1 as below:

NOTE 8:	For BS operating in Band n1, specific solutions may be required to fulfil the spurious emissions

limits for NR BS co-existence with Band 65/n65 UL operating band, where requirement applies for 1980 MHz to 2010 MHz.

Observation 3: For frequency range of uplink operating band of co-existence systems or bands:

For NR operating band except n28, n67, n68, n1, n2, and n5, if NR downlink operating band or NR uplink operating band partially covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band.

Proposal 6: RAN4 need to discuss the note in note column in the following table to properly reflect it in new format table.

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band n82 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz. For BS operating in Band n68, it applies for 728 MHz to 733 MHz. | n28 | 703 – 748 | 758 – 803 | FDD | Why? | Why? |
|  |  |  |  |  | n67 | N/A | 738 – 758 | SDL | No | Why? |
|  |  |  |  |  | n68 | 698 – 728 | 753 – 783 | FDD | Why? | Why? |

Observation 4: For frequency range of uplink operating band of co-existence systems or bands:

For NR operating band except n29 and n28, if the frequency gap between NR downlink operating band and uplink operating band of co-existence systems or bands is less than or equal to [4MHz], this requirement does not apply to NR BS operating in this NR operating band.

Proposal 7: State the following note in note column for frequency range of uplink operating band of co-existence systems or bands as below:

For NR operating band, if NR uplink operating band or NR downlink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band.

For NR operating band except n28, n67, n68, n1, n2, and n5, if NR downlink operating band or NR uplink operating band partially covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band.

For NR operating band except n29 and n28, if the frequency gap between NR downlink operating band and uplink operating band of co-existence systems or bands is less than or equal to [4MHz], this requirement does not apply to NR BS operating in this NR operating band.

Proposal 8: Use following simplified tables for co-existence requirement

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Operating band to co-exist with | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Frequency range of co-existence downlink operating band | GSM850, CDMA850 or GSM900 | -57 | 100 kHz | For NR operating band, if the frequency gap between NR downlink operating band and downlink operating band of co-existence systems or bands is less than or equal to [5MHz], or NR downlink operating band full covers downlink operating band of co-existence systems or bands, or NR downlink operating band partially covers downlink operating band of co-existence systems or bands (including only one frequency), this requirement does not apply to NR BS operating in this NR operating band. Note 1 |
|  | DCS1800 or PCS1900 | -47 | 100 kHz |  |
|  | Other | -52 | 1 MHz |  |
| Frequency range of co-existence uplink operating band | GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | 100 kHz | For NR operating band, if NR uplink operating band or NR downlink operating band full covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band. For NR operating band except n28, n67, n68, n1, n2, and n5, if NR downlink operating band or NR uplink operating band partially covers uplink operating band of co-existence systems or bands, this requirement does not apply to NR BS operating in this NR operating band. For NR operating band except n29 and n28, if the frequency gap between NR downlink operating band and uplink operating band of co-existence systems or bands is less than or equal to [4MHz], this requirement does not apply to NR BS operating in this NR operating band. Note 1, Note 2, Note 3, Note 4, Note 5, Note 6, Note 7. |
|  | Other | -49 | 1 MHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in this table does not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for the excluded frequency range may be covered by local or regional requirements. NOTE 2: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 or NR Band n68 UL operating band, where requirement applies for between 698 MHz and 703 MHz NOTE 3: For a BS operating in Band n29, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 4: For BS operating in Band n67, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. NOTE 5: For BS operating in Band n68, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n109 UL operating band, where requirement applies for 728 MHz to 733 MHz. NOTE 6: For BS operating in Band n2, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXV/25/n25 UL operating band, where requirement applies for 1910 MHz to 1915 MHz. NOTE 7: For BS operating in Band n5, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXVI/26/n26 UL operating band, where requirement applies for 814 MHz to 824 MHz. NOTE 8: For BS operating in Band n1, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 65/n65 UL operating band, where requirement applies for 1980 MHz to 2010 MHz. |  |  |  |  |

Proposal 9: RAN4 can further consider the convenience for readers.

- References

- R4-2504741, Way forward for [114bis][331] BSRF_Simplify_CoLo_Coex, Moderator (Ericsson), RAN4#114-bis

- R4-2502306, Way forward for [114][331] BSRF_Simplify_CoLo_Coex, Moderator (Ericsson), RAN4#114

- RP-243288, Moderator's summary of discussion about R19-RAN4-BSRF, RAN4 Chair (Huawei), RAN#106

- R4-2500102, On RAN task to work on framework simplification for co-location/co-existence requirements, Nokia

- R4-2500880, On the topic of BS RF specification structure improvements, Ericsson

- RP-242767, On new objective under NR_BS_RF_req_evo, Nokia, Ericsson, ZTE, Huawei, HiSilicon, RAN#106