6.6.5.2.3	Additional spurious emissions requirements

For frequency range of co-existence downlink operating band, the note analysis for note in note column is in following Table 6.6.5.2.3-1.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 921 – 960 MHz | -57 dBm | 100 kHz | This requirement does not apply to BS operating in band n8 | n8 | 880 – 915 | 925 – 960 | FDD | No | Full overlapping |
| DCS1800 | 1805 – 1880 MHz | -47 dBm | 100 kHz | This requirement does not apply to BS operating in band n3. | n3 | 1710 – 1785 | 1805 – 1880 | FDD | No | Full overlapping |
| PCS1900 | 1930 – 1990 MHz | -47 dBm | 100 kHz | This requirement does not apply to BS operating in band n2, n25 or band n70. | n2 | 1850 – 1910 | 1930 – 1990 | FDD | No | Full overlapping |
|  |  |  |  |  | n25 | 1850 – 1915 | 1930 – 1995 | FDD | No | Full overlapping |
|  |  |  |  |  | n70 | 1695 – 1710 | 1995 – 2020 | FDD | No | Frequency gap is 5Mz from 1990 to 1995, the DL co-existence frequency range is located 5MHz below DL operating band. |
| GSM850 or CDMA850 | 869 – 894 MHz | -57 dBm | 100 kHz | This requirement does not apply to BS operating in band n5 or n26. | n5 | 824 – 849 | 869 – 894 | FDD | No | Full overlapping |
|  |  |  |  |  | n26 | 814 – 849 | 859 – 894 | FDD | No | Full overlapping |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 2110 – 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65 | n1 | 1920 – 1980 | 2110 – 2170 | FDD | No | Full overlapping |
|  |  |  |  |  | n65 | 1920 – 2010 | 2110 – 2200 | FDD | No | Full overlapping |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1930 – 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n2 or n70. | n2 | 1850 – 1910 | 1930 – 1990 | FDD | No | Full overlapping |
|  |  |  |  |  | n70 | 1695 – 1710 | 1995 – 2020 | FDD | No | Frequency gap is 5Mz from 1990 to 1995, the DL co-existence frequency range is located 5MHz below DL operating band. |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1805 – 1880 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. | n3 | 1710 – 1785 | 1805 – 1880 | FDD | No | Full overlapping |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n66 | n66 | 1710 – 1780 | 2110 – 2200 | FDD | No | Full overlapping |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 869 – 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n5 or n26. | n5 | 824 – 849 | 869 – 894 | FDD | No | Full overlapping |
|  |  |  |  |  | n26 | 814 – 849 | 859 – 894 | FDD | No | Full overlapping |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 or NR Band n18 | 860 – 890 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n18. | n18 | 815 – 830 | 860 – 875 | FDD | No | Partial overlapping, the rest is 875-890MHz. |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2620 – 2690 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n7. | n7 | 2500 – 2570 | 2620 – 2690 | FDD | No | Full overlapping |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 925 – 960 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n8 or n100. | n8 | 880 – 915 | 925 – 960 | FDD | No | Full overlapping |
|  |  |  |  |  | n100 | 874.4 – 880 | 919.4 – 925 | FDD | No | Partial overlapping, only 925MHz is overlapping, the rest is 925MHz to 960MHz.. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 – 1879.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. | n3 | 1710 – 1785 | 1805 – 1880 | FDD | No | Full overlapping |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n66 | n66 | 1710 – 1780 | 2110 – 2200 | FDD | No | Full overlapping |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 – 1510.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n74, n75, n92, n94 or n109. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | No | Full overlapping |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 729 – 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85. | n12 | 699 – 716 | 729 – 746 | FDD | No | Full overlapping |
|  |  |  |  |  | n85 | 698 – 716 | 728 – 746 | FDD | No | Full overlapping |
| UTRA FDD Band XIII or E-UTRA Band 13 or NR Band n13 | 746 – 756 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n13. | n13 | 777 – 787 | 746 – 756 | FDD | No | Full overlapping |
| UTRA FDD Band XIV or E-UTRA Band 14 or NR band n14 | 758 – 768 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n14. | n14 | 788 – 798 | 758 – 768 | FDD | No | Full overlapping |
| E-UTRA Band 17 | 734 – 746 MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 791 – 821 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n20 or n28. | n20 | 832 – 862 | 791 – 821 | FDD | No | Full overlapping |
|  |  |  |  |  | n28 | 703 – 748 | 758 – 803 | FDD | No | Partial overlapping, the rest is 803-821MHz. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n48, n77 or n78. | n48 | 3550 – 3700 | 3550 – 3700 | TDD | No | Partial overlapping, the rest is 3510-3550MHz. |
|  |  |  |  |  | n77 | 3300 – 4200 | 3300 – 4200 | TDD | No | Full overlapping |
|  |  |  |  |  | n78 | 3300 – 3800 | 3300 – 3800 | TDD | No | Full overlapping |
| E-UTRA Band 24 or NR Band n24 | 1525 – 1559 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n24. | n24 | 1626.5 – 1660.5 | 1525 – 1559 | FDD | No | Full overlapping |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 – 1995 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, n25 or n70. | n2 | 1850 – 1910 | 1930 – 1990 | FDD | No | Partial overlapping, the rest is 1990-1995MHz. |
|  |  |  |  |  | n25 | 1850 – 1915 | 1930 – 1995 | FDD | No | Full overlapping |
|  |  |  |  |  | n70 | 1695 – 1710 | 1995 – 2020 | FDD | No | Partial overlapping, only one frequency 1995MHz is overlapping, the rest is 1930-1950MHz. |
| UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 859 – 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n5 or n26. | n5 | 824 – 849 | 869 – 894 | FDD | No | Full overlapping |
|  |  |  |  |  | n26 | 814 – 849 | 859 – 894 | FDD | No | Full overlapping |
| E-UTRA Band 27 | 852 – 869 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n5. | n5 | 824 – 849 | 869 – 894 | FDD | No | Partial overlapping, only one frequency 869MHz is overlapping, the rest is 952-869MHz. |
| E-UTRA Band 28 or NR Band n28 | 758 – 803 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n20, n28, n67 or n68. | n20 | 832 – 862 | 791 – 821 | FDD | No | Partial overlapping, the rest is 758-791MHz |
|  |  |  |  |  | n28 | 703 – 748 | 758 – 803 | FDD | No | Full overlapping |
|  |  |  |  |  | n67 | N/A | 738 – 758 | SDL | No | Partial overlapping, only one frequency 758MHz is overlapping, the rest is 758-803MHz. |
|  |  |  |  |  | n68 | 698 – 728 | 753 – 783 | FDD | No | Partial overlapping, the rest is 783-803MHz |
| E-UTRA Band 29 or NR Band n29 | 717 – 728 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n29 or n85 | n29 | N/A | 717 – 728 | SDL | No | Full overlapping |
|  |  |  |  |  | n85 | 698 – 716 | 728 – 746 | FDD | No | Partial overlapping, only one frequency 728MHz is overlapping, the rest is 717-728MHz. |
| E-UTRA Band 30 or NR Band n30 | 2350 – 2360 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n30 | n30 | 2305 – 2315 | 2350 – 2360 | FDD | No | Full overlapping |
| E-UTRA Band 31 or NR Band n31 | 462.5 – 467.5 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n31 or n72. | n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD | No | Full overlapping |
|  |  |  |  |  | n72 | 451 – 456 | 461 – 466 | FDD | No | Partial overlapping, the rest is 466-467.5MHz |
| UTRA FDD band XXXII or E-UTRA band 32 | 1452 – 1496 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n74, n75, n92, n94 or n109 | n50 | 1432 – 1517 | 1432 – 1517 | TDD | No | Full overlapping |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | No | Partial overlapping, the rest is 1452-1475MHz |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 – 1920 MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 – 2025 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n34. | n34 | 2010 – 2025 | 2010 – 2025 | TDD | No | Full overlapping |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n2 or n25. | n2 | 1850 – 1910 | 1930 – 1990 | FDD | No | Full overlapping |
|  |  |  |  |  | n25 | 1850 – 1915 | 1930 – 1995 | FDD | No | Full overlapping |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n38. | n38 | 2570 – 2620 | 2570 – 2620 | TDD | Same as DL operating band | Full overlapping |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n39. | n39 | 1880 – 1920 | 1880 – 1920 | TDD | Same as DL operating band | Full overlapping |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n30 or n40. | n30 | 2305 – 2315 | 2350 – 2360 | FDD | No | Partial overlapping, the rest is 2300-2350MHz, and 2360-2400MHz |
|  |  |  |  |  | n40 | 2300 – 2400 | 2300 – 2400 | TDD | No | Full overlapping |
| E-UTRA Band 41 or NR Band n41, n90 | 2496 – 2690 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n41, n53 or [n90]. | n41 | 2496 – 2690 | 2496 – 2690 | TDD | No | Full overlapping |
|  |  |  |  |  | n53 | 2483.5 – 2495 | 2483.5 – 2495 | TDD | No | Partial overlapping, the rest is 2495-2690MHz |
|  |  |  |  |  | n90 | 2496 – 2690 | 2496 – 2690 | TDD | Same as DL operating band | Full overlapping |
| E-UTRA Band 42 | 3400 – 3600 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n48, n77 or n78. | n48 | 3550 – 3700 | 3550 – 3700 | TDD | Same as DL operating band | Partial overlapping, the rest is 3400-3550MHz |
|  |  |  |  |  | n77 | 3300 – 4200 | 3300 – 4200 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n78 | 3300 – 3800 | 3300 – 3800 | TDD | Same as DL operating band | Full overlapping |
| E-UTRA Band 43 | 3600 – 3800 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n48, n77 or n78. | n48 | 3550 – 3700 | 3550 – 3700 | TDD | Same as DL operating band | Partial overlapping, the rest is 3700-3800MHz |
|  |  |  |  |  | n77 | 3300 – 4200 | 3300 – 4200 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n78 | 3300 – 3800 | 3300 – 3800 | TDD | Same as DL operating band | Full overlapping |
| E-UTRA Band 44 | 703 – 803 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n28. | n28 | 703 – 748 | 758 – 803 | FDD | No | Partial overlapping, the rest is 703-758MHz |
| E-UTRA Band 45 | 1447 – 1467 MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n46, n96 or n102. | n46 | 5150 – 5925 | 5150 – 5925 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n96 | 5925 – 7125 | 5925 – 7125 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping. |
|  |  |  |  |  | n102 | 5925 – 6425 | 5925 – 6425 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping. |
| E-UTRA Band 47 | 5855 – 5925 MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n48, n77 or n78. | n48 | 3550 – 3700 | 3550 – 3700 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n77 | 3300 – 4200 | 3300 – 4200 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n78 | 3300 – 3800 | 3300 – 3800 | TDD | Same as DL operating band | Full overlapping |
| E-UTRA Band 50 or NR band n50 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93, n94, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping. |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | No | Partial overlapping, the rest is 1432-1475MHz |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Partial overlapping, only one frequency is overlapping. |
|  |  |  |  |  | n91 | 832 – 862 | 1427 – 1432 | FDD | No | Partial overlapping, only one frequency is overlapping. |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n93 | 880 – 915 | 1427 – 1432 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Partial overlapping, the rest is 1435-1517MHz |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76, n91, n92, n93, n94, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping. |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Full overlapping. |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Partial overlapping, only one frequency is overlapping. Partial overlapping, only one frequency is overlapping. |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Full overlapping. |
|  |  |  |  |  | n91 | 832 – 862 | 1427 – 1432 | FDD | No | Full overlapping. |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Partial overlapping, only one frequency is overlapping. |
|  |  |  |  |  | n93 | 880 – 915 | 1427 – 1432 | FDD | No | Full overlapping. |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Partial overlapping, only one frequency is overlapping. |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Partial overlapping, only one frequency is overlapping. |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Partial overlapping, only one frequency is overlapping. |
| E-UTRA Band 53 or NR Band n53 | 2483.5 - 2495 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n41, n53 or n90. | n41 | 2496 – 2690 | 2496 – 2690 | TDD | Same as DL operating band | Frequency gap is 1MHz. |
|  |  |  |  |  | n53 | 2483.5 – 2495 | 2483.5 – 2495 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n90 | 2496 – 2690 | 2496 – 2690 | TDD | Same as DL operating band | Frequency gap is 1MHz. |
| E-UTRA Band 54 or NR Band n54 | 1670 – 1675 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n54 | n54 | 1670 – 1675 | 1670 – 1675 | TDD |  |  |
| E-UTRA Band 65 or NR Band n65 | 2110 – 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65. | n1 | 1920 – 1980 | 2110 – 2170 | FDD | No | Partial overlapping, the rest is 2170-2200MHz |
|  |  |  |  |  | n65 | 1920 – 2010 | 2110 – 2200 | FDD | No | Full overlapping |
| E-UTRA Band 66 or NR Band n66 | 2110 – 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n66. | n66 | 1710 – 1780 | 2110 – 2200 | FDD | No | Full overlapping |
| E-UTRA Band 67 or NR Band n67 | 738 – 758 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n28 or n67. | n28 | 703 – 748 | 758 – 803 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n67 | N/A | 738 – 758 | SDL | No | Full overlapping |
| E-UTRA Band 68 or NR Band n68 | 753 -783 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n28 or n68. | n28 | 703 – 748 | 758 – 803 | FDD | No | Partial overlapping, the rest is 753-758MHz |
|  |  |  |  |  | n68 | 698 – 728 | 753 – 783 | FDD | No | Full overlapping |
| E-UTRA Band 69 | 2570 – 2620 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n38. | n38 | 2570 – 2620 | 2570 – 2620 | TDD | No | Full overlapping |
| E-UTRA Band 70 or NR Band n70 | 1995 – 2020 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, n25 or n70 | n2 | 1850 – 1910 | 1930 – 1990 | FDD | No | Frequency gap is 5MHz |
|  |  |  |  |  | n25 | 1850 – 1915 | 1930 – 1995 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n70 | 1695 – 1710 | 1995 – 2020 | FDD | No | Full overlapping |
| E-UTRA Band 71 or NR Band n71 | 617 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or n105 | n71 | 663 – 698 | 617 – 652 | FDD | No | Full overlapping |
|  |  |  |  |  | n105 | 663 – 703 | 612 – 652 | FDD | No | Full overlapping |
| E-UTRA Band 72 or NR Band n72 | 461 – 466 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n31 or n72. | n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD | No | Partial overlapping, the rest is 461-462.5MHz |
|  |  |  |  |  | n72 | 451 – 456 | 461 – 466 | FDD | No | Full overlapping |
| E-UTRA Band 74 or NR Band n74 | 1475 – 1518 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n74, n75, n92, n94 or n109. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Partial overlapping, the rest is 1517-1518MHz |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | No | Full overlapping |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Partial overlapping, the rest is 1517-1518MHz |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Partial overlapping, the rest is 1517-1518MHz |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Partial overlapping, the rest is 1517-1518MHz |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Partial overlapping, the rest is 1517-1518MHz |
| E-UTRA Band 75 or NR Band n75 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93 or n94, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | No | Partial overlapping, the rest is 1432-1475MHz |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n91 | 832 – 862 | 1427 – 1432 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n93 | 880 – 915 | 1427 – 1432 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Partial overlapping, the rest is 1435-1517MHz |
| E-UTRA Band 76 or NR Band n76 | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76, n91, n92, n93, n94, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Full overlapping |
|  |  |  |  |  | n91 | 832 – 862 | 1427 – 1432 | FDD | No | Full overlapping |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n93 | 880 – 915 | 1427 – 1432 | FDD | No | Full overlapping |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Partial overlapping, only one frequency is overlapping |
| NR Band n77 | 3.3 – 4.2 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n48, n77 or n78 | n48 | 3550 – 3700 | 3550 – 3700 | TDD | Same as DL operating band | Partial overlapping, the rest is 3300-3550MHz and 3700-4200MHz |
|  |  |  |  |  | n77 | 3300 – 4200 | 3300 – 4200 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n78 | 3300 – 3800 | 3300 – 3800 | TDD | Same as DL operating band | Partial overlapping, the rest is 3800-4200MHz |
| NR Band n78 | 3.3 – 3.8 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n48, n77 or n78 | n48 | 3550 – 3700 | 3550 – 3700 | TDD | Same as DL operating band | Partial overlapping, the rest is 3300-3550MHz and 3700-3800MHz |
|  |  |  |  |  | n77 | 3300 – 4200 | 3300 – 4200 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n78 | 3300 – 3800 | 3300 – 3800 | TDD | Same as DL operating band | Full overlapping |
| NR Band n79 | 4.4 – 5.0 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n79 | n79 | 4400 – 5000 | 4400 – 5000 | TDD | Same as DL operating band | Full overlapping |
| E-UTRA Band 85 or NR Band n85 | 728 – 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n12, n29 or n85. | n12 | 699 – 716 | 729 – 746 | FDD | No | Partial overlapping, the rest is 728-729MHz |
|  |  |  |  |  | n29 | N/A | 717 – 728 | SDL | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n85 | 698 – 716 | 728 – 746 | FDD | No | Full overlapping |
| E-UTRA Band 87 or NR Band n87 | 420 – 425 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n87 or n88. | n87 | 410 - 415 | 420 – 425 | FDD | No | Full overlapping |
|  |  |  |  |  | n88 | 412 - 417 | 422 - 427 | FDD | No | Partial overlapping, the rest is 420-422MHz |
| E-UTRA Band 88 or NR Band n88 | 422 – 427 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n87 or n88. | n87 | 410 - 415 | 420 – 425 | FDD | No | Partial overlapping, the rest is 425-427MHz |
|  |  |  |  |  | n88 | 412 - 417 | 422 - 427 | FDD | No | Full overlapping |
| NR Band n91 | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Partial overlapping, only one frequency is overlapping |
| NR Band n92 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | No | Partial overlapping, the rest is 1432-1475MHz |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Partial overlapping, the rest is 1435-1517MHz |
| NR Band n93 | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Partial overlapping, only one frequency is overlapping |
| NR Band n94 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | No | Partial overlapping, the rest is 1432-1475MHz |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Partial overlapping, the rest is 1435-1517MHz |
| NR Band n95 | 2010 – 2025 MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| NR Band n96 | 5925 – 7125 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n46, n96, n102 or n104. | n46 | 5150 – 5925 | 5150 – 5925 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n96 | 5925 – 7125 | 5925 – 7125 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n102 | 5925 – 6425 | 5925 – 6425 | TDD | Same as DL operating band | Partial overlapping, the rest is 6425-7125MHz |
|  |  |  |  |  | n104 | 6425 – 7125 | 6425 – 7125 | TDD | Same as DL operating band | Partial overlapping, the rest is 5925-6425MHz |
| NR Band n97 | 2300 – 2400MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| NR Band n98 | 1880 – 1920MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| NR band n100 | 919.4 – 925 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n8 or n100. | n8 | 880 – 915 | 925 – 960 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n100 | 874.4 – 880 | 919.4 – 925 | FDD | No | Full overlapping |
| NR band n101 | 1900 – 1910 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n101. | n101 | 1900 – 1910 | 1900 – 1910 | TDD | No | Full overlapping |
| NR Band n102 | 5925 – 6425 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n46, n96, n102 or n104. | n46 | 5150 – 5925 | 5150 – 5925 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n96 | 5925 – 7125 | 5925 – 7125 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n102 | 5925 – 6425 | 5925 – 6425 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n104 | 6425 – 7125 | 6425 – 7125 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
| E-UTRA Band 103 | 757 – 758 MHz | -52 dBm | 1 MHz |  |  |  |  |  |  |  |
| NR Band n104 | 6425 – 7125 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n96, n102 or n104 | n96 | 5925 – 7125 | 5925 – 7125 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n102 | 5925 – 6425 | 5925 – 6425 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n104 | 6425 – 7125 | 6425 – 7125 | TDD | Same as DL operating band | Full overlapping |
| NR Band n105 | 612 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n71 or n105 | n71 | 663 – 698 | 617 – 652 | FDD | No | Partial overlapping, the rest is 612-617MHz |
|  |  |  |  |  | n105 | 663 – 703 | 612 – 652 | FDD | No | Full overlapping |
| E-UTRA Band 106 or NR Band n106 | 935 - 940 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n106 | n106 | 896 – 901 | 935 – 940 | FDD | No | Full overlapping |
| NR Band n109 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93, n94, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | No | Partial overlapping, the rest is 1432-1475MHz |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n91 | 832 – 862 | 1427 – 1432 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n93 | 880 – 915 | 1427 – 1432 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Partial overlapping, the rest is 1435-1517MHz |
| NR Band n110 | 1432 – 1435 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76, n91, n92, n93, n94, n109 or n110. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Same as DL operating band | Full overlapping |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Same as DL operating band | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n91 | 832 – 862 | 1427 – 1432 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n93 | 880 – 915 | 1427 – 1432 | FDD | No | Partial overlapping, only one frequency is overlapping |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n110 | 1390 - 1395 | 1432 - 1435 | FDD | No | Full overlapping |
| E-UTRA Band 111 | 1820 – 1830 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. | n3 | 1710 – 1785 | 1805 – 1880 | FDD | No | Full overlapping |