6.6.5.2.3	Additional spurious emissions requirements

For frequency range of co-existence uplink operating band, the note analysis for note in note column is in following Table 6.6.5.2.3-1.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Uplink (UL) operating band overlapping | Downlink (DL) operating band overlapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876 – 915 MHz | -61 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.2.2. | n8 | 880 – 915 | 925 – 960 | FDD | Partial overlapping, the rest is 876-880MHz | No |
| DCS1800 | 1710 – 1785 MHz | -61 dBm | 100 kHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.2.2. | n3 | 1710 – 1785 | 1805 – 1880 | FDD | Full overlapping | No |
| PCS1900 | 1850 – 1910 MHz | -61 dBm | 100 kHz | This requirement does not apply to BS operating in band n2 or n25 since it is already covered by the requirement in clause 6.6.5.2.2. | n2 | 1850 – 1910 | 1930 – 1990 | FDD | Full overlapping | No |
|  |  |  |  |  | n25 | 1850 – 1915 | 1930 – 1995 | FDD | Full overlapping | No |
| GSM850 or CDMA850 | 824 – 849 MHz | -61 dBm | 100 kHz | This requirement does not apply to BS operating in band n5 or n26, since it is already covered by the requirement in clause 6.6.5.2.2. | n5 | 824 – 849 | 869 – 894 | FDD | Full overlapping | No |
|  |  |  |  |  | n26 | 814 – 849 | 859 – 894 | FDD | Full overlapping | No |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in clause 6.6.5.2.2. | n1 | 1920 – 1980 | 2110 – 2170 | FDD | Full overlapping | No |
|  |  |  |  |  | n65 | 1920 – 2010 | 2110 – 2200 | FDD | Full overlapping | No |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1850 – 1910 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, since it is already covered by the requirement in clause 6.6.5.2.2. | n2 | 1850 – 1910 | 1930 – 1990 | FDD | Full overlapping | No |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1710 – 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.2.2. | n3 | 1710 – 1785 | 1805 – 1880 | FDD | Full overlapping | No |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 – 1755 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.6.5.2.2. | n66 | 1710 – 1780 | 2110 – 2200 | FDD | Full overlapping | No |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 824 – 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n5 or n26, since it is already covered by the requirement in clause 6.6.5.2.2. | n5 | 824 – 849 | 869 – 894 | FDD | Full overlapping | No |
|  |  |  |  |  | n26 | 814 – 849 | 859 – 894 | FDD | Full overlapping | No |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 or NR Band n18 | 815 – 830 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n18, since it is already covered by the requirement in clause 6.6.5.2.2. | n18 | 815 – 830 | 860 – 875 | FDD | Full overlapping | No |
|  | 830 – 845 MHz | -49 dBm | 1 MHz |  |  |  |  |  |  |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2500 – 2570 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n7, since it is already covered by the requirement in clause 6.6.5.2.2. | n7 | 2500 – 2570 | 2620 – 2690 | FDD | Full overlapping | No |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 880 – 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.2.2. | n8 | 880 – 915 | 925 – 960 | FDD | Full overlapping | No |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 – 1784.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.2.2. | n3 | 1710 – 1785 | 1805 – 1880 | FDD | Full overlapping | No |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 – 1770 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.6.5.2.2. | n66 | 1710 – 1780 | 2110 – 2200 | FDD | Full overlapping | No |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1427.9 – 1447.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n51, n74, n75, n76, n91, n92, n93, or n109. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Partial overlapping, the rest is 1427.9-1432MHz | Same as UL operating band |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Partial overlapping, the rest is 1432-1447.9MHz | Same as UL operating band |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | Full overlapping | No |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Partial overlapping, the rest is 1427.9-1432MHz |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Partial overlapping, the rest is 1432-1447.9MHz |
|  |  |  |  |  | n91 | 832 – 862 | 1427 – 1432 | FDD | No | Partial overlapping, the rest is 1432-1447.9MHz |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Partial overlapping, the rest is 1427.9-1432MHz |
|  |  |  |  |  | n93 | 880 – 915 | 1427 – 1432 | FDD | No | Partial overlapping, the rest is 1432-1447.9MHz |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Partial overlapping, the rest is 1427.9-1432MHz |
|  | 1447.9 – 1462.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n74, n75, n92, n94 or n109. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Full overlapping | Same as UL operating band |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | Full overlapping | No |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Full overlapping |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Full overlapping |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Full overlapping |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 699 – 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.6.5.2.2. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). | n12 | 699 – 716 | 729 – 746 | FDD | Full overlapping | No |
|  |  |  |  |  | n85 | 698 – 716 | 728 – 746 | FDD | Full overlapping | No |
|  |  |  |  |  | n29 | N/A | 717 – 728 | SDL | No | Frequency gap is 1MHz, the UL co-existence frequency is located at 1 MHz below the Band n29 downlink operating band |
| UTRA FDD Band XIII or E-UTRA Band 13 or NR Band n13 | 777 – 787 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n13, since it is already covered by the requirement in clause 6.6.5.2.2. | n13 | 777 – 787 | 746 – 756 | FDD | Full overlapping | No |
| UTRA FDD Band XIV or E-UTRA Band 14 or NR band n14 | 788 – 798 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n14, since it is already covered by the requirement in clause 6.6.5.2.2. | n14 | 788 – 798 | 758 – 768 | FDD | Full overlapping | No |
| E-UTRA Band 17 | 704 – 716 MHz | -49 dBm | 1 MHz | For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). | n29 | N/A | 717 – 728 | SDL | No | Frequency gap is 1MHz, the UL co-existence frequency is located at 1 MHz below the Band n29 downlink operating band. |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.6.5.2.2. | n20 | 832 – 862 | 791 – 821 | FDD | Full overlapping | No |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n77 or n78. | n77 | 3300 – 4200 | 3300 – 4200 | TDD | Full overlapping | Same as UL operating band |
|  |  |  |  |  | n78 | 3300 – 3800 | 3300 – 3800 | TDD | Full overlapping | Same as UL operating band |
| E-UTRA Band 24 or NR Band n24 | 1626.5 – 1660.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n24, since it is already covered by the requirement in clause 6.6.5.2.2. | n24 | 1626.5 – 1660.5 | 1525 – 1559 | FDD | Full overlapping | No |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 – 1915 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n25 since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.6.5.2.2. | n25 | 1850 – 1915 | 1930 – 1995 | FDD | Full overlapping | No |
|  |  |  |  |  | n2 | 1850 – 1910 | 1930 – 1990 | FDD | Partial overlapping, the rest is 1910-1915MHz. | No |
| UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n26 since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.5.2.2. | n26 | 814 – 849 | 859 – 894 | FDD | Full overlapping | No |
|  |  |  |  |  | n5 | 824 – 849 | 869 – 894 | FDD | Partial overlapping, the rest is 814-824MHz. | No |
| E-UTRA Band 27 | 807 – 824 MHz | -49 dBm | 1 MHz | This requirement also applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band (Note 4). | n28 | 703 – 748 | 758 – 803 | FDD | No | Frequency gap is 4MHz, the UL co-existence frequency is located at 4 MHz above the Band n28 downlink operating band. |
| E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz. For BS operating in Band n68, it applies for 728 MHz to 733 MHz. | n28 | 703 – 748 | 758 – 803 | FDD | Full overlapping | No |
|  |  |  |  |  | n67 | N/A | 738 – 758 | SDL | No | Partial overlapping, the rest is 703-738MHz, but it(requirment) applies for 703 MHz to 736 MHz, what is the reason for the rest 2MHz(738-736)? |
|  |  |  |  |  | n68 | 698 – 728 | 753 – 783 | FDD | Partial overlapping, the rest is 728-748MHz, but it(requirment) applies for 728 MHz to 733 MHz. what is reason for the rest 16MHz(748-733)? | No |
| E-UTRA Band 30 or NR Band n30 | 2305 – 2315 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n30, since it is already covered by the requirement in clause 6.6.5.2.2. | n30 | 2305 – 2315 | 2350 – 2360 | FDD | Full overlapping | No |
| E-UTRA Band 31 or NR Band n31 | 452.5 – 457.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n31, since it is already covered by the requirement in clause 6.6.5.2.2. This requirement does not apply to BS operating in band n72. | n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD | Full overlapping | No |
|  |  |  |  |  | n72 | 451 – 456 | 461 – 466 | FDD | Partial overlapping, the rest is 456-457.5MHz. | No |
| E-UTRA Band 65 or NR Band n65 | 1920 – 2010 MHz | -49 dBm | 1 MHz | For BS operating in Band n1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in clause 6.6.5.2.2. This requirement does not apply to BS operating in band n65, since it is already covered by the requirement in clause 6.6.5.2.2. | n1 | 1920 – 1980 | 2110 – 2170 | FDD | Partial overlapping, the rest is 1980-2010MHz. it applies for 1980 MHz to 2010 MHz. | No |
|  |  |  |  |  | n65 | 1920 – 2010 | 2110 – 2200 | FDD | Full overlapping | No |
| E-UTRA Band 66 or NR Band n66 | 1710 – 1780 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.6.5.2.2. | n66 | 1710 – 1780 | 2110 – 2200 | FDD | Full overlapping | No |
| E-UTRA Band 68 or NR Band n68 | 698-728 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n68, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n28, this requirement applies between 698 MHz and 703 MHz, while the rest is covered in clause 6.6.5.2.2. | n68 | 698 – 728 | 753 – 783 | FDD | Full overlapping | No |
|  |  |  |  |  | n28 | 703 – 748 | 758 – 803 | FDD | Partial overlapping, the rest is 698-703MHz, this requirement applies between 698 MHz and 703 MHz. | No |
| E-UTRA Band 70 or NR Band n70 | 1695 – 1710 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n70, since it is already covered by the requirement in clause 6.6.5.2.2. | n70 | 1695 – 1710 | 1995 – 2020 | FDD | Full overlapping | No |
| E-UTRA Band 71 or NR Band n71 | 663 – 698 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or n105, since it is already covered by the requirement in clause 6.6.5.2.2. | n71 | 663 – 698 | 617 – 652 | FDD | Full overlapping | No |
|  |  |  |  |  | n105 | 663 – 703 | 612 – 652 | FDD | Full overlapping | No |
| E-UTRA Band 72 or NR Band n72 | 451 – 456 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n72, since it is already covered by the requirement in clause 6.6.5.2.2. This requirement does not apply to BS operating in band n31. | n72 | 451 – 456 | 461 – 466 | FDD | Full overlapping | No |
|  |  |  |  |  | n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD | Partial overlapping, the rest is 451-452.5MHz. | No |
| E-UTRA Band 74 or NR Band n74 | 1427 – 1470 MHz | -49 dBm | 1MHz | This requirement does not apply to BS operating in band n50, n51, n74, n75, n76, n91, n92, n93 or n94 or n109. | n50 | 1432 – 1517 | 1432 – 1517 | TDD | Partial overlapping, the rest is 1427-1432MHz. | Same as UL operating band |
|  |  |  |  |  | n51 | 1427 – 1432 | 1427 – 1432 | TDD | Partial overlapping, the rest is 1432-1470MHz | Same as UL operating band |
|  |  |  |  |  | n74 | 1427 – 1470 | 1475 – 1518 | FDD | Full overlapping | No |
|  |  |  |  |  | n75 | N/A | 1432 – 1517 | SDL | No | Partial overlapping, the rest is 1427-1432MHz. |
|  |  |  |  |  | n76 | N/A | 1427 – 1432 | SDL | No | Partial overlapping, the rest is 1432-1470MHz |
|  |  |  |  |  | n91 | 832 – 862 | 1427 – 1432 | FDD | No | Partial overlapping, the rest is 1432-1470MHz |
|  |  |  |  |  | n92 | 832 – 862 | 1432 – 1517 | FDD | No | Partial overlapping, the rest is 1427-1432MHz. |
|  |  |  |  |  | n93 | 880 – 915 | 1427 – 1432 | FDD | No | Partial overlapping, the rest is 1432-1470MHz |
|  |  |  |  |  | n94 | 880 – 915 | 1432 – 1517 | FDD | No | Partial overlapping, the rest is 1427-1432MHz. |
|  |  |  |  |  | n109 | 703 – 733 | 1432 – 1517 | FDD | No | Partial overlapping, the rest is 1427-1432MHz. |
| NR Band n80 | 1710 – 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.2.2. | n3 | 1710 – 1785 | 1805 – 1880 | FDD | Full overlapping | No |
| NR Band n81 | 880 – 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.2.2. | n8 | 880 – 915 | 925 – 960 | FDD | Full overlapping | No |
| NR Band n82 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz. For BS operating in Band n68, it applies for 728 MHz to 733 MHz. | n28 | 703 – 748 | 758 – 803 | FDD | Why? | Why? |
|  |  |  |  |  | n67 | N/A | 738 – 758 | SDL | No | Why? |
|  |  |  |  |  | n68 | 698 – 728 | 753 – 783 | FDD | Why? | Why? |
| NR Band n83 | 703 – 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz. | n28 | 703 – 748 | 758 – 803 | FDD | Full overlapping | No |
|  |  |  |  |  | n67 | N/A | 738 – 758 | SDL | Partial overlapping, the rest is 703-738MHz, but it applies for 703 MHz to 736 MHz. what is reason for the rest 2MHz(738-736)? | No |
| NR Band n84 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1, since it is already covered by the requirement in clause 6.6.5.2.2. | n1 | 1920 – 1980 | 2110 – 2170 | FDD | Full overlapping | No |
| E-UTRA Band 85 or NR Band n85 | 698 – 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.6.5.2.2. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). | n12 | 699 – 716 | 729 – 746 | FDD | Full overlapping | No |
|  |  |  |  |  | n85 | 698 – 716 | 728 – 746 | FDD | Full overlapping | No |
|  |  |  |  |  | n29 | N/A | 717 – 728 | SDL | No | Frequency gap is 1Mhz from 716MHz to 717MHz. the UL co-existence frequency range is located at 1 MHz below the Band n29 downlink operating band. |
| NR Band n86 | 1710 – 1780 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.6.5.2.2. | n66 | 1710 – 1780 | 2110 – 2200 | FDD | Full overlapping | No |
| E-UTRA Band 87 or NR Band n87 | 410 – 415 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n87, since it is already covered by the requirement in clause 6.6.5.2.2. | n87 | 410 - 415 | 420 – 425 | FDD | Full overlapping | No |
| E-UTRA Band 88 or NR Band n88 | 412 – 417 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n88, since it is already covered by the requirement in clause 6.6.5.2.2. This requirement does not apply to BS operating in Band n87. | n88 | 412 - 417 | 422 - 427 | FDD | Full overlapping | No |
|  |  |  |  |  | n87 | 410 - 415 | 420 – 425 | FDD | Partial overlapping, the rest is 415-417MHz. | Frequency gap is 3MHz, the UL co-existence frequency range is located at 3MHz below band n87 downlink operating band, but no special note? |
| NR Band n89 | 824 – 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n5, since it is already covered by the requirement in clause 6.6.5.2.2. | n5 | 824 – 849 | 869 – 894 | FDD | Full overlapping | No |
| NR Band n91 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.6.5.2.2. | n20 | 832 – 862 | 791 – 821 | FDD | Full overlapping | No |
| NR Band n92 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.6.5.2.2. | n20 | 832 – 862 | 791 – 821 | FDD | Full overlapping | No |
| NR Band n93 | 880 – 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.2.2. | n8 | 880 – 915 | 925 – 960 | FDD | Full overlapping | No |
| NR Band n94 | 880 – 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.2.2. | n8 | 880 – 915 | 925 – 960 | FDD | Full overlapping | No |
| NR Band n99 | 1626.5 – 1660.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n24, since it is already covered by the requirement in clause 6.6.5.2.2. | n24 | 1626.5 – 1660.5 | 1525 – 1559 | FDD | Full overlapping | No |
| NR band n100 | 874.4 – 880 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n100, since it is already covered by the requirement in clause 6.6.5.2.2. | n100 | 874.4 – 880 | 919.4 – 925 | FDD | Full overlapping | No |
| E-UTRA Band 103 | 787 – 788 MHz | -49 dBm | 1 MHz |  |  |  |  |  |  |  |
| NR Band n105 | 663 – 703 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in n105, since it is already covered by the requirement in clause 6.6.5.2.2. | n105 | 663 – 703 | 612 – 652 | FDD | Full overlapping | No |
| E-UTRA Band 106 or NR Band n106 | 896 – 901 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in Band n5 or n26. This requirement does not apply to BS operating in n106, since it is already covered by the requirement in clause 6.6.5.2.2. | n5 | 824 – 849 | 869 – 894 | FDD | No | Frequency gap is 2MHz, the UL co-existence frequency range is located at 2MHz above DL operating band. |
|  |  |  |  |  | n26 | 814 – 849 | 859 – 894 | FDD | No | Frequency gap is 2MHz, the UL co-existence frequency range is located at 2MHz above DL operating band. |
|  |  |  |  |  | n106 | 896 – 901 | 935 – 940 | FDD | Full overlapping | No |
| NR Band n109 | 703 –733MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n68, it applies for 728 MHz to 733 MHz. | n28 | 703 – 748 | 758 – 803 | FDD | Full overlapping | No |
|  |  |  |  |  | n68 | 698 – 728 | 753 – 783 | FDD | Partial overlapping, the rest is 728-733MHz, it applies for 728 MHz to 733 MHz. | No |
| NR Band n110 | 1390 – 1395 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n110, since it is already covered by the requirement in clause 6.6.5.2.2. | n110 | 1390 - 1395 | 1432 - 1435 | FDD | Full overlapping | No |
| E-UTRA Band 111 | 1800 – 1810 MHz | -49 dBm | 1 MHz |  |  |  |  |  |  |  |