3GPP TSG-RAN WG4 Meeting #115                                                                                             R4-2506043

Malta, MT, May. 19 –May. 23, 2025

Source: 	CATT

Title: 	Discussion on UE Rx requirement for NTN support HPUE

Agenda item:	7.10.3.2.2

Document for:	Approval

- Introduction

In RAN4 #114-bis meeting, a WF [1] on UE Rx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE Rx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Rx requirements for NTN support HPUE was achieved during the previous meetings, as shown below:

### Sub-topic 1-3 Rx requirements Issue 1-10: General discussion of RSD analysis Ad-hoc agreements: - In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation. Issue 1-11: General discussion of RSD of PC1.5 and PC1 Ad-hoc agreements: - Companies agree to bring more analysis for the RSD values for PC1.5/PC1 non-handheld UE. - Further check these in next meeting. Issue 1-13: RSD requirements for PC2 on n255 Ad-hoc agreements: - For n255 PC2: 5MHz; 10MHz; 15 MHz; 20 MHz. RSD@1Tx; [0.8]; [0.8]; [0.9]; [0.9]. RSD@2Tx; [1.6]; [1.6]; [1.7]; [1.7]. Issue 1-14: RSD requirements for PC1.5 and PC1. Way forward: - Encourage companies to further discuss the following proposals: - Option 1: For PC1.5 2Tx (MTK) Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; 2.2; 2.2; 2.2; 2.2. n255; 3.2; 3.2; 3.3; 3.3. n254; 2.2; 2.2; 2.2. - Option 2: For PC1 1Tx (MTK) Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3]; [3]; [3]; [3]. n254; [3]; [3]; [3]. Issue X: Proposal to allow power reduction beyond certain duty cycle Way forward: - Encourage companies to further discuss the following proposal: - Option 1: For NR NTN PC2 and PC1.5, and IoT NTN PC2, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded. (Qualcomm)

Since FR1 NR NTN UE and FR1 NR TN UE are implemented in a similar approach, the study of TN HPUE operating in single band FDD mode could be reused for NR NTN HPUE. Referring to the RSD study TR 38.861 and TR 38.896, the 2Tx architecture causes more severe interference compared to the 1Tx architecture, but the larger Tx-Rx separation prevents potential interference from the Tx chain.

For PC1 NTN HPUE, the Tx architectures will be non-handheld with 1Tx or [4Tx]. For non-handheld 1Tx PC 1 NTN HPUE, since there is only 1 Tx chain, we believed that the RSD is not required. For non-handheld [4Tx] PC 1 NTN HPUE, it is important to reach a final agreements whether the PC1 NTN HPUE will supports the 4Tx architecture or not, then the corresponding RSD value could be derived.

Proposal 1: For NR NTN 1Tx PC 1 non-handheld HPUE, the RSD value could be 0 dB. Meanwhile, it is necessary to determine if the NR NTN PC 1 non-handheld HPUE supports 4Tx.

- Conclusion

This contribution provides our initial views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: For NR NTN 1Tx PC 1 non-handheld HPUE, the RSD value could be 0 dB. Meanwhile, it is necessary to determine if the NR NTN PC 1 non-handheld HPUE supports 4Tx.

- References

- R4-2505113, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #114-bis