3GPP TSG RAN WG4 Meeting #115		R4-2506063

St Julian, Malta, May 19th – 23rd , 2025

Title: 	Further discussion on BS RF requirements for 7MHz channel bandwidth

Agenda Item:	7.8.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, a WF was approved on requirements for 7MHz channel bandwidth, where one issue on BS RF requirements is still open [1]:

3- Remaining open issue: ICS requirement

In this contribution, we present our proposals on ICS requirement for 7MHz channel bandwidth.

# 2 Discussion

According to current specs, ICS requirements for Wide-Area BS are defined as the following table with designated FRC for wanted signal and interfering signal [2].

Table – 1 legacy ICS requirements

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 (Total: 15 PRBs) | 15 | G-FR1-A1-20 (9 PRBs) | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 (Total: 25 PRBs) | 15 | G-FR1-A1-7 (15 PRBs) | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 (25 PRBs) | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |

From the above table, it can be observed that for a small channel bandwidth such as 3MHz and 5MHz, wanted signal and interfering signal occupy the whole transmission bandwidth configuration, which is illustrated in Fig. 1(a) and 1(b) respectively.

Observation 1: For ICS requirements for 3MHz or 5MHz channel bandwidth, wanted signal and interfering signal occupy the whole transmission bandwidth configuration.

- ICS for 3MHz channel bandwidth

- ICS for 5MHz channel bandwidth

- ICS for channel bandwidths 10/15/20/25/30/35MHz

Fig. 1, Legacy wanted signal and interfering signal placement for ICS

As also shown in Fig. 1(c), for ICS requirements for a larger channel bandwidth such as 10, 15, 20, 25,30 and 35MHz, wanted signal and interfering signal have the same number of PRBs and placed symmetrically around the channel centre, leaving unused RBs in the channel bandwidth.

Observation 2: For ICS requirements for a larger channel bandwidth such as 10, 15, 20, 25,30 and 35MHz, wanted signal and interfering signal have the same number of PRBs and placed symmetrically around the channel centre, leaving un-allocated RBs in the channel bandwidth.

If reusing 5MHz ICS for 7MHz channel bandwidth, then there are 10 PRBs unused, however wanted signal and interfering signal do not have the same number of PRBs, hence it is not a consistent way.

Observation 3: It is not a consistent way if reusing 5MHz ICS for 7MHz channel bandwidth having different PRB numbers for wanted and interfering signal and un-allocated RBs.

To introduce ICS requirements for 7MHz channel bandwidth in a consistent way as 3MHz and 5MHz channel bandwidth, one way could be that the number of PRBs for wanted signal is 25, and that of interfering signal is 10, making the whole transmission bandwidth configuration allocated. Fig. 2 illustrates the placement, and the table below shows the configuration.

Fig. 2, Proposed wanted signal and interfering signal placement for ICS for 7MHz channel bandwidth

Table – 2 Proposed ICS requirements for 7MHz channel bandwidth

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 (Total: 15 PRBs) | 15 | G-FR1-A1-20 (9 PRBs) | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 (Total: 25 PRBs) | 15 | G-FR1-A1-7 (15 PRBs) | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 (Total: 35 PRBs) | 15 | G-FR1-A1-1 (25 PRBs) | TBD | TBD | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 (25 PRBs) | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |

In short, for ICS requirements for 7MHz channel bandwidth, RAN4 may reuse RMC for 10/15/20/25/30/35MHz channel bandwidth and reuse interfering signal for 5MHz channel bandwidth.

Proposal 1: RAN4 to define ICS requirements for 7MHz channel bandwidth by reusing RMC for 10/15/20/25/30/35MHz channel bandwidth and interfering signal for 5MHz channel bandwidth.

To do that, simulation campaign could be conducted under such configuration.

Proposal 2: RAN4 to launch simulation campaign under such configuration to specify ICS requirements for 7MHz channel bandwidth.

# 3 Conclusion

In this contribution, we have the following observations and proposals for ICS requirements for 7MHz channel bandwidth:

Observation 1: For ICS requirements for 3MHz or 5MHz channel bandwidth, wanted signal and interfering signal occupy the whole transmission bandwidth configuration.

Observation 2: For ICS requirements for a larger channel bandwidth such as 10, 15, 20, 25,30 and 35MHz, wanted signal and interfering signal have the same number of PRBs and placed symmetrically around the channel centre, leaving un-allocated RBs in the channel bandwidth.

Observation 3: It is not a consistent way if reusing 5MHz ICS for 7MHz channel bandwidth having different PRB numbers for wanted and interfering signal and un-allocated RBs.

Proposal 1: RAN4 to define ICS requirements for 7MHz channel bandwidth by reusing RMC for 10/15/20/25/30/35MHz channel bandwidth and interfering signal for 5MHz channel bandwidth.

Proposal 2: RAN4 to launch simulation campaign under such configuration to specify ICS requirements for 7MHz channel bandwidth.

# Reference

[1] R4-2505098, “WF on requirements for 7MHz channel bandwidth”, Ericsson

[2] TS 38.104, v19.0.0