3GPP TSG-RAN WG4 Meeting #115	R4-2506109

# Malta , MT, 19th – 23rd May, 2025

Title: 	Discussion on (e)RedCap RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.31.5.1

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part1 was approved and some agreements were reached, but there still are a few issues left [1]. This document will discuss (e)RedCap RRM requirements for Rel-19 NTN phase 3 and give our understanding and proposals.

- Discussion

  - General consideration

- 2.1.1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

SDT requirements

For CG-SDT, RAN4 has agreed to remove TA validation based on the RSRP change criterion for (e)RedCap UE with FR1-NTN and to refer to the requirements on TA validation for the PUR requirements for IoT-NTN. Whether to send RAN2 LS were still FFS. The Way Forward in last meeeting is copied as follows [1]:

<Way Forward>: - Proposal 1: For the conclusion on how to define CG-SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2. - The agreement reached in the last meeting only focus on RAN4’s requirements. - Proposal 2: RAN4 to send LS to RAN2 informing them about the RAN4 agreement on the removal of the RSRP change criterion for TA validation in CG-SDT for (e)RedCap UE in FR1-NTN and on the reuse of the TA validation criteria as specified for PUR in IOT-NTN.

In our view, from RAN2’s signaling perspective, they will not to differtiate RedCap UEs and non-RedCap UEs, the agreement only focus on RAN4’s requirements.

The SDT was supported in R17 NTN in RAN2, which was not limited to UE types, and the specific content is no different from SDT defined in TN. According to the RAN2’s spec, if the field of cg-SDT-TA-ValidationConfig not configured, then the UE does not perform RSRP based TA validation. Therefore, from a network perspective, there is no need to restrict configuration, and RAN4 can only make our decision from the perspective of RRM requirements. Therefore, we think that triggering RAN2's discussion on this issue is meaningless and RAN4 does not need to send LS to RAN2.

Observation 1: From RAN2’s signaling perspective, they will not to differtiate RedCap UEs and non-RedCap UEs, the agreement for TA validation in CG-SDT requirements only focus on RAN4’s requirements.

Proposal 1: For TA validation in CG-SDT requirements for (e)RedCap NTN, it is not necessary to send LS to RAN2, and the agreement reached in RAN4#114 meeting only focus on RAN4’s requirements.

- 2.1.2 Features to be considered for (e)RedCap UE with FR1-NTN

The proximity distance between two SMTCs

In #114 meeting, RAN4 discussed the proximity distance between two SMTCs for (e)RedCap UE with FR1-NTN. RAN4 has agreed to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN and the distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, the Way Forward is copied as follows [1]:

<Way Forward>: - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms. - Y=5 ms

In the last meeting, RAN4 has discussed this issue, and most companies can compromise to 1 ms relaxation for proximity compared to legacy 4ms. Since the UE side believed that the baseband capability of RedCap UE will also be correspondingly weakened, so we can also compromise to relax 1ms for the proximity distance between two SMTCs for (e)RedCap UEs in NTN, which is 5ms.

Proposal 2: When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

- the two SMTC occasions are fully or partially overlapping in time domain, or

- the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 5 ms.

Other features

In the last meeting, RAN4 discussed other features, and the agreement is copied as below [1]:

Agreement: - SMTC adjustment in idle/inactive is kept [mandatory] for (e)RedCap UEs with NTN. - From RAN4 perspective, location-based measurement report trigger is kept optional for (e)RedCap UEs with NTN.

For SMTC adjustment in idle/inactive, some companies believed that it is optional for (e)RedCap UEs with NTN since it is optional in TS 38.822 as following [2]:

| 34-11 | SMTC adjustment in idle/inactive | It's optional for UE to perform SMTC adjustment in RRC_IDLE/RRC_INACTIVE | Optional without capability signalling |
| --- | --- | --- | --- |

However, most of R17 features are optional for UE regardless whether it is mandatary or optional for UE supporting NTN. Therefore, we don’t think it is a reason to regard this UE feature as an optional UE capability for NTN UE.

In addition, some of description for SMTC adjustment in TS 38.331 is copied as below [3]:

smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

In our view, it is quite important feature in RRM measurements for NTN, if it is not supported for UE, the UE will not be able to adjust SMTC time offset to measure neighbour cell due to the time difference between serving cell and neighbour cell will change. Therefore, we think that SMTC adjustment in idle/inactive should be mandatory in NTN regardless of the UE type and it also doesn’t need very high complexity for RedCap UE.

Proposal 3: SMTC adjustment in idle/inactive should be supported by (e)RedCap UEs with NTN.

However, if companies have different views about the legacy SMTC adjustment in idle/inactive feature, whether optional or mandatory, this feature was depends on RAN2 in R17, and RAN4 will not make any modifications on this feature for RedCap UE with NTN.

Observation 2: The feature of SMTC adjustment in idle/inactive was depends on RAN2 in R17.

Proposal 4: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN.

  - The impact of HD-FDD

    - General HD-FDD related impact

In the last meeting, RAN4 discussed the general HD-FDD related impact for RedCap UE with NTN, the agreements on some requirements were reached, and there was still some requirements for further discussion. The agreements are copied as below [1]:

Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measurement interval. - FFS the same or different option for DRX and non-DRX

According to the above, some requirements has been agreed to reuse the legacy requirements and applicable conditions, there was still FFS for minimum requirement for L1 indication for RLM and LR and scheduling availability of UE during L1-RSRP measurement.

Regarding how to reflect the HD-FDD impact, there was three options.

For Option a, which means the requirement is extended and the samiliar approach is used as NR-U for defining the HD-FDD impact. In our view, the NR-U approach is very complicated, also considering the time point for completing this WI, we don’t want to spend too much time to discuss how to extend the requirement.

Proposal 5: For Option a, we don’t want to spend too much time to discuss how to extend the requirement.

For Option b, we prefer to use this clarification. Specifically, in the existing scheduling availability of UE during L1-RSRP measurement in HD-FDD bands, CBD evaluation is prioritized over UL transmission, and the FR1 requirements are copied as below [4]:

#### 9.5B.6.1 Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1 For FD-FDD and TDD RedCap UEs, there are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1. For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1. #### 9.5B.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement. - For FD-FDD and TDD RedCap UEs, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement. - For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1.

However, according to the agreement reached in RAN1:

RAN1#118bis: Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS RAN1#119: Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

For collision case 3, semi-statically configured CSI-RS and PRS reception may be deprioritized compared to UL transmission, the network is allowed to indicate UL overriding DL, so if  CSI-RS and PRS resources collide with UL transmission, CSI-RS and PRS based measurement will be impacted. In our view, a simple way is to clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

Proposal 6: For scheduling availability of UE during L1-RSRP measurement in HD-FDD bands, a simple way is to clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

For option c, we think that there is no much difference compared with the legacy condition. For minimum requirement for L1 indication for RLM and LR, we think that the following condition can be reused for HD-FDD RedCap UEs with NTN:

RLM: When DRX is not used for HD-FDD RedCap UEs, TIndication_interval,RedCap is max(10 ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 8.1B.2 if the RLM-RS resource is SSB, or TCSI-RS specified in clause 8.1B.3 if the RLM-RS resource is CSI-RS, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period. In case DRX is used for HD-FDD RedCap UEs, TIndication_interval,RedCap is Max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX_cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX_cycle_length is greater than 320ms, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period. LR: For HD-FDD UE, the above conditions and requirements TIndication_interval_BFD_Redcap apply given that for each BFD-RS configuration, at least one BFD-RS sample must fall with DL occasion within an indication period

Proposal 7: For minimum requirement for L1 indication for RLM and LR, the legacy condition can be reused for HD-FDD RedCap UEs with NTN.

  - UE capabilities

    - General principle for designing the UE capabilities for (e)RedCap UE with NTN

In the last meeting, the WF on UE capabilities for (e)RedCap UE with FR1-NTN bands is copied as below [1]:

<Way Forward>: - Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - Proposal 2: Introduce new capability to declare support over RedCap over NTN. - Proposal 3: RAN4 to not discuss or conclude on UE capability signaling and structure for RedCap NTN, unless specific requests are received from other working groups via LSs.

Firstly, both Proposal 1 and Proposal 2 are workable and what we are discussing here is which method is better.

In our view, the advantage of Proposal 1 is simplicity and greater rationality, as currently no new features have been introduced for (e)RedCap UE with FR1-NTN, only some features may need to be modified. Afterwards, we can define corresponding signaling for certain UE capabilities that need to be modified or added.

Observation 3: Currently no new features have been introduced for (e)RedCap UE with FR1-NTN, only some existing features may need to be modified.

The company that support Proposal 2 is mainly concerned that some of the requirements defined for (e)RedCap UE in TN networks may not be supported or defined in NTN scenarios, and some of the requirements defined for normal UE in NTN networks may no longer be supported for (e)RedCap UE. Therefore, they want to define a new set of UE capabilities for (e)RedCap UE with FR1-NTN. However, this is all from the RAN4 perspective of defining requirements.

However, from the perspective of RAN2’s signaling, all features introduced for (e)RedCap UEs in TN and normal UEs in NTN have not been excluded. In addition, most of the features of (e)RedCap UE in NTN will reuse legacy UE capabilities for NTN and RedCap, so it may not be necessary to define new capabilities as a whole, as there may be duplication and waste, it will also increase the workload of RAN2.

Observation 4: From the perspective of RAN2’s signaling, all features introduced for (e)RedCap UEs in TN and normal UEs in NTN have not been excluded.

Observation 5: Most of the features of (e) RedCap UE in NTN will reuse legacy UE capabilities, so it may not be necessary to define new capabilities as a whole.

Proposal 8: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

    - ParallelSMTC for (e)RedCap UE with NTN

For Feature 25-1(parallelSMTC), the agreement reached in the last meeting is copied as below [1]:

Agreement: - For Feature 25-1 (parallelSMTC), not differentiate 1Rx and 2Rx RedCap UEs for this UE capability. - RedCap UE is mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier - ‘Parallel’ means the SMTCs are TDM with proximity distance.

According to the above agreement, there is a little difference between the UE capabilities for (e)RedCap UE with NTN and normal UE with NTN, which is the number of supported SMTC in parallel. Even though most of description of this feature is the same as R17 NTN, a new UE capability for (e)RedCap UE with NTN also need to be defined. The new UE capability can be defined as follow:

Observation 6: There is a little difference between the UE capabilities for (e)RedCap UE with NTN and normal UE with NTN, which is the number of supported SMTC in parallel.

Proposal 9: Introduce new UE capability to indicate the supported SMTC in parallel for (e)RedCap UE with NTN.

| Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| Parallel measurements on multiple SMTC-s for a single frequency carrier | Support of measurements on target cells belonging to 4 SMTC-s on a single frequency carrier |  | FDD only | FR1 only | RedCap UE is mandatory to support 2 if the feature is supported | Optional with capability signaling |

- Conclusion

This document discussed the issues for (e)RedCap RRM requirements for R19 NR NTN and presented following observations and proposals:

Observation 1: From RAN2’s signaling perspective, they will not to differtiate RedCap UEs and non-RedCap UEs, the agreement for TA validation in CG-SDT requirements only focus on RAN4’s requirements.

Proposal 1: For TA validation in CG-SDT requirements for (e)RedCap NTN, it is not necessary to send LS to RAN2, and the agreement reached in RAN4#114 meeting only focus on RAN4’s requirements.

Proposal 2: When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

- the two SMTC occasions are fully or partially overlapping in time domain, or

- the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 5 ms.

Proposal 3: SMTC adjustment in idle/inactive should be supported by (e)RedCap UEs with NTN.

Observation 2: The feature of SMTC adjustment in idle/inactive was depends on RAN2 in R17.

Proposal 4: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN.

Proposal 5: For Option a, we don’t want to spend too much time to discuss how to extend the requirement.

Proposal 6: For scheduling availability of UE during L1-RSRP measurement in HD-FDD bands, a simple way is to clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

Proposal 7: For minimum requirement for L1 indication for RLM and LR, the legacy condition can be reused for HD-FDD RedCap UEs with NTN.

Observation 3: Currently no new features have been introduced for (e)RedCap UE with FR1-NTN, only some existing features may need to be modified.

Observation 4: From the perspective of RAN2’s signaling, all features introduced for (e)RedCap UEs in TN and normal UEs in NTN have not been excluded.

Observation 5: Most of the features of (e) RedCap UE in NTN will reuse legacy UE capabilities, so it may not be necessary to define new capabilities as a whole.

Proposal 8: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

Observation 6: There is a little difference between the UE capabilities for (e)RedCap UE with NTN and normal UE with NTN, which is the number of supported SMTC in parallel.

Proposal 9: Introduce new UE capability to indicate the supported SMTC in parallel for (e)RedCap UE with NTN.

| Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| Parallel measurements on multiple SMTC-s for a single frequency carrier | Support of measurements on target cells belonging to 4 SMTC-s on a single frequency carrier |  | FDD only | FR1 only | RedCap UE is mandatory to support 2 if the feature is supported | Optional with capability signaling |

- Reference

- R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- 3GPP TS 38.822, NR User Equipment (UE) feature list

- 3GPP TS 38.331, NR Radio Resource Control (RRC) protocol specification

- 3GPP TS 38.133, NR Requirements for support of radio resource management