3GPP TSG-RAN WG4 Meeting #115	R4-2506121

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	PC2 RedCap UE RF requirements

Agenda item:	7.6.2

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

Issue 1-1-1: REFSENS for 1Rx in FD-FDD Agreement: Consider the following formats for specifying REFSENS for 1Rx PC2 RedCap in FD-FDD - Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on RSD ranges - Flat 3dB ΔR 1R for all CBW no larger than 20MHz or use values for FDD bands in table 7.3I.2-1 for bands whose PC2 1Tx/2RX RSD is 0dB. - Companies to evaluate REFSENS for other bands. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. - Option2: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on the following conditions - ΔR 1R value may be considered from LTE TS 36.101 Table 7.3.1E-1A for bands and CBW which are already defined in TS 36.101 Table 7.3.1E-1A. The ΔR 1R value is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1. - Companies are encouraged to bring measurement based evaluation for ΔR1R for other bands and CBW which are not covered by LTE TS 36.101 Table 7.3.1E-1A. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. Way forward: It is encouraged for companies to provide analysis for option 1 and 2. Note: Companies are encouraged to evaluate all specified PC2 FDD bands. The WI is intended to be completed in May meeting according to WI plan.

Discussion

One RX REFSENS in dBm

In the previous meeting, two options were agreed to derive the REFSENS based on whether the RSD is zero or a single RX relaxation exist in the LTE specs. In the LTE specifications, single RX relaxation exists for n2, n3, n8 (channel bandwidth of 5 and 10 MHz only), n13 (channel bandwidth of 5 and 10 MHz only), and n25. Further, in NR specification RSD is zero for n1, n66, and n70.

For the rest of the bands, following approach was used to calculate the REFSENS.

- For the desired band, calculate the thermal noise keeping into account any change in noise figure assumption.

- For the desired band, calculate PC2 REFSENS for 2 RX (SPC2) by adding the 2 RX RSD (reference sensitivity degradation) to the PC3 REFSENS (SPC3).

- Now using this PC2 REFSENS for 2 RX (SPC2), estimate the self-interference power in the primary receiver (Pp) assuming MRC combining and that the power delta between primary and diversity receiver is 6 dB (Δpri-div).

- Using this primary path self-interference along with the thermal noise, calculate the REFSENS for PC2 1 RX (SPC2-Δ1R)

Based on these steps along with the previously agreed WF, we propose the following REFSENS values for 1 RX PC2 (e)RedCap devices. Based on these, ΔR1R can be derived.

Table 1 Single RX REFSESN in dBm for bands at 15 KHz SCS for which PC2 is defined.

| Band | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | -97.5 | -93.8 | -92.0 | -90.8 |
| n2 | -94.7 | -91.4 | -89.4 | -88.1 |
| n3 | -94.0 | -90.8 | -89.0 | -87.8 |
| n7 | -94.6 | -91.4 | -89.6 | -88.4 |
| n8 | -94.0 | -90.6 | -87.2 | -79.3 |
| n13 | -93.2 | -89.9 |  |  |
| n14 | -93.5 | -90.1 |  |  |
| n25 | -93.2 | -90.0 | -88.1 | -86.7 |
| n66 | -97.0 | -93.3 | -91.5 | -90.3 |
| n70 | -97.5 | -93.8 | -92.0 | -90.8 |
| n71 | -93.6 | -89.9 | -87.4 | -79.5 |
| n85 | -93.3 | -89.6 | -77.0 |  |

Agree to the proposed REFSENS numbers for one RX PC2.

Specification impact

One option is to add the absolute REFSENS numbers in dBm to the specification section 7.3I.2. However, then the table has to be include values for both 15 KHz and 30 KHz SCS. Another approach is to add a new single antenna port reference sensitivity allowance ΔR1R,PC2 table specifically for PC2 FDD bands. Although, it does create a delta over delta situation, but it maintains clarity and the intuitive comprehension of the specifications.

Agree to add a new single antenna port reference sensitivity allowance ΔR1R,PC2 table specifically for PC2 FDD bands in section 7.3I.2.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Agree to the proposed REFSENS numbers for one RX PC2.

Proposal 2: Agree to add a new single antenna port reference sensitivity allowance ΔR1R,PC2 table specifically for PC2 FDD bands in section 7.3I.2.

- References

- R4-2505231, WF on PC2 RedCap UE RF requirements, China Telecom, RAN4#114bis, Wuhan, Hubei, China, 07th – 11th April, 2025.