3GPP TSG-RAN WG4 Meeting#115	R4-2506181

# Malta, MT, 19th  – 25th  May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1: PC1.5 for 2Tx inter-band uplink CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A Draft CR on duty cycle solution for PC1.5 of CA was previously endorsed in R4-2502865. However, it did not include needed changes to Table 6.2A.1.3-1 or changes for ΔPPowerClass,CA and there was no example band combination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds PC1.5 columns to Table 6.2A.1.3-1 and removes unused columns for PC5 for compactness. and adds needed changes for ΔPPowerClass,CA Also adds CA_n41A-n77A PC1.5 as an example band combination. Modifies Note 14 for the tables in 5.5A.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1.5 for 2Tx inter-band UL CA not fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.3.1, 6.2A.1.3, 6.2A.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<First Change>

#### 5.5A.3.1	Configurations for inter-band CA (two bands)

<Unchanged Tables omitted>

Table 5.5A.3.1-1j: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n48A | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n48B | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n41A-n48C | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n48 | CA_n48C_BCS1 |  |
| CA_n41A-n48(2A) | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
| CA_n41C-n48A | CA_n41A-n48A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
| CA_n41C-n48B | CA_n41A-n48A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n41C-n48C | CA_n41A-n48A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n48 | CA_n48C_BCS1 |  |
| CA_n41(2A)-n48A | CA_n41A-n48A | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n48B | CA_n41A-n48A | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n41(2A)-n48C | CA_n41A-n48A | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n48 | CA_n48C_BCS1 |  |
| CA_n41(2A)-n48(2A) | CA_n41A-n48A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
| CA_n41A-n50A | CA_n41A-n50A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n50 | 5, 10, 15, 20, 40, 50, 60, 801 |  |
| CA_n41A-n66A | n418,9 n668 CA_n41A-n66A8,13,14 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A | n418, 9 n668CA_n41A-n66A8,13,14 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A) | n418, 9 n668 CA_n41A-n66A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41C-n66A | n418, 9 n668 CA_n41A-n66A8,13,14 CA_n41C | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | n418, 9 CA_n41A-n66A8,13,14 CA_n41C8 CA_n41C-n66A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A) | n418, 9 n668 CA_n41A-n66A8 CA_n41C8 CA_n41C-n66A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n66(2A) | n418, 9 n668 CA_n41A-n66A8 | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41(3A)-n66A | n418, 9 n668 CA_n41A-n66A8 | n41 | CA_n41(3A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40, |  |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n66(2A) | n418,9 n668 CA_n41A-n66A8 | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41(A-C)-n66A | n418, 9 n668 CA_n41C8 CA_n41A-n66A8 CA_n41C-n66A | n41 | CA_n41(A-C)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n66(2A) | n418,9 n668 CA_n41C8 CA_n41A-n66A8 CA_n41C-n66A | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41A-n70A | CA_n41A-n70A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n41A-n71A | n418,9 n718 CA_n41A-n71A8,13,14 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B | n418,9 n718 CA_n41A-n71A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41A-n71(2A) | n418,9 n718 CA_n41A-n71A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41C-n71A | n418, 9 n718 CA_n41A-n71A8,13 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | n418, 9 n718 CA_n41A-n71A8,13 CA_n41C8 CA_n41C-n71A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71(2A) | n418,9 n718 CA_n41A-n71A8 CA_n41C-n71A CA_n41C8 | n41 | CA_n41C_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n71A | n418, 9 n718 CA_n41A-n71A8,13,14 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A)_BCS3 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n71(2A) | n418,9 n718 CA_n41A-n71A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n71B | n418,9 n718 CA_n41A-n71A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41(3A)-n71A | n418,9 n718 CA_n41A-n71A8 | n41 | CA_n41(3A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n71B | n418,9 n718 CA_n41A-n71A8 | n41 | CA_n41(3A)_BCS4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS4 and 5 |  |
| CA_n41(3A)-n71(2A) | n418,9 n718 CA_n41A-n71A8 | n41 | CA_n41(3A)_BCS4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS4 and 5 |  |
| CA_n41(A-C)-n71A | n418,9 n718 CA_n41A-n71A8 CA_n41C8 CA_n41C-n71A | n41 | CA_n41(A-C)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n71B | n418,9 n718 CA_n41A-n71A8 CA_n41C8 CA_n41C-n71A | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41(A-C)-n71(2A) | n418,9 n718 CA_n41A-n71A8 CA_n41C8 CA_n41C-n71A | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41C-n71B | n418,9 n718 CA_n41A-n71A8 CA_n41C-n71A CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41A-n74A | n418 CA_n41A-n74A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n41A-n77A | n418,9 n778,9 CA_n41A-n77A8, 13,14 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41B-n77A | CA_n41A-n77A | n41 | CA_n41B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41(2A)-n77A | n418,9 n778,9 CA_n41A-n77A8,13 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n77A | n418,9 n778,9 CA_n41A-n77A8 | n41 | CA_n41(3A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n77A | n418,9 n778,9 CA_n41C8 CA_n41A-n77A8 CA_n41C-n77A | n41 | CA_n41(A-C)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n77A | n418,9 n778,9 CA_n41A-n77A8,13 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n418,9 n778,9 CA_n41A-n77A8,13 CA_n41C8 CA_n41C-n77A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n77(2A) | n418,9 n778,9 CA_n77(2A)8 CA_n41A-n77A8,13 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  | n418,9 n778,9 CA_n41A-n77A8,13 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41C-n77(2A) | n418,9 n778,9 CA_n41A-n77A8 CA_n41C8 CA_n41C-n77A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41A-n77(3A) | n418,9 n778,9 CA_n41A-n77A8 CA_n77(2A) | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(3A)_BCS 4 and 5 |  |
| CA_n41(2A)-n77(2A) | - | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  | n418,9 n778,9 CA_n41A-n77A8 | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41A-n77C | CA_n41A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n41A-n78A | CA_n41A-n78A | n41 | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n78(2A) | CA_n41A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n41A-n78C | CA_n41A-n78A CA_n41A-n78C CA_n78C | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 | 1 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n41A-n79A | n418,9 n798,9 CA_n41A-n79A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 2 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n79C | n418,9 n798,9 CA_n41A-n79A8 CA_n79C8 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  | CA_n79C CA_n41A-n79A CA_n41A-n79C | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n41C-n79A | n418,9 n798,9 CA_n41A-n79A8 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n79C | CA_n41C CA_n79C CA_n41A-n79A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n41A-n85A | n418,9 CA_n41A-n85A8,13,14 | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n85A | CA_n41A-n85A CA_n41C | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n85A | CA_n41A-n85A | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n85A | CA_n41A-n85A | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n85A | CA_n41A-n85A CA_n41C | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n104A | CA_n41A-n104A | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n104 | 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41C-n104A | CA_n41C CA_n41A-n104A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n104 | 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n104C | CA_n104C CA_n41A-n104A | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n104 | CA_n104C_BCS0 |  |

<Unchanged Tables omitted>

The following notes are applied to the above tables:

NOTE 1:	This UE channel bandwidth is applicable only to downlink.

NOTE 2:	The minimum requirements for intra-band contiguous or non-contiguous CA apply.

NOTE 3:	For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs.

NOTE 4:	This UE channel bandwidth is optional in this release of the specification.

NOTE 5:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration.

NOTE 6:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration

NOTE 7:	Limited to operation at 3450-3550 MHz and 3700–3980 MHz.

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

NOTE 11: The CA configurations are given in Table 5.5A.1-1 or Table 5.5A.2-1 in this specification

NOTE 12: Void.

NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band.

NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 1 Tx or 2Tx antenna connectors in the other band.

NOTE 15: Uplink is only in n5 for CA_n5-n8.

NOTE 16: For UEs only supporting DL CA_n26-n28, uplink support in band n26 is optional, if the UE supports CA_n26-n28 UL configuration, it should also support UL in band n26 and n28.

<Next Change>

#### 6.2A.1.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in Table 6.2.1-1 apply for power class 3 and other power classes if indicated in clause 5.5A.3.

For inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the transmitter power requirements specified in subclause 6.2A.1.1 apply.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmitter power requirements specified in subclause 6.2A.1.2 apply. For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.3-1.

| NOTE 1: Void NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports a PC2 uplink CA configuration with single carrier for each individual band and a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 8: The UE that supports a PC3 uplink CA configuration with a composite of supporting PC3 within an NR TDD or FDD band and PC5 within a second NR band listed in Table 6.2F.1-1 may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 9: The UE supports either PC3 or PC2 within each NR TDD band. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 6.2A.1.3-1: UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n5A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n7A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n8A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n18A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n20A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n26A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n28A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n40A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n41A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n1A-n46A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n71A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n74A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n1A-n78A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n1A-n78C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n79A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n1A-n102A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n102B |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n102C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n1A-n105A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n5A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n7A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n12A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n14A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n30A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n41A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n48A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n71A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n2A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n2A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n5A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n7A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n8A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n18A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n20A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n26A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n28A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n34A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n38A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n40A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n3A-n41A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n3A-n41C |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n3A-n71A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n74A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n3A-n78A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n3A-n78C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n79A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n3A-n79C |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n3A-n102A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n102B |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n102C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n104A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n3A-n105A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n7A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n12A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n13A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n14A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n25A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n28A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n30A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n40A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n41A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n48A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n5A-n78A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n5A-n78C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n79A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n5A-n105A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n8A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n20A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n25A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n26A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n28A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n40A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n46A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n71A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n7A-n78A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n7A-n102A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n102B |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n102C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n7A-n105A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n8A-n20A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n8A-n28A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n8A-n34A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n8A-n39A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n8A-n40A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n8A-n41A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n8A-n41C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n8A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n8A-n78A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n8A-n78C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n8A-n79A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n8A-n79C |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n8A-n104A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n12A-n25A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n12A-n30A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n12A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n12A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n12A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n13A-n25A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n13A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n13A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n14A-n30A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n14A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n14A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n18A-n28A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n18A-n40A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n18A-n41A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n18A-n74A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n18A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n18A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n20A-n28A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n20A-n41A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n20A-n71A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n20A-n77A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n20A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n24A-n41A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n24A-n48A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n24A-n77A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n25A-n38A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n25A-n41A |  |  |  |  | 266 | +2/-32 | 23 | +2/-3 |
| CA_n25A-n41C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_25A-n48A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n25A-n66A |  |  |  |  | 269 | +2/-32 | 23 | +2/-3 |
| CA_n25A-n71A |  |  |  |  | 269 | +2/-32 | 23 | +2/-3 |
| CA_n25A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n78A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n85A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n26A-n28A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n26A-n48A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n26A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n26A-n70A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n26A-n77A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n26A-n78A |  |  |  |  | 266 | +2/-32 | 23 | +2/-3 |
| CA_n28A-n34A |  |  |  |  | 266 | +2/-32 | 23 | +2/-3 |
| CA_n28A-n39A |  |  |  |  | 266 | +2/-32 | 23 | +2/-3 |
| CA_n28A-n40A |  |  |  |  | 266 | +2/-32 | 23 | +2/-3 |
| CA_n28A-n41A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n41C |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n46A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n28A-n50A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n28A-n74A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n28A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n78A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n79A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n79C |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n102A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n28A-n102B |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n28A-n102C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n34A-n79A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n30A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n30A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n34A-n39A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n34A-n40A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n34A-n41A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n34A-n41C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n34A-n79A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n34A-n79C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n38A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n38A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n39A-n40A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n39A-n41A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n39A-n41C |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n39A-n79A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n39A-n79C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n40A-n41A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n40A-n41C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n40A-n77A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n40A-n78A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n40A-n79A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n40A-n79C |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n40A-n105A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n41A-n48A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n41A-n50A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n41A-n66A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n70A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n41A-n71A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n74A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n77A |  |  | 29x | +2/-3 | 269 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n41A-n79A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n79C |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n41C-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n41C-n71A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n41C-n77A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n41C-n79A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n85A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n104A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n46A-n48A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n46A-n48B |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n46A-n77A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n46A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n48A-n66A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n48A-n70A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n48A-n71A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n48A-n96A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n48B-n96A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n48A-n96B |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n50A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n66A-n71A |  |  |  |  | 269 | +2/-32 | 23 | +2/-3 |
| CA_n66A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n78A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n85A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n70A-n71A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n70A-n77A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n70A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n71A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n71A-n78A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n74A-n77A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n74A-n78A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n77A-n79A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n77A-n85A |  |  |  |  | 266 | +2/-3 | 23 | +2/-3 |
| CA_n77A-n102A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n77A-n102B |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n77A-n102C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n78A-n79A |  |  |  |  | 269 | +2/-3 | 23 | +2/-3 |
| CA_n78A-n92A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n78A-n102A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n78A-n102B |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n78A-n102C |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n78A-n104A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n78A-n105A |  |  |  |  |  |  | 23 | +2/-3 |
| CA_n100A-n101A |  |  |  |  |  |  | 23 | +2/-3 |
| NOTE 1: Void NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports a PC2 uplink CA configuration with single carrier for each individual band and a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 8: The UE that supports a PC3 uplink CA configuration with a composite of supporting PC3 within an NR TDD or FDD band and PC5 within a second NR band listed in Table 6.2F.1-1 may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 9: The UE supports either PC3 or PC2 within each NR TDD band. NOTE x: The UE supports PC2 within each NR band. |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.3-1 and the supported power class enables the higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	 if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3;

–	else if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38. 306 (The exact evaluation period is no less than one radio frame); or

–	 if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 26dBm or lower;

–	shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3;

–	else;

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 50%  ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

Table 6.2A.1.3-2 Void

<Unchanged clauses omitted>

##### 6.2A.4.1.3	Configured transmitted power for Inter-band CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in clause 6.2.4, except that the UE power class for serving cell c on the specific operating band shall be determined by the ue-PowerClassPerBandPerBC-r17 IE [7] as indicated for the band combination if signalled.

For uplink inter-band carrier aggregation, MPRc and A-MPRc apply per serving cell c and are specified in clause 6.2.2 and clause 6.2.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c  is calculated under the assumption that the transmit power is increased independently on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink inter-band carrier aggregation with one serving cell c per operating band when same slot symbol pattern is used in all aggregated serving cells,

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c/ (tC,c),  pPowerClass.c/(MAX(mprc·∆mprc, a-mprc)·tC,c ·tIB,c·tRxSRS,c) , pPowerClass,c/pmprc], PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.3-1 without taking into account the tolerance specified in the Table 6.2A.1.3-1; If the UE indicates higherPowerLimit-r17 for an eligible CA configuration as specified in Table 6.2A.1.3-1 and ΔPPowerClass, CA = 0, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.

-	pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1-1 according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise without taking into account the tolerance;

-	For uplink inter-band UL carrier aggregation with a single uplink component carrier configured in any bands;

-	If the UE indicates the support of higherPowerLimit-r17 for this band combination which is an eligible CA configuration as specified in Table 6.2A.1.3, and;

-	If the UE further indicates support of powerBoosting-pi2BPSK-QPSK-Modified-r18 or powerBoosting-pi2BPSK-QPSK-r18 for any of the bands that comprise the band combination, pPowerClass,c is replaced by pPowerClass,c ∙ ∆pPowerBoost,c, for the one band for which IE powerBoostPi2BPSK-r18 or powerBoostQPSK-r18  is set to 1, where ∆pPowerBoost,c is linear value of ΔPPowerBoost.c as specified in 6.2.4, with the power class according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise.

-	ΔPPowerClass,CA:=

-	For a power class 2 UE it is 3 dB when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.3; otherwise ΔPPowerClass, CA = 0 dB;

–	For a power class 1.5 UE, it is 6dB when the requirements of default power class are applied as specified in sub-clause 6.2A.1.3; and it is 3dB when the requirements of power class 2 are applied as specified in sub-clause 6.2A.1.3; otherwise ΔPPowerClass, CA = 0 dB;

NOTE:	UE reports ∆PPowerClass,CA when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the powerClass after the duty cycle exceedance.

-	mpr c and a-mpr c are the linear values of MPR c and A-MPR c as specified in clause 6.2.2 and clause 6.2.3, respectively;

-	∆mpr c is the linear value of ∆MPR c as specified in clause 6.2.2;

-	pmprc is the linear value of P-MPR c;

-	∆tRxSRS,c  is the linear value of ∆TRxSRS,c;

-	tC,c is the linear value of TC,ctC,c = 1.41 when NOTE 2 in Table 6.2A.1.3-1 applies for a serving cell c, otherwise tC,c = 1;

-	tIB,c  is the linear value of the inter-band relaxation term TIB,c of the serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; otherwise tIB,c In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.For uplink inter-band carrier aggregation with one serving cell c per operating band when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in clause 6.2.4, except that the UE power class for the serving cell c(i) on the specific operating band shall be determined by the ue-PowerClassPerBandPerBC-r17 IE [7] as indicated for the band combination if signalled.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i),i (p) + pCMAX_ H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

where pCMAX_L,f,c (i),i  and pCMAX_ H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale and pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1-1 according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise without taking into account the tolerance; If the UE indicates higherPowerLimit-r17, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.

For combinations of intra-band and inter-band carrier aggregation with UE configured for transmission on three serving cells (up to two contiguously aggregated carriers per operating band), the following apply:

The UE power class for the serving cell(s) on the operating band Bi including intra-band carrier aggregation shall be determined by the ue-PowerClassPerBandPerBC-r17 IE [7] as indicated for the band combination if signalled.

For the case when the UE indicates higherPowerLimit-r17, PPowerClass,CA is replaced by 10 log10 (pPowerClass,A + pPowerClass,CA,B).

Where

-	pPowerClass,A is the linear value of the maximum UE power for serving cell c on the operating band A specified in Table 6.2.1-1 according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise without taking into account the tolerance;

-	pPowerClass,CA,B is the linear value of the maximum UE power for serving cell(s) on the operating band B including intra-band carrier aggregation specified in Table 6.2F.1A.2-1 according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass, otherwise without taking into account the tolerance.

For the case when p and q belong to the same band and k belongs to a different band, but p, q and k are of the same numerology and slot patterns.

PCMAX_L = MIN {10log10∑( pCMAX_L, Bi), PEMAX,CA, PPowerClass.CA }

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass.CA }

Where

-	pCMAX_L, Bi is the linear values of PCMAX_L specified for the specific operating band Bi.

-	The linear value of PCMAX_L specified for uplink intra-band contiguous carrier aggregation in subclause 6.2A.4.1.1 applies for operating band supporting two contiguous serving cells, designated by its band index Bi. The linear value of PCMAX_L specified for single carrier in subclause 6.2.4 applies for operating band Bj supporting one serving cell.

For the case when p and q belong to the same band and are of the same numerology i and slot patterns (p,q),while k belong to a different band and is of different numerology j and/or slot pattern on the 3rd cell then:

PCMAX_L (p,q,k) = MIN {10 log10 [pCMAX_L,Bi,i(p,q) + pCMAX_L,c(3),Bj,j(k)], PEMAX,CA, PPowerClass.CA }

PCMAX_H (p,q,k) = MIN {10 log10 [pCMAX_ H,Bi,i (p,q) + pCMAX_ H,c(3), Bj,j(k)], PEMAX,CA, PPowerClass.CA }

Where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PEMAX,CA is p-UE-FR1 value signalled by RRC and defined in [38.331];

-	PPowerClass.CA is the maximum UE power specified in Table 6.2A.1.3-1 without taking into account the tolerance specified in the Table 6.2A.1.3-1 or Table 6.2F.1A.1-1 for shared spectrum bands;

-	pCMAX_L,c(3),Bj,j(k) and pCMAX_ H,c(3), Bj,j(k) are the linear values of PCMAX_L and PCMAX_H respectively, specified for single carrier in subclause 6.2.4 and applies for operating band supporting one serving cell in the Bj band on numerology j, using slot pattern k;

-	pCMAX_L,Bi,i(p,q)  and pCMAX_ H,Bi,i (p,q) are the linear values of PCMAX_L respectively PCMAX_H for uplink intra-band contiguous carrier aggregation specified in subclause 6.2A.4.1.1 which applies for operating band Bi on numerology i, supporting two contiguous serving cells, using the same slot pattern (p,q).

TREF and Teval are specified in Table 6.2A.4.1.3-0 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

Table 6.2A.4.1.3-0: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for slots i and i + 1 applies for any overlapping portion of slots i and i + 1. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.1.3-1. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.3-1 for inter-band carrier aggregation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or symbol pattern, shall be within the following range:

P'CMAX_L–  MAX{TL, TLOW (P'CMAX_L)} ≤  P'UMAX  ≤  P'CMAX_H + THIGH (P'CMAX_H)

P'UMAX = 10 log10 ∑ p'UMAX,c

where p'UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P'CMAX) and THIGH(P'CMAX) for applicable values of P'CMAX are specified in Table 6.2A.4.1.3-1 for inter-band carrier aggregation. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.3-1 for inter-band carrier aggregation.

where:

P'CMAX_L  = MIN{ MIN {10log10∑( pCMAX_L,f,c(i),i), PPowerClass,CA} over all overlapping slots in TREF}

P'CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c , PPowerClass,CA} over all overlapping slots in TREF}

If the UE indicates higherPowerLimit-r17, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c

Table 6.2A.4.1.3-1: PCMAX tolerance for uplink inter-band CA (two bands)

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

<End of changes>