3GPP TSG-RAN WG4 Meeting #115	R4-2506237

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Ku-band NTN

Agenda item:	7.11.7

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM requirements for Ku-band NTN based on approved WF [1].

Discussion

Assupmtion for defining RRM requirments in Ku-band NTN

Before discussing RRM requirements for Ku-band NTN, RAN4 needs to discuss the assumption for VSAT UE to support Ku-band. In Rel-18 Ka-band NTN, RRM requirements have defined based on following assumption:

- Single SAN Tx beam per radio cell in DL

- For inter satellite HO, only blind HO and no neighbor cell measurement

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping)

Since the UE type (VSAT UE) of Ka-band and Ku-band is the same, the same assumption considered in Rel-18 can be considered for Ku-band NTN.

- Proposal 1: The same assumptions considered for Rel-18 Ka-band RRM requirements should apply to Ku-band RRM requirements for both FR1-NTN and FR2-NTN numerologies;

- Single SAN Tx beam per radio cell in DL

- For inter satellite HO, only blind HO and no neighbor cell measurement

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping)

Based on the assumption mentioned above, the RRM requirements defined for Ka-band could be resued for Ku-band RRM requirements, except for timing requirements.

- Proposal 2: FR2 (Ka-band) NTN RRM requirements could be reused for Ku-band NTN RRM requirements including both FR1-NTN and FR2-NTN numerologies (not to introduce separate RRM requirements for Ku-band NTN), except for UL transmit timing requirements.

Inter-satellite measurement

In the last meeting, RAN4 discussed whether to consider RRM requirements for inter-satellite cell reslection and satellite switching with re-sync. Even if FR1-NTN numerology is used in Ku-band, the UE behavior is the same VSAT defined in FR2-NTN Ka-band. If RAN4 consideres to define inter-satellite measurement in Ku-band, only for VSAT type 5 which has electronic steering antenna might be feasible, and it would be good chance to introduce new VSAT UE behavior in Ku-band . However, based on Rel-19 timeline, only two meetings are left, so we prefer to do this discussion properly in Rel-20.

- Proposal 3: RAN4 to consider inter-satellite measurement related discussion for Ku-band NTN in a future release (e.g., Rel-20).

Timing accuracy requirements for case 4

Agreement: - The same Te_NTN requirement applies to case 4. On top of the side conditions agreed for Rel-18 Ka band, RAN4 to add the following side condition to Te_NTN requirements for Case 4 - Option 1: Capture it as a NOTE on the test case in the core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec in the core corresponding test case. - Option 2: Capture it as a NOTE in core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec.

In Ka-band, the transmit timing error requirement 7.5Ts for mobile VSAT UE served by GSO has been defined with some side conditions to reduce UE posisioning error in core requirement. In Ku-band, since mobile VSAT UE served by NGSO is introduced, the positioning error for NGSO would increase compared to GSO due to the fast moving of NGSO. Therefore, to reduce the positioning error for NGSO, ephemeris information of the satellite should be provided with short interval. In the last RAN4 meeting, two options were discussed below, and the interval between adjacent epoch times should not exceed 2.56sec and it should be defined in core requirement same as defined by Ka band side conditions.

- Proposal 4: RAN4 to capture side condition as a NOTE in core requirement: The interval between adjacent epoch times should not exceed 2.56 sec.

Conclusion

In this contribution, we provide our views on RRM requirements for Ku-band NTN, and we propose

- Proposal 1: The same assumptions considered for Rel-18 Ka-band RRM requirements should apply to Ku-band RRM requirements for both FR1-NTN and FR2-NTN numerologies;

- Single SAN Tx beam per radio cell in DL

- For inter satellite HO, only blind HO and no neighbor cell measurement

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping)

- Proposal 2: FR2 (Ka-band) NTN RRM requirements could be reused for Ku-band NTN RRM requirements including both FR1-NTN and FR2-NTN numerologies (not to introduce separate RRM requirements for Ku-band NTN), except for UL transmit timing requirements.

- Proposal 3: RAN4 to consider inter-satellite measurement related discussion for Ku-band NTN in a future release (e.g., Rel-20).

- Proposal 4: RAN4 to capture side condition as a NOTE in core requirement: The interval between adjacent epoch times should not exceed 2.56 sec.

Reference

[1] R4-2504897, “WF on RRM requirements for NR_NTN_Ku_bands,” Apple