3GPP TSG-RAN WG4 Meeting #115	R4- 2506304

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Agenda item:	7.28.5.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on some issues as below of RRM core requirements for NR LP-WUS/WUR based on open issues in last meeting’s moderator summary [2] and WF [1].

- Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

- Issue 1-1-2: Threshold for switch between different cases

- Issue 1-1-8: LP-WUR operation carrier frequency

- Issue 1-2-6: On MR wake up delay related requirements for LP-WUR

Discussion

- Topic #1: RRM core requirements for LP-WUS/WUR

General aspects

### Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

RAN4 has been agreed to support case#1 and #3. But regarding case#2 scenario, RAN4 doesn’t make agreement and it can follow the RAN2 decision. For now RAN2 has discussed entry/exit condition separately for serving cell RRM relaxation and neighbor cell RRM relaxation as below:

RAN2#128 (Nov.2024) meeting - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation. RAN2#129 (Feb.2025) meeting - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

From the RAN2’s discussion, if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation, then it means not to support case#2. Else, it means to support case#2.

Our preference is following the RAN2’s conclusion. But we are also fine to not considering the case#2.

Proposal 1: Our preference is following the RAN2’s conclusion. But we are also fine to not considering the case#2.

Issue 1-1-2: Threshold for switch between different cases

- Proposals - P1: Up to RAN2 decision (CMCC LG CT Ericsson, Nokia) - P2: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (vivo Apple ZTE) - P2-1: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1). LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). If RAN4 cannot make conclusion on issue 1-1-2, RAN4 sending LS to RAN2 for guidance (Apple) - P2-2: Details on threshold are up to RAN2 (vivo) - P2-3: Add an offset for existing threshold which is compared to MR based measurement for LP-RSRP and LP-RSRQ. (ZTE) - P3: To avoid the ping-pong, the LR and MR measurement result should be comparable or equivalent with certain offset/margin and additionally UEs are allowed to adjust the configured LR threshold within specific range; NW should configure independently the LR threshold according to LR types (OOK based LR and OFDM based LR) (LG) - P4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1. (MTK)

Regarding threshold for switch between different cases, it is up to RAN2 decision. But RAN4 also need to consider how to avoid the ping-pong problem. To avoid the ping-pong problem, at least the LR and MR measurement result should be comparable or equivalent with certain offset/margin. But we think it’s not enough. Because if we consider only the configured LR and MR threshold for avoiding the ping-pong problem, the threshold shoud be conservative because each UE might have different offset/margin between MR and LR. And it can make the service area for case#1 extremely small. So, to increase the service area for case#1, the UE should be allowed to adjust the configured LR threshold.

Below figures are example of MR and LR coverage by network configured MR and LR threshold. Let’s suppose that the MR case1 coverage are same based on the same MR threshold. Each UE have different offset between MR and LR.

Figure 1. Scenario 1

UE1: Worse performance of LR compared to MR.

UE2: Normal performance of LR compared to MR.

UE3: Better performance of LR compared to MR.

Considering this situation the UE1 can struggle by ping-pong problem at blue area as below.

So, to avoid the ping-pong problem the network should configure the MR threshold to higher value, so the MR coverage could be reduced like below.

Figure 2. Scenario 2

This means that the MR threshold should consider worst case UE, and it can reduce the case1 area. But if the UEs are allowed to adjust the configured LR threshold within specific range, then the service area for case1 can be extended without ping-pong problem.

From the figure1, if the UE1 allowed to reduce the LR threshold by x dB, then the LR coverage can extended as below. And the ping-pong problem can be addressed.

Proposal 2: The threshold for switching between cases is up to RAN2 decision.

Proposal 3: RAN4 to consider how to avoid the ping-pong problem as below

- the LR and MR measurement result should be comparable or equivalent with certain offset/margin and additionally UEs are allowed to adjust the configured LR threshold within specific range.

  - i.e. When the LR threshold is configured by network, the UE is allowed to adjust the configured LR threshold within [6] dB.

Meanwhile, when the NW configure the LR threshold it should consider the UE types. Becase, according to the RF agreements the OOK based LR and OFDM based LR can have different noise figure. So, the NW should configure independently the LR threshold according to LR types.

Proposal 4: NW should configure independently the LR threshold according to LR types (OOK based LR and OFDM based LR).

Issue 1-1-8: LP-WUR operating carrier frequency

From the RAN2 reply LS R4-2503116, RAN2 shared that the UE can report the supporting LR band to NW and the overload issues also could be addressed by current mechanism in spec as below.

- RAN2 understands that UE can report which band(s) is supported by LR to NW. - RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

According to the RAN2’s conclusion as above, there is no impact on RAN4 RRM requirements.

Proposal 5: There is no impact on RAN4 RRM requirements.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

### Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624

### Issue 1-2-6-1: On the number of SSBs

- Ad-hoc minutes discussion For 10ms, 400ms 800ms ramping up time, the number of SSBs: Option 1: [ 3 5 5] (Apple vivo, LG, HW, MTK, oppo, CMCC) Side condition [>= -3 dB] Option 2: [3 3 3] (ZTE) (Side condition -3 dB) Option 3: [5 5 5] (xiaomi) (-6 dB) Option 4: [2 2 2] (Nokia) (-2 dB) Regarding SINR Option 1: -3 dB (Apple vivo LG CMCC Xiaomi oppo HW Ericsson QC) Option 2: -2 dB (Nokia QC) Option 3: -6 dB (MTK) AGC: 1-2 samples for side conditions (-3 dB)

During last meeting, RAN4 discussed about the number of SSBs for wake up time but could not make conclusion. We think the 3, 5, 5 SSBs are reasonable for 10ms, 400ms, 800ms ramping up time respectively, at -3dB SINR side condition.

Proposal 6: Support 3, 5, 5 SSBs for 10ms, 400ms, 800ms ramping up time respectively at -3dB SINR side condition.

Conclusion

In this contribution, we provide our views on the remain issues listed in previous meeting’s moderator summary[2], and we propose

Proposal 1: Our preference is following the RAN2’s conclusion. But we are also fine to not considering the case#2.

Proposal 2: The threshold for switching between cases is up to RAN2 decision.

Proposal 3: RAN4 to consider how to avoid the ping-pong problem as below

- the LR and MR measurement result should be comparable or equivalent with certain offset/margin and additionally UEs are allowed to adjust the configured LR threshold within specific range.

  - i.e. When the LR threshold is configured by network, the UE is allowed to adjust the configured LR threshold within [6] dB.

Proposal 4: NW should configure independently the LR threshold according to LR types (OOK based LR and OFDM based LR).

Proposal 5: There is no impact on RAN4 RRM requirements.

Proposal 6: Support 3, 5, 5 SSBs for 10ms, 400ms, 800ms ramping up time respectively at -3dB SINR side condition.

Reference

[1] R4-2504908, “WF on RRM requirements for NR_LPWUS,” Moderator (vivo)

[2] R4-2503627, “Topic summary for [114bis][217] NR_LPWUS,” Moderator (vivo)