# 3GPP TSG-RAN WG4 Meeting #115                               R4-2506313

# St Julian’s, Malta, 19th-23th May, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on BS RF requirements for LP-WUS/WUR

# Agenda Item:   7.28.4

# Document for:  Approval

- Introduction

In previous meetings, RAN4 agreed not to define LP-WUS power boosting as a core requirement but the minimum power boosting level should be included in the BS manufacture declaration table. How to specify the declaration is still under discussion. Besides, whether to define EVM requirements for LP-WUS still needs to be further checked.

- Discussion

## 2.1 BS manufacturer declaration

As discussed in last meeting, the details of BS manufacturer declaration needs to be specified[1].

Issue 1-1: How to specify the BS manufacturer declaration for LP-WUS - WF - Manufacturer shall declare the support of LP-WUS for each supported band, [frequency range] and channel bandwidth. - If the BS supports LP-WUS power boosting, manufacturer shall declare the LP-WUS power boosting level it could support with a minimum of +[X]dB. - X can be different for different channel bandwidth. - FFS on the value of X and use 3 as starting point.

We think different channel bandwidth can support different levels of LP-WUS power boosting because of the power share between NR signal and LP-WUS. The simulation results in our previous contribution [2] verifies this view. Therefore, we prefer LP-WUS power boosting level can be different for different BS channel bandwidth. As for whether manufacturers would like to declare same boosting level or different boosting levels for different channel bandwidth, this depends on manufacturers themselves. Namely, X can be different for different channel bandwidth. Besides, minimum power boosting level as 3dB is challenging for small channel bandwidth, and small power boosting is still helpful. Thus, we propose to adopt the following manufacture declaration in Table 1.

Table 1  Manufacturer declarations for LP-WUS operation and LP-WUS dynamic range

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.x | LP-WUS operation | Manufacturer shall declare the support of LP-WUS operation in NR in-band for each supported band, frequency range and channel bandwidth. | x |  |
| D.y | LP-WUS power dynamic range | If the BS supports LP-WUS operation in NR in-band, manufacturer may declare the minimum power dynamic range for each supported channel bandwidth(Note Z). | x |  |
| Note Z: The default LP-WUS dynamic range is 0 dB. |  |  |  |  |

Proposal 1: It is proposed to adopt the manufacture declarations for LP-WUS operation and LP-WUS power dynamic range in Table 1.

## 2.2 Transmit Signal Quality

Whether transmit signal quality should be defined and whether equivalent EVM requirements should be defined are still under discussion [1]. Our simulation results are based on the assumptions shown in Table 2.

Table 2	Link level simulation assumptions for LP-WUS BS transmission

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz |
| Channel structure | 5 information bits with 32bit RM coding No CRC is configured |
| Chip rate | M=1/2/4 |
| Coding | 1/2 rate Manchester coding |
| Repetition | None |
| Time error | 0 |
| Residual Frequency error | 0/10/20 ppm |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB) |
| WUS RB | - Fixed 11RB |
| Position within channel | - Center |
| Guard RB | - 0RB between LP-WUS and NR signal |
| Non-linearity | Not modeled |

Traditional Tx EVM is tested based on frequency-domain constellations. For LP-WUS, the symbols before RB mapping can be regarded as “constellations”. We added EVM on the LP-WUS frequency-domain symbols before RE mapping, as shown in Fig. 1, and then check the impact of EVM to Rx MDR. The simulation result is shown in Fig. 2. It can be found that frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR.

Fig. 1  Add EVM to LP-WUS frequency-domain symbols

Fig. 2  Impact of EVM on OOK-4 M=4 receiving MDR for OOK-based receiver and OFDM-based receiver

Observation 1: Frequency-domain EVM on LP-WUS has minor impact on the receiving MDR.

Therefore, we prefer that there is no need to define EVM requirement for LP-WUS.

Proposal 2: It is proposed not to define EVM requirements for LP-WUS.

On the other hand, RAN4 cares about the LP-WUS impact on NR signals within the same carrier. For the mixed signal, PAPR is the most important metric to judge the impact from LP-WUS. Therefore, we compared the 99%PAPR of pure NR CP-OFDM signal and mixed NR CP-OFDM and LP-WUS signal under NFFT=2048 as shown in Table 3 and Fig 3.

Table 3  99% PAPR (dB) comparison between pure NR CP-OFDM signal and NR+LP-WUS

| NR Modulation | Signal component | Channel Bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10MHz | 20MHz | 50MHz |
| QPSK | Pure NR | 11.53 | 11.70 | 11.75 |
|  | NR + LP-WUS | 10.96 | 11.62 | 11.76 |
| 16QAM | Pure NR | 11.55 | 11.70 | 11.76 |
|  | NR + LP-WUS | 10.98 | 11.62 | 11.75 |
| 64QAM | Pure NR | 11.52 | 11.69 | 11.77 |
|  | NR + LP-WUS | 10.98 | 11.61 | 11.77 |
| 256QAM | Pure NR | 11.55 | 11.70 | 11.77 |
|  | NR + LP-WUS | 11.00 | 11.61 | 11.76 |
| Note: The 99% PAPR of pure LP-WUS is 6.12dB. |  |  |  |  |

Fig. 3  CDF curve of PAPR under condition of 20MHz CBW and 16QAM NR Modulation

It can be found that LP-WUS in NR in-band has minor impact on PAPR. Therefore, we don’t think LP-WUS have much impact on the signal quality of NR signal.

Observation 2: LP-WUS operation in NR in-band has minor impact on PAPR.

Proposal 3: It is proposed that no EVM verification is needed.

- Conclusion

Proposal 1: It is proposed to adopt the manufacture declarations for LP-WUS operation and LP-WUS power dynamic range in Table 1.

Table 1  Manufacturer declarations for LP-WUS operation and LP-WUS dynamic range

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.x | LP-WUS operation | Manufacturer shall declare the support of LP-WUS operation in NR in-band for each supported band, frequency range and channel bandwidth. | x |  |
| D.y | LP-WUS power dynamic range | If the BS supports LP-WUS operation in NR in-band, manufacturer may declare the minimum power dynamic range for each supported channel bandwidth(Note Z). | x |  |
| Note Z: The default LP-WUS dynamic range is 0 dB. |  |  |  |  |

Observation 1: Frequency-domain EVM on LP-WUS has minor impact on the receiving MDR.

Proposal 2: It is proposed not to define EVM requirements for LP-WUS.

Observation 2: LP-WUS operation in NR in-band has minor impact on PAPR.

Proposal 3: It is proposed that no EVM verification is needed.

- Reference

- R4-2504744 WF on NR LPWUS BS RF, Huawei, RAN4#114bis, Wuhan, China, Apr. 2025.

- R4-2418827 Discussion on BS RF requirements for LP-WUS/WUR, ZTE, RAN4#113, Orlando, USA, Nov. 2024.