# 3GPP TSG-RAN WG4 Meeting #115	R4-2506331

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.18.3.1

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor issues captured in the WF [1] and summary [2].

- Discussion

Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Status in the WF [1] and summary [2]:

### Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Further discuss this issue. - Proposals for scenario 1/2/3/4 - Option 1: UE autonomous quitting. - Option 1a (vivo, LGE, HW, Intel, Samsung, Ericsson): - UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results. - (vivo): If there is no consensus, it is preferred to only consider threshold conditions as the exit conditions. - Option 1b (LGE): RAN4 to leave it to UE implementation whether to keep or re-active FR2 FBS when serving cell’s signal quality is lower than the threshold configured by network and after quit conditions being discussed have been met. - Option 1c (HW): - UE is allowed to fall back to normal measurement when UE has performed fast measurement for at least T1. - After quitting fast measurement, UE shall re-activate fast measurement no longer than T2 from the quitting time. - Option 1d (Intel): - The reduced beam sweeping factor state continues to apply until the UE reports the measurement results to the network. And the UE makes another exam on the criteria that are configured for the serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the quality criteria are no longer met. - Option 1e (Samsung): - Define the maximum period of time of UE to continue BSF measuring after it has reported the qualified measurement result as a timer. - Option 1f (Ericsson): - UE may quit FBS if the UE has completed at least one measurement in the FBS way and the UE identifies that there isn’t any urgent task to continue FBS, e.g., the UE remains a stationary state or no qualified neighbour cells are available for switching. - Option 2: UE indication to the NW is used to deactivate FBS based L3 measurements. - Option 2a (OPPO): The detailed signaling can leave to RAN2 decision. - Option 2b (LGE): Reuse Rel-18 UAI ‘multiRx-PreferenceFR2’. - Option 2c (CMCC): Introduce UAI similar as “multiRx-PreferenceFR2” to inform network there is power consumption issue. - Option 2d (CT, Samsung, MTK): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction. - Option 2e (Apple): To consider UE power consumption or overheating, UAI mechanism should be used. Either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed. - To address the power consumption issues in the two cases (Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving; Case 2: UE is still in the cell centre and the triggering conditions are met due to block.), it is proposed to consider the following two options: - Option 1: No specific action is taken, assuming UE can use UAI to inform the network that it needs to fall back to Non-FBS mode (i.e., deactivate FBS operation) if power consumption/overheating becomes an issue. - Option 2: After the UE sends a qualified measurement report, the UE can start a timer, and when the timer expires, the UE exits from FBS operation. - Option 3: Timer-based solution to deactivate Rx BSF reduction. - Option 3a (CATT) - UE deactivates Rx BSF reduction feature based on timer-solution, i.e., when either of the conditions are met: - Serving cell quality threshold condition is still met, but timer expires. - While timer is still running, but serving cell quality threshold condition is no long met. - Introduce another timer, and UE re-evaluates the serving cell quality threshold condition after it expires. This timer starts to run when UE deactivates Rx BSF reduction feature. - Option 3b: (Ericsson) - Instead of TTT, RAN4 to introduce ‘time to deactivate’ parameter, which limits the minimal FBS measurement time after activated/enabled. - UE may quit FBS in certain case with or without other conditions, but at least after the minimal time period ‘time to deactivate’. - Proposals for scenario 5/7 - Option 4 (OPPO): - For scenario 5/7, UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection. - Recommended WF - Discuss the option(s) to finalize quitting conditions and re-evaluation mechanism. - Note: Option 1c/1d/3a/3b includes re-evaluation mechanism, which may be coupled with the quit conditions or not, they are collected as follow: - Option 1c: After quitting fast measurement, UE shall re-activate fast measurement no longer than T2 from the quitting time. - Option 1d: (after quitting), the UE makes another exam on the criteria that are configured for the serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the quality criteria are no longer met. - Option 3a: Introduce another timer, and UE re-evaluates the serving cell quality threshold condition after it expires. This timer starts to run when UE deactivates Rx BSF reduction feature. - Option 3b: After deactivating FBS, the UE may re-evaluate the condition of enabling FBS, at least after a minimal time interval ‘time to active’ after deactivated or disabled.

For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”, we understand the intention is considering UE power consumption, which is similar as Rel-18 Multi-Rx UE power consumption issues, the UAI ‘multiRx-PreferenceFR2’ is introduced to solve the issue of UE power consumption. In our views, when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction, UE autonomous quitting should be avoided since it may cause inconsistency between NW and UE, and it’s preferred to introduce UE indication to the NW to assist to NW’s better decision and scheduling, therefore, UAI mechanism is proposed to indicate the UE power consumption issue and to quit BSF mode, besides, a new UAI is preferred to decouple with features of Rel-18 Multi-Rx WI.

Proposal 1: UAI mechanism is proposed to indicate the UE power consumption issue and to quit BSF mode, and a new UAI is preferred.

Requirement of L3 measurement delay reduction by optimizing Rx BSF

Status in the WF [1] and summary [2]:

### Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement in the ad-hoc: - Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4. ### Issue 1-2-4: Other aspects of UE capability design Further discuss this issue. - Proposals - Option 1: (QC) - RAN4 introduces per-band UE capability for FR2 RX beam sweeping factor optimization and L3 measurement delay reduction. - Option 2: (HW) - Define separate UE capability for intra-frequency and inter-frequency. - Option 3: (Ericsson) - We recommend initially limiting the consideration of FBS to a single FR2 serving cell and excluding the scenario of multiple FR2 serving cells from the BSF capability assessment at the first step. - Recommended WF - Discuss the option(s).

For the granularity of the BSF UE capability, per-band is preferred since different FR2-1 bands may apply different beam sweeping factors, it’s reasonable to consider per-band granularity.

Proposal 2: Per-band granularity is considered for BSF UE capability.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, with the following proposals:

Proposal 1: UAI mechanism is proposed to indicate the UE power consumption issue and to quit BSF mode, and a new UAI is preferred.

Proposal 2: Per-band granularity is considered for BSF UE capability.

- References

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.

- R4-2503619, Topic summary for [114bis][209] NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.