3GPP TSG-RAN4 Meeting # 115	R4-2506377

Malta, May.19th – 23th, 2025

# Title: 	View on the methods for reducing the number of UE Rx chain

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.3

# Document for:	Approval

# 1 Introduction

In the agreed WF[1], there were some agreements achieved, but there still some open issues which was FFS.

In this contribution, we give some views on some left open issues on the methods for reducing the number of Rx chain for fragment carriers.

# 2	Discussion

- 2.1 Triggering condition

Regarding the triggering condition to enable “one RF Rx chain” or “shared RF Rx chain”, the agreements were:

- Issue 2-3: Triggering condition to enable ”One Rx RF chain” RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode: - Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed - Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified - Note: detailed conditions to enable/disable can be further discussed and decided separately

In our understanding, the triggering conditions are needed to enable UE ‘one RF Rx chain’, and such conditions shall be under the control of network. According to the initial evaluations from companies in the past meeting, the DL PCC or SCC performance would be degraded based on the existing test point for a UE enables ‘one RF Rx chain’, then if there are no other methods such as more MIMO layer which can compensate the performance, then NW can make decision to enable or disable UE ‘one RF Rx chain’ capability with the consideration of the whole system performance. For example, when there are some conditions such as the power measured in the gap is larger which makes the DL CC performance poorer comparing with traditional way or single carrier due to poor filter attenuation in the gap, the benefit of shared Rx RF chain would loss. In this case, NW should disable the shared RF Rx chain feature to maintain the DL performance.

The purpose for the condition is to make NW configure/schedule UE band configuration correctly, which is when UE reports the capability of shared Rx RF chain to the NW after initial access, NW would configure/schedule the band configuration for the UE who reports the capability based on the trigger conditions are met or not. We think NW can trigger UE to do power measurement in the gap via the existing RSSI measurement method.

Proposal 1: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified.

For the triggering condition, it is straightforward to use the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC as the trigger condition. In addition to that, we think the frequency span should be also considered as one of the condition. In this SID, only frequency span≤100MHz between two DL CC is considered. However, up to 200MHz for one single Rx RF chain can be supported in current design. If we only focus on frequency span ≤100MHz, then the ≤100MHz frequency span can be seen as the precondition for the shared RF Rx chain. For those UE who can support >100MHz frequency range but report the capability of shared Rx RF chain, NW should disable this capability.

Proposal 2. Both frequency span and the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled.

- 2.2 UE capability for supporting “one Rx RF chain”

In addition to the above condition, how UE reports the shared Rx RF chain to inform the network of appropriate CA configuration it can support is also discussed in last meeting, but no consensus were achieved. The WF is:

- How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers Option 1: Legacy band combination reporting scheme + Rx sharing capability indication - FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”. Other options are not precluded - Discussion on semi-statically switch hardware resources procedure and fallback considerations - Consider the following alternatives - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication - Alternative 1 has no modification of CA configurations in RAN4 - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers. - Alternative 1a has no modification of CA configurations in RAN4 - Alternative 2: use the new notation for CA configurations with single Rx RF Chain - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

As we discuss in [2][3], the inter-band DL NC CA band configurations supported by a UE capable of shared Rx RF chain are essentially traditional CA band configuration, where the existing Tx requirement shall be applied, and also the intra-band UL NC CA configurations like BCS associated with supported channel bandwidth should be the same. In other words, there are no impacts on Tx requirements and the only impacts are the Rx requirements, and also the configuration supported inter-band DL NC CA band configurations supported by a UE should comes from the existing configurations defined in the specification.

if new bandwidth class ‘F’ is introduced or new notation is introduced for a configuration supporting single Rx RF chain, it would be seen as a brand new CA configuration, which needs more information like supported channel bandwidths and BCS, and also it should describe that the relationship between the traditional CA band configuration and new CA band configuration, even it needs basket WID procedure. Moreover, a new table may be needed to include the configurations with single Rx chain, it would make things a bit more complicated.

Therefore, we think legacy band combination reporting scheme + Rx sharing capability indication is straightforward and simple, and also the impacts on RAN2 is minimum since it is normal procedure similar as other new feature introduce in later releases.

In addition to that, as we discussed as above, frequency span/separation between two carriers can be used as one of the trigger conditions, so Rx sharing capability and other related information including the frequency separation between two carriers would be fine to us.

For the capability granularity, since there are only two DL carriers considered in fragment carrier SID, so ‘per-band per-BC’ capability’ is enough. However, if considering future proof which is more than two DL carriers are considered but there are some of DL carrier can be implemented in fragment carrier mode, in this case ‘per-band per-BC’ capability’ is not enough, instead ‘per CC per band per BC’ would be needed. Therefore, considering it is SID, and future proofing should be taken into accounted, ‘per CC per band per BC’ would be fine.

Proposal 3. To adopt legacy band combination reporting scheme + Rx sharing capability indication and other related information including the frequency separation between two carriers.

- ‘per CC per-band per-BC’ capability for future proofing

2.3 Fallback consideration

Another thing is fallback solution, which is when a UE report the shared Rx RF chain related capability

- Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations - Further discussions on - CA fallback with/without UE architecture switching - Consider the fall back - from high order CA to lower order CA - from CA to single carrier mode - Decided by network - UE architecture switching: - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA - FFS on whether it is decided by UE itself or decided by network - FFS on whether the additional indication from UE side is needed

Regarding the CA band combination fallback for fragment mode, in our understanding the CA band combination fallback principle shall abide by the fallback band combination definition as in TS38.306. If we only focus on the SI scope where only two carriers scenario is considered, i.e. only CA_nX(2A) is considered, then we think when we talk about the CA band combination fallback, i.e. from CA_nX(2A) to nXA, then it shall be from CA to single carrier mode. If the high order CA is considered, for example CA_nX(2A)-nYA, where CA_nX(2A) is in fragment mode. In this case, both CA_nXA-nYA and CA_nX(2A) should be the fallback configuration (i.e. from high order CA to lower order CA) from the aspect of the fallback, and CA_nXA-nYA is a configuration without ‘one Rx RF chain’, and CA_nX(2A) is a fragment mode configuration with ‘one Rx RF chain’. Which fallback configurations can be supported are decided by NW, which is the same approach with normal configuration.

- Observation 1. CA band combination fallback principle as in TS38.306 is applied for fragment carrier CA fallback.

Regarding the UE architecture switching, it seems it would cause UE’s complexity since legacy Rx RF architecture (i.e. partial shared) and one Rx RF architecture should be supported for UE, and also switching between these two architectures are needed to be considered and LO retuning may be needed which would cause like-Rx switching related discussion. Meanwhile, the condition for the switching between the architectures may need to be discussed. But if UE architecture switching is really needed to be considered, we think it should be decided by UE itself since the UE implementation should be transparent to NW, NW may only care whether the switching period is needed.

Observation 2. UE architecture switching means LO retuning as well as the switching period may be needed.

Proposal 4: If UE architecture switching is really needed to be considered, it should be decided by UE itself.

# 3 Conclusion

In this contribution, we give some discussions on some left open issue on the methods for reducing the number of Rx chain for fragment carriers:

Proposal 1: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified.

Proposal 2. Both frequency span and the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled.

Proposal 3. To adopt legacy band combination reporting scheme + Rx sharing capability indication and other related information including the frequency separation between two carriers.

- ‘per CC per-band per-BC’ capability for future proofing

- Observation 1. CA band combination fallback principle as in TS38.306 is applied for fragment carrier CA fallback.

Observation 2. UE architecture switching means LO retuning as well as the switching period may be needed.

Proposal 4: If UE architecture switching is really needed to be considered, it should be decided by UE itself.

# 4	Reference

[1] R4-2505109, WF on fragmented DL, Mediatek

[2] R4-2503963, Discussion the methods for reducing the number of UE Rx chain, ZTE Corporation, Sanechips

[3] R4-2500941, Discussion the methods for reducing the number of UE Rx chain, ZTE Corporation, Sanechips