3GPP TSG-RAN4 Meeting # 115	R4-2506382

Malta, May.19th – 23th, 2025

# Title: 	View on low-low band CA via switching

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.7.3

# Document for:	Approval

# 1 Introduction

In the approved WF [2], some agreements were included:

Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination. Issue 2: Switching period location - Await outcome of RAN1 design Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded Issue 5: Test methodology -Companies are encouraged to contribute proposals related to the testability of the requirement on the time mask for switching

In this contribution, we gives some further discussions on low-low band CA via switching (i.e. downlink carrier switching) .

# 2	Discussion

In our understanding, the purpose of this WID is to generate the carrier switching method in the case of the shared antenna for the switched-from and switched-to carriers. In other words, there is no need to consider the downlink carrier switching for separated antennas implementations since the Rx chain/antennas are totally separated for the two bands low-low CA, or the carrier switching period for separated antennas implementations is nearly 0us.

Observation 1.There is no need to consider the downlink carrier switching for separated antennas implementations for low-low CA band combination.

As we discussed in [2], for downlink carrier switching, similarly we cannot restrict the implementation for UE supporting low-low CA band combination, but UE needs to report the downlink carrier switching capability to NW. This is also mentioned in the WF.

As there is no simultaneous Tx/Rx between the PCell and the SCell. Rx is only happened in either switch-from carrier or switch-to carrier. In other words, only switchedDL option (similar to switchedUL for Tx switching) can be considered for low-low CA downlink carrier switching in this WID. For the ‘dualDL’ option, it could be considered in later release associated with the new band combination such as FDD-FDD low-low band CA via switching.

Proposal 1. Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID.

In last meeting, it was agreed to define Tx on-off time mask requirement for Rx switching , and how to define it is FFS.

Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded

We think the Tx on-off time mask for Rx switching depends on the switching period location which is related to the switching pattern in RAN1’s discussion. The latest agreements in RAN1 are shown below:

1. For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier 2. For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

It can be seen that for switching from SDL carrier to FDD carrier, RAN1 has already reached a consensus that the switching gap (noted that the switching gap includes switching period as well as UL TA(if needed)) is always assumed at the “switch from” carrier, i.e., SDL carrier. When considering the Tx on-off time mask associated with the switching period location, off->on time mask for switching should be specified from SDL to FDD carrier.

For switching from FDD carrier to SDL carrier, the PA for FDD carrier should be from ON->OFF when considering the Tx on-off time mask. However, the switching period/gap location for switching from FDD carrier to SDL carrier is still under discussion in RAN1, and there are three alternatives in RAN1’s agreements. In terms of these three alternatives, it seems the switching gap can be located at only FDD carrier, or only SDL carrier, or either FDD carrier or SDL carrier.

Proposal 2. Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier

The time mask for switching from SDL carrier to FDD carrier could be found in the Fig 1.

Switching from SDL carrier ->FDD carrier:

Fig 1. Time mask for switching from SDL carrier 2 to FDD DL carrier 1 where the switching period is located in SDL carrier 2

For the time mask for switching from FDD carrier to SDL carrier, there are two options which could be found in the Fig 2.

(a)

(b)

We think it is premature to define which options should be adopted in RAN4 since it relies on the RAN1’s consensus on the switching period/gap location for switching from FDD carrier to SDL carrier.

Proposal 3. The time mask for switching from FDD carrier to SDL carrier should wait for RAN1 consensus of switching pattern design.

In the WID, there is only one example band combination (i.e. CA_n5A-n29A), the other band combination to be handled via the basket work item approach. Although it is not clear how to pursue it or what to be discussed in basket WID approach at this stage, it seems RAN4 will introduce tables to specify which band combination can use downlink carrier switching method via basket WID. If so, it will cause specification cumbersome if the downlink carrier switching method is defined as band combination manner.

Actually whether to use basket WID approach to introduce additional band combination was also discussed in Rel-18 3/4 bands Tx switching discussion. But in the end, no basket WID approach is introduced for 3/4 bands Tx switching since it will be subjected to basket WID procedure/guidance, which needs at least 3 months delay when operator request it, and also it will increase RAN4 workload. Instead it depends on whether the switching capability is present for a certain band configuration.

For downlink carrier switching, we think the same approach should be adopted, which is the band configuration which can be enabled low NR band carrier aggregation via switching capability is the subset of the existing band configurations which are introduced by normal PC3/HPUE/SUL band basket WIDs. Otherwise, it seems the work for basket WID approach for low NR band carrier aggregation via switching would overlap/conflict with the normal PC3/HPUE basket WID since the MSD work may be needed if a brand new band configuration is introduced (means this configuration is not introduced via normal basket WID) via carrier switching basket WID. In general, such MSD work is discussed in the normal PC3/HPUE basket WID.

Moreover, when downlink carrier switching happens, whether the UL interruption is allowed should be defined per band combination.

Therefore, we think RAN4 should try to define generic low NR band carrier aggregation via switching requirements for all the band combinations, rather than defining band combination specified requirements.

Observation 2. No band combination tables for Tx switching are included in the spec, instead it depends on whether the switching capability is present for a certain band combination.

Proposal 4: RAN4 should try to define generic low NR band carrier aggregation via switching requirements, rather than band combination specified requirements.

# 3 Conclusion

In this contribution, we give some further discussion on the downlink carrier switching to enable low-low band CA band combination.

The observations, proposals and conclusions are:

Observation 1.There is no need to consider the downlink carrier switching for separated antennas implementations for low-low CA band combination.

Observation 2. No band combination tables for Tx switching are included in the spec, instead it depends on whether the switching capability is present for a certain band combination.

Proposal 1. Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID.

Proposal 2. Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier

Proposal 3. The time mask for switching from FDD carrier to SDL carrier should wait for RAN1 consensus of switching pattern design.

Proposal 4: RAN4 should try to define generic low NR band carrier aggregation via switching requirements, rather than band combination specified requirements.

# 4	Reference

[1] R4-2505223, WF on UE RF requirements for LB CA via switching

[2] R4-2503962, View on low-low band CA via switching, ZTE Corporation, Sanechips,