3GPP TSG-RAN WG4 Meeting #115    	    	 R4-2506417

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.17.3

Source: 	CMCC

Title: 			 Discussion on interference suppressing performance for 8Rx UE

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed test parameters about interference suppressing performance for 8Rx UE, the WF has been approved in [1].

In this contribution, we further discuss the remaining open issues about test applicability and release independent.

- Discussion

Test applicability to skip 2/4Rx requirements

Proposals

- Option 1:
For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not.

- Option 2:
To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

- Option 3:
For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

- Option 4: 
Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test:
- UE support both 8Rx and 2/4Rx
- The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement.

- Option 5:
RAN4 shall have the following applicability rules:
- UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx.
- UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx.

For demodulation test of inter-cell interference scenario, the test configuration difference between 8Rx and 2/4Rx tests are as follows:

|  |  | 2Rx | 4Rx | 8Rx |
| --- | --- | --- | --- | --- |
| Rank | HomNet | 1 | 1 | 2 |
|  | HetNet | 1 | 1 | 4 |
| MCS | HomNet | MCS 13 | MCS 13 | MCS 17 |
|  | HetNet | MCS 13 | MCS 13 | MCS 13 |
| Propagation condition | HomNet | TDLC300-100 | TDLC300-100 | TDLA30-10 |
|  | HetNet | TDLA30-10 | TDLA30-10 | TDLA30-10 |

For demodulation test of intra-cell inter-user interference scenario, the test configuration difference between 8Rx and 2/4Rx tests are mainly as follows:

|  | 2Rx | 4Rx | 8Rx |
| --- | --- | --- | --- |
| Rank | 1+1 | 1+1 2+2 | 2+2 |
| MCS | MCS 13 | MCS 13 | MCS 17 |
| Propagation condition | TDLC300-100 | TDLC300-100 TDLA30-10 | TDLA30-10 |

From the above tables, we observed that except for propagation condition, other test configurations for 8Rx give a higher test requirement compared with 2Rx and 4Rx. With this observation, for demodulation test, we are fine to introduce a test applicability rule that if 8Rx tests are passed, then the test of 2/4Rx requirements can be skipped.

- Proposal 1: Introduce a test applicability rule that if 8Rx tests are passed, then the test of 2/4Rx requirements can be skipped.

Release independency

Agreement:

- The new Rel-19 requirements shall at least be release independent from Rel-18.

Remaining proposals:

- Option 1:
The new Rel-19 requirements shall also be release independent from Rel-17.

- Option 2:
The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

As specified in TS 38.307, the Demodulation requirements for 8Rx UE is mandatory release independent from Rel-17.

Besides, the 2/4Rx UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference are mandatory release independent from Rel-17.

Since no new signaling, UE capability, UE receiver implementation will be introduced, for the new demodulation requirements introduced from R19, it shall also be release independent from R17.

- Proposal 2: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

- Conclusion

In this contribution, we provide our opinion about test applicability rule and release independent. The proposals are:

- Proposal 1: Introduce a test applicability rule that if 8Rx tests are passed, then the test of 2/4Rx requirements can be skipped.

- Proposal 2: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

- Reference

[1] R4-2504694, Way forward for [114bis][322] NR_demod_Ph5_Part2_UE, Moderator (Nokia)