3GPP TSG-RAN WG4 Meeting # 115											R4-2506427

# Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.28.5.1

Source: 	CMCC

Title: 			  Simulation assumption and results for LP-WUR

Document for:	Discussion

- Introduction

In RAN4#114 meeting, RAN4 provided the updated RRM simulation assumption. The WF has been approved in [1].

In this contribution, we give our simulation results based on RAN4 simulation assumption.

- Simulation result

Our simulation results are provided below by using the simulation assumption in Annex.1.

Table 1: SINR = -3dB, Interference Source 6 dB lower compared with cell 1

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | channel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X = 1 |  |  |  |  |  |  | X = 2 |  |  |  |  |  |  |  | X = 3 |  |  |  |  |  |  |  | X = 4 |  |  |  |  |  |  |  |  |
|  |  | 5% | 50% |  | 95% |  | ΔRSRP |  | 5% |  | 50% |  | 95% |  | ΔRSRP |  | 5% |  | 50% |  | 95% |  | ΔRSRP |  | 5% |  | 50% |  | 95% |  | ΔRSRP |  |  |
| {1，4} | S: [0101] I: [0110] | -0.12 |  | 0.4 |  | 0.84 |  | 0.84 |  | -0.29 |  | 0.24 |  | 0.68 |  | 0.75 |  | -0.1 |  | 0.28 |  | 0.65 |  | 0.65 |  | 0.05 |  | 0.28 |  | 0.51 |  | 0.51 | TDL-C 300ns |
| {1，6} | S: [101010] I: [010101] | -3.14 |  | -0.93 |  | -0.18 |  | 3.14 |  | -2.39 |  | -0.97 |  | -0.35 |  | 2.39 |  | -2.09 |  | -1 |  | -0.45 |  | 2.09 |  | -1.79 |  | -0.95 |  | -0.49 |  | 1.79 | TDL-C 300ns |
| {1，8} | S:[10100101] I:[10101001] | 0.1 |  | 0.87 |  | 2.12 |  | 2.12 |  | 0.3 |  | 0.7 |  | 1.27 |  | 1.27 |  | 0.23 |  | 0.6 |  | 1.09 |  | 1.09 |  | 0.22 |  | 0.46 |  | 0.8 |  | 0.8 | TDL-C 300ns |
| {2，8} | S:[01011001] I: [01100101] | -0.99 |  | -0.02 |  | 0.78 |  | 0.99 |  | -0.6 |  | -0.09 |  | 0.35 |  | 0.6 |  | -0.29 |  | -0.08 |  | 0.12 |  | 0.29 |  | -0.2 |  | -0.05 |  | 0.08 |  | 0.2 | TDL-C 300ns |
| {2，16} | S:[1001010110011001] I:[1001100101100101] | -0.43 |  | 0.02 |  | 0.44 |  | 0.44 |  | -0.26 |  | -0.12 |  | 0.02 |  | 0.26 |  | -0.05 |  | 0.11 |  | 0.21 |  | 0.21 |  | -0.03 |  | 0.06 |  | 0.15 |  | 0.15 | TDL-C 300ns |
| {4，16} | S:[0110100110101010] I:[0110101010011010] | -0.24 |  | 0.71 |  | 2.09 |  | 2.09 |  | -0.17 |  | 0.36 |  | 1.05 |  | 1.05 |  | -0.04 |  | 0.36 |  | 0.88 |  | 0.88 |  | -0.01 |  | 0.22 |  | 0.52 |  | 0.52 | TDL-C 300ns |
| {4，32} | S:[01011010101010011010011001100101] I:[01100101011001011001101010100101] | -0.98 |  | -0.1 |  | 0.62 |  | 0.98 |  | -0.57 |  | -0.18 |  | 0.16 |  | 0.57 |  | -0.4 |  | -0.13 |  | 0.13 |  | 0.4 |  | -0.26 |  | -0.15 |  | -0.05 |  | 0.26 | TDL-C 300ns |
| {1，4} | S: [0101] I: [0110] | -0.37 |  | 0.05 |  | 0.42 |  | 0.42 |  | -0.23 |  | 0.05 |  | 0.32 |  | 0.32 |  | -0.19 |  | 0.05 |  | 0.27 |  | 0.27 |  | -0.15 |  | 0.05 |  | 0.24 |  | 0.24 | AWGN |
| {1，6} | S: [101010] I: [010101] | -1.81 |  | -1.29 |  | -0.87 |  | 1.81 |  | -1.65 |  | -1.29 |  | -1 |  | 1.65 |  | -1.58 |  | -1.29 |  | -1.04 |  | 1.58 |  | -1.55 |  | -1.29 |  | -1.08 |  | 1.55 | AWGN |
| {1，8} | S: [10100101] I:[10101001] | 0.23 |  | 0.49 |  | 0.72 |  | 0.72 |  | 0.31 |  | 0.49 |  | 0.66 |  | 0.66 |  | 0.35 |  | 0.49 |  | 0.63 |  | 0.63 |  | 0.37 |  | 0.49 |  | 0.61 |  | 0.61 | AWGN |
| {2，8} | S:[01011001] I: [01100101] | -0.51 |  | -0.1 |  | 0.3 |  | 0.51 |  | -0.39 |  | -0.1 |  | 0.19 |  | 0.39 |  | -0.35 |  | -0.1 |  | 0.13 |  | 0.35 |  | -0.31 |  | -0.1 |  | 0.1 |  | 0.31 | AWGN |
| {2，16} | S:[1001010110011001] I:[1001100101100101] | -0.34 |  | -0.05 |  | 0.23 |  | 0.34 |  | -0.25 |  | -0.05 |  | 0.15 |  | 0.25 |  | -0.21 |  | -0.05 |  | 0.11 |  | 0.21 |  | -0.19 |  | -0.05 |  | 0.09 |  | 0.19 | AWGN |
| {4，16} | S:[0110100110101010] I:[0110101010011010] | 0.1 |  | 0.42 |  | 0.77 |  | 0.77 |  | 0.18 |  | 0.42 |  | 0.68 |  | 0.68 |  | 0.22 |  | 0.42 |  | 0.63 |  | 0.63 |  | 0.25 |  | 0.42 |  | 0.6 |  | 0.6 | AWGN |
| {4，32} | S:[01011010101010011010011001100101] I:[01100101011001011001101010100101] | -0.37 |  | -0.08 |  | 0.19 |  | 0.37 |  | -0.28 |  | -0.08 |  | 0.11 |  | 0.28 |  | -0.25 |  | -0.08 |  | 0.08 |  | 0.25 |  | -0.22 |  | -0.08 |  | 0.06 |  | 0.22 | AWGN |

- Discussion

During our simulation, both I and Q branch are used in simulation results. Ideal RSRP is derived from the serving cell signal after the fading channel;

From our simulation results, we observed that the RSRP accuracy are highly depends on the OOK sequence between serving and neighbour. The accuracy performance is best when the sequences are half same and half opposite, the accuracy performance get worse for other cases, the accuracy performance is worst when two sequences is same or opposite.

To fulfill the ± 3.5dB accuracy requirement, we think 2 samples is needed.

Observation 1: Under same measurement sample, the RSRP accuracy performance are highly depends on the OOK sequence between serving and neighbour.

Observation 2: For OOK based LP-WUR, 2 samples can fulfill the accuracy requirement of ±3.5dB for LP-RSRP.

- Conclusion

Based on the discussion above, the following observations are concluded.

Observation 1: Under same measurement sample, the RSRP accuracy performance are highly depends on the OOK sequence between serving and neighbour.

Observation 2: For OOK based LP-WUR, 2 samples can fulfill the accuracy requirement of ±3.5dB for LP-RSRP.

- Reference

- R4-2504908, WF on RRM requirements for NR_LPWUS, vivo

- Annex.A

Simulation assumption

Note: Highlight part is updated base on the WF of last meeting

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30kHz |
| Measurement period (in number of measurement samples) | - [1, 2, 3, 4] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially |
| SSS | 30KHz for SSS |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-4 with M = 1, 2, 4 | OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: T=5us for OOK-4 with M=1 T=2us for OOK-4 with M=2 T=1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -6 dB - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-5.7] | When Ês/Iot = -6 dB - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-11.7] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB ; -6dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [7.68MHz] |