3GPP TSG RAN WG4 Meeting #115	R4-2506460

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:			7.29.2

Source:	vivo

Title:	On UE capability for mobility enhancement phase 4 WI

Document for:	Discussion

# Introduction

In the previous RAN4 meetings, requirements for event triggered reporting for LTM and CSI-RS based LTM were discussed.

In this contribution, we provide our views on UE capabilities for event triggered reporting and CSI-RS based L1-RSRP measurement for LTM.

# Discussion

CSI-RS based L1-RSRP measurement for LTM

In the last RAN1 meeting, FG 63-1 and FG 63-2 for mobility enhancement phase 4 WI was agreed. The details are as in the following table.

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource |
| --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource |

From RAN4 perspective, similar UE capabilities as for SSB-based L1-RSRP measurement for LTM need to be introduced. Since the definition of intra-frequency/inter-frequency measurement is under discussion, it would be difficult to decide the UE capabilities as some of them are relevant to number of frequency layers.

From our point of view, the intra/inter frequency measurement definition based on active BWP is better. Thus, we provide the UE capabilities under the assumption that intra/inter frequency measurement is based on active BWP. With this assumption, only 1 layer of intra-frequency measurement based on CSI-RS need to be supported. The UE capabilities could be simpler.

Proposal 1: If intra/inter frequency measurement definition is based on active BWP, UE feature for Rel-19 mobility enhancement phase WI is agreed as in Table 1.

Event triggered reporting for LTM

For event triggered reporting for LTM, the reporting criteria will be defined in the spec. Besides that, we don’t see any additional UE capabilities being needed.

Observation 1: No UE capabilities need to be introduced for event triggered reporting for LTM.

Table 1: UE feature for mobility enhancement phase 4 WI

- NR_Mob_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_Mob_Ph4 | 56-1-1 | Number of neighbour cells to be measured per frequency layer | The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for CSI-RS based intra-frequency | 63-1 or 63-2 | Yes | No | There is no limitation on the number of neighbour cells per frequency layer for CSI-RS based L1 measurement. | Per BC | No | No | N/A | Candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signalling |
| 56. NR_Mob_Ph4 | 56-1-2 | Number of CSI-RS resources for L1-RSRP measurement within a slot | The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra-frequency L1-RSRP measurement | 63-1 or 63-2 | Yes | No | There is no limitation on the number of CSI-RS resources of intra-frequency for L1 measurement within a slot. | Per BC | No | Yes | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} | Optional with capability signalling |
| 56. NR_Mob_Ph4 | 56-1-3 | Number of CSI-RS resources for L1-RSRP measurement per frequency layer | The max number of CSI-RS resources UE can measure for L1-RSRP per frequency layer for intra frequency | 63-1 or 63-2 | Yes | No | There is no limitation on the number of CSI-RS resources per frequency layer for L1 measurement. | Per BC | No | No | N/A | Candidate value: {1,2,3,4,5,6,7,8} | Optional with capability signalling |

# Summary

In this contribution, we further provided views on UE capabilities for event triggered reporting and CSI-RS based L1-RSRP measurement for LTM. Following proposals and observations are present.

Proposal 1: If intra/inter frequency measurement definition is based on active BWP, UE feature for Rel-19 mobility enhancement phase WI is agreed as in Table 1.

Observation 1: No UE capabilities need to be introduced for event triggered reporting for LTM.

- References