3GPP TSG-RAN WG4 Meeting #115	R4-2506798

Malta, MT, 19th - 23rd, May, 2025

Agenda item:	7.18.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part for RRM Phase 5

Document for:	Discussion

- Introduction

According to the TU schedule, the performance part for RRM phase 5 has started. In this contribution, we provide our views on the performance impacts of the WI.

- Discussion

L3 measurement enhancement by optimising Rx BSF

The performance impacts were initially discussed in the last meeting with the following agreements:

Issue 2-1-1: TC list for Rx BSF optimization Agreement in online session: Further discuss based on the following table: Index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CMCC. 3; Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CATT. 2; Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Huawei. 4; Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Samsung. 5; Scenario 5: Handover; 1 or 2 TCs; TBA(e.g., Intra-frequency or inter-frequency, unknown); OPPO. 6; Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection; 1 or 2 TCs; TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown); Intel.

Regarding the detailed configurations, from our understanding, since the motivation of the feature is to speed up the L3 measurement, there is no need to verify the performance in DRX mode. Regarding whether to testing quitting FBS, since the quitting conditions is only to allow UE to fall back to normal L3 measurement behaviour and UE is not forced to quit fast L3 measurement, thus there is no need to verity the quitting behaviour. However, if there is further conclusion in core requirements discussion that UE shall re-active fast measurement, such behaviour may need to be verified.

Proposal 1: For scenario 1/2/3/4, only define TC for non-DRX mode.

Proposal 2: FFS whether to verify UE behaviour on quitting FBS, which depends on whether and how to introduce quitting conditions for FBS in core requirements.

Regarding the detailed timeline design for FBS TC, following time periods are proposed:

- T1: UE is configured with FR2-1 PCell (Cell1) and is configured with two MO: one is the MO on frequency carrier of PCell, and the other MO on the target FR2-1 carrier (Cell2). Before T2, the conditions on PCell is configured that it will not trigger FBS. And the conditions of Cell2 is configured that it will not trigger event reporting.

- T2: During T2, the conditions on PCell is configured that it will trigger FBS, and the length of T2 is longer than the measurement period.

- T3: At the beginning of T3, the conditions on Cell2 is configured that it will trigger event reporting. And UE is required to trigger event with the measurement period with reduce N.

Proposal 3: The detailed timeline design for FBS TC are proposed as follows:

- T1: UE is configured with FR2-1 PCell (Cell1) and is configured with two MO: one is the MO on frequency carrier of PCell, and the other MO on the target FR2-1 carrier (Cell2). Before T2, the conditions on PCell is configured that it will not trigger FBS. And the conditions of Cell2 is configured that it will not trigger event reporting.

- T2: During T2, the conditions on PCell is configured that it will trigger FBS, and the length of T2 is longer than the measurement period.

- T3: At the beginning of T3, the conditions on Cell2 is configured that it will trigger event reporting. And UE is required to trigger event with the measurement period with reduce N.

L3 measurement delay by optimising CSSF outside gap in CA/DC

TC for solution 1

In the last meeting, RAN4 RRM concluded how to define the test case for solution 1, which has been captured in the following:

Issue 2-2-1: TC list for CSSF optimization - TC for solution 1: Agreement in online session: - Define the test cases for solution 1 of CSSF enhancement following the table below: TC index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; ZTE.

Regarding the test cases for Solution 1, we would like to provide our views on the details of test scope and test configuration.

The test configuration for Solution 1 can utilize the following types:

15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode

15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode

30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode

Proposal 4: The test configuration for Solution 1 can utilise the following types

15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode

15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode

30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode

For the test scope, solution 1 needs a new test set to verify the enhancement that measures one CC per band. A new network capability will enable or disable the UE to do the enhancement that measures one serving CC per band when multiple serving CCs are configured in the same band. Based on this design, this enhancement requires a test set to verify its performance. For current test cases, the test set is, for example, one PCell and one SCell to test the report delay, so RAN 4 needs to discuss whether or not to define a new test set for Solution 1 to verify whether or not UE achieve this enhancement.

Proposal 5: To define a new test set for Solution 1 and verify whether the UE achieves the desired enhancement.

In the last meeting, RAN4 started to discuss the performance part of the RRM enhancement Phase 5. The test case discussion for solution 3 is captured in the following part:

- TC for solution 3: [Way forward]: - FFS: - Define the test cases for solution 3 of CSSF enhancement following the table below: TC index; Aspects; CA/DC mode; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA.

The test configuration for Solution 3 can utilize the following types:

15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode

15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode

30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode

Proposal 6: The test configuration for Solution 3 can utilise the following types

15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode

15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode

30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode

Since Solution 3 reduces the measurement delay of SCells, the test set for Solution 3 should be designed to evaluate how effectively this enhancement improves the measurement process of SCells. For the solution 3, at least set two SCells with neighbour Cell. Here is an example of a test set:

Five cells are needed to deploy in this test, FR1 PCell (Cell 1), an FR1 or FR2 deactivated SCell (Cell 2) and an FR1 or FR2 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2), another FR1 or FR2 deactivated SCell (Cell 4) and an FR1 or FR2 neighbour cell (Cell 5) on the same frequency as the SCell (Cell 4).

Proposal 7: The new test set of solution 3 is needed and the cell set is supposed to be:

Five cells are needed to deploy in this test, FR1 PCell (Cell 1), an FR1 or FR2 deactivated SCell (Cell 2) and an FR1 or FR2 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2), another FR1 or FR2 deactivated SCell (Cell 4) and an FR1 or FR2 neighbour cell (Cell 5) on the same frequency as the SCell (Cell 4).

- Conclusions

Proposal 1: For scenario 1/2/3/4, only define TC for non-DRX mode.

Proposal 2: FFS whether to verify UE behaviour on quitting FBS, which depends on whether and how to introduce quitting conditions for FBS in core requirements.

Proposal 3: The detailed timeline design for FBS TC are proposed as follows:

- T1: UE is configured with FR2-1 PCell (Cell1) and is configured with two MO: one is the MO on frequency carrier of PCell, and the other MO on the target FR2-1 carrier (Cell2). Before T2, the conditions on PCell is configured that it will not trigger FBS. And the conditions of Cell2 is configured that it will not trigger event reporting.

- T2: During T2, the conditions on PCell is configured that it will trigger FBS, and the length of T2 is longer than the measurement period.

- T3: At the beginning of T3, the conditions on Cell2 is configured that it will trigger event reporting. And UE is required to trigger event with the measurement period with reduce N.

Proposal 4: The test configuration for Solution 1 can utilise the following types

15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode

15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode

30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode

Proposal 5: To define a new test set for Solution 1 and verify whether the UE achieves the desired enhancement.

Proposal 6: The test configuration for Solution 3 can utilise the following types

15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode

15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode

30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode

Proposal 7: The new test set of solution 3 is needed and the cell set is supposed to be:

Five cells are needed to deploy in this test, FR1 PCell (Cell 1), an FR1 or FR2 deactivated SCell (Cell 2) and an FR1 or FR2 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2), another FR1 or FR2 deactivated SCell (Cell 4) and an FR1 or FR2 neighbour cell (Cell 5) on the same frequency as the SCell (Cell 4).

References

[1] R4-2504899 WF on RRM requirements for NR_RRM_Ph5_Part1