3GPP TSG-RAN WG4 #115	R4-2506803

Malta, MT, 19th - 23rd, May, 2025

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM performance requirements for Rel-19 ATG

Agenda Item:	7.12.6

Document for:	Discussion

Introduction

In the last meeting, we discussed the Rel-19 ATG CA enhancement. The TU schedule of #114bis shows that the performance part for Rel-19 ATG is starting. This paper presents our observations and views on RRM impacts of performance parts on ATG CA.

RRM performance requirement analysis

In this part, we provide our analysis of the performance requirements to support CA in case-by-case manner in the following part.

Test applicability rule

The agreement reached in the last meeting is concluded with the test configuration that is captured below. And there is one issue that the test applicability rule needs to discuss.

Issue 2-2-1: Test configuration Agreement: - Following three test configurations are supported for both PCell and SCell: - Configuration 1:15kHz SSB SCS, 10MHz bandwidth, FDD duplex mode - Configuration 2: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode - Configuration 3: 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode - If legacy TN test cases don’t have this test configuration, then no need to introduce it into the corresponding ATG test cases. Issue 2-2-2: Test applicability rule - Proposal 1: - UE is only required to be tested in one of the supported test configurations - UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

Whether or not to let UE require to teste one configuration only, the inter-band CA SCell deactivation and activation need to be considered. For SCell activation and deactivation, the PCell and SCell’s configuration should cover all TDD and FDD duplex modes. So, the test applicability rule should be: UE is required to be tested in one of the supported test configurations randomly.

Proposal 1: UE is required to be tested in one of the supported test configurations randomly.

Test setups

In the core part discussion, the SSB-less SCell activation requirement is agreed upon and can be applied to Rel-19 ATG. The following part is the agreement of the core part.

Issue 2-2-3: Test setups Agreement: - Following test setups agreed in R18 can be reused: - TDD pattern: The legacy TDD pattern are used for PCell, FFS for SCell - Channel model: AWGN with residual doppler channel model - Number of neighbour cells: one neighbour cell in the test if needed - GNSS and UE mobility setup: UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system. - BS location: The specific gNB reference location is emulated by test system - RMC: the legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases. - OCNG: - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133. - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133. - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

Regarding ATG CA enhancement is DL CA, the SCell using the TDD pattern should use the DL-only FDD pattern. In terms of the chapter A.3.1.4 in 38.133[2], the TDD pattern defined as below:

Table A.3.1.4-1: TDD UL/DL configuration for SCS=15 kHz Parameter; Unit; Value. Reference channel; TDDConf.1.1; TDDConf.1.2. referenceSubcarrierSpacing; kHz; 15; 15. TDD UL/DL pattern 1 Note 2; ‘DSUU’ S=’10DL:2GP:2UL’; ‘DSUU’ S=’ 6DL: 2GP: 6UL’. dl-UL-TransmissionPeriodicity; ms; 4; 4. nrofDownlinkSlots; 1; 1. nrofDownlinkSymbols; 10; 6. nrofUplinkSlot; 2; 2. nrofUplinkSymbols; 2; 6. TDD UL/DL pattern 2 Note 2; ‘D’; ‘D’. dl-UL-TransmissionPeriodicity; ms; 1; 1. nrofDownlinkSlots; 1; 1. nrofDownlinkSymbols; 0; 0. nrofUplinkSlot; 0; 0. nrofUplinkSymbols; 0; 0. NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information.. Table A.3.1.4-2: TDD UL/DL configuration for SCS=30 kHz Parameter; Unit; Value. Reference channel; TDDConf.2.1; TDDConf.2.2; TDDConf.2.3. referenceSubcarrierSpacing; kHz; 30; 30; 30. TDD UL/DL pattern 1 Note 2; ‘3D1S4U’ S=’6DL:4GP:4UL’; ‘1D1S2U’ S=’11DL: 1GP:2UL’; ‘3D1S4U’ S=’4DL:4GP:6UL’. dl-UL-TransmissionPeriodicity; ms; 4; 2; 4. nrofDownlinkSlots; 3; 1; 3. nrofDownlinkSymbols; 6; 11; 4. nrofUplinkSlot; 4; 2; 4. nrofUplinkSymbols; 4; 2; 6. TDD UL/DL pattern 2 Note 2; ‘DD’; Not configured; ‘DD’. dl-UL-TransmissionPeriodicity; ms; 1; Not configured; 1. nrofDownlinkSlots; 2; Not configured; 2. nrofDownlinkSymbols; 0; Not configured; 0. nrofUplinkSlot; 0; Not configured; 0. nrofUplinkSymbols; 0; Not configured; 0. NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information.

In release 19, the enhancement of ATG incorporates DL CA, with the SCell configured to exclusively receive downlink data. The legacy TDD pattern employs the interleaved uplink and downlink slot configuration, such as ‘DDDSU’. While the SCell can be restricted to downlink operations. But defining a new TDD pattern has been discussed in the Rel-18, so it is necessary to further discuss whether or not to define a new TDD pattern for ATG.

Proposal 2: RAN4 to discuss whether or not to define a new TDD pattern for ATG.

Test method and scaling factor for UE with antenna array

The

Issue 2-2-4: Test method and scaling factor for UE with antenna array FFS: - The scaling factor related with antenna array will not be applied in test requirements - The test parameters, test requirements for UE with antenna array on both PCC and SCC

Since the test case in Release 18 does not consider the scaling factor for the antenna array of the ATG UE, the scaling factor in Release 19 is not required in the test requirements. Therefore, the scaling factor does not need to be considered in the test requirements.

Proposal 3: The scaling factor does not need to be considered in the test requirements.

Test scope

The following table was created from the last meeting. We would like to update our comments in this table. Meanwhile, several test cases required further discussion, the details are provided below:

      - Inter-band SSB-less SCell Activation test case

Regarding the Release 19 ATG CA, which introduced the SSB-less SCell activation requirement, the details on how ATG UE with an antenna array in FR1 can achieve this requirement remain unclear or based on the UE implementation. This aspect may impact the validity of SSB-less test cases. In our view, SSB-less SCell activation test cases should be restricted to ATG UE explicitly supporting [Case 4 capability].

Proposal 4: Inter-band SSB-less SCell Activation test cases should be restricted to ATG UE explicitly supporting [Case 4 capability].

      - Test case for Pre-configured measurement gap test requirement

This test case issue is related to the pre-configured measurement gap, which seems this is not to be required for CA enhancement. Discuss whether or not to define this test requirement in this release.

Proposal 5: Discuss whether or not to define Pre-configured measurement gap test requirements in this release.

Proposal 6: We updated our comments in the test scope table.

| Test category | Test cases | Company comments |
| --- | --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 | CMCC: Support Ericsson: Support HW: Support |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot | CMCC: Support HW: Support Ericsson: Support |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot | CMCC: Support HW: Support Ericsson: Support |
|  | interruption at NR SRS antenna port switching | HW: postpone |
|  | Interruptions at NR SRS carrier based switching | CATT: FFS |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support Ericsson: Support HW: Support |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | CMCC: Support CATT: Support Ericsson: Support HW: Support |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX | CMCC: Support CATT: Support HW: Support Ericsson: Support |
|  | Direct SCell activation at SCell addition of known SCell in FR1 | CMCC: Support CATT: Support HW: Support |
|  | Direct SCell activation at handover with known SCell in FR1 | CMCC: Support CATT: Support HW: FFS |
|  | Direct SCell Activation at RRC Resume | CMCC: FFS CATT: Support HW: FFS |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | CMCC: Support CATT: Support HW: FFS |
|  | Inter-band SSB-less SCell Activation based on A-TRS | CMCC: Support CATT: Support HW: FFS |
| Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | Ericsson: Support HW: Support |
|  | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | HW: Support |
|  | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode | HW: Support |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs | CMCC: Support CATT: Support Ericsson: Support |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs | CMCC: Support CATT: Support |
|  | non-simultaneous Timer based BWP switch delay on multiple CCs | CMCC: FFS |
| SCell dormancy switch | SCell dormancy switch | CMCC: Support HW: Support |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX | CMCC: Support HW: Support |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement | CMCC: FFS HW: FFS |
| Others | Intra-frequency measurement(with scheduling restriction verified as well) | ZTE: Support |
|  | Inter-frequency measurement(with scheduling restriction verified as well) | ZTE: Support |
|  | L1-RSRP measurement | ZTE: Support |
|  | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX | Ericsson: Support |
|  | The interruption of SCell addition/release | ZTE: Support |
|  | The interruption of SCell activation/deactivation | ZTE: Support |

Conclusion

Proposal 1: UE is required to be tested in one of the supported test configurations randomly.

Proposal 2: RAN4 to discuss whether or not to define a new TDD pattern for ATG.

Proposal 3: The scaling factor does not need to be considered in the test requirements.

Proposal 4: Inter-band SSB-less SCell Activation test cases should be restricted to ATG UE explicitly supporting [Case 4 capability].

Proposal 5: Discuss whether or not to define Pre-configured measurement gap test requirements in this release.

Proposal 6: We updated our comments in the test scope table.

| Test category | Test cases | Company comments |
| --- | --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 | CMCC: Support Ericsson: Support HW: Support |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot | CMCC: Support HW: Support Ericsson: Support |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot | CMCC: Support HW: Support Ericsson: Support |
|  | interruption at NR SRS antenna port switching | HW: postpone |
|  | Interruptions at NR SRS carrier based switching | CATT: FFS |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support Ericsson: Support HW: Support |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | CMCC: Support CATT: Support Ericsson: Support HW: Support |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX | CMCC: Support CATT: Support HW: Support Ericsson: Support |
|  | Direct SCell activation at SCell addition of known SCell in FR1 | CMCC: Support CATT: Support HW: Support |
|  | Direct SCell activation at handover with known SCell in FR1 | CMCC: Support CATT: Support HW: FFS |
|  | Direct SCell Activation at RRC Resume | CMCC: FFS CATT: Support HW: FFS |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | CMCC: Support CATT: Support HW: FFS |
|  | Inter-band SSB-less SCell Activation based on A-TRS | CMCC: Support CATT: Support HW: FFS |
| Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | Ericsson: Support HW: Support |
|  | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | HW: Support |
|  | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode | HW: Support |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs | CMCC: Support CATT: Support Ericsson: Support |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs | CMCC: Support CATT: Support |
|  | non-simultaneous Timer based BWP switch delay on multiple CCs | CMCC: FFS |
| SCell dormancy switch | SCell dormancy switch | CMCC: Support HW: Support |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX | CMCC: Support HW: Support |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement | CMCC: FFS HW: FFS |
| Others | Intra-frequency measurement(with scheduling restriction verified as well) | ZTE: Support |
|  | Inter-frequency measurement(with scheduling restriction verified as well) | ZTE: Support |
|  | L1-RSRP measurement | ZTE: Support |
|  | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX | Ericsson: Support |
|  | The interruption of SCell addition/release | ZTE: Support |
|  | The interruption of SCell activation/deactivation | ZTE: Support |

References

[1] R4-2504898 WF on RRM requirements for NR_ATG_enh

[2] 3GPP TS 38.133 V18.8.0