3GPP TSG-RAN WG4 Meeting #115 	R4-2506843

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on the RF requirement for CW

Agenda Item:	7.26.2.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on the boundary of emission requirement and transmit OFF power for the CW node.

- Discussion

- Phase noise

In the last meeting, RAN1 agreed a Table for the candidate D2R transmission bandwidth, and the minimum BW could be 15kHz for 2SB transmission. For the phase noise, smaller BLF will suffer from higher interference, and the 15kHz D2R can be considered as the worst case to set the requirement boundary after adding 10% SFO impact.

However, we also need a starting point for the phase noise since the CW itself should be excluded and 1kHz can be a possible choice. The measurement bandwidth can same as transmission bandwidth of D2R

Proposal 1: The phased noise is set within [fcw-8.5kHz, fcw +1kHz] and [fcw+1kHz, fcw +8.5kHz], with 7.5kHz measurement bandwidth.

In [2], we analyze the minimum D2R power at reader based on the deployment assumed in SI, and the minimum D2R power could be -108.6 dBm (consider 10 dB backscatter loss). Considering 20m NLOS pathloss (around 61 dB), 4dB shadow fading, 10dB spatial isolation due to the reader Rx antenna, the residual power of 33 dBm CW at reader could be – 42 dBm. The desens due to phase noise can be calculated as below:

| D2R transmission bandwidth | Wanted signal | iPN within D2R transmission channel bandwidth | Noise power | SNR | D2R SNR@ 10% BLER | Desens/dB |
| --- | --- | --- | --- | --- | --- | --- |
| 15 kHz | -108.6 dBm | -60 dBc | -102 dBm | -6.6 | -1 dB (consider 6dB desens due to the CW cancellation) | 5.6 |
|  |  | -65 dBc | -107 dBm | -1.6 |  | 0.6 |
|  |  | -70 dBc | -112 dBm | 3.4 |  | <0 |
|  |  | -75 dBc | -117 dBm | 8.4 |  | <0 |
|  |  | -80 dBc | -122 dBm | 13.4 |  | <0 |

We can find that within the 15m coverage, the D2R signal power is much higher than the AIoT BS receiver capability, so a large margin exists for D2R to endure with the CW phase noise and -60 dBc can be considered as the requirement.

Observation 1: Under the deployment assumption in SI, the D2R signal power received at reader is much higher than the reader sensitivity, and the margin exist for D2R endure with the phase noise.

Proposal 2: The limit of phase noise can be -30 dBm/7.5kHz.

- Unwanted emission

As we analyzed in the last meeting, the emission of CW should ensure the D2R performance under difference small frequency shift, otherwise the FDMA may hard to work.

Observation 2：The emission of CW should guarantee the D2R performance within the CBW when FDMA is used.

So the boundary of emission should within the D2R channel bandwidth and ignore the region where the phase noise is already applied.

Figure III Illustration of the CW SEM boundary

Proposal 3: The unwanted emission of CW is defined from fcw ± 8.5kHz to the boundary of maximum D2R channel bandwidth.

As we analyzed in phase noise part, 70 dBc iPN can ensure the no desens, and this can be translated to the absolute power, i.e., 40 dBm/7.5kHz, which can be considered as the starting point of emission requirement.

Proposal 4: -40 dBm/7.5kHz can be considered as the unwanted emission requirement for CW.

- Spurious emission

In the last meeting, we agreed that NR spurious emission is used as starting point. The NR spurious emission comes from ITU-R and it can be applied out of D2R Channel bandwidth.

Figure IV Illustration of the phase noise requirement boundary

Proposal 5: The spurious emission of CW starts from the boundary of maximum D2R channel bandwidth.

- Conclusion

In this contribution, we provide our views on CW RF requirement.

Proposal 1: The phased noise is set within [fcw-8.5kHz, fcw +1kHz] and [fcw+1kHz, fcw +8.5kHz], with 7.5kHz measurement bandwidth.

Observation 1: Under the deployment assumption in SI, the D2R signal power received at reader is much higher than the reader sensitivity, and the margin exist for D2R endure with the phase noise.

Proposal 2: The limit of phase noise can be -30 dBm/7.5kHz.

Observation 2：The emission of CW should guarantee the D2R performance within the CBW when FDMA is used.

Proposal 3: The unwanted emission of CW is defined from fcw ± 8.5kHz to the boundary of maximum D2R channel bandwidth.

Proposal 4: -40 dBm/7.5kHz can be considered as the unwanted emission requirement for CW.

- References

- R4-2505097 	 WF on A-IoT BS and CW requirements	Huawei

- R4-2506842	Discussion on the RF requirement for device 1	vivo