3GPP TSG-RAN WG4 Meeting #115				R4-2506907

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.27.2

Source: 	Ericsson

Title: 	Discussion on UE feature list for Rel-19 NES

Document for:	Approval

- Introduction

This contribution includes the RAN4 UE feature list for enhancements of network energy savings for NR in Rel-19. In the below table we propose to define the following capabilities based on RAN4 previous agreements for the work item.

Proposal 1:

#### On-demand SSB SCell operation

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. Netw_Energy_NR_enh | 61-5 | On-demand SSB for deactivated SCell measurement | 1. Support to perform on-demand SSB based deactivated SCell measurement based on OD-SSB configuration without lower bound within fast measurement window in both Case #1 (No always-on SSB on the cell) and Case #2 (with always-on SSB on the cell) | 61-1, 61-2, 61-2a, 61-3, 61-4, 61-4a | Yes | n/a | UE performs on-demand SSB based deactivated SCell measurement with a lower bound of 10ms within fast measurement window in Case #1 and Case #2 | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-6 | On-demand SSB [activation] processing time | - Support to reception of on-demand SSB bursts indicated via a MAC CE not less than T_min=m+3Nslotsubframe,μ+1 with additional [2]ms processing time. | 61-3, 61-4, 61-4a | Yes | n/a | UE receives on-demand SSB bursts indicated via a MAC CE not less than T_min=m+3Nslotsubframe,μ+1 with additional 5ms processing time | Per band | n/a | n/a | n/a |  | Optional with capability signaling |