# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2506930

# Malta, 19th – 23rd May, 2025

Agenda item:	7.25.3.1

Source: 	Qualcomm Incorporated

Title: 	Views on new requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#114bis, a WF was agreed in [2]. In this contribution we share our views on remaining aspects in new BS RF requirements for Rel-19 SBFD.

# New BS RF requirements for SBFD operation

## Requirements for Transient period between SBFD and non-SBFD

As captured in [2], RAN4 has agreed to the following:

- For the location of transient period between SBFD and non-SBFD symbols, - No need to be specified in RAN4 core requirement;

RAN4 has already informed RAN1 about the above agreement and the different four cases for transitioning between SBFD and non-SBFD symbols. Since RAN4 is focused on the gNB aspects regarding the introduction of the new transient period requirements between SBFD and non-SBFD operation, there is ambiguity in how the UE is supposed to be either informed or operates during the abovementioned transient periods. Additionally, the location of the transition period is not defined for all scenarios. For cases A and D, the transition period location is clearly specified, whereas for cases B and C, it is determined by the gNB implementation.

Observation 1: UE’s knowledge of the location of the transient period for Case B and C is important to ensure proper transition between SBFD and non-SBFD symbols.

In this regard, from the UE's perspective, the knowledge for the locations and lengths of the transient/guard periods between SBFD and non-SBFD symbols is necessary. Thus, it is important to discuss if RAN4 needs to further discuss if such discussion is best to take place in other RAN WGs (e.g., RAN1), since RAN4 in the Rel-19 work item is focused on the standardization of the SBFD-capable gNB requirements.

Proposal 1: RAN4 to discuss if the UE’s knowledge of location of the transient periods between SBFD and non-SBFD is relevant to Rel-19 WI discussions.

## In-channel adjacent subband leakage ratio

During the study item phase, legacy ACLR and ACS was assumed for inter-channel leakage and inter-channel selectivity, this was mainly assumed to progress the work on the coexistence between legacy TDD and new SBFD deployments for the different simulation scenarios. In our views, it is a viable way forward to follow what was done in the study item as a baseline and further agree on methodologies that could be agreed within RAN4 to confirm the feasibility of such way forward. One challenge for the in-channel new requirements is the dependency on the self-interference capability yields the definition of a new inter-subband requirement difficult to agree between different companies. As observed in Section 9 in TR 38.853, different implementations can be assumed among vendors, leading to different self-interference, co-sector interference, and inter-site interference mitigation capabilities. Moreover, employed technologies and mitigation techniques in the SBFD-capable gNB vary within RAN4, and thus, room for implementation differences should be considered when defining the new inter-subband leakage and blocking requirements.

Observation 2: The in-channel subband sub-band Tx leakage and blocking depend on the self-interference, inter-site and inter-sector interference considerations; thus, they are implementation dependent.

Proposal 2: RAN4 to agree on reusing ACLR absolute basic limit given in Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2-1, for the absolute limit for in-channel adjacent subband leakage.

# Conclusion

In this contribution, we provided our views on new BS RF requirements for SBFD with the following observations and proposals:

Observation 1: UE’s knowledge of the location of the transient period for Case B and C is important to ensure proper transition between SBFD and non-SBFD symbols.

Proposal 1: RAN4 to discuss if the UE’s knowledge of location of the transient periods between SBFD and non-SBFD is relevant to Rel-19 WI discussions.

Observation 2: The in-channel subband sub-band Tx leakage and blocking depend on the self-interference, inter-site and inter-sector interference considerations; thus, they are implementation dependent.

Proposal 2: RAN4 to agree on reusing ACLR absolute basic limit given in Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2-1, for the absolute limit for in-channel adjacent subband leakage.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, RAN4#114bis, Samsung.