3GPP TSG-RAN WG4 Meeting #115	R4-2506970

# Malta, MT, 19th – 23rd May, 2025

Title:	Discussion on testability for LP-WUR

Source:	Samsung

Agenda item:	7.28.3.4

Document for:	Discussion

1.	Introduction

Testability is one of the scopes for this work item but no much progress yet, it is important to converge on high level framework and then move forward onto details.

In this contribution, we share our views on LP-WUR testability.

2. 	Discussion

2.1 Testing MDR of LP-WUS or PDCCH

In RAN4#112bis meeting, 1% MDR is agreed as metric of LP-WUR UE RF for both requirements and testing [1, R4-2417112]:

Issue 1-1-3: MDR value for RF requirements Agreements: - 1% MDR. Same for requirements and conformance testing.

According to TR38.869, MDR here means the miss-detection rate (MDR) of LP-WUS, rather than PDCCH.

In last meeting, MDR of PDCCH was proposed for discussion and the way forward is as following [2, R4-2505234]:

Issue 4-1-1: Whether LR can be tested directly Agreements: - Down select below two approaches for LP-WUR UE RF requirements: - Option 1: stick to 1% MDR LP-WUS as performance metric, but with a note in TS that LP-WUR ON is not mandated. - The note depends on RAN1 conclusion - Option 2: update it to 1% MDR of PDCCH triggered by LP-WUR

For the newly proposed option 2 which is to adopt MDR of PDCCH, it is a kind of function test of a feature rather than RF conformance test of transmitter or receiver. For conformance test, repeatability is required. But for option 2, because the LP-WUR ON/OFF status is unpredictable, so the MDR results of PDCCH are not repeatable when testing many times.

Observation 1:	MDR testing based on PDCCH is not repeatable which contradicts the principle of conformance testing.

Moreover, MDR of PDCCH as performance metric for LP-WUR is contradicted with the coverage target. The designed coverage target is for LP-WUS, it was also agreed that the REFSENS derivation should also consider the LP-WUS coverage target. If adopting MDR of PDCCH as performance metric for LP-WUR, the coverage for REFSENS is actually becoming the mix of LP-WUS and PDCCH. It would be not aligned with all previous RAN1 and RAN4 MDR simulations.

Observation 2:	MDR of PDCCH as performance metric for LP-WUR is contradicted with the coverage target and not aligned with previous RAN1 and RAN4 simulations.

Considering above reasons, it is proposed to stick to previous agreement, i.e., stick to 1% MDR LP-WUS as performance metric.

Proposal 1:	RAN4 to stick to previous agreement, i.e., stick to 1% MDR LP-WUS as performance metric.

2.2 RRC states

There kinds of RRC states are involved for LP-WUS feature, i.e., RRC_Idle, RRC_inactive and RRC_Connected. For requirement and test, a high-level agreement was achieved in RAN4#113 meeting [3, R4-2420528]:

Issue 4-1-1: alignment of common understanding on LP-WUS requirements and corresponding testing Agreements: - The same LP-WUS RF requirements for all three RRC states are assumed. But only one RRC state needs to be tested.

Only one RRC state needs to be tested. Then it is next question which RRC state is to be selected or prioritized for testing. Progress on this aspect faces obstacle due to no clear consensus on UE capabilities, the way forward of RAN4#114 meeting is to further check whether all UE types can support IDLE and/or CONNECTED mode [4, R4-2503028]:

Issue 4-1-2: Separate RF test case for idle and connection mode Way forward: - RAN4 further discuss and check whether all UE types can support IDLE and/or CONNECTED mode?

According to the UE feature list [5] attached in the RAN1 LS [6], UE has full flexibility in terms of its supported RRC states. UE can support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) separately, and UE can also support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) together, i.e., there could be 3 types of UE in terms of supported RRC states:

- UEs supporting IDLE/INACTIVE mode only

- UEs supporting CONNECTED mode only

- UEs supporting both IDLE/INACTIVE mode and CONNECTED mode

| Features | Index | Feature group |
| --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

There was proposals to rely on MR in connected mode to loop back the LR detection test results to TE. Based on above UE capabilities, it is not feasible for UEs supporting IDLE/INACTIVE mode only.

Observation 3:	By using connected mode to achieve loopback is not feasible since there are UEs supporting IDLE/INACTIVE mode only.

For FR1, supporting LP-WUS in RRC_Idle states showed most obvious power saving benefits [7]. Though UE may support not all 3 RRC states, the large commercialization possibility is that the FR1 LP-WUS UE would either support RRC_Idle or support RRC_Idle & RRC_Connected states. Therefore, it can be expected that test method based on RRC_Idle could cover most available UEs. It is reasonable to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing.

Proposal 2:	RAN4 to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing.

2.3 Test mode

High level framework has been discussed for several meeting cycles, and the latest agreements are as following [3, R4-2420528]:

Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance Agreements: - Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point: - Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance. - FFS if the MR should only be woken up when data transfer the system simulator is needed. - Other options are not precluded.

There have been sufficient discussions on this issue and it can be concluded that waking up MR for each detected LP-WUS is not practical for conformance test. It is more practical to just counting the detected LP-WUS signal and not to wake up the MR before the test counting is over. It is proposed to agree on this high-level framework and then move forward.

Proposal 3:	RAN4 to counter the detection rate without waking up the MR to verify LP-WUS RF performance.

This implies that a test mode is needed to force the LP-WUR always on during the MDR test. The way forward of last meeting is as following [2, R4-2505234]:

Issue 4-1-4: UE test mode for LP-WUR Agreements: - Test mode design as well as the details of test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

According to the agreement, a test mode is needed but detailed design is up to RAN5 based on RAN4 input. From RAN4 perspective, it is necessary to conclude in RAN4 that a test mode is needed to force the LP-WUR always on during the MDR test, and to loopback test results to TE after MDR test over.

Proposal 4:	RAN4 inputs on the test mode include at least to force the LP-WUR always on during the MDR test, and to loopback test results to TE after MDR test over.

3. 	Conclusion

Observation 1:	MDR testing based on PDCCH is not repeatable which contradicts the principle of conformance testing.

Observation 2:	MDR of PDCCH as performance metric for LP-WUR is contradicted with the coverage target and not aligned with previous RAN1 and RAN4 simulations.

Proposal 1:	RAN4 to stick to previous agreement, i.e., stick to 1% MDR LP-WUS as performance metric.

Observation 3:	By using connected mode to achieve loopback is not feasible since there are UEs supporting IDLE/INACTIVE mode only.

Proposal 2:	RAN4 to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing.

Proposal 3:	RAN4 to counter the detection rate without waking up the MR to verify LP-WUS RF performance.

Proposal 4:	RAN4 inputs on the test mode include at least to force the LP-WUR always on during the MDR test, and to loopback test results to TE after MDR test over.

4. 	References

- R4-2417112   “WF on UE RF requirements for LP-WUS”, vivo

- R4-2505234    “WF for [114bis][131] NR_LPWUS_UERF”, vivo

- R4-2420528    “WF on UE RF requirements for LP-WUS”, vivo

- R4-2503028    “WF on UE RF requirements for LP-WUS”, vivo

- R1-2501397    “Initial RAN1 UE features list for Rel-19 NR after RAN1 #120”, Moderators (AT&T, NTT DOCOMO, INC.)

- R4-2503104_R1-2501399    “LS on Rel-19 RAN1 UE features list for NR after RAN1#120”, RAN1

- TR 38.869