3GPP TSG-RAN WG4#115	R4-2507072

Malta, MT, May 19th – 23rd, 2025

Agenda item:	7.6.2

Source: 	Qualcomm France

Title: 	RedCap PC2 FDD REFSENS

Document for:	Approval

# Introduction

RedCap PC2 1Rx FD-FDD REFSENS for is analyzed in this contribution.

- WF from RAN4#114bis

# WF for REFSENS is as follows [1]

Issue 1-1-1: REFSENS for 1Rx in FD-FDD

Agreement: Consider the following formats for specifying REFSENS for 1Rx PC2 RedCap in FD-FDD

- Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on RSD ranges

  - Flat 3dB ΔR 1R for all CBW no larger than 20MHz or use values for FDD bands in table 7.3I.2-1 for bands whose PC2 1Tx/2RX RSD is 0dB.

  - Companies to evaluate REFSENS for other bands.

  - The format in which the REFSENS is shown in the specs is TBD pending evaluations.

- Option2: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on the following conditions

  - ΔR 1R value may be considered from LTE TS 36.101 Table 7.3.1E-1A for bands and CBW which are already defined in TS 36.101 Table 7.3.1E-1A. The ΔR 1R value is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1.

  - Companies are encouraged to bring measurement based evaluation for ΔR1R for other bands and CBW which are not covered by LTE TS 36.101 Table 7.3.1E-1A.

  - The format in which the REFSENS is shown in the specs is TBD pending evaluations.

Way forward: It is encouraged for companies to provide analysis for option 1 and 2.

Note: Companies are encouraged to evaluate all specified PC2 FDD bands. The WI is intended to be completed in May meeting according to WI plan.

- Discussion

In WF there were a couple of options with some degree of freedom for baseline ΔR 1R between 2.5dB and 3dB for certain bands.

In our calculations, we assumed 6dB total interference difference between PRX and DRX. We calculated the PC2 1Rx REFSENS in two ways: with thermal noise equaling 9dB NF excluding interference and with thermal noise equaling up to 12dB NF excluding interference. For both cases the interference was set such that UE just meets 2Rx PC2 REFSENS. Using the same calculation, PRX REFSENS was calculated. The rationale for using two different thermal noise levels is as follows. Bands whose REFSENS for 5MHz is -100dBm are assumed to have thermal noise equaling 9dB NF. Then bands whose 5MHz REFSENS is worse than -100dBm, have thermal noise equaling up to 12.5dB NF (n25) but the increase in thermal noise can be constructed in multiple ways, i.e. it could entirely due to interference increase or partially or even entirely due to IL increase. Hence using two extremes one gets good overall view of the situation.

With the more aggressive approach among the two explained, the minimum ΔR 1R becomes 3dB.

We use tighter values of these two methods in our proposal. The reason for this is that the PC2 2RX RSD’s are at least adequate, so in our view using 9dB NF for all bands topped by interference to meet 2RX PC2 REFSENS would result in a tad pessimistic numbers. For simplicity, we show our proposal in dBm’s for each band/bandwidth, but we are open to further discuss the exact format.

Figure 1 PC2 1Rx REFSENS for RedCap

- Conclusion

PC2 1RX FD-FDD REFSENS is analyzed in this contribution with the following proposals.

Proposal 1: Use the following numbers for FD-FDD PC2 1Rx RedCap REFSENS evaluation.

- Reference

[1] R4-2505231, “WF on PC2 RedCap UE RF requirements”, China Telecom, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple, Nokia

- Annex

Calculations for some bands.

.