3GPP TSG-RAN WG4 Meeting #115	R4-2507105

St. Julian’s, Malta, 19 May – 23 May 2025

Agenda Item:	7.21.3

Source:	Ericsson

Title:	Discussion on CSI prediction

Document for:	Discussion

# 1	Introduction

RAN4#114bis agreed with the way forward on AI/ML for NR air interface [1]. This contribution discusses the remaining issues on the CSI reporting requirement and testing framework for CSI prediction.

# 2	Performance monitoring

RAN1 agreed to use Type 3 performance monitoring [2] for the UE-assisted performance monitoring metric:

- UE calculates the performance metric(s)

- UE reports performance metric(s) to the NW

- NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

RAN1 also agreed to select SGCS as the performance metric, but RAN1 needs more time to decide the detailed definition of SGSC and reporting methods.

Figure 1 shows our understanding of the performance monitoring framework based on the Type 3 performance monitoring. When the gNB configures the CSI report with AI/ML-based CSI prediction, the UE needs to report the performance monitoring quantity to the gNB in addition to the CSI report. The gNB may use the reported performance monitoring quantity to judge whether the prediction functionality works or not. If the gNB judges the prediction functionality does not work, e.g., the performance monitoring quantity is less than a threshold, then the gNB deactivates the prediction functionality. When the gNB deactivates the AI/ML-based CSI prediction functionality, the UE uses the legacy CSI reporting method to report PMI.

Figure 1	Performance monitoring of AI/ML-based CSI prediction model.

The performance monitoring is applicable not only for CSI prediction, but also for other AI/ML-based functionalities such as beam management. RAN1 has already agreed that the performance monitoring report for beam management is a part of CSI reporting, called ‘Reference signal prediction accuracy indicator, or RS-PAI. If RAN1 reuses the same reporting framework for performance monitoring of CSI prediction functionality, RAN4 may need the performance monitoring accuracy requirements for CSI prediction.

On the other hand, we think it is difficult to verify the performance monitoring accuracy based on the SGCS. Although RAN1 is still discussing the detailed definition of SGCS, the UE calculates the SGCS from the predicted PMI and ground-truth PMI (PMI derived from the measured CSI-RS). This means TE should know the ground-truth PMI at the UE. The ground-truth PMI is same as the PMI calculated from the legacy CSI estimation. However, in the existing specifications, RAN4 does not verify the accuracy of the reported PMI, but checks the throughput ratio between the reported PMI and random PMI.

If we cannot verify the accuracy of performance monitoring quantity for CSI prediction, alternatively, we may check the reported quantity together with the throughput ratio. For example, we may add the following requirements in the PMI reporting performance requirements.

- Reported performance monitoring quantity shall be more than X at least Y% of the test time (e.g., X=0.7 and Y=80%)

Proposal 1: RAN4 discusses the testability of the performance monitoring accuracy requirement for CSI prediction.

# 3	Generalization aspects, static/non-static scenarios/conditions, and propagation conditions for testing

For the non-AI-based (legacy) PMI reporting performance, RAN4 defines one PMI reporting test case per precoder type, e.g., Type I and eTypeII. It is because we assume UE applies the same PMI reporting algorithm regardless of the test scenarios (e.g., antenna configuration, PMI configuration, SNR, channel model).

On the other hand, the AI/ML-based CSI prediction performance depends on the training dataset. Even if a UE reports the right PMI in one specific scenario, this UE may report PMI that degrades the performance in other scenarios. One way to ensure the model generality is to define PMI reporting requirements with more than one channel model, such as TDLA30 and TDLC300, and antenna configurations, 16 CSI-RS ports and 32 CSI-RS ports.

Proposal 2: RAN4 should consider more than one condition to guarantee the generality of the AI-based CSI prediction model. At least RAN4 should consider more than one channel model (e.g., TDLA30 and TDLC300) and antenna configuration (e.g., 16 CSI-RS ports and 32 CSI-RS ports).

One concern of multiple requirements is an increase in the total number of test cases. RAN4 discussed the non-static scenarios/conditions to save the total number of test cases. Although RAN4 has already agreed to use static conditions as baseline for CSI prediction [3], it is still FFS on whether to have non-static condition. One way is to combine more than one fixed condition test. This is an example of test procedure:

- Step1: In period T1, the TE sets TDLA30-10 and schedules the fixed MCS with 64QAM/0.5 and measures the PDSCH throughput with follow PMI. TE then decides the SNR test point (=SNR1) based on the throughput in T1. In period T2, the TE sets TDLC300-100 and schedules fixed MCS with QPSK/0.3 and decides the SNR test point (=SNR2) based on the throughput in T2. In period T3, …

- Step 2: After deciding the test points SNR1, SNR2, …, TE reruns the same tests with random TypeI PMI with TDLA30-10/64QAM/0.5 with SNR1 in T1, with TDLC30-100/QPSK/0.3 with SNR2 in T2, …

- Step 3: TE derives the throughput ratios of follow PMI and random PMI for each period T1, T2, …

- Step 4: UE passes the test if the throughput ratio exceeds the required gamma for each period.

Since TE can change channel model and MCS without RRC reconfiguration, we don’t think the test time is increased significantly.

Table 1 is an example of PMI reporting test requirements with semi-static condition. In this example, TE sets different scenarios in time duration T1, T2, and T3.

Table 1	Example of PMI test with semi-static scenario/condition

|  | Channel model | MCS | Gamma |
| --- | --- | --- | --- |
| T1 (e.g., 1000 slots) | TDLA30-10 | 64QAM, 0.5 | 2.0 |
| T2 (e.g., 1000 slots) | TDLC300-100 | QPSK 0.3 | 1.3 |
| T3 (e.g., 1000 slots) | TDLA30-50 | 16QAM 0.5 | 1.7 |

Proposal 3: For CSI prediction performance requirements, RAN4 discusses the feasibility of combing more than one static condition test to ensure the generality of the prediction model.

In RAN4#113, RAN4 agreed to use TDL channel model for AI/ML-based CSI prediction tests. We think the AI-based CSI reporting performance test should apply to more realistic channel model than TDL channel model. One candidate model is CDL, but RAN4 is discussing the feasibility for performance requirements with CDL channel model in Rel-19 SI. We propose RAN4 should apply the CDL channel model to the PMI reporting test when a suitable CDL model is available.

Proposal 4: RAN4 should apply CDL channel model to the PMI reporting requirements with the AI/ML-based CSI prediction when a suitable CDL model is available.

# 5	Summary

Proposals:

Proposal 1: RAN4 discusses the testability of the performance monitoring accuracy requirement for CSI prediction.

Proposal 2: RAN4 should consider more than one condition to guarantee the generality of the AI-based CSI prediction model. At least RAN4 should consider more than one channel model (e.g., TDLA30 and TDLC300) and antenna configuration (e.g., 16 CSI-RS ports and 32 CSI-RS ports).

Proposal 3: For CSI prediction performance requirements, RAN4 discusses the feasibility of combing more than one static condition test to ensure the generality of the prediction model.

Proposal 4: RAN4 should apply CDL channel model to the PMI reporting requirements with the AI/ML-based CSI prediction when a suitable CDL model is available.

# References

- R4-2505105, “WF on requirements for AI/ML air interface normative work”, Qualcomm Incorporated.

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

- R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm Incorporated.