3GPP TSG-RAN WG4 Meeting #115	R4-2507138

# St Julian’s, Malta, May 19th – 23rd 2025

Source:	Ericsson

Title:	Discussion on the methodology of NR SCM for demodulation

Agenda item:	7.16.3

Document for:	Discussion

# Introduction

In the RAN4#114bis meeting, companies mainly discussed the detailed of derivation of CDL based model and TDL based model. In WF [1], agreed CDL model truncation procedures are captured. The model profiles per CDL model options are also discussed by email before the RAN4#115 meeting.

In this contribution, the open issues on common methodologies, pros and cons of CDL/TDL based model and metrics for alignment and comparison are furtherly discussed.

# Discussion

PDSCH ‘Fixed’ PMI Selection

Way Forward

For alignment a random PMI will be used, for comparison cases with a ‘fixed’ PMI FFS on approach.

2.1	Common for all Methodologies

It is observed that the spatial preference during a relative long time is typical in both simulations and field measurements. In that case, it is possible to use a fixed PMI configuration rather than random PMI to check the receiver performance under a specified (randomness reduced) spatial channel model. Since the chosen PMI is based on the follow-PMI procedure, the fixed PMI test could be more practical which can show receiver behavior in real network.

The issue of considering fixed PMI is how to align this configuration among companies’ implementations. Some solutions are provided in RAN4#114-bis meeting, but there is no much time to discuss it. Generally, it could be possible to align PMI hypothesis used in simulation if companies get alignment from CSI-port mapping to PMI selection method. As a comparison method for PDSCH performance checking under SCM, we suggest companies continue discussing the alignment procedure.

Proposal 1	Consider following steps for fixed PMI simulation alignment.

•	Step 1: Align PMI hypothesis including CSI-port mapping and PMI index content.

•	Step 2: Align PMI histograms from companies under high SNR or high normalized throughput range (>=80%).

•	Step 3: Choose the highest possibility PMI from aligned PMI histogram for performance alignment.

PMI Granularity in PMI reporting cases

Way Forward

RAN4 to use wideband PMI for Type 1 Follow PMI cases

RAN4 to use wideband PMI for Type 1 Random PMI cases

Note: Previous requirements have used both wideband and sub-band PMI

For Type-I codebook, wideband is typical configuration regarding limited indices can be chosen. Even we use sub-band PMI for Type-I codebook, the performance difference would be very limited. Based on our simulation results, there is no difference observed from all alignment/comparison test cases [2].

There is no obvious performance difference between wideband and sub-band for Type-I codebook in all alignment and comparison test cases.

## 2.2 	TDL based methodologies

TDL Model Extensions

Way Forward

Companies to assess Cluster Models 1, 2, and 3 – focus on option 2 and 1 for alignment, with 4 clusters for all cases, based on companies’ preference.

It is encouraged that all companies assess all options to understand the pros and cons of each option.

We have discussed a lot on pros and cons of TDL based model in previous meetings [3, 4]. These models can show expected performance by creating beams according test cases. One important issue is we should not change channel when we change test case, otherwise the performance is suspicious. If TDL based model would have sufficient spatial richness for all cases (at least in this study scope), the version for 8 layers transmission can be applied for all other test cases. Companies could check the performance accordingly.

Proposed TDL based models need to change beam configuration according to different test cases which is not practical.

Proposal 2	Companies to check if one TDL based model can be used for 2/4/8 layers test cases.

## 2.3	CDL based methodologies

CDL Table Truncation

Way Forward

As a starting point, a single CDL Table taken forward to include truncation and cluster combining (details included in Annex A). The number of clusters used will be 12.

This will be assessed during RAN4#115 if it can support 8 layers; and if a untruncated option is required.

The CDL model cluster truncation method is proposed during the last meeting regarding TE vendor’s argument on implementation complexity when supporting conformance tests, and new truncated model profiles are proposed by the end of meeting. Before RAN4#115, companies did calculation based on proposed truncation method and almost all companies get alignment finally on un-truncated (20 clusters) and truncated (12 clusters) model profiles based on CDL-C model in TR38.901 (with randomness reduction framework in TR38.827).

Based on our simulation results [2], the performance between un-truncated model and truncated model are very similar. Truncated 12 cluster model show a slightly better performance on 8 layers PDSCH case on worse condition CW. The reason behind might be that the less clusters could achieve higher transmitting power when Tx power is the same. See figure below.

The 8 layers PDSCH performances are very similar between un-truncated 20 cluster model and truncated 12 cluster model with same AAV configuration. The truncated model has a slightly better performance.

It seems the truncated cluster model profile can be used for further alignment and comparison. However, the un-truncated 20 clusters model profile is also useful since it is independent from spatial filter caused by AAV configurations. From potential more AAV variants in future study, it would be better to keep this un-truncated model as a general model in the TR together with truncated models.

For the truncation procedure proposed by companies, we see how to apply spatial filter on top of cluster combined model is not so clear. It would be better to give more detailed instructions in final TR.

Proposal 3 	Keep both un-truncated model profile and truncated model profile in TR for future study.

Proposal 4	Give more clear instruction on spatial filter application in truncation procedure in final TR.

#### AAV Configuration for comparison

Way Forward

Options:

- Option 1 (previously 1Y): with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

  - For interested companies: (M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only

- Option 3: with AAV with the following configuration

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

As the many simulation results showed, the AAV 1Y could achieve lower target SNR comparing to AAV3 due to more antenna elements are used. In real network, AAV is a basic configuration for AAS radio. But the issue here is the AAV configuration could be quite different based on different product in real network. There will be different antenna element numbers for different TRX branches with same number of total antenna elements, e.g., 128AE 64TRX (M, N, P, Ms, Ns) = (8, 8, 2, 2, 1), 128AE 32TRX (M, N, P, Ms, Ns) = (8, 8, 2, 4, 1), 128AE 16TRX (M, N, P, Ms, Ns) = (8, 8, 2, 8, 1), etc.

However, all AAV configurations would achieve similar performance curve after the SNR normalization for AAV. The impact might not be so critical, but the larger AAV would lead to worse SNR value after normalization which is not align with real network results (larger AAV should achieve lower target SNR in field). In that case, we propose focusing on AAV3 for all alignment and comparison with TDL based models. As for other AAV configurations, if companies could align normalization factor calculation method, it could easy to predict the target SNR after normalization and could be possible to directly compare SNR before normalization.

Proposal 5 	Keep focusing on AAV3 for further alignment and comparison with TDL based model.

Proposal 6	RAN4 consider include gain of AAV configuration to be more practical after companies could align with SNR normalization method.

#### Angles of Departure and Angles of Arrival

Agreement

For PDSCH comparison μϕ,desired =  μϕ,model

For PMI further discussion required.

For TR 38.753, RAN4 to explicitly specify angle wrapping rules.

For PMI tests, legacy test setup uses time variable beam steering matrices to have different angle of departure during whole test. The intention is to have better test coverage on multiple PMI conditions. The corresponding channel model profile is actually time variable either, but the benefit is the calculation might be easy when TDL model is used.

In current CDL based model, the desired departure/arrival angles are fixed. If we want to have similar test coverage, a set of channel model profiles with different departure/arrival angles might be needed. The tricky part here is that the corresponding derivation effort would be much higher than TDL model. How to configure a set of desired departure/arrival angles and make it possible to cover most of PMI conditions (e.g., histogram from follow PMI simulation can show different preference) is a big question. High effort on trial simulations and alignment would be expected.

However, if TE could support using multiple CDL models during PMI tests, it could be worthy to evaluate multiple CDL models. To reduce effort, it could be possible to only derive limited number of CDL models on top of current TDL model based PMI tests. Regarding the timeline of this study, this topic could be furtherly discussed in future release.

Much higher effort would be needed for CDL model based PMI test if similar test coverage as legacy TDL model based PMI test is assumed.

It could be possible to use small set of CDL models for PMI test to reduce effort if TE can support using multiple CDL models during tests.

#### SNR Normalisation with CDL

Way Forward

Assess results alignment at RAN4#115, FFS on the method to define normalisation method if required.

Encourage companies to provide normalisation factor at next meeting and align accordingly.

Agreement

Simulation results to be provided with AAV gain (including AE pattern) normalized, such that AAV gain is taken away as part of the channel normalization.

In previous meeting, there is no consensus on how to normalize SNR value based on simulations. One issue could be where we measure the power in whole simulation link. Generally, we agree with some companies’ argument that the long-term averaged signal power at pre-EQ part could be a fair comparison point. But there are still different implementations on SNR in companies’ simulator, e.g., stable signal power with variable noise power and variable signal power with stable noise power. In the former implementation, the pre-EQ power normalization might be suitable. For the latter implementation, consider normalization in antenna gain part would be more suitable and easier.

Since RAN4 demodulation work have to align the final SNR values for requirement, it might be possible to capture different normalization factors in model profile, and let users choose according to their implementations. This is just the workaround way for this SI. Companies can furtherly discuss how to capture it in the specification in the future once RAN4 agree to use CDL based model for the requirement.

It seems hard to align normalization calculation method due to different simulator implementations.

In our simulator, we use variable received signal power and stable noise power. In that case, we would use a gain factor to do normalization, see below table includes normalization factor for AAV 1Y and AAV 3. The gain ratio of AAV1Y to AAV3 is constant as 8.21dB and the gain ratio of 3GPP AE pattern to isotropic AE pattern is also constant as 0.72dB. The gain ratio of AAV1Y to isotropic AE is ~8.93dB.

Table 2.3-1 SNR Normalization factor derivation for 8TR 8 layers PDSCH

| Config. | SNR [dB] | A: AAV1Y + 3GPP AE | B: AAV3 + 3GPP AE | C: AAV3 + Isotropic AE | Gain of A to B [dB] | Gain of B to C [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Avg. received signal power | 15 | 0.155 | 0.0234 | 0.0198 | 8.21 | 0.72 |
|  | 30 | 3.892 | 0.588 | 0.498 | 8.21 | 0.72 |

The received gain ratio between different AAV configuration could be constant.

If companies have different simulator implementation and bring out different factor calculation methods and different scale factor, companies could check if the final SNR are aligned. If they can be aligned, TR could capture different calculation methods and corresponding factors for future reference anyway.

Proposal 7	If companies can’t get aligned on normalization factor calculation due to different simulator implementations, TR could capture these methods and corresponding factors for future reference.

## 2.4	Comparison of methodologies

#### Performance Metrics

Way Forward:

Channel Model candidates to be compared in two aspects:

- Performance Properties

- Channel Properties (captured in an annex)

Performance Properties include:

- [TPUT with follow precoding]

- [TPUT with fixed precoding]

- [TPUT with random precoding]

- [Metrics Statistics and Spread]

- [SNR Operating Point]

- [Convergence time]

Channel Properties include:

- [Eigenmodes and Eigenvectors]

- [Power Angular Distribution]

- [Time Coherence]

- [Frequency Coherence]

- [PMI Preference]

There are many comparison metrics are proposed in the previous meeting, but companies don’t have aligned view. We would like to bring our understanding on these metrics for further discussion.

We think the channel properties are important metrics when we compare CDL based model or TDL based model. For CDL based model, current models used for alignment come from TR38.901 which is maintained by RAN1 team for many years. We could trust the model can reflect the real network scenario at a certain level. And companies have delivered some analysis on CDL based models and field measurement results on channel properties. It could show the similarity between models and measurements. For TDL based models, it could be important to show such kind of analysis to prove they are aligned with field measurement and can be trusted.

The performance metrics could be considered from improvement perspective. For example, we decide to use one CDL based model regarding its practical channel properties, but its target SNR is very high which is hard for conformance tests, then we could modify some parameters of the model to have lower target SNR. We can’t deny the whole model derivation method just depending on its throughput. That’s why we suggest companies to be careful if only performance properties are focused.

As for the detailed items per properties, we think all candidates here are important. But the effort would be very high to analyze all of them, so companies could choose several items from performance or channel properties which might need less effort for comparison. For performance properties, the convergence time might need more effort and could be lower the priority, but other items can be obtained from companies results. For channel properties, maybe power angular distribution and PMI preference are more relevant to spatial characters which can be prioritized.

The channel properties are very important for comparison between current CDL based models and TDL based models.

The performance properties are more relevant to improvement metrics regarding to testability.

Proposal 8 	Use channel properties mainly for comparison between CDL and TDL based methodologies. Use performance properties for comparison within same methodology scope, e.g., not compare throughput between CDL and TDL based models.

# Conclusions

In the previous sections we made the following observations:

Observation 1	There is no obvious performance difference between wideband and sub-band for Type-I codebook in all alignment and comparison test cases.

Observation 2	Proposed TDL based models need to change beam configuration according to different test cases which is not practical.

Observation 3	The 8 layers PDSCH performances are very similar between un-truncated 20 cluster model and truncated 12 cluster model with same AAV configuration. The truncated model has a slightly better performance.

Observation 4	Much higher effort would be needed for CDL model based PMI test if similar test coverage as legacy TDL model based PMI test is assumed.

Observation 5	It could be possible to use small set of CDL models for PMI test to reduce effort if TE can support using multiple CDL models during tests.

Observation 6	It seems hard to align normalization calculation method due to different simulator implementations.

Observation 7	The received gain ratio between different AAV configuration could be constant.

_Toc197715121

Observation 8	The channel properties are very important for comparison between current CDL based models and TDL based models.

Observation 9	The performance properties are more relevant to improvement metrics regarding to testability.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Consider following steps for fixed PMI simulation alignment.

•	Step 1: Align PMI hypothesis including CSI-port mapping and PMI index content.

•	Step 2: Align PMI histograms from companies under high SNR or high normalized throughput range (>=80%).

•	Step 3: Choose the highest possibility PMI from aligned PMI histogram for performance alignment.

Proposal 2	Companies to check if one TDL based model can be used for 2/4/8 layers test cases.

_Toc197715129

Proposal 3 	Keep both un-truncated model profile and truncated model profile in TR for future study.

Proposal 4	Give more clear instruction on spatial filter application in truncation procedure in final TR.

Proposal 5 	Keep focusing on AAV3 for further alignment and comparison with TDL based model.

Proposal 6	RAN4 consider include gain of AAV configuration to be more practical after companies could align with SNR normalization method.

_Toc197715134

_Toc197715135

Proposal 7	If companies can’t get aligned on normalization factor calculation due to different simulator implementations, TR could capture these methods and corresponding factors for future reference.

Proposal 8 	Use channel properties mainly for comparison between CDL and TDL based methodologies. Use performance properties for comparison within same methodology scope, e.g., not compare throughput between CDL and TDL based models.

# References

[1] 	R4-2502378, Way Forward for [114][322] FS_NR_demod_SCM, Nokia, RAN4#114

[2]	R4-2504382, Simulation results for NR SCM for demodulation, Ericsson, RAN4#114bis

[3]	R4-2501787, Discussion on Rel-19 SCM methodology for demodulation, Ericsson, RAN4#114

[4]	R4-2501788, Simulation results for Rel-19 SCM for demodulation, Ericsson, RAN4#114

[5] 	R4-2501745, Measurement results for MIMO channels in band n78, BT, Ericsson, RAN4#114

[6]	R4-2419338, Further observations on the properties of MIMO channels in typical deployments, BT, Ericsson, RAN4#113

[7]	R4-2504431, PMI statistics for measured MIMO channels in band n78, BT, Ericsson, RAN4#114bis