3GPP TSG-RAN WG4 Meeting #115	R4-2507348

Malta, MT, 19th – 23rd May, 2025

Source:	Nokia

Title:	Discussion on gap association

Agenda item:	4.5.2

Document for:	Discussion

Introduction

In this paper we come back to a discussion at RAN4#114 related to the WI NR_MG_enh2 [1] on the term “gap association”.

In this contribution we analyze the usage of the term in TS 38.133 and provide a proposal to introduce a definition for the terms “gap association” and “gap association rule” for Rel-17 measurement gap types, which will then also provide a basis for Rel-18 measurement gap types, considered in Case 1 and Case 2.

Discussion

The discussion on gap association was triggered at RAN4 #114 when Case 2 requirements for Rel-18 were discussed.

In particular, CR [2] was agreed including the term “gap association”. It was noticed that a definition for the term “gap association” is missing in TS 38.133. A draft CR was then provided to RAN4#114 [3] for information, which introduces the definition of the terms “gap association” and “gap association rule” for Rel-17 measurement gap types.

When analyzing the usage of the term in TS 38.133, the following is observed.

- Clause 9 in Rel-17 of TS 38.133 includes the term “association” only twice, namely in clauses 9.1.5.2 and 9.1.5.2.2.

- Clause 9 in Rel-18 of TS 38.133 has 6 occurrences of the term “association” and 4 occurrences of the term “gap association”, namely in clauses 9.1.5.3 (1), 9.1.12.2 (2) related to concurrent gaps for Case 1 and 9.1.13.2 (1) related to concurrent gaps for Case 2.

- For Rel-17, there are only two occurrences of the term “association” in clause 9 of TS 38.133, whilst for Rel-18 there are 6 occurrences of the term “association” and 4 occurrences of the term “gap association” in clause 9 of TS 38.133.

Thus, definitions of the terms “gap association” and “gap association rule” should be added from Rel-17, where concurrent gaps, Pre-MG and NCSG gap types were added.

Therefore, it is proposed to add these definitions in clause 3.1.

Add following definitions for “gap association” and “gap association rule” to TS 38.133 clause 3.1 from Rel-17 to improve readability of the spec:

- Gap association: the association of a configured measurement object to a configured measurement gap.

- Gap association rule: the gap association rule for pre-configured measurement gap is defined in clause 9.1.7.2, for concurrent measurement gaps in clause 9.1.8.2 and for network controlled small gap in clause 9.1.9.2.

The accompanied CR [4] for Rel-17 adds these definitions and includes the gap association rule for Rel-17 measurement gap types in clause 9.1.7.2 for Pre-MG, in clause 9.1.8.2 for concurrent measurement gaps and in clause 9.1.9.2 for NCSG.

Conclusion

In this contribution we analyzed the usage of the term “gap association” in TS 38.133 and provide a proposal to introduce a definition for the terms “gap association” and “gap association rule” for Rel-17 measurement gap types, i.e. concurrent measurement gaps, Pre-MG and NCSG, which will then also provide a basis for Rel-18 measurement gap types, considered in Case 1 and Case 2.

As part of this discussion, the following observations and proposals were made:

- For Rel-17, there are only two occurrences of the term “association” in clause 9 of TS 38.133, whilst for Rel-18 there are 6 occurrences of the term “association” and 4 occurrences of the term “gap association” in clause 9 of TS 38.133.

- Add following definitions for “gap association” and “gap association rule” to TS 38.133 clause 3.1 from Rel-17 to improve readability of the spec:

- Gap association: the association of a configured measurement object to a configured measurement gap.

- Gap association rule: the gap association rule for pre-configured measurement gap is defined in clause 9.1.7.2, for concurrent measurement gaps in clause 9.1.8.2 and for network controlled small gap in clause 9.1.9.2.

The accompanied CR [4] adds these definitions and includes the gap association rule for Rel-17 measurement gap types in clause 9.1.7.2 for Pre-MG, in clause 9.1.8.2 for concurrent measurement gaps and in clause 9.1.9.2 for NCSG.

- References

- RP-232440, “Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, Intel, MediaTek

- R4-2502670, “(NR_MG_enh2-Core) CR on 38.133 MG enh2 on NCSGs“, Ericsson, Nokia, CMCC, OPPO, RAN4#114

- R4-2502686, “Draft CR38.133 Addition of definition for gap association”, Nokia, RAN4#114

- R4-2507349, “(NR_MG_enh-Core) CR38.133 Gap association for concurrent MGs, Pre-MG and NCSG”, Nokia, RAN4#115