3GPP TSG-RAN WG4 Meeting #115	R4-2507388

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	7.30.1

Source:	Nokia, Qualcomm

Title:	Rapporteur Inputs

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Information

# 1	Introduction

This document contains the following rapporteur’s input to this meeting:

- A list of XR agreements reached in RAN4 so far;

- The information for RAN1 and RAN2 can be found at

- R2-2503563, Rapporteur Inputs, Nokia, Qualcomm (Rapporteurs)

- R1-2504156, Rapporteur Inputs, Nokia, Qualcomm (WI rapporteurs)

# 2	List of Agreements

## 2.1	Scenarios for measurement skipping

### 2.1.1	L1, L3 and LTM

Agreement from RAN4 #112 R4-2414043

Issue 1-1: Workscope < Agreement>: - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed.

Agreement from RAN4 #114 R4-2502700

Issue 1-2: Types of measurements to be considered <Agreement>: - The following measurement types are considered for XR measurement skipping - Intra-frequency - Inter-frequency - LTE-inter- RAT (NR-EUTRAN FDD/TDD) - The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS.

Agreement from RAN4 #114 R4-2502700

Issue 1-3: L3 Measurements without gaps with scheduling restrictions Issue 1-4: L1 measurements with scheduling restrictions and procedures Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

Agreement from RAN4 #114-bis R4-2504911

## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - Requirements in clause 9.15 NR inter-frequency L1 measurement with gaps are updated for gap skipping.

### 2.1.2	Standalone, dual connectivity scenarios and frequency ranges

Agreement from RAN4 #112bis R4-2416864

Issue 1-1: Deployment scenarios < Agreement>: - Frequency ranges - Agree on FR1 and FR2-1

Agreement from RAN4 #113 R4-2420103

Issue 1-1: Deployment scenarios < Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID

Agreement from RAN4 #114bis R4-2504911

## Issue 1-1: Deployment scenarios < Agreement>: - Skipping requirements applies for NR-SA

## 2.2	Measurements with gaps

Agreement from RAN4 #113 R4-2420103

## Issue 2-1: General approach for defining requirements for measurements with gaps <Agreement>: - For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI.

Agreement from RAN4 #114-bis R4-2504911

# Topic #2: Measurements with gaps ## Issue 2-1: Types of measurement gaps In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. <Agreement>: - Gaps which shall not be cancelled - MUSIM - Positioning - For other gap types: - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19 - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows.

## 2.3	XR skipping requirements impact

Agreement from RAN4 #113 R4-2420103

## Issue 3-1: How to capture impact of measurement skipping <Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay

Agreement from RAN4 #114 R4-2502700

## Issue 3-1: Principles for measurement delay calculation <Agreement>: Follow NR-U-like approach for designing the formulas for measurement delay extension.

## 2.4	Time offset for measurement skipping processing

Agreement from RAN4 #113 R4-2420103

## Issue 4-1: Time offset requirement for measurement gap skipping <Agreement>: - Baseline feature 5 ms - Optional capability 3 ms, FFS if any condition is needed

Agreement from RAN4 #114-bis R4-2504911

# Topic #4: Time offset for measurement skipping processing ## Issue 4-1: Time offset requirement for measurement gap skipping <Agreement>: - Not define additional condition for the optional capability of 3 ms. ## Issue 4-2: Time offset requirement for cancelling skip indication <Conclusion>: The following is the common understanding in RAN4 RRM session: - The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where - X=5ms as baseline and X=3ms as optional UE capability. - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

## 2.5	Need/feasibility of UE assistance information

Agreement from RAN4 #114 R4-2502700

## Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.

Agreement from RAN4 #114-bis R4-2504911

# Topic #5: Need/feasibility of UE assistance information ## Issue 5-1: General on UAI <Agreement>: Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting. - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

## 2.6	List of impacted clauses and work split

Agreement from RAN4 #114 R4-2502700, was updated in RAN4 #114 bis

Agreement from RAN4 #114-bis R4-2504911

- Rapporteur comment: Inter-freq CR includes both clauses 9.3.4 and 9.3.5

# Topic 8 List of impacted clauses and worksplit Clause; Topic; Volunteer. 8.xr (new) Or 9.1; Time offset for DCI based measurement skipping; Nokia. 9.2.6; Intra-freq measurements with gaps; ZTE. 9.3.4; Inter-freq measurements with gaps; Ericsson. 9.4.2; Inter-RAT measurements E-UTRAN FDD measurements; Huawei. 9.4.3; Inter-RAT measurements E-UTRAN TDD measurements; Apple. 9.15.5; Inter-frequency L1-RSRP with MG; vivo. NOTE: The full list of clauses with potential impact due to XR skipping is found in the Ad-hoc minutes R4-2502580.

# 3	Workplan

Table 3-1: Workplan for the XR WI

| Date | RAN1 | RAN2 | RAN3 | RAN4 | RAN |
| --- | --- | --- | --- | --- | --- |
| 2024-02 | RAN1#116 Start discussion on a design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. |  |  |  |  |
| 2024-03 |  |  |  |  | RAN#103 Status Report (target of 2% corresponding to 0.5 TU spent over an initial allocation of 24.8) |
| 2024-04 | RAN1#116bis Continue the discussion on the design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Start agreeing on solution(s). | RAN2#125bis Endorse workplan Start the work on all 4 objectives except measurement gaps, for which RAN1 conclusions first need to be reached. For multi-modality and scheduling enhancements, start the study phase. | RAN3#123bis Start discussion on RAN3 related objectives on NR-DC, e.g. PDU set based handling, ECN marking, etc. Agree the DC bearer types to be supported. Designate responsible companies for BL Stage-2 & 3 CRs. |  |  |
| 2024-05 | RAN1#117 Continue developing the solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#126 Continue the work on all 3 objectives. | RAN3#124 Finalize the solution on required enhancements. Finalize Stage-2 CR(s) Agree the skeleton of Stage-3 CR(s). |  |  |
| 2024-06 |  |  |  |  | RAN#104 Status Report (target of 18% corresponding to 4.5 TUs spent over an initial allocation of 24.8) Take into account outcome of SA2 study if available. |
| 2024-08 | RAN1#118 Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#127 Continue the work on all 3 objectives. For multi-modality and scheduling enhancements, conclude the study phase and convey the outcome to RAN#105. | RAN3#125 Agree Stage-2 CR(s) and Stage-3 CR(s) | RAN4#112 Approval of workplan Initial discussion on scenarios for measurement skipping based on RAN1 conclusions. |  |
| 2024-09 |  |  |  |  | RAN#105 Status Report (target of 30% corresponding to 7.5 TUs spent over an initial allocation of 24.8) Checkpoints: 1. Multi-modality (RAN2) 2. Scheduling enhancements (RAN2) |
| 2024-10 | RAN1#118bis Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Check if RAN1 involvement related to objective on multi-modality is needed. | RAN2#127bis Check with RAN1 the progress on measurement gaps and start discussing in RAN2. Continue the work on UL scheduling enhancements according to the refined objective. Start the work on uplink congestion signalling. | RAN3#125bis Discuss multi-modality when handling incoming LS (no additional TU allocated to the new objective). | RAN4#112bis Conclusions on scenarios for measurement skipping based on RAN1 conclusions. Discussion on the need for UE assistance information regarding measurements occasions needed. |  |
| 2024-11 | RAN1#119 Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#128 Narrow down possible solutions. First version of Stage 2 CR | RAN3#126 Discuss LS on PDU Set Information Marking Support without QoS parameters, and usage of multi-modality information (no additional TU allocated to the new objective). | RAN4#113 Discussion on RRM requirements with measurement skipping. Discussion on the necessary UE assistance information. |  |
| 2024-12 |  |  |  |  | RAN#106 Status Report (target of 55% corresponding to 13.5 TUs spent over an initial allocation of 24.8) Checkpoint on multi-modality (including possible allocation of additional TUs for RAN3) |
| 2025-02 | RAN1#120 Finalize, if not already, the discussion on the design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Provide RAN1 input to RAN2 on RRC configuration parameters, UE capabilities, and Stage 2 TPs. | RAN2#129 Continue the work on the details of the agreed solutions. First versions of Stage 2 & 3 CR. | RAN3#127 Discuss LS on UL bit rate control, available data rate reporting and multi-modality awareness (no additional TU allocated to the new objective). | RAN4#114 Conclusion on RRM requirements with measurement skipping. Discussion on the need for UE assistance information regarding measurement skipping. Agreement of CR work split. |  |
| 2025-03 |  |  |  |  | RAN#107 Checkpoint on multi-modality (including possible allocation of additional TUs for RAN3) Status Report (target of 67% corresponding to 16.5 TUs spent over an initial allocation of 24.8) |
| 2025-04 | RAN1#120bis XR will not be discussed. | RAN2#129bis Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in running Stage 2 & 3 CRs. | RAN3#127bis Start discussion on RAN3 related objectives on uplink rate control, PDU set based QoS handling enhancement, TTNB and burst size, exposure of available data rate etc. Designate responsible companies for Stage-2 & 3 BL CRs. | RAN4#114bis Conclusion on the necessary UE assistance information. First Draft CRs for RRM core requirements (draft CRs for 38.133 expected) First big Draft CR. |  |
| 2025-05 | RAN1#121 | RAN2#130 Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in running Stage 2 & 3 CRs. | RAN3#128 Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in Stage 2 & 3 BL CRs. | RAN4#115 Discussion on remaining open issues for RRM core. |  |
| 2025-06 |  |  |  |  | RAN#108 Status Report (target of 91% corresponding to 22.5 TUs spent over an initial allocation of 24.8) |
| 2025-08 |  | RAN2#131 Finalise Stage 2 & 3 CRs Agree Stage 2 & 3 CRs | RAN3#129 Finalise Stage 2 & 3 CRs Agree Stage 2 & 3 CRs | RAN4#116 Discussion on remaining open issues for RRM core. Revision of CRs for RRM core requirements. Planning of RRM performance. Initial discussion on RRM test cases. |  |
| 2025-09 |  |  |  |  | RAN#109 Status Report (target of 100% corresponding to 24.8 TUs spent over an initial allocation of 24.8) CRs received for approval. |