3GPP TSG-RAN WG4 Meeting #115	R4-2507431

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	RRM performance requirements for beam management and positioning accuracy

Agenda item:	7.21.6

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM and Positioning use cases were continued in RAN4#114bis in the scope of Rel-19 WI NR_AIML_air ‎[1] based on the objectives agreed in RAN4#107. Agreements achieved during RAN4#114bis meeting are summarized in the WF ‎[2].

In this paper, we address RRM performance requirements for AI/ML enabled beam management and positioning use cases. We discuss following main aspects:

- Performance requirements for AI/ML BM

- Testing framework for AI/ML based Beam Management

- Performance requirements for AI/ML Positioning

Discussion

Performance requirements for AI/ML BM

Measurement error impact and the ground truth

During RAN4#112 and RAN4#112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements of RAN4 #113 are provided below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training (set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

Using the agreed evaluation methodology as detailed in R4-2505152 from the last RAN4#114bis meeting, we present the simulation results for KPI1 which is the Beam Id prediction accuracy. The results are provided for a subset of the candidate options: case 1, case 2a and case 3(a) where no error is considered in the ground truth. As of now, the evaluations for these cases are done for K = 1.

Description of the candidate options and the different alternatives for errors considered in the simulations are as follows:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth

- Case 2a: No error will be considered in training dataset and ground-truth. Error will be considered in model input during inference

  - Alt 1: Only baseband errors are included

  - Alt 2: Only RF errors are included

  - Alt 3: Both baseband errors and RF errors are included

- Case 3a: Error will be considered in training dataset, model input during inference BUT not in ground-truth

  - Alt 3: Both baseband errors and RF errors are included

Note that case 3(a) was derived from case3 by assuming no errors in ground-truth

Table 1: Evaluation of KPI1 for K=1

| Candidate Options | X = 1 | X = 2 | X = 3 |
| --- | --- | --- | --- |
| Case 1 | 0.84 | 0.88 | 0.91 |
| Case 2a: Alt 1 | 0.83 | 0.88 | 0.90 |
| Case 2a: Alt 2 | 0.74 | 0.80 | 0.84 |
| Case 2a: Alt 3 | 0.74 | 0.79 | 0.83 |
| Case 3a: Alt 3 | 0.71 | 0.78 | 0.82 |

Table 1 provides the simulation results with KPI1 for top-1 predicted beam. We observe that RF errors dominate the impact on KPI1 for Case 2a, while base-band error has little impact. Differing from case 2a with errors in the training dataset, case 3a shows comparable performance.

Performance metrics

Following agreement was reached in RAN4#114bis meeting:

Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values - Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Based on the latest agreements on the beam ID prediction KPIs and its discussion in our previous TDoc [1], we can conclude that the evaluation of success rate of correct prediction should consider other strong-enough top-K predicted beams, apart from the beam with maximum RSRP. Hence, we put forth the following proposals

The success rate of correct prediction can be computed as a weighted average (Sw) of the strongest N beams among the top-K predicted beams, where 2≤N≤K

- Weighted success rate, Sw=1Nt n=1Nti=1Nwi,n lRSRPbi,n≥RSRPbi,n-X

- Where,

- wi,n: the weights can be implementation specific.

- For example, we use wi,n=RSRPbi,nRSRPbi,ni=1NRSRPbi,nRSRPbi,n with RSRPs in linear scale

RSRPbi: RSRP of ith ranked beam, bi

RSRPb̂i: RSRP of ith ranked predicted beam, bi

X: allowed RSRP margin

l(C): Indicator function. It equals to 1 when the condition C is true, else 0

N: Number of strong-enough beams considered among top-K predicted beams

Nt: Number of inference instances

- NOTE: All the RSRPs and their ranking in the above are determined from the ground-truth values

RAN4 to discuss adaptation of Option 3 with the following changes:

- At least N strongest beams among the top-K predicted beams must be considered in the success rate evaluation, where N≥2

- RSRP of each of the strongest N predicted beams must be compared with the RSRP of its equal ranked genie-aided beam - X

Value(s) of X to be determined from simulations of the ongoing study

Testing framework for AI/ML based Beam Management

The testability issues for AI/ML based BM use case were highlighted by multiple companies during previous few RAN4 meetings but the discussions continued without any concrete conclusion on the topic. Below are the agreements/summary of the progress on this topic during the past RAN4 meetings which are documented in the corresponding WFs.

Agreements from RAN4#110 meeting Issue 2-3: Test setup feasibility for FR2 Companies are invited to provide further analysis on what the test setup should enable in terms of test environment Agreements from RAN4#113 meeting 2.3.2 Agreements in main session: Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. Agreements from RAN4#114 meeting 2.4 Testing for one sided models Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point Agreements from RAN4#114-bis meeting Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

Recent RAN4 agreements show that there is a need for a solution for the conformance testing of AI/ML based BM use case which not only be based on the existing test solutions but should also be AI/ML BM performance centric.

As there was not much progress on finalising the testing setup in the previous RAN4#114bis meeting, we reiterate some of our previous views along with providing our further views and solutions for the conformance testing of AI/ML based BM use case.

    - Test setup based on single or multiple AoA(s)

- For a while, RAN4 has been actively discussing whether the set up for the conformance testing of AI/ML based beam management use case should be based on single AoA or multiple AoAs. Undoubtedly, each of these test setups have their own pros and cons, which were discussed during the past RAN4 meetings. The main challenge is the emulation of Set A beams during the conformance testing to extract the ground truth which depends on the UE architecture and implementation. Also, RAN4 has agreed that Set A should have 32 beams for the conformance testing. So, the question is how to generate 32 beams inside the OTA test chamber during the conformance testing?

- Ideally, all the beams in Set A and/or Set B should be generated without the UE rotation as the similar scenario will be observed in the real deployments. However, based on the TE vendors’ input, only a limited number of beams with different AoAs (RX-PAS profiles) can be emulated inside the test chamber due to which multiple beams in Set A and/or Set B will arrive at the DUT from the same/similar direction(s).

We below provide two alternatives to emulate the Set A and Set B beams during the conformance testing, one of which is based on the concept of UE rotation while the other solution does not assume any kind of UE rotation.

      - Emulation of Set A and/or Set B beams without using UE rotation

- This solution provides a method, which is applicable for the multiple AoAs based test systems, to emulate the Set A and/or Set B beams without using the UE rotation. Considering a test chamber equipped with ‘M’ test probe antennas where each test probe antenna is contributing to form a definite Tx antenna pattern and/or Tx beam(s) which is also affected by the amount of Tx power corresponding to the signals applied to each test probe antenna. A particular Tx antenna pattern (containing at least a Tx beam) can be formed by selecting and activating any of the ‘N’ test probes out of the available ‘M’ probes, where 1≤N≤M, i.e.,

- MC1 Tx antenna patterns can be formed if only 1 probe is selected/activated.

- …

- MCM Tx antenna patterns can be formed if ‘M’ probes are selected/activated.

Hence, a test system with ‘M’ test probe antennas in the test chamber can emulate (MC1 + … +MCM) Tx antenna radiation patterns (or beams). For example, a practical test system equipped with 6 test probe antennas can emulate upto 6C1 + 6C2 + 6C3 + 6C4 + 6C5 + 6C6 = (6 + 15 + 20 + 15 + 6 + 1) = 63 Tx antenna radiation patterns (or beams). And, for a test system having 6 test probes, simultaneously activating at least 3 test probe antennas will result in the creation of (20+15+6+1) = 42 Tx antenna patterns which might be sufficient to emulate 32 Set A beams during the conformance testing.

Therefore, with this solution, the existing MIMO-OTA and/or multi-Rx test system(s) which are equipped with 6 test probes can emulate 32 or more Tx Beams (or Tx antenna patterns) with at least 3 simultaneously activated probes. This is also explained below with the help of few examples.

In example 1 (see Figure 2), 3 test probes (Probe 1, Probe 2 and Probe 6) are used to emulate a unique Tx antenna pattern by exploiting the constructive/destructive interference created by the multi-antenna/probe signals simultaneously transmitted by the Probe 1, Probe 2 and Probe 6. The resulting beam/antenna pattern also depends on the P_11, P_21 and P_61, which are the transmit powers of the Probe 1, Probe 2, and Probe 6, respectively.

- In example 2 (see Figure 3), 5 test probes (Probe 1, Probe 2, Probe 3, Probe 4 and Probe 5) are used to emulate a unique Tx antenna pattern by simultaneously transmitting the test signal by using the Probe 1, Probe 2, Probe 3, Probe 4 and Probe 5. The resulting beam/antenna pattern also depends on the P_12, P_22, P_32, P_42, and P_52, which are the transmit powers of the Probe 1, Probe 2, Probe 3, Probe 4 and Probe 5, respectively.

| Figure 2: Example 1 showing emulation of Antenna Pattern ‘1’ or Beam ‘1’ using 3 test probes. | Figure 3: Example 2 showing emulation of Antenna Pattern 2 or Beam ‘2’ using 5 test probes. |
| --- | --- |

- But, how to decide or select the required Tx power of the selected probes is FFS.

RAN4 to discuss and explore the feasibility of generating Set A and Set B beams, during the conformance testing of the AI/ML-BM use case, by selecting the number of probes and controlling the transmit power of each probe. How to decide or select the required Tx power of the selected probes is FFS.

It is also important to note that this solution may need a special beam codebook for the generation of Set A and Set B beams and UE needs to be (pre)-trained for such a codebook. To ensure that the UE is (pre)-trained for this type of Set A and Set B beam(s) generation during the conformance testing, a pre-alignment check may be needed before the exact start of the conformance test where the objective of the pre-alignment check is to ensure that the TE provided parameters (e.g., parameters corresponding to the beam codebook) and the DUT’s model/functionality/configuration’s current/preferred parameter (e.g., the parameters using which the training was performed) are in agreement with each other. This is needed to ensure that the test does not fail because of the (AI/ML) parameter(s) mismatch between the DUT (pre)-training and conformance testing.

RAN4 to discuss and decide the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

      - Emulation of Set A/Set B beams with UE rotation

In this solution, an alternative method to emulate the Set A and Set B beams in an iterative manner by using the concept of UE rotation is explained. This solution is applicable for both single AoA based test systems as well as for the multiple AoA based test systems.

To enable the beam sweeping of Set A and/or Set B beams in an iterative manner, a combination of the available test probes and different possible UE rotations is exploited where single or multiple test probe(s) are available. One benefit of using the UE rotation is that it enables the test system to simulate the AoA angles outside the angular range of test probes (from UE’s point of view) and hence the test system can generate any possible AoA beam angles.

The proposed solution assumes: (1) OTA test chamber is equipped with single or multiple test probe(s), (2) any of the CDL/simplified CDL/TDL/AWGN channel model is used during the test, (3) One beam is generated by using only one of the available test probe(s) during the conformance test, (4) or one beam is generated by using a probe array for any possible AoA angles. The solution proposes the DUT to perform beam sweeping in multiple iterations and to measure the RSRP of the corresponding beams in Set A and/or Set B during the conformance test for which a new test command is to be defined and to be sent to the DUT. After the DUT receives this test command from the TE and configures itself to perform beam sweeping in multiple iterations, one of the possible UE orientations is selected and TE selects the power level (typically different) for each of the available test probes to form different beams towards the UE. It is worth mentioning that in this solution, only one test probe is active at any given point in time and represents one Tx beam. This implies that with ‘P’ test probes in the OTA testing (anechoic) chamber, using one probe at a time for one beam will implicitly mean emulating ‘P’ different Tx beams (one at a time) for a given UE orientation. Thereafter, if more than ‘P’ Tx beams are required, TE can change the UE orientation but uses the same set of available test probes (with the Tx power of each test probe to be held constant as previously selected or to be set differently) to emulate another set of ‘P’ Tx beams. In this fashion, multiple UE orientations can be selected depending on the number of Tx beams required. The above solution is valid for emulating all Set A and/or Set B beams in an iterative manner to perform beam sweeping during the conformance testing.

Figure : An example of the proposed test system with UE rotation.

Given the Set A and/or Set B beams can be generated iteratively during the conformance test as proposed above, DUT can measure the L1-RSRP of each beam in Set B and provides it as input to its AI/ML model for Set A beam prediction.

RAN4 to discuss and explore the feasibility of performing beam sweeping using UE rotation in multiple iterations during the conformance testing of AI/ML based BM use case.

    - Conformance testing without transmitting Set A beams during the test

- It is a common understanding among companies participating in RAN4 that the TE requires the ground-truth to compare the DUT’s predictions with the corresponding ground truth to decide whether the DUT has passed or failed the test/test iteration. As the ground truth depends on DUT’s architecture and implementation, TE first needs to generate each Set A beam during the conformance test after which DUT will measure and report the ground truth (e.g., L1-RSRP) measurement corresponding to each transmitted Set A beam back to the TE. However, the emulation of Set A beams during the AI/ML based BM conformance test results in several issues:

- As the number of Set A beams to be emulated during the conformance test is 32, many test probes are required which is neither supported by the existing test systems nor preferred by the TE vendors due to the increased cost and complexity associated with the enhancement of existing system.

- As the DUT itself is reporting the ground truth back to the TE, it is possible that DUT either manipulates the reported ground truth values or the reported prediction values to pass the test. This will raise concern about UE cheating during the test.

- If using DUT rotations during the conformance test for measuring the ground truth of Set A beams, multiple DUT rotations will be required resulting in a large testing time.

- So, an important question to ask is: Can ground truth be generated at the TE without transmitting Set A beams? If yes, how? In the following, we present our views on this and present a solution.

We know that a practical OTA testing chamber has only a limited number of test probe antennas (e.g., 4, 6, or 8) using which the test signals will be transmitted to the DUT. The common understanding for the AI/ML-enabled beam management conformance testing is that both Set A beams as well as Set B beams need to be transmitted to the DUT during the conformance test for ground truth extraction and AI/ML prediction, respectively. As the number of available test probe antennas at the testing chamber is limited, all Set A and Set B beams need to be transmitted using those limited test probe antennas by reusing each test probe antenna for transmitting more than one beam. Therefore, if the characteristics of the OTA test chamber (e.g., path loss from each test probe antenna to the DUT) and the UE Rx antenna pattern and/or Rx beamforming gain towards the direction of test probe antennas can be obtained either prior to the test or during the starting phase of the test, UE’s ground truth for the Set A beams can be generated by using the obtained UE Rx antenna pattern and/or Rx beamforming gain knowledge and OTA characteristics knowledge at the TE, and without actually transmitting the Set A beams during the conformance testing. The detailed derivation is shown below for the case where only one test probe antenna is used to form a Tx beam towards the UE. The below shown derivation can easily be extended for the case where multiple test probe antenna are used to form a Tx beam towards the UE.

Let the test signal#‘i’ contains the unit energy symbol xi whose transmit power is ρi Watts, and the channel implemented by the channel emulator (CE) to transmit ρixi be a scalar hi due to single activated probe antenna. Hence, the baseband equivalent of the signal transmitted by the test probe antenna#‘i’ can be written as ρihixi which then propagates through the over the air (OTA) wireless channel of the testing chamber and is received by the DUT’s antenna/antenna array. The baseband equivalent of the signal received at the DUT is given by

yi=wihOTA,iρihixi+ni

where wi captures the impact of UE Rx antenna pattern and/or Rx beamforming gain in the direction of test probe#‘i’, hOTA denotes the wireless channel of the testing chamber, and ni represents the additive noise added by the UE hardware. Hence, the RSRP of the received signal yi can be approximated as

RSRPTSi= Eyi2= EwihOTA,iρihixi+ni2≈wihOTA,i2×ρihi2xi2+σ2

where E[.] denotes the expectation operation and is performed by taking the linear average over the power distribution (in [W]) over all resource elements that carry the RSs configured by the network for RSRP measurements, and σ2 denotes the average noise power or noise variance. It is worth noting that in the above mathematical expression for RSRPTSi, the only unknowns parameters for TE are wi and hOTA,i as TE knows the test parameters it has selected, i.e., ρi, hi, xi for transmitting the test signal#‘i’, and noise power/variance can be estimated by the DUT using the already known techniques. Therefore, DUT’s ground truth can be generated by the TE if the information wihOTA,i2 is available at the TE for all possible/relevant test directions ‘i’. One possible way of obtaining this information at the TE is to take the ratio of RSRPTSi and TPTSi where TPTSi is the transmit power of the test signal#‘i’ transmitted by the test probe antenna ‘i’ and can be written as TPTSi= ρihi2xi2. Therefore, the ratio is given by

γi≝RSRPTSiTPTSi=wihOTA,i2×ρihi2xi2+σ2ρihi2xi2= wihOTA,i2+σ2ρihi2xi2≈wihOTA,i2

as the term σ2ρihi2xi2 will be very small due to high ρi and small σ2.

If the test probe antennas have a directional gain, it can be assumed that the joint impact of test probe antenna and characteristics of the OTA testing chamber for test probe antenna#‘i’ is captured in hOTA,i. The DUT will be configured to also report the information of Rx beam (e.g., Rx beam ID) it has used to calculate the RSRPTSi which is required to generate the ground truth at the TE without actually transmitting Set A beams during the test.

Generation of UE side ground truth at the TE for Set A beams using the ratio γi

The TE uses the obtained wihOTA,i2 to generate the UE side ground truth as follows:

For the Set A Tx beam corresponding to the RS id#‘m’ having a transmit power of ρm, the corresponding test signal (RS) is being mapped to test probe antenna#‘j’, where j = (‘m’ modulo ‘t’), t is the max. number of test probe antennas in the test chamber to be used for the conformance test. If the channel model to be used is hj, the TE already knows the test probe information, hj and ρm. Therefore, the UE side ground truth for this Set A beam can be generated by the TE without actually transmitting this beam as:

RSRPTSm,j≈ wjhOTA,j2×ρmhj2xm2+σ2

In this way, the ground truth for all Set A beams can be generated at the TE without actually transmitting them.

Configuration for measuring Set B beams for consistent prediction of top-K Set A beams

As the ground truth was generated by the TE, it needs to configure the DUT for measuring Set B beams for the consistent prediction of top-K Set A beams based on the ground truth generated by the TE. For this, TE needs to indicate the DUT that when a Set B beam is transmitted by the test probe antenna#‘l’, the DUT must use the antenna pattern and/or UE Rx beamforming gain which results in wlhOTA,l2 being the overall impact of OTA testing chamber and UE hardware for the corresponding Set B beam.

    - Generalization Aspects

In the RAN4#114-bis meeting, the following agreement was achieved on the generalization aspects.

Relation between generalization testing and additional conditions signalling Agreement: - There shall be consistency between applicable conditions signalling and testing - No test will be defined that implies a change of network condition or associated ID during the test. Non-static scenarios Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario

The above agreements show that the default assumption is to use static scenarios during the test and non-static scenarios can be used on need basis. To test the generalization, RAN4 may define the test having multiple sub-tests with each with a different static condition, i.e., TE configures the same specified UE configuration for AI/ML BM in each of the test where the same UE configuration is tested under different scenarios (in different subtests) to verify the UE model’s capability to generalize.

In the tables below, we attempt to capture different parameters that may be needed to define different generalization scenarios for AI/ML enabled BM use case based on set of parameters/configurations to both DUT/TE. For identified scenarios in AI/ML enabled BM-Case1 and BM-Case2, the minimum level of performance may need to be defined. Similarly acceptable tolerance margin for other scenarios may also need to be agreed, potentially based on simulation results.

Table 1: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR (Deployment Scenario) | Good / Bad Radio conditions |
| Propagation Model | TDL/CDL |

For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define the generalization test which consists of multiple subtests each associated with the testing of different  static scenarios based on parameters listed in Table 1.

In order to address the testability issue of generalization aspects of AI/ML BM use case, we propose a new metric to indicate the generalization capabilities of an AI/ML functionality, which may be referred as “generalization score”. This may be used to verify/check whether the BM functionality generalize well or not. As an example, if there are 3 scenarios of interest, any one scenario can be the baseline setup and the remaining 2 scenarios will then be used as other set ups to test the generalization aspects. The generalization score can be calculated as a function of individual scenario verdicts, e.g., as simple average or weighted average of the individual generalization verdict of each Scenario ‘j’ (i.e., Scenario ‘1’ to Scenario ‘N), where the verdict of Scenario ‘j’ = 1 if the functionality generalizes to Scenario ‘j’ else it is equal to 0.

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

RAN4 to consider designing a new metric to verify the generalization performance of the AI/ML BM functionality in different (static) scenarios.

Performance requirements for AI/ML Positioning

Performance requirements in Case 1

Positioning Accuracy Requirements

In RAN4#110bis, it has been agreed on the testability and interoperability issue for AIML positioning.

Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme(if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced

In RAN4#112bis, it has been agreed on the testability and interoperability issue for AIML positioning while no agreements on RRM core requirement were achieved.

Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible

In Case 1 for direct AI/ML positioning, the output of the AI/ML functionality is UE position. As a metric for positioning requirements and tests, we propose the positioning accuracy metric as follows over the selected test positions:

- At each test position, the following steps will be followed:

  - Get the output from AI/ML functionality inference with the input of radio measurements,

  - Compute a positioning error, e.g., absolute error value between estimated position and true position, and

  - Confirm that the positioning error is within the acceptable range (i.e., positioning accuracy requirements).

For instance, positioning accuracy requirements are already specified for NR A-GNSS where the minimum performance requirements are defined in TS 37.571-1 Clause 13.3. Therefore, a similar approach should be envisaged in the context of AI/ML-enabled direct positioning.

RAN4 to use positioning accuracy as the performance metric in Case 1.

Additionally, in the performance requirements in NR positioning, it has been assumed that AWGN is used in test cases. Any performance requirements on AIML positioning may also have the same assumption the NR positioning.

[legacy assumption is AWGN + among subset of channels, we can select AWGN + at least in AWGN, AIML is better than legacy + AWGN is an easy case for comparison]

RAN4 to assume an AWGN channel in performance requirements if any applicable scenario exists in Cases 1 and 3.

Requirements in Case 3a/3b

Report Mapping Aspects in Case 3b

In RAN4#112bis, it has been agreed on the testability and interoperability issue for AIML positioning [9] while no agreements on RRM core requirement were achieved.

Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.

In RAN4#114, it has been agreed on the testability and interoperability issue for AIML positioning.

Issue 3-4: Report mapping for UL SRS-RSRP Agreement: Reuse the report mapping for UL SRS-RSRP to Case 3a

In RAN4#114bis, it has been agreed on the testability and interoperability issue for AIML positioning.

Issue 3-3: Rx-Tx time difference for case 3a Agreement: - Introduce gNB Rx-Tx time difference reporting accuracy requirements - Reuse legacy accuracy requirements Issue 3-4: Report mapping for gNB Rx-Tx time difference Agreement: - Reuse the existing report mapping for gNB Rx-Tx time difference measurements

Based on the agreements above, we discuss the report mapping aspects in Case 3b. In particular, it is agreed that UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b. Also, RAN4 agreed to reuse the report mapping for UL SRS-RSRP in Case 3a. Moreover, RAN4 agreed to reuse the legacy requirements of gNB Rx-Tx time difference reporting in Case 3a. Since UL SRS-RSRP measurement and gNB Rx-Tx time difference can be reported in Case 3b, it is needed to clarify whether to define the requirements for UL SRS-RSRP and gNB Rx-Tx time difference in Case 3b.

RAN4 to reuse the report mapping for UL SRS-RSRP to Case 3b.

RAN4 to introduce gNB Rx-Tx time difference reporting accuracy requirements (i.e., path-based measurements) by reusing the legacy accuracy requirements in Case 3b.

Agreement (RAN1#120) For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. Agreement (RAN1#120bis) For Rel-19 AI/ML based positioning, for Case 3b, “FFS: whether transmit offset from gNB to LMF” in RAN1#119 agreement is resolved by: - No offset is transmitted from gNB to LMF

In RAN1#120 meeting, an agreement has been made for the measurements in AI/ML based positioning Case 3b. There are two channel measurement types (A) or type (B). Type (A) is a path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), and type (B) is Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). Therefore, the descriptive information of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. In the channel measurement report includes the descriptive information measurement type, e.g., type (A) path-based measurement or type (B) Rel-19 enhanced measurement. Also, the report includes the measurement parameters, e.g., the measurement parameters such as the number of additional paths k in type (A) or Nt' and k in type (B). With two defined measurement types, additional benefits can be achieved with AIML positioning. For instance, if UE can use adaptive reporting by taking advantage of LMF’s signaling on the measurement configuration/indication on how to select a suitable reporting type based on the quality of a received reference signal.

In Case 3b, RAN1 also concludes that, if the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that the gNB cannot provide measurement with the requested measurement type. Therefore, the gNB does not respond to LMF with a measurement report where the measurement type is different from requested. Moreover, in RAN1#120bis, it is agreed that, for channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19.

When the RAN1 agreement has two measurement types, type (A) measurement does not involve an enhancement and existing specifications are applicable. For instance, type (A) path-based measurements can be reported by similarly using the reporting method of PRS-RSRP measurements. Unlike type (A) measurements, Rel-19 enhanced measurement, sample-based type (B) measurements may need the requirements specified in RAN4. In type (B) case, each report may include the multiple measured values of the samples, and, therefore, reporting overhead increases with Nt'. If the samples have the similar values, each measurement report will include the duplicated values of multiple samples. Therefore, it is possible to consider how to reduce the reporting overhead by defining a report mapping table. For instance, a report can include a part of measured sample values if those partial values can represent the overall measurement of all samples. Therefore, RAN4 can study how to define the reporting requirements for the new measurement types (A) and (B).

RAN4 to study feasibility of reusing the legacy performance requirements for the sample-based measurement, and how to specify separate performance requirements for sample-based measurement if reusing is not feasible in Case 3b.

Requirements on ground truth accuracy in data collection

In RAN1#116-bis meeting, training data collection of AI/ML based positioning has been discussed to identify the collect ed data samples as follows.

Agreement(RAN1#116bis) For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement(RAN1#120) - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above. Agreement(RAN1#120bis) For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

Also, an agreement is made in RAN1#120 on the label and quality indicator of label.

Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. · From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

In Part B, data collection with ground-truth dataset is essential for training. In particular, the training data needs to include both the ground-truth UE location and measurements associated with the UE’s location. To enhance the accuracy of AIML positioning, it is important to have accurate ground-truth information for positioning labelling. When collecting ground-truth information from the UE side, a high accuracy positioning information needs to be acquired by the UE. If UE is unable to acquire a high accuracy ground-truth information, the network/testing equipment may assist UE to acquire a high accuracy ground-truth positioning label. For instance, UE may obtain positioning labels from multiple sources e.g., non-NR and/or NR RAT-dependent positioning methods. Therefore, the degree of accuracy on ground-truth positioning label can be defined as a quality indicator. As captured in RAN1’s agreement, if the existing IE is used with a geographic shape in TS 23.032, the uncertainty/confidence on location estimate can be used as a quality indicator of the label. In this case, the legacy positioning does not specify a requirement on the quality indicator. Also, the ground-truth accuracy can be ensured by the definition on labels. Hence, a requirement on training data collection is not required in Case 3a/3b.

RAN4 to not specify a requirement on training data collection of Part B in Case 3a/3b.

Conclusion

In this paper we share our views on RRM performance requirements for AI/ML enabled beam management and positioning use cases. Specifically, we cover following aspects:

- Performance requirements for AI/ML BM

- Testing framework for AI/ML based Beam Management

- Performance requirements for AI/ML Positioning

The following Proposals were made for AI/ML BM use case:

Proposal 1: The success rate of correct prediction can be computed as a weighted average (S_w) of the strongest N beams among the top-K predicted beams, where 2≤N≤K

- Weighted success rate, Sw=1Nt n=1Nti=1Nwi,n lRSRPbi,n≥RSRPbi,n-X

Proposal 2: RAN4 to discuss adaptation of Option 3 with the following changes:

- At least N strongest beams among the top-K predicted beams must be considered in the success rate evaluation, where N≥2

- RSRP of each of the strongest N predicted beams must be compared with the RSRP of its equal ranked genie-aided beam - X

Proposal 3: Value(s) of X to be determined from simulations of the ongoing study

Proposal 4: RAN4 to discuss and explore the feasibility of generating Set A and Set B beams, during the conformance testing of the AI/ML-BM use case, by selecting the number of probes and controlling the transmit power of each probe. How to decide or select the required Tx power of the selected probes is FFS.

Proposal 5: RAN4 to discuss and decide the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

Proposal 6: RAN4 to discuss and explore the feasibility of performing beam sweeping using UE rotation in multiple iterations during the conformance testing of AI/ML based BM use case.

Proposal 7: For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define the generalization test which consists of multiple subtests each associated with the testing of different static scenarios based on parameters listed in Table below.

Table: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR (Deployment Scenario) | Good / Bad Radio conditions |
| Propagation Model | TDL/CDL |

Proposal 8: RAN4 to consider designing a new metric to verify the generalization performance of the AI/ML BM functionality in different (static) scenarios.

The following Proposals were made for AI/ML Positioning use case:

Proposal 9: RAN4 to use positioning accuracy as the performance metric in Case 1.

Proposal 10: RAN4 to assume an AWGN channel in performance requirements if any applicable scenario exists in Cases 1 and 3.

Proposal 11: RAN4 to reuse the report mapping for UL SRS-RSRP to Case 3b.

Proposal 12: RAN4 to introduce gNB Rx-Tx time difference reporting accuracy requirements (i.e., path-based measurements) by reusing the legacy accuracy requirements in Case 3b.

Proposal 13: RAN4 to study feasibility of reusing the legacy performance requirements for the sample-based measurement, and how to specify separate performance requirements for sample-based measurement if reusing is not feasible in Case 3b.

Proposal 14: RAN4 to not specify a requirement on training data collection of Part B in Case 3a/3b.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2505105, “WF on the requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#114bis meeting, 7th – 11th April 2025, Wuhan, China.