3GPP TSG-RAN WG4 Meeting #115                                      R4-2507477

St Julian’s, Malta, May 19th – May 23rd, 2025

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.755 on RF performance

Agenda Item:	7.5.4

Document for:	Discussion

Introduction

This contribution proposes the text proposal in the TR 38.755 to capture the RF performance evaluation.

Text Proposal

<<Start of Change 1 for TR 38.755>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”.

[3] 	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4] 	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5] 	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[6] 	R4-2413339 Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, RAN4#112, Nokia

[7] 	R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112bis, Nokia

[8] 	RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

[9] 	R4-2411691 On general aspects of fragmented carriers, RAN4#112, Huawei

[10] 	RP-241269, On Fragmented Carriers in Rel-19, Apple

[11] 	RP-241361, Recommended WF on Fragmented Carriers, MediaTek

[12] 	R4-2417201 WF on NR FR1 DL fragmented carriers study, RAN4#112bis

[13] 	R4-2413031 On architecture options for fragmented spectrum reception, RAN4#112, Skyworks

[14] 	R4-2416138 Discussion on UE Rx chain architecture and network assumption, RAN4#112bis, MediaTek

[15] 	R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

[16] 	R4-2415804 On methods for reducing the number of UE Rx chain, RAN4#112bis, Huawei

[17]	R4-2503479 TP to TR 38.755 on Clause 5, RAN4#114bis, CATT

[18]	R4-2503558 TP for TR38.755: Company C evaluation results, RAN4#114bis, Qualcomm

[19]	R4-2504452 TP on UE RF requirements impact for fragmented carriers, RAN4#114bis, Apple

[20]		R4-2504214 Further discussion of impacts on UE RF requirements and DL performance impacts, RAN4#114bis, MediaTek

[21]	R4-2504129 General aspects of Fragmented CA, RAN4#114bis, Nokia

[x]	R4-2504129 General aspects of Fragmented CA, RAN4#114bis, Nokia

- 7.X	RF requirements evaluation from company C

The performance evaluation in this clause is generated based on agreed link budget template in [15]. The evaluation covers REFSENS, ACS and IBB requirements.

- 7.X.1	Band n3

The REFSENS performance of Single Rx RF Chain will be impacted by the Tx leakage when the UL signal is close to the DL carrier as shown in below Figure 2.1-1.

Figure 7.X.1-1 REFSENS on band n3 with Tx leakage

Taking band n3 as an example, the estimated SNR on PCC and SCC could be derived as in Table 7.X.1-1.

Table 7.X.1-1. Link budget of Self-interference for 1st configuration on band n3

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise (12 bit): -74.0dBFs (6.02N+1.76) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to -6dBFs | Tx leakage mapping to -12dBFS |
| out | 27 | - | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - | - |
| Rx Antenna input | - | -97 | -92.3 | - | - | - |
| FE loss | - | 4 | 4 | - | - | - |
| Duplexer output | -23 | -101 | -96.3 | - | - | - |
| LPF attenuation | 12.8 | 0.5 | 0.5 | - | - | - |
| LPF output (dBm) | -35.8 | -101.5 | -96.8 | - | - | - |
| Noise floor for basic REFSENS | - | - | - | -100.5 | - | - |
| NF increase due to AGC adjustment | - | - | - | 5 | - | - |
| Noise floor | - | - | - | -95.5 | - | - |
| In-band noise (Derived based on MSD and SNR = -1dB) | - | - | -97.6 | - | - | - |
| ADC output (dBm) | - | - | - | - | -103.8 | -97.8 |
| SNR after ADC (dB) | - | - | - | - | -6.6(PCC) | -8.0(PCC) |
|  |  |  |  |  | -4.8(SCC) | -5.7(SCC) |

Table 7.X.1-2 summaries the evaluation using different assumption of the key parameters. To achieve the -1dB demodulation threshold, extra 3.1~7.0dB relaxation would be required on PCC, and 1.3~4.7dB relaxation is required on SCC.

Table 7.X.1-2 Estimated SNR on band n3 with a strong Tx leakage

|  |  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- |
| Duplexer isolation @Tx Fc |  | 50 | 50 | 50 | 50 |
| LPF attenuation @Tx Fc |  | 12 | 12 | 12 | 12 |
| NF increase due to AGC adjustment |  | 5 | 5 | 2 | 2 |
| ADC (12bit) |  | -12dBFS | -6dBFS | -12dBFS | -6dBFS |
| SNR (dB) | REFSENS (PCC) | -8.0 | -6.6 | -6.4 | -4.1 |
|  | REFSENS (SCC) | -5.7 | -4.8 | -3.6 | -2.3 |

Observation 1: When the UL signal is close to DL carrier, the REFSENS performance is impacted by the Tx leakage. The estimated SNR varies with different assumptions of key parameters, including the duplexer isolation @Tx, the LPF attenuation @Tx, the NF increase due to AGC adjustment and the ADC setting.

Observation 2: On band n3, extra 4.6~7.0dB relaxation would be required on PCC, and 2.7~3.7dB relaxation is required on SCC to achieve the -1dB demodulation threshold with the existence of a strong Tx leakage.

Using the similar link budget method, the SNR of ACS/IBB cases with parameters of Case 4 in Table 7.X.1-2 are summarized as below Table 7.X.1-3.

Table 7.X.1-3	SNR for ACS/IBB with fully shared RF chain

|  | ACS case 1 | ACS case 2 | IBB case 1 | IBB case 2 |
| --- | --- | --- | --- | --- |
| PCC | -1.4 | -1.0 | -3.2 | -3.2 |
| SCC | 3.0 | 3.7 | 0.4 | 0.4 |

The Tx leakage has impact on RF performance especially when the PCC is located close to the DL spectrum. If the PCC and SCC are swapped, there will be less RF performance loss. Table 7.X.1-4 summarises the estimated SNR. No extra relaxation is needed.

Table 7.X.1-4 Estimated SNR on band n3 when PCC and SCC are swapped

| Parameters |  | Value |
| --- | --- | --- |
| Duplexer isolation @Tx Fc |  | 50 |
| LPF attenuation @Tx Fc |  | 34.9 |
| NF increase due to AGC adjustment |  | 0 |
| ADC (12bit) |  | -6dBFS |
| SNR (dB) | REFSENS (PCC) | -1.0 |
|  | REFSENS (SCC) | -1.0 |
|  | ACS Case 1 (PCC) | -1.0 |
|  | ACS Case 1 (SCC) | -1.0 |
|  | ACS Case 2 (PCC) | -1.0 |
|  | ACS Case 2 (SCC) | -1.0 |
|  | IBB Case 1 (PCC) | -1.0 |
|  | IBB Case 1 (SCC) | -1.0 |
|  | IBB Case 2 (PCC) | -1.0 |
|  | IBB Case 2 (SCC) | -1.0 |

As a summary, the performance on band n3 is captured in below Table 7.X.1-5.

Table 7.X.1-5 Estimated SNR on band n3

|  | band | PCC bandwidth | SCC bandwidth | Wgap | PCC SCC Swap | LPF antennuation @Tx leakage | Tx leakage post LPF | PCC @ Rx antenna | SCC @ Rx antenna | noisefloor | NF increase comparing with separate RF chain | coChanNoise SCC | coChanNoise PCC | ADCnoise | SNR PCC | SNR SCC | MSD PCC | MSD SCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS | 3 | 5 | 5 | 65 | 0 | -12.8 | -35.8 | -101.5 | -101.5 | -100.5 | 5.0 | -97.6 | N/A | -103.8 | -6.6 | -8.5 | 5.6 | 7.5 |
| ACS case 1 | 3 | 5 | 5 | 65 | 0 | -12.8 | -35.8 | -87.5 | -87.5 | -100.5 | 5.0 | -97.6 | -86.7 | -103.8 | -1.4 | -1.7 | 0.4 | 0.7 |
| ACS case 2 | 3 | 5 | 5 | 65 | 0 | -12.8 | -35.8 | -61.0 | -61.0 | -100.5 | 12.0 | -97.6 | -60.0 | -96.6 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 3 | 5 | 5 | 65 | 0 | -12.8 | -35.8 | -95.5 | -95.5 | -100.5 | 5.0 | -97.6 | -95.8 | -103.8 | -3.2 | -4.3 | 2.2 | 3.3 |
| IBB case 2 | 3 | 5 | 5 | 65 | 0 | -12.8 | -35.8 | -95.5 | -95.5 | -100.5 | 5.0 | -97.6 | -95.8 | -103.6 | -3.2 | -4.3 | 2.2 | 3.3 |
| REFSENS | 3 | 5 | 5 | 65 | 1 | -35.0 | -58.0 | -101.5 | -101.5 | -100.5 | 0.0 | N/A | N/A | -126.0 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 3 | 5 | 5 | 65 | 1 | -35.0 | -58.0 | -87.5 | -87.5 | -100.5 | 0.0 | N/A | -86.7 | -125.6 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 2 | 3 | 5 | 5 | 65 | 1 | -35.0 | -58.0 | -61.0 | -61.0 | -100.5 | 12.0 | N/A | -60.0 | -97.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 3 | 5 | 5 | 65 | 1 | -35.0 | -58.0 | -95.5 | -95.5 | -100.5 | 0.0 | N/A | -95.8 | -124.0 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 2 | 3 | 5 | 5 | 65 | 1 | -35.0 | -58.0 | -95.5 | -95.5 | -100.5 | 0.0 | N/A | -95.8 | -116.1 | -1.0 | -1.0 | 0.0 | 0.0 |
| REFSENS | 3 | 5 | 5 | 45 | 0 | -25.6 | -48.6 | -101.5 | -101.5 | -100.5 | 0.0 | N/A | N/A | -116.6 | -1.1 | -1.1 | 0.1 | 0.1 |
| ACS case 1 | 3 | 5 | 5 | 45 | 0 | -25.6 | -48.6 | -87.5 | -87.5 | -100.5 | 0.0 | N/A | -86.7 | -116.6 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 2 | 3 | 5 | 5 | 45 | 0 | -25.6 | -48.6 | -61.0 | -61.0 | -100.5 | 12.0 | N/A | -60.0 | -97.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 3 | 5 | 5 | 45 | 0 | -25.6 | -48.6 | -95.5 | -95.5 | -100.5 | 0.0 | N/A | -95.8 | -116.4 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 2 | 3 | 5 | 5 | 45 | 0 | -25.6 | -48.6 | -95.5 | -95.5 | -100.5 | 0.0 | N/A | -95.8 | -113.6 | -1.1 | -1.1 | 0.1 | 0.1 |
| REFSENS | 3 | 5 | 5 | 45 | 1 | -42.8 | -65.8 | -101.5 | -101.5 | -100.5 | 0.0 | N/A | N/A | -133.8 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 3 | 5 | 5 | 45 | 1 | -42.8 | -65.8 | -87.5 | -87.5 | -100.5 | 0.0 | N/A | -86.7 | -131.9 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 2 | 3 | 5 | 5 | 45 | 1 | -42.8 | -65.8 | -61.0 | -61.0 | -100.5 | 12.0 | N/A | -60.0 | -97.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 3 | 5 | 5 | 45 | 1 | -42.8 | -65.8 | -95.5 | -95.5 | -100.5 | 0.0 | N/A | -95.8 | -127.4 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 2 | 3 | 5 | 5 | 45 | 1 | -42.8 | -65.8 | -95.5 | -95.5 | -100.5 | 0.0 | N/A | -95.8 | -116.4 | -1.0 | -1.0 | 0.0 | 0.0 |

- 7.X.2	Band n2

With the same link budget method, the performance on band n2 is evaluated and summarized in below Table 7.X.2-1.

Table 7.X.2-1 Estimated SNR on band n2

|  | band | PCC bandwidth | SCC bandwidth | Wgap | PCC SCC Swap | LPF antennuation @Tx leakage | Tx leakage post LPF | PCC @ Rx antenna | SCC @ Rx antenna | noisefloor | NF increase comparing with separate RF chain | coChanNoise SCC | coChanNoise PCC | ADCnoise | SNR PCC | SNR SCC | MSD PCC | MSD SCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS | 2 | 5 | 5 | 50 | 0 | -15.2 | -38.2 | -102.5 | -102.5 | -101.5 | 2.0 | -98.2 | N/A | -106.2 | -3.8 | -7.1 | 2.8 | 6.1 |
| ACS case 1 | 2 | 5 | 5 | 50 | 0 | -15.2 | -38.2 | -88.5 | -88.5 | -101.5 | 2.0 | -98.2 | -87.7 | -106.2 | -1.2 | -1.5 | 0.2 | 0.5 |
| ACS case 2 | 2 | 5 | 5 | 50 | 0 | -15.2 | -38.2 | -61.0 | -61.0 | -101.5 | 12.0 | -98.2 | -60.0 | -97.0 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 2 | 5 | 5 | 50 | 0 | -15.2 | -38.2 | -96.5 | -96.5 | -101.5 | 2.0 | -98.2 | -96.8 | -106.2 | -1.9 | -3.5 | 0.9 | 2.5 |
| IBB case 2 | 2 | 5 | 5 | 50 | 0 | -15.2 | -38.2 | -96.5 | -96.5 | -101.5 | 2.0 | -98.2 | -96.8 | -105.8 | -1.9 | -3.5 | 0.9 | 2.5 |
| REFSENS | 2 | 5 | 5 | 50 | 1 | -36.1 | -59.1 | -102.5 | -102.5 | -101.5 | 0.0 | N/A | N/A | -127.1 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 2 | 5 | 5 | 50 | 1 | -36.1 | -59.1 | -88.5 | -88.5 | -101.5 | 0.0 | N/A | -87.7 | -126.7 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 2 | 2 | 5 | 5 | 50 | 1 | -36.1 | -59.1 | -61.0 | -61.0 | -101.5 | 12.0 | N/A | -60.0 | -97.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 2 | 5 | 5 | 50 | 1 | -36.1 | -59.1 | -96.5 | -96.5 | -101.5 | 0.0 | N/A | -96.8 | -124.7 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 2 | 2 | 5 | 5 | 50 | 1 | -36.1 | -59.1 | -96.5 | -96.5 | -101.5 | 0.0 | N/A | -96.8 | -116.2 | -1.0 | -1.0 | 0.0 | 0.0 |
| REFSENS | 2 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -102.5 | -102.5 | -101.5 | 0.0 | N/A | N/A | -122.7 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 2 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -88.5 | -88.5 | -101.5 | 0.0 | N/A | -87.7 | -122.6 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 2 | 2 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -61.0 | -61.0 | -101.5 | 12.0 | N/A | -60.0 | -97.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 2 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -96.5 | -96.5 | -101.5 | 0.0 | N/A | -96.8 | -121.7 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 2 | 2 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -96.5 | -96.5 | -101.5 | 0.0 | N/A | -96.8 | -115.6 | -1.0 | -1.0 | 0.0 | 0.0 |
| REFSENS | 2 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -102.5 | -102.5 | -101.5 | 0.0 | N/A | N/A | -138.0 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 2 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -88.5 | -88.5 | -101.5 | 0.0 | N/A | -87.7 | -134.6 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 2 | 2 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -61.0 | -61.0 | -101.5 | 12.0 | N/A | -60.0 | -97.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 2 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -96.5 | -96.5 | -101.5 | 0.0 | N/A | -96.8 | -128.1 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 2 | 2 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -96.5 | -96.5 | -101.5 | 0.0 | N/A | -96.8 | -116.5 | -1.0 | -1.0 | 0.0 | 0.0 |

- 7.X.3	Band n7

With the same link budget method, the performance on band n7 is evaluated and summarized in below Table 7.X.3-1.

Table 7.X.3-1 Estimated SNR on band n7

|  | band | PCC bandwidth | SCC bandwidth | Wgap | PCC SCC Swap | LPF antennuation @Tx leakage | Tx leakage post LPF | PCC @ Rx antenna | SCC @ Rx antenna | noisefloor | NF increase comparing with separate RF chain | coChanNoise SCC | coChanNoise PCC | ADCnoise | SNR PCC | SNR SCC | MSD PCC | MSD SCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS | 7 | 10 | 5 | 55 | 0 | -25.1 | -48.1 | -99.3 | -102.5 | -98.3 | 0.0 | N/A | N/A | -116.1 | -1.1 | -1.1 | 0.1 | 0.1 |
| ACS case 1 | 7 | 10 | 5 | 55 | 0 | -25.1 | -48.1 | -85.3 | -88.5 | -98.3 | 0.0 | N/A | -84.5 | -116.1 | -1.0 | -4.1 | 0.0 | 3.1 |
| ACS case 2 | 7 | 10 | 5 | 55 | 0 | -25.1 | -48.1 | -58.0 | -61.2 | -98.3 | 12.0 | N/A | -57.0 | -97.4 | -1.0 | -4.2 | 0.0 | 3.2 |
| IBB case 1 | 7 | 10 | 5 | 55 | 0 | -25.1 | -48.1 | -93.3 | -96.5 | -98.3 | 0.0 | N/A | -93.6 | -115.9 | -1.0 | -3.6 | 0.0 | 2.6 |
| IBB case 2 | 7 | 10 | 5 | 55 | 0 | -25.1 | -48.1 | -93.3 | -96.5 | -98.3 | 0.0 | N/A | -93.6 | -113.3 | -1.0 | -3.6 | 0.0 | 2.6 |
| REFSENS | 7 | 10 | 5 | 55 | 1 | -41.5 | -64.5 | -99.3 | -102.5 | -98.3 | 0.0 | N/A | N/A | -132.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 7 | 10 | 5 | 55 | 1 | -41.5 | -64.5 | -85.3 | -88.5 | -98.3 | 0.0 | N/A | -84.5 | -131.2 | -1.0 | -4.1 | 0.0 | 3.1 |
| ACS case 2 | 7 | 10 | 5 | 55 | 1 | -41.5 | -64.5 | -58.0 | -61.2 | -98.3 | 12.0 | N/A | -57.0 | -97.5 | -1.0 | -4.2 | 0.0 | 3.2 |
| IBB case 1 | 7 | 10 | 5 | 55 | 1 | -41.5 | -64.5 | -93.3 | -96.5 | -98.3 | 0.0 | N/A | -93.6 | -127.1 | -1.0 | -3.6 | 0.0 | 2.6 |
| IBB case 2 | 7 | 10 | 5 | 55 | 1 | -41.5 | -64.5 | -93.3 | -96.5 | -98.3 | 0.0 | N/A | -93.6 | -116.4 | -1.0 | -3.6 | 0.0 | 2.6 |
| REFSENS | 7 | 10 | 5 | 30 | 0 | -43.5 | -66.5 | -99.3 | -102.5 | -98.3 | 0.0 | N/A | N/A | -134.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 7 | 10 | 5 | 30 | 0 | -43.5 | -66.5 | -85.3 | -88.5 | -98.3 | 0.0 | N/A | -84.5 | -132.6 | -1.0 | -4.1 | 0.0 | 3.1 |
| ACS case 2 | 7 | 10 | 5 | 30 | 0 | -43.5 | -66.5 | -58.0 | -61.2 | -98.3 | 12.0 | N/A | -57.0 | -97.5 | -1.0 | -4.2 | 0.0 | 3.2 |
| IBB case 1 | 7 | 10 | 5 | 30 | 0 | -43.5 | -66.5 | -93.3 | -96.5 | -98.3 | 0.0 | N/A | -93.6 | -127.5 | -1.0 | -3.6 | 0.0 | 2.6 |
| IBB case 2 | 7 | 10 | 5 | 30 | 0 | -43.5 | -66.5 | -93.3 | -96.5 | -98.3 | 0.0 | N/A | -93.6 | -116.5 | -1.0 | -3.6 | 0.0 | 2.6 |
| REFSENS | 7 | 10 | 5 | 30 | 1 | -56.4 | -79.4 | -99.3 | -102.5 | -98.3 | 0.0 | N/A | N/A | -147.2 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 7 | 10 | 5 | 30 | 1 | -56.4 | -79.4 | -85.3 | -88.5 | -98.3 | 0.0 | N/A | -84.5 | -136.7 | -1.0 | -4.1 | 0.0 | 3.1 |
| ACS case 2 | 7 | 10 | 5 | 30 | 1 | -56.4 | -79.4 | -58.0 | -61.2 | -98.3 | 12.0 | N/A | -57.0 | -97.5 | -1.0 | -4.2 | 0.0 | 3.2 |
| IBB case 1 | 7 | 10 | 5 | 30 | 1 | -56.4 | -79.4 | -93.3 | -96.5 | -98.3 | 0.0 | N/A | -93.6 | -128.5 | -1.0 | -3.6 | 0.0 | 2.6 |
| IBB case 2 | 7 | 10 | 5 | 30 | 1 | -56.4 | -79.4 | -93.3 | -96.5 | -98.3 | 0.0 | N/A | -93.6 | -116.5 | -1.0 | -3.6 | 0.0 | 2.6 |

- 7.X.4	Band n25

With the same link budget method, the performance on band n25 is evaluated and summarized in below Table 7.X.4-1.

Table 7.X.4-1 Estimated SNR on band n25

|  | band | PCC bandwidth | SCC bandwidth | Wgap | PCC SCC Swap | LPF antennuation @Tx leakage | Tx leakage post LPF | PCC @ Rx antenna | SCC @ Rx antenna | noisefloor | NF increase comparing with separate RF chain | coChanNoise SCC | coChanNoise PCC | ADCnoise | SNR PCC | SNR SCC | MSD PCC | MSD SCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS | 25 | 5 | 5 | 55 | 0 | -11.9 | -34.9 | -101.0 | -101.0 | -100.0 | 5.0 | -96.7 | N/A | -102.9 | -6.7 | -8.7 | 5.7 | 7.7 |
| ACS case 1 | 25 | 5 | 5 | 55 | 0 | -11.9 | -34.9 | -87.0 | -87.0 | -100.0 | 5.0 | -96.7 | -86.2 | -102.9 | -1.4 | -1.8 | 0.4 | 0.8 |
| ACS case 2 | 25 | 5 | 5 | 55 | 0 | -11.9 | -34.9 | -61.0 | -61.0 | -100.0 | 12.0 | -96.7 | -60.0 | -96.4 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 25 | 5 | 5 | 55 | 0 | -11.9 | -34.9 | -95.0 | -95.0 | -100.0 | 5.0 | -96.7 | -95.3 | -102.9 | -3.2 | -4.5 | 2.2 | 3.5 |
| IBB case 2 | 25 | 5 | 5 | 55 | 0 | -11.9 | -34.9 | -95.0 | -95.0 | -100.0 | 5.0 | -96.7 | -95.3 | -102.7 | -3.2 | -4.5 | 2.2 | 3.5 |
| REFSENS | 25 | 5 | 5 | 55 | 1 | -34.2 | -57.2 | -101.0 | -101.0 | -100.0 | 0.0 | N/A | N/A | -125.2 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 25 | 5 | 5 | 55 | 1 | -34.2 | -57.2 | -87.0 | -87.0 | -100.0 | 0.0 | N/A | -86.2 | -124.9 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 2 | 25 | 5 | 5 | 55 | 1 | -34.2 | -57.2 | -61.0 | -61.0 | -100.0 | 12.0 | N/A | -60.0 | -97.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 25 | 5 | 5 | 55 | 1 | -34.2 | -57.2 | -95.0 | -95.0 | -100.0 | 0.0 | N/A | -95.3 | -123.6 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 2 | 25 | 5 | 5 | 55 | 1 | -34.2 | -57.2 | -95.0 | -95.0 | -100.0 | 0.0 | N/A | -95.3 | -116.0 | -1.0 | -1.0 | 0.0 | 0.0 |
| REFSENS | 25 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -101.0 | -101.0 | -100.0 | 0.0 | N/A | N/A | -122.7 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 25 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -87.0 | -87.0 | -100.0 | 0.0 | N/A | -86.2 | -122.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 2 | 25 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -61.0 | -61.0 | -100.0 | 12.0 | N/A | -60.0 | -97.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 25 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -95.0 | -95.0 | -100.0 | 0.0 | N/A | -95.3 | -121.7 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 2 | 25 | 5 | 5 | 30 | 0 | -31.7 | -54.7 | -95.0 | -95.0 | -100.0 | 0.0 | N/A | -95.3 | -115.6 | -1.0 | -1.0 | 0.0 | 0.0 |
| REFSENS | 25 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -101.0 | -101.0 | -100.0 | 0.0 | N/A | N/A | -138.0 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 25 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -87.0 | -87.0 | -100.0 | 0.0 | N/A | -86.2 | -133.8 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 2 | 25 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -61.0 | -61.0 | -100.0 | 12.0 | N/A | -60.0 | -97.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 1 | 25 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -95.0 | -95.0 | -100.0 | 0.0 | N/A | -95.3 | -128.1 | -1.0 | -1.0 | 0.0 | 0.0 |
| IBB case 2 | 25 | 5 | 5 | 30 | 1 | -47.0 | -70.0 | -95.0 | -95.0 | -100.0 | 0.0 | N/A | -95.3 | -116.5 | -1.0 | -1.0 | 0.0 | 0.0 |
| REFSENS | 25 | 40 | 5 | 20 | 0 | -11.9 | -34.9 | -84.0 | -101.0 | -83.0 | 5.0 | -75.4 | N/A | -102.9 | -6.0 | -25.6 | 5.0 | 24.6 |
| ACS case 1 | 25 | 40 | 5 | 20 | 0 | -11.9 | -34.9 | -70.0 | -87.0 | -83.0 | 5.0 | -75.4 | -69.2 | -102.9 | -1.4 | -18.8 | 0.4 | 17.8 |
| ACS case 2 | 25 | 40 | 5 | 20 | 0 | -11.9 | -34.9 | -51.5 | -68.5 | -83.0 | 12.0 | -75.4 | -50.5 | -96.4 | -1.0 | -18.0 | 0.0 | 17.0 |
| IBB case 1 | 25 | 40 | 5 | 20 | 0 | -11.9 | -34.9 | -78.0 | -95.0 | -83.0 | 5.0 | -75.4 | -78.3 | -102.9 | -2.9 | -21.4 | 1.9 | 20.4 |
| IBB case 2 | 25 | 40 | 5 | 20 | 0 | -11.9 | -34.9 | -78.0 | -95.0 | -83.0 | 5.0 | -75.4 | -78.3 | -102.7 | -2.9 | -21.4 | 1.9 | 20.4 |
| REFSENS | 25 | 40 | 5 | 20 | 1 | -22.8 | -45.8 | -84.0 | -101.0 | -83.0 | 0.0 | N/A | N/A | -113.8 | -1.0 | -1.2 | 0.0 | 0.2 |
| ACS case 1 | 25 | 40 | 5 | 20 | 1 | -22.8 | -45.8 | -70.0 | -87.0 | -83.0 | 0.0 | N/A | -69.2 | -113.6 | -1.0 | -17.8 | 0.0 | 16.8 |
| ACS case 2 | 25 | 40 | 5 | 20 | 1 | -22.8 | -45.8 | -51.5 | -68.5 | -83.0 | 12.0 | N/A | -50.5 | -97.4 | -1.0 | -18.0 | 0.0 | 17.0 |
| IBB case 1 | 25 | 40 | 5 | 20 | 1 | -22.8 | -45.8 | -78.0 | -95.0 | -83.0 | 0.0 | N/A | -78.3 | -113.7 | -1.0 | -16.8 | 0.0 | 15.8 |
| IBB case 2 | 25 | 40 | 5 | 20 | 1 | -22.8 | -45.8 | -78.0 | -95.0 | -83.0 | 0.0 | N/A | -78.3 | -111.9 | -1.0 | -16.8 | 0.0 | 15.8 |

- 7.X.5	Band n26

With the same link budget method, the performance on band n26 is evaluated and summarized in below Table 7.X.5-1.

Table 7.X.5-1 Estimated SNR on band n26

|  | band | PCC bandwidth | SCC bandwidth | Wgap | PCC SCC Swap | LPF antennuation @Tx leakage | Tx leakage post LPF | PCC @ Rx antenna | SCC @ Rx antenna | noisefloor | NF increase comparing with separate RF chain | coChanNoise SCC | coChanNoise PCC | ADCnoise | SNR PCC | SNR SCC | MSD PCC | MSD SCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS | 26 | 15 | 10 | 10 | 0 | -14.6 | -37.6 | -97.2 | -99.0 | -96.2 | 2.0 | -72.8 | N/A | -105.6 | -3.3 | -26.2 | 2.3 | 25.2 |
| ACS case 1 | 26 | 15 | 10 | 10 | 0 | -14.6 | -37.6 | -83.2 | -85.0 | -96.2 | 2.0 | -72.8 | -82.4 | -105.6 | -1.1 | -12.7 | 0.1 | 11.7 |
| ACS case 2 | 26 | 15 | 10 | 10 | 0 | -14.6 | -37.6 | -55.0 | -56.8 | -96.2 | 12.0 | -72.8 | -54.0 | -96.9 | -1.0 | -2.9 | 0.0 | 1.9 |
| IBB case 1 | 26 | 15 | 10 | 10 | 0 | -14.6 | -37.6 | -91.2 | -93.0 | -96.2 | 2.0 | -72.8 | -91.5 | -105.6 | -1.7 | -20.3 | 0.7 | 19.3 |
| IBB case 2 | 26 | 15 | 10 | 10 | 0 | -14.6 | -37.6 | -91.2 | -93.0 | -96.2 | 2.0 | -72.8 | -91.5 | -105.3 | -1.7 | -20.3 | 0.7 | 19.3 |
| REFSENS | 26 | 15 | 10 | 10 | 1 | -29.0 | -52.0 | -97.2 | -99.0 | -96.2 | 0.0 | N/A | N/A | -120.0 | -1.0 | -1.0 | 0.0 | 0.0 |
| ACS case 1 | 26 | 15 | 10 | 10 | 1 | -29.0 | -52.0 | -83.2 | -85.0 | -96.2 | 0.0 | N/A | -82.4 | -119.9 | -1.0 | -2.7 | 0.0 | 1.7 |
| ACS case 2 | 26 | 15 | 10 | 10 | 1 | -29.0 | -52.0 | -55.0 | -56.8 | -96.2 | 12.0 | N/A | -54.0 | -97.5 | -1.0 | -2.8 | 0.0 | 1.8 |
| IBB case 1 | 26 | 15 | 10 | 10 | 1 | -29.0 | -52.0 | -91.2 | -93.0 | -96.2 | 0.0 | N/A | -91.5 | -119.4 | -1.0 | -2.4 | 0.0 | 1.4 |
| IBB case 2 | 26 | 15 | 10 | 10 | 1 | -29.0 | -52.0 | -91.2 | -93.0 | -96.2 | 0.0 | N/A | -91.5 | -114.9 | -1.0 | -2.4 | 0.0 | 1.4 |

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024

- R4-2505109, “WF on NR FR1 DL fragmented carriers study”, Moderator (Mediatek Inc.), Apple, Qualcomm, RAN4#114bis Meeting, April 2025