3GPP TSG-RAN WG4 Meeting #115				R4-2507478

Malta, MT, 19th – 23rd May, 2025

Agenda item:	8

Source: 	Huawei, HiSilicon

Title: 	View on Rel-19 RAN4 UE feature list

Document for:	Discussion

- Introduction

This contribution provides our view of the RAN4 UE feature list for Rel-19 NR.

- NR_ENDC_RF_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | MPR reduction for single carrier-fixed | Indicates whether UE supports MPR reduction for single UL carrier with symmetric and asymmetric 1/2*UE CBW extension. |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | Yes (only FR1) | N/A | This capability is applicable for single UL CC case | Optional |
|  | 46-2 | MPR reduction for single carrier-flexible | Indicates whether UE supports MPR reduction for single UL carrier with symmetric and asymmetric 1/X*UE CBW extension, where 1/X is a set of ratios smaller than 1/2 [{1/4, …}]. | 46-1 | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | Yes (only FR1) | N/A | This capability is applicable for single UL CC case | Optional |
|  | 46-3 | MPR enhancement for activated carrier | Indicates whether UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA. |  | Yes | N/A | The FR1 UE shall support MPR requirement based on UL CA configuration even for single activated CC. | Per BC | No | Yes (only FR1) | N/A |  | Optional |
|  | 46-4 | FR2 MPR-Improvement Downlink Independent | Indicates whether UE supports reduced MPR by removing dependence on DL CA configuration. |  | Yes | N/A | The FR2 UE only supports MPR requirement with dependence of DL CA configuration. | Per BC | Yes | Yes (only FR2) | N/A |  | Optional |
|  | 46-5 | FR2 MPR Improvement Activation Dependent | Indicates whether UE supports MPR based on activation status of its configured CCs |  | Yes | N/A | The FR2 UE shall support MPR requirement based on UL CA configuration. | Per BC | Yes | Yes (only FR2) | N/A |  | Optional |
|  | 46-6 | Support maximum 6 MIMO layers for DL reception | Indicates whether the UE supports maximum 6 spatial multiplexing layers for DL reception for FWA. |  | Yes | N/A | UE does not support maximum 6 MIMO layers for DL reception | Per FSPC | N/A | Yes (only FR1) | N/A |  | Optional |
|  | 46-7 | SRS antenna switching for 6Rx | Indicates supporting of t3r6 SRS antenna switching. |  | Yes | N/A | UE does not support t3r6 SRS antenna switching | Per BC | N/A | Yes (only FR1) | N/A | This capability cannot be indicated simultaneously with Tx diversity | Optional |
|  | 46-8 | [SRS IL imbalance compensation] | Indicates whether UE performs SRS IL self-compensation | 2-55 or 23-8-3 | Yes | N/A |  | Per BC | N/A | Yes (only FR1) | N/A |  | Optional |

- NonCol_intraB_ENDC_NR_CA_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NonCol_intraB_ENDC_NR_CA_Ph2 | 47-1 | Support of intra-band non-collocated NR CA operation with 4 layers/CC | Indicates the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 (type-4) [2]. And the UE also should indicate intraBandNR-CA-non-collocated-r18 | 33-1 | Yes | N/A | Intra-band non-collocated NR CA operation with 4 layers/CC is not supported. | Per BC | N/A | FR1 only | N/A | Supported for band n77/n78 only | Optional with capability signaling |

- Void

- Void

- NR_ATG_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_ATG_enh | 50-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_RRM_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_RRM_Ph5 | 51-1 | FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor | Support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor for: - SSB based Intra-frequency measurement without MG - SSB based Intra-frequency measurement with MG - Handover - RRC Re-establishment RRC Connection Release with Redirection | n/a | Y | n/a | UE does not meet the requirements of FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Per Band | n/a | The feature only applies to FR2-1 | n/a | Candidate value of N in L3 measurement delay requirements: {2,4,6}. The feature applies to the scenario when UE is configured with only one FR2-1 Cell (i.e. FR2-1 PCell without CA/DC) and is configured with only one FR2-1 carrier for L3 SSB measurement. | Optional with capability signalling |
|  | 51-2 | FR2-1 inter-frequency L3 measurement delay reduction by optimizing Rx beam sweeping factor | Support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor for: - SSB based Inter-frequency measurement without MG - SSB based Inter-frequency measurement with MG | 51-1 | Y | n/a | UE does not meet the requirements of FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Per Band | n/a | The feature only applies to FR2-1 | n/a | Candidate value of N in L3 measurement delay requirements: {2,4,6}. The feature applies to the scenario when UE is configured with only one FR2-1 Cell (i.e. FR2-1 PCell without CA/DC) and is configured with only one FR2-1 carrier for L3 SSB measurement. | Optional with capability signalling |

- NR_demod_Ph5-Perf

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_demod_Ph5-Perf | 52-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_MIMO_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. NR_MIMO_Ph5 | 53-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- Netw_Energy_NR_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. Netw_Energy_NR_enh | 54-1 | Lower bound on OD-SSB periodicity for measurement in fast measurement window | A lower bound on OD-SSB periodicity of [10] ms for measurement in deactivated SCell fast measurement window | At least one among 61-1/ 61-2/61-2a/ 61-3/ 61-4/61-4a | Yes | n/a | There is no lower bound on OD-SSB periodicity for for measurement in deactivated SCell fast measurement window | Per BC | n/a | n/a | Yes | If UE does not indicate this capability, there is no lower bound on OD-SSB periodicity for for measurement in deactivated SCell fast measurement window. | Optional with capability signalling |
|  | 54-2 | Additional processing timing for OD-SSB activation | Additional OD-SSB processing time for OD-SSB reception. | At least one among 61-3/ 61-4/61-4a | Yes | n/a | UE is not able to receive OD-SSB before the processing time | Per UE | n/a | n/a | Yes | Candidate value :{[2 or 3]ms, 5ms}. The additional processing time does not apply when OD-SSB activation comes together with SCell activation. | Optional with capability signalling |

- NR_LPWUS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55.NR_LPWUS | 55-1 | LP-WUR types | Support of different LP-WUR UE type, aiming at different power consumption designs: a) Type 1: LP-WUR targets ultra-low power consumption, assuming a combined noise figure (NF) and implementation margin (IM) of 18dB. b) Type 2: LP-WUR targets moderate power consumption, assuming a combined noise figure (NF) and implementation margin (IM) of 13.5dB. | N/A | Yes | N/A |  | Per UE | No | Yes (FR1 only) | N/A |  | Optional |

- NR_Mob_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_Mob_Ph4 | 56-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_XR_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57.NR_XR_Ph3 | 57-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_NTN_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58.NR_NTN_Ph3 | 58-1 |  |  |  |  |  |  |  |  |  |  |  |  |