3GPP TSG-RAN WG4 Meeting#115                                R4-2507505

Malta , MT, 19th – 23th May, 2025

Agenda item:	7.26.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussions on General aspect for A-IoT

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

- Topic 2-1: System parameters Issue 2-1-1: Operating band Agreement: - NR band n8 can be used as an example band, and other FDD bands below 1GHz are not precluded. Issue 2-1-2: R2D transmission bandwidth definition Agreement: - Reuse the following definition of R2D transmission bandwidth in TR38.769 - Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D. - RAN4 will re-visit the two agreed bullets above, if RAN1 has the different conclusion. Issue 2-1-3: R2D channel bandwidth definition Agreement: - Define R2D channel bandwidth as: - Channel bandwidth, BCH,R2D from a reader perspective: The frequency resources used for transmitting R2D, and potential guard band. Issue 2-1-4: Values of R2D bandwidth Agreement: - Define R2D transmission bandwidth configuration: - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS - FFS on other RB numbers - Further discuss the R2D channel bandwidth considering additional guard band. - If RAN1 has the agreement different than above, RAN4 will revisit the agreements Issue 2-1-5: R2D channel raster and channel spacing Agreement: - Do not define channel spacing for R2D transmission - Define channel raster for R2D transmission. - Details need further discussion. Issue 2-1-6: D2R bandwidth Agreement: - D2R channel bandwidth for A-IoT device - Define D2R channel bandwidth for A-IoT device considering the following aspects: - [1SB or] 2SB - D2R maximum data rate (depending on RAN1 conclusion) - SFO - FDMA, i.e., considering the different device 1 has different frequency shift - Reference Filter performance - Other aspects are not precluded Issue 2-1-7: D2R channel raster and channel spacing Agreement: Do not define channel raster and channel spacing for D2R

Discussion

- 2.1 General aspects for A-IoT BS in D1T1

Issue 2-1-4: Values of R2D bandwidth

- For R2D transmission, from our initial understanding, the minimum channel bandwidth is supposed to be defined based on the potential minimum PRB configuration for R2D and additional guard band on each side (e.g. 1/18 of R2D transmission bandwidth which is the same as LTE spectrum utilization) to define the ACLR and OBUE requirement.

- Proposal 1: propose to define the minimum carrier bandwidth for R2D transmission as following with 10kHz, 20kHz, 30kHz, 40kHz GB on each side of R2D transmission reserved for 1/2/3/4 PRB transmission.

Figure 1. the proposed carrier bandwidth for R2D transmission

Issue 2-1-5: R2D channel raster and channel spacing

- Regarding the channel raster for A-IoT service, if the standalone spectrum is allocated for A-IoT R2D transmission, then channel raster for R2D transmission should be at least 100KHz. If the in-band spectrum is allocated for A-IoT R2D transmission, then channel raster could be defined as following:

- N*100kHz channel raster+delta_offset

- where offset could be -47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5KHz

Figure 2. the illustration of OOK signal generation for D2R signal [freq error of D2R transmission should be considered within GB]

- Proposal 2: for R2D transmission in Standalone operation mode, to define the channel raster as 100KHz; for R2D transmission in in-band operation mode, to define the channel raster as 100KHz*N+delta_offset where delta_offset could be as [-47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5]kHz.

Issue 2-1-6: D2R bandwidth

- According to the latest RAN1 agreement, the ideal D2R bandwidth without considering SFO,FDMA etc is summarized as following. However if RAN4 multiple D2R bandwidth as in the following table, then it will be quite difficult to implement these different combinations, from our understanding, it might be good to define some set of carrier bandwidth from A-IoT BS perspective. D2R bandwidth and freq shift to CW is considered as scheduling bandwidth and freq position within the carrier bandwidth.

Figure 3 Relationship with D2R transmission bandwidth, D2R chip duration, SFS factor R

- Table 1. potential bandwidth for D2R reflection

|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Freq. shift to CW =1/2Tchip | 3.75 kHz | 7.5 kHz | 15 kHz | 30 kHz | 45 kHz | 60 kHz | 90 kHz | 120 kHz | 180 kHz | 240 kHz | 360 kHz | 480 kHz | 720 kHz | 960 kHz |
| Btx,D2R =4/Tb | Tb (μs) | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
| 15 kHz | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  |  |
| 30 kHz | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
| 60 kHz | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
| 120 kHz | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
| 180 kHz | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
| 240 kHz | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
| 360 kHz | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
| 480 kHz | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
| 720 kHz | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
| 960 kHz | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 1440 kHz | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
| 1920 kHz | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 2880 kHz | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
| 3840 kHz | 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |

NOTE: CBW=(Freq. shift to CW*2+DSB/2)*(1+2*SFO)*(1+2*Guard_ band)

Where

- Freq. shift to CW is equal to 1/2Tchip ;

- DSB is equal to 4/Tb

- SFO could be assumed as 10%

- Guard band for filtering could be FFS

- Proposal 3: propose RAN4 to define the limited carrier bandwidth from BS receiver perspective to simplify the BS implementation and treat the D2R bandwidth and freq shift to CW as scheduling bandwidth and freq position within the carrier bandwidth.

Conclusions

In this contribution, we shared some initial views on the RF requirement for A-IoT BS in D1T1 and proposals are made as following:

- Proposal 1: propose to define the minimum carrier bandwidth for R2D transmission as following with 10kHz, 20kHz, 30kHz, 40kHz GB on each side of R2D transmission reserved for 1/2/3/4 PRB transmission.

- Proposal 2: for R2D transmission in Standalone operation mode, to define the channel raster as 100KHz; for R2D transmission in in-band operation mode, to define the channel raster as 100KHz*N+delta_offset where delta_offset could be as [-47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5]kHz.

- Proposal 3: propose RAN4 to define limited carrier bandwidth from BS receiver perspective to simplify the BS implementation and treat the D2R bandwidth and freq shift to CW as scheduling bandwidth and freq position within the carrier bandwidth.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282, RF requirements for Ambient IoT BS, Huawei,

- [4] R4-2417088, WF on RF requirement impact of A-IoT, Huawei, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Huawei, approved.

- R4-2410597,WF on impacts of A-IoT on RF requirements, Approved.

- RP-243326, New WID on Rel-19 Ambient IoT, Approved.

- TR 38.769,Study on solutions for Ambient IoT (Internet of Things) in NR

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.

- R4-2505230, WF on A-IoT device requirements, CMCC, approved.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.