3GPP TSG-RAN WG4 Meeting #115 	R4-2507740

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.19.3

Source: 	Ericsson

Title:	On RRM requirements BDS B2b Signal in A-GNSS

Document for:	Approval

# Introduction

In [1], a WI was agreed on introducing in Rel-19 3GPP specifications the support for BDS (BeiDou Navigation Satellite System) B2b Signal in A-GNSS, applicable for both NR and LTE. The WI has the core and performance part objectives in [1].

Core part objectives:

Specify A-BDS enhancements in Rel-19, including:

- Supporting new signal of BDS B2b in A-GNSS for both E-UTRA and NR [RAN2];

- Specify corresponding assistant information in dedicated signalling [RAN2].

Performance part objectives:

Specify UE performance requirements for support of BDS B2b in A-GNSS for both E-UTRA and NR using existing A-GNSS minimum performance requirements framework [RAN4].

The expected output on the RAN4-related performance part is as below [1]:

| TS 38.171 | Add corresponding references of B2b and define corresponding requirements. | RAN#109 |
| --- | --- | --- |
| TS 36.171 | Add corresponding references of B2b and define corresponding requirements. | RAN#109 |

The RAN4 work is expected to be finalized, according to the time plan, in RAN4#116 (Aug. 2025) and reported to the RAN Plenary at RAN#109 (Sep. 2025).

In this contribution, we discuss the potential updates to TS 38.171 and TS 36.171, according to the WI objective [1].

# Discussion

The justification for the 3GPP work from [1] is as follows:

The Interface Control Document (ICD) of the BeiDou Navigation Satellite System (BDS) Signal In Space (SIS) for Open Service Signal B2b was published by the China Satellite Navigation Office in July, 2020 (http://www.beidou.gov.cn/xt/gfxz/202008/P020230516558683155109.pdf). There are also requirements to introduce it in A-GNSS in 3GPP to increase the positioning accuracy, provide timing for UEs, etc. The main work is similar to what we have done for B1C, B2a, B3I signals. Both E-UTRA and NR system need to support providing the assistant information of B2b signal.

The endorsed RAN2 CR to TS 37.355 (LPP) from RAN2#129 is in [2].

The corresponding necessary changes in TS 38.171 include at least:

- Section 4.8, Table 4.8-1 (Relative signal power levels for each signal type for each GNSS) needs to be updated with the corresponding power level for B2b.

- In Annex C, Table C.2 (Parameter values): a row for B2b needs to be added.

- In Annex C, Table C.3 (Ratio between carrier frequency and chip rate): a row for B2b needs to be added.

- In Annex E.1, Table E.1 (Assistance data to be provided to the UE): B2b needs to be included in Note 2.

- In Annex E.2, Table E.4: the GNSS-IonosphericModel IE needs to be updated to account for B2b in Note 2.

- In Annex E.2, Table E.7: the GNSS clock and orbit model need to be added for B2b.

- In Annex E.2, Table E.10: the GNSS Almanac choice needs to be added for B2b.

- In Annex E.2, Table E.12: the GNSS UTC Model choice needs to be added for B2b.

For LTE, TS 36.171 needs to be updated similar to the above for TS 38.171.

# Summary

- Proposal 1: At least the following changes are necessary for introducing the support for BDS B2b for NR in TS 38.171:

  - Section 4.8, Table 4.8-1 (Relative signal power levels for each signal type for each GNSS) needs to be updated with the corresponding power level for B2b.

  - In Annex C, Table C.2 (Parameter values): a row for B2b needs to be added.

  - In Annex C, Table C.3 (Ratio between carrier frequency and chip rate): a row for B2b needs to be added.

  - In Annex E.1, Table E.1 (Assistance data to be provided to the UE): B2b needs to be included in Note 2.

  - In Annex E.2, Table E.4: the GNSS-IonosphericModel IE needs to be updated to account for B2b in Note 2.

  - In Annex E.2, Table E.7: the GNSS clock and orbit model need to be added for B2b.

  - In Annex E.2, Table E.10: the GNSS Almanac choice needs to be added for B2b.

  - In Annex E.2, Table E.12: the GNSS UTC Model choice needs to be added for B2b.

- Proposal 2: For LTE, TS 36.171 needs to be updated to align with the updates in TS 38.171.

# References

[1] RP-250767, Revised WID: Introduction of BDS (BeiDou Navigation Satellite System) B2b Signal in A-GNSS for LTE and NR, CAICT, March 2025.

[2] R2-2501435, Introduction of B2b signal in BDS system in A-GNSS, CATT, CAICT, Ericsson, Huawei, HiSilicon, Feb. 2025.