3GPP TSG-RAN WG4 Meeting #115	R4-2507752

# St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:		6.13.1

Source: 		Qualcomm Inc.

Title:	 	A-MPR for n252

Document for:		Approval

## 1	Introduction

UE RF requirements for n252 (UL 2000-2020, DL 2180-2200) has been discussed for a few meetings, and decision on UE-to-UE coexistence limits was reached in previous meeting. Therefore, in this contribution we present our view on required A-MPR. Another band-specific important aspect without conclusion is OOB blocking requirements and whether co-banding is considered similar to n256 requirements.

## 2	Discussion

In RAN4#114 UE co-existence requirements were agreed as follows.

The need for A-MPR was studied using the following assumptions:

- PA calibrated to 30 dB ACLR with 100 RB DFT-s-OFDM QPSK waveform with 22 dBm at antenna connector

- -28 dBc IQ-image and DC-leakage

- -60 dBc CIM3

The proposed A-MPR is summarized in Tables 1 to 4.

Table 1: A-MPR regions for -40 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | < 2007.5 | ≥ 0 | ≥ 2.7 | A1 |
| 10MHz | < 2010 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 3.6 | A1 |
|  |  | > 7.74 | ≤ 0.72 | A1 |
|  | 2010 ≤ Fc ≤ 2015 | ≥ 0 | ≥ 4.5 | A1 |
| 15MHz | < 2012.5 | ≤ 3.06 | ≥ 0 | A3 |
|  |  | > 3.06 | 4.5 ≤ LCRB*12*SCS ≤ 9 | A1 |
|  |  | > 3.06 | > 9 | A2 |
|  |  | > 10.98 | ≤ 0.72 | A1 |
|  | 2012.5 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 6.48 | A1 |
|  |  | > 12.6 | ≤ 0.72 | A1 |
| 20 MHz | 2010 | ≤ 4.68 | ≥ 0 | A3 |
|  |  | > 4.68 | 4.5 ≤ LCRB*12*SCS ≤ 9 | A1 |
|  |  | > 4.68 | > 9 | A2 |
|  |  | > 12.6 | ≤ 0.72 | A1 |

Table 2: A-MPR for -40 dBm/MHz

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 6 | ≤ 7 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 6.5 | ≤ 7.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 7.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 7 | ≤ 8 |
| CP-OFDM QPSK | ≤ 6 | ≤ 8.5 | ≤ 9 |
| CP-OFDM 16 QAM | ≤ 6 | ≤ 8.5 | ≤ 9 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 8.5 | ≤ 9 |

Table 3: A-MPR regions for -30 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | - | - | - | - |
| 10MHz | < 2010 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | 2.7 < LCRB*12*SCS < 4.5 | A2 |
|  |  | ≤ 1.44 | ≥ 4.5 | A1 |
|  |  | ≥ 1.44 | ≥ 5.4 | A2 |
| 15MHz | < 2012.5 | ≤ 3.06 | ≤ 2.7 | A1 |
|  |  | ≤ 3.06 | 2.7 < LCRB*12*SCS < 4.5 | A2 |
|  |  | ≤ 3.06 | ≥ 4.5 | A1 |
|  |  | ≥ 3.06 | ≥ 5.4 | A2 |
|  | 2012.5 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | 2.7 < LCRB*12*SCS < 5.4 | A2 |
|  |  | ≤ 1.44 | ≥ 5.4 | A1 |
|  |  | ≥ 1.44 | ≥ 7.2 | A2 |
| 20 MHz | 2010 | ≤ 4.68 | ≤ 2.7 | A1 |
|  |  | ≤ 4.68 | 2.7 ≤ LCRB*12*SCS ≤ 5.4 | A2 |
|  |  | ≤ 4.68 | > 5.4 | A1 |
|  |  | > 4.68 | ≥ 7.2 | A2 |

Table 4: A-MPR for -30 dBm/MHz

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 2.5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 4 |
| CP-OFDM QPSK | ≤ 6 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 6 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 4.5 |

In RAN4#114bis a WF [1] was agreed which captured a compromise over different companies results for A-MPR. Especially for -40 dBm/MHz limit A2 and A3 A-MPRs are in some cases even 4 dB larger than our results indicate

A-MPR for -40 dBm/MHz from [1] compared to our results in red

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ [8] ≤ 6 | ≤ [11] ≤ 7 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ [8] ≤ 6.5 | ≤ [11] ≤ 7.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ [8] ≤ 6.5 | ≤ [11] ≤ 7.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ [8] ≤ 7 | ≤ [11] ≤ 8 |
| CP-OFDM QPSK | ≤ 7 | ≤ [10] ≤ 8.5 | ≤ [12] ≤ 9 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ [10] ≤ 8.5 | ≤ [12] ≤ 9 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ [10] ≤ 8.5 | ≤ [12] ≤ 9 |

Proposal 1: Strive to further reduce the A-MPR for A2 and A3 regions with -40 dBm/MHz limit, as results indicate up to 4 dB better performance. Consider also any new results provided in this meeting by other companies

## 3	Conclusion

In this contribution UE RF requirements for n252 were discussed. Following observations and proposals were made:

Proposal 1: Strive to further reduce the A-MPR for A2 and A3 regions with -40 dBm/MHz limit, as results indicate up to 4 dB better performance. Consider also any new results provided in this meeting by other companies

A-MPR for -40 dBm/MHz from [1] compared to our results in red

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ [8] ≤ 6 | ≤ [11] ≤ 7 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ [8] ≤ 6.5 | ≤ [11] ≤ 7.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ [8] ≤ 6.5 | ≤ [11] ≤ 7.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ [8] ≤ 7 | ≤ [11] ≤ 8 |
| CP-OFDM QPSK | ≤ 7 | ≤ [10] ≤ 8.5 | ≤ [12] ≤ 9 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ [10] ≤ 8.5 | ≤ [12] ≤ 9 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ [10] ≤ 8.5 | ≤ [12] ≤ 9 |

## 4	References

- R4-2505235, WF on NR and IoT NTN bands, Moderator (Apple)