3GPP TSG-RAN WG4 Meeting #115	R4-2507880

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.8.5

Source:	Qualcomm Incorporated

Title: 		RRM requirements for 7MHz Channel BW

Document for:	Discussion

# Introduction

For release 19 work item on NR FR1 7 MHz Channel Bandwidth, one of the objectives for the performance part is to specify necessary RRM performance requirements, if needed.

During Rel-18 RAN4 specified the core and performance requirements to support UE operation with 5 MHz channel BW. These included new RLM OOS and IS evaluation delay and BFD evaluation delay requirements. For 7MHz BW, we can potentially use the same requirements as there is no new physical layer signal design considered by RAN1 for 7MHz BW, e.g., no new puncturing mechanisms to CORESET#0, etc. So, we don’t need new core and performance requirements for a UE to support 7MHz BW.

Observation 1: RAN1 hasn’t introduced any new physical layer signal designs specifically for 7MHz BW.

Observation 2: In Rel-18, RAN4 defined RLM and BFD requirements for 5 MHz channel bandwidth.

Proposal 1: No need to define new/additional RRM core and performance requirements to support 7 MHz BW.

Proposal 2: RAN4 to reuse RLM and BFD requirements defined for 5 MHz channel BW for 7 MHz BW as well.

However, in TS 38.133, we use the terminology “… for a UE operating on a cell with less than 5 MHz BW …” to refer to 3 MHz or 5 MHz channel BW requirements. So, we still need a CR to enable these requirements for 7MHz as well.

Proposal 3: Text changes (e.g., to update existing text “… for a UE operating on a cell with less than 5 MHz BW …”) are needed in TS 38.133 to enable RLM and BFD requirements with 5MHz BW for 7MHz BW as well.

# Conclusion

Observation 1: RAN1 hasn’t introduced any new physical layer signal designs specifically for 7MHz BW

Observation 2: In Rel-18, RAN4 defined RLM and BFD requirements for 5 MHz channel bandwidth.

Proposal 1: No need to define new/additional RRM core and performance requirements to support 7 MHz BW.

Proposal 2: RAN4 to reuse RLM and BFD requirements defined for 5 MHz channel BW for 7 MHz BW as well.

Proposal 3: Text changes (e.g., to update existing text “… for a UE operating on a cell with less than 5 MHz BW …”) are needed in TS 38.133 to enable RLM and BFD requirements with 5MHz BW for 7MHz BW as well.

# References

[1] 	RP-250373, “Revised WID on NR FR1 7 MHz Channel Bandwidth”, T-Mobile USA, China Telecom, RAN#107